3GPP TSG-RAN WG2 Meeting #123	R2-2308005

Toulouse, France, August 21-25, 2023

Agenda item:		7.4.4

Source:		Lenovo

Title:			Discussion on CHO with candidate SCG

Document for:		Discussion and Decision

- Introduction

We have the following agreement so far.

- RAN2#119 meeting: - CHO configuration referring to or including CPC/CPA configuration (intended to be applicable together) can be supported. - FFS: When triggering CHO, UE perform CPC/CPA configuration to start CPC/CPA evaluation, FFS if CHO evaluation and CPC/CPA evaluation is concurrent or sequential. - RAN2#120 meeting: - Execution order: the UE doesn’t execute CPC/CPA unless CHO condition is fulfilled (regardless parallel or sequential evaluation) - RAN2#121 meeting: - RAN2 agrees to support the simultaneous evaluation of CHO and CPC in Rel-18. - The UE should not need to unpack any of the nested conditionalconfiguration containers in order to measure, acc to agreement above - RAN2#121bis meeting: - For the CHO+CPC case: - When both CHO and CPC conditions are met, both CHO and CPC cell change is executed. - Baseline: The UE waits until both CHO and CPC conditions are met (always). (furthermore, it is assumed that if needed the network can provide a complementary CHO-only configuration, to avoid failures in deployments where failure would otherwise be likely to happen). - Alternative: FFS if When CHO condition is met, but CPC condition is not met, CHO execution is triggered (and somehow source SCG can be released). IF allowed in the new configuration the UE may continue evaluation of CPC/CPA conditions. - RAN2#122 meeting: - P3: The CHO execution conditions (for candidate PCells) and CPA/CPC execution conditions (for candidate PSCells) are provided based on the source MeasConfig. - P4: For CHO execution conditions, the source MN determines the execution conditions on candidate PCells, based on the source MCG MeasConfig. - P5: For CPA/CPC execution conditions, the candidate MN determines the parameters of the execution conditions for candidate PSCells (e.g. event A4 threshold). - P6: The candidate MN informs the source MN about the prepared candidate PSCells and parameters of the associated execution conditions (e.g. event A4 threshold). According to the received information from the candidate MN, the source MN generates the corresponding execution conditions based on the source MCG MeasConfig to the UE. - FFS how, if to support event A3/A5. - P8: For CHO with candidate SCGs for CPA/CPC, the RRCReconfigurtaion message in one CHO container includes one MCG configuration and one SCG configuration (i.e. similar to Rel-17 CHO with SCG configuration). - P9: The execution conditions associated with one CHO container includes both CHO execution condition(s) and CPA/CPC execution condition(s), i.e. triggering conditions on both candidate PCell and candidate PSCell. - P10: If there are multiple candidate PSCells associated with one candidate PCell, the NW can provide multiple CHO configurations for the same candidate PCell, i.e. each one contains one MCG configuration (for the same candidate PCell) and one SCG configuration (for different candidate PSCell). - P12: When the CPA/CPC execution condition is met but no CHO execution condition is met, the UE continues to evaluate both CHO and CPA/CPC execution conditions. - For CHO+CPC we only consider execution when BOTH conditions are met. - (When the CHO execution condition is met but no CPC execution condition is met, if there is an available CHO-only or Rel-17 CHO with SCG configuration for which the CHO condition is met, the UE performs the CHO-only or Rel-17 CHO with SCG execution, and THUS the network can handle such situation by providing proper configurations).

This contribution aims to discuss the potential impacts for the case that CHO includes candidate SCGs for CPC/CPA.

- Discussion

2.1 existence between CHO-only and CHO with candidate SCG

It was agreed that both CHO and CPC should be executed only when both CHO and CPC conditions are met. Furthermore, a complementary CHO-only configuration could be provided to UE to avoid potential failure. For example, the condition for candidate SCG is not suitable, which results in that CHO condition is met but the condition of candidate SCG is not met. Based on this intention, the execution condition for CHO-only configuration should be same as the condition for candidate PCell from CHO with candidate SCG.

Observation 1: The execution conditions for complementary CHO-only configuration is same as the condition for candidate PCell from CHO with candidate SCG since the candidate PCell is same.

In the case that CHO-only is configured, the execution condition for candidate PCell from CHO-only and CHO with candidate SCG will be met at the same time, especially same condition is configured for both CHO-only and CHO with candidate SCG. In this case, if both CHO-only and CHO with candidate SCG can be performed based on the perspective condition, it is straight for UE to perform CHO with candidate SCG in priority. If the execution condition of candidate SCG is not met, RAN2 needs to discuss if CHO-only can be executed immediately. It is possible that the condition for candidate SCG will be met late after the conditions for candidate PCell from both CHO-only and CHO with candidate SCG are met. Alternatively, one time offset can be configured to CHO-only. Namely, if the execution condition for candidate PCell from CHO-only and CHO with candidate SCG will be met at the same time but the condition of candidate SCG is not met yet, UE needs to wait for a period. If the condition of candidate SCG is still not met, UE is allowed to perform CHO-only.

Observation 2: The execution conditions for candidate PCell from CHO-only and CHO with candidate SCG may be fulfilled at the same time.

Proposal 1: In the case that both CHO-only and CHO with candidate SCG are configured to UE, if the execution condition for candidate PCell from CHO-only and candidate PCell and PSCell from CHO with candidate SCG are met, UE performs CHO with candidate SCG in priority.

Proposal 2: RAN2 to discuss how to handle the case that the execution conditions for candidate PCell from CHO-only and candidate PCell from CHO with candidate SCG are met at the same time but the condition PSCell is not met yet.

2.2 CHO Recovery

In legacy, if UE selects a CHO candidate cell during re-establishment procedure, UE will perform CHO instead of re-establishment procedure. In legacy CHO recovery, CHO can be performed if CHO candidate cell meets the S-criteria. if CHO with candidate SCG can be used for CHO recovery, the first question is that whether only candidate PCell configuration is applied or not. Another way is that the whole configuration for CHO with candidate SCG is applied. Regarding the latter, whether the condition for candidate SCG should be considered or not when UE performs cell selection during re-establishment procedure.

Proposal 3: CHO with candidate SCG configuration can be used for CHO recovery purpose.

Proposal 4: Assuming CHO with candidate SCG can be used for CHO recovery, RAN2 to discuss whether the condition for candidate SCG should be evaluated or not when UE performs cell selection during re-establishment procedure.

Proposal 5: RAN2 to discuss whether candidate SCG configuration can be applied as well besides candidate PCell configuration when UE performs CHO recovery via a CHO with candidate SCG.

- Conclusion

In this contribution, the conclusion and proposal are made based on the discussion:

Observation 1: The execution conditions for complementary CHO-only configuration is same as the condition for candidate PCell from CHO with candidate SCG since the candidate PCell is same.

Observation 2: The execution conditions for candidate PCell from CHO-only and CHO with candidate SCG may be fulfilled at the same time.

Proposal 1: In the case that both CHO-only and CHO with candidate SCG are configured to UE, if the execution condition for candidate PCell from CHO-only and candidate PCell and PSCell from CHO with candidate SCG are met, UE performs CHO with candidate SCG in priority.

Proposal 2: RAN2 to discuss how to handle the case that the execution conditions for candidate PCell from CHO-only and candidate PCell from CHO with candidate SCG are met at the same time but the condition PSCell is not met yet.

Proposal 3: CHO with candidate SCG configuration can be used for CHO recovery purpose.

Proposal 4: RAN2 to discuss whether the condition for candidate SCG should be considered or not when UE performs cell selection during re-establishment procedure.

Proposal 5: RAN2 to discuss whether candidate SCG configuration can be applied as well besides candidate PCell configuration when UE performs CHO recovery via a CHO with candidate SCG.

References

Chairman notes for RAN2#122