3GPP TSG-RAN WG2 Meeting #123	R2-2308009

Toulouse, France, August 21-25, 2023

Agenda item:		7.6.4

Source:		Lenovo

Title:	Some remaining issues for discontinuous coverage

Document for:		Discussion

- Introduction

The previous RAN2 meetings made the following agreements regarding discontinuous coverage for IoT NTN:

RAN2#121 agreements - RAN2 can continue to check whether dedicated RRC signalling can be used for providing satellite information corresponding to discontinuous coverage. - RAN2 will support enhancements in paging and eDRX, in alignment with the work in SA2 and CT1. FFS on the details - RAN2 may consider enhancements for connected UE upon detecting discontinuous coverage (e.g., suspend RLM, RLF detection, and RRC re-establishment process) - Companies supporting the store and forward approach can bring a proposal to the plenary for TEI18 or for updating the WID. RAN2#121bis agreements - RAN2 will not introduce any enhancement to allow a UE in RRC Connected to stay in RRC_CONNECTED during/after a coverage gap (e.g. suspend RLM/RLF, activation time in RRC Reconfiguration, CHO enhancement) - RAN2 to introduce enhancement to RRC Release using one of the following options (FFS which one): - Explicit RRC Release using a new RRC Release cause - UE Autonomous release (e.g. timer based or upon detection of coverage gap)

Considering that some aspects including paging enhancements, RRC connection release enhancements and UE behaviour when in discontinuous coverage	have been discussed in email discussion [Post122][113][IoT NTN Enh] Discontinuous coverage, in this contribution we further discuss some remaining issues beyond the scope of the email discussion.

- Discussion

AS operations to be suspended in RRC_CONNECTED

In Rel-17 to avoid unnecessary measurements and to reduce UE power consumption in IDLE due to coverage interruption in discontinuous coverage, the UE is not required to perform AS functions when out of coverage. However, unnecessary procedures including measurements may also occur before due to coverage interruption.

The criteria of triggering neighbour cell measurement in RRC_CONNECTED for NB-IoT is a combination of serving cell RSRP and its variance. That is, in CONNECTED mode if the measured serving cell RSRP is lower than a threshold and its variance compared to a reference RSRP is higher than a threshold, the UE performs neighbour cell measurements. This solution is designed for continuous coverage deployment, wherein the RSRP and its variation can well reflect whether an NB-IoT UE is possibly approaching RLF (i.e., due to leaving the serving cell), and the scenarios of discontinuous coverage have not been considered.

The discontinuous coverage may happen in space and time domain due to sparse constellation of satellites and satellite movement. It may lead to additional and unnecessary power consumption which is at least essential to NB-IoT devices. Unlike the temporary coverage interruptions in TN, in NTN the coverage interruptions are expected to last for a longer time duration and are predictable with the help of satellite assistance information. In such scenario there could be no neighbour cell available to be measured, but unnecessary neighbour cell measurement could be still triggered by NB-IoT UE.

Observation 1: CONNECTED neighbour cell measurement for NB-IoT UE will be unnecessarily triggered due to coverage discontinuity.

Therefore, CONNECTED neighbour cell measurement triggering for NB-IoT shall be suspended or disabled according to UE estimated coverage interruption in discontinuous coverage.

Proposal 1: CONNECTED neighbour cell measurement triggering for NB-IoT can be suspended when no neighbour cell coverage is expected in discontinuous coverage.

Similarly, for eMTC, unnecessary measurement reporting for CONNECTED mobility could be still triggered when there is no available neighbour cell.

Observation 2: Measurement reporting for eMTC UE will be unnecessarily triggered due to coverage discontinuity.

And:

Proposal 2: Measurement reporting for eMTC can be suspended when no neighbour cell coverage is expected in discontinuous coverage.

In discontinuous coverage scenario, the suspending of CONNECTED neighbour cell measurement triggering or measurement reporting could be based on the duration of coverage discontinuity as well. Two possible options can be considered: 1) UE determines when to disable the measurement triggering by its prediction, or 2) network determines when to disable the measurement triggering by its prediction or UE’s report.

Proposal 3: UE can report its estimation on coverage interruption e.g., to assist network in suspending AS operations in CONNECTED.

- Conclusion

In this contribution we analyze some remaining issues for discontinuous coverage. It is observed that:

Observation 1: CONNECTED neighbour cell measurement for NB-IoT UE will be unnecessarily triggered due to coverage discontinuity.

Observation 2: Measurement reporting for eMTC UE will be unnecessarily triggered due to coverage discontinuity.

The following proposals are given:

Proposal 1: CONNECTED neighbour cell measurement triggering for NB-IoT can be suspended when no neighbour cell coverage is expected in discontinuous coverage.

Proposal 2: Measurement reporting for eMTC can be suspended when no neighbour cell coverage is expected in discontinuous coverage.

Proposal 3: UE can report its estimation on coverage interruption e.g., to assist network in suspending AS operations in CONNECTED.