3GPP TSG-RAN WG2 Meeting #123	R2-2308017

Toulouse, FR, 21st – 25th August 2023

Agenda item:	 7.13.5

Source:		Lenovo

Title:		Discussion on MRO for NR-U

Document for:		Discussion and Decision

1	Introduction

In RAN2#119bis-e meeting [1], agreements for MRO for NR-U were achieved:

Agreements:

1	The UE will log information of multiple RA procedures related to consistent LBT failures. FFS details.

Agreements:

1	Introduce a new raPurpose in the RA-Report to indicate that the RA was initiated following a “consistent LBT failures” in the SpCell.

2	RAN2 agree to log kind of “the number of LBT failures” in the RA report.

LBT failure is the failure to access the channel before transmission.

The definition of “the number of LBT failures” should be clarified.

FFS how to log the number of LBT failures in the RA report.

=>	FFS: how to fulfil RAN3 request in logging RSSI.

=>	RAN2 consult RAN3 to whether it is possible to know the lbt-FailureRecoveryConfig used for execution of the RA procedure and evaluate the cost for the solution without UE reporting.

In RAN2#121 meeting [2], further agreements were achieved:

Agreements:

1: 	Log the last successful RA procedure related information in the RA report. Only some information to be logged for multiple successive RA procedures failed due to LBT issue. FFS what information.

In RAN2#122 meeting [3], further agreements were achieved:

Agreements:

1	Only the preamble transmission attempts for which LBT was successful are represented in the “per RA attempt info list” for a given beam.

2	On how to represent the preamble transmission attempts blocked by LBT,

Introduce a field (or reusing the existing field) that counts the number of preamble transmissions blocked by LBT per RA procedure, and a flag indicating transmission failures experienced right before beam switching. Details can FFS.

3	For the RA-Report, the enhancements on the handling of the “per RA attempt info list” (i.e. as per Proposal 1) apply only to the last RA procedure in the last BWP prior to the random access success.

4	For the other BWPs in which the UE experienced the consistent LBT failure, the UE logs in the RA-InformationCommon:

a.	The locationAndBandwidth information of the BWP

b.	The subcarrierSpacing information of the BWP

c.	The absoluteFrequencyPointA information of the BWP ( How to log once for all the BWPs of the cell is FFS)

5	As baseline, RAN2 assumes the following:

a.	Enhancements discussed for the RA-InformationCommon for the RA-Report are applicable also to the RLF-Report

b.	The detailed “per RA attempt info” are only reported in the RLF-Report for the last RA procedure before RLF/HOF, FFS whereas limited information are reported for the other BWPs in which consistent LBT failure is detected

c.	The above bullets may be revisited case by case depending on future agreements.

6	The UE logs RA-InformationCommon including LBT info in the RLF-Report, in case of HOF and when the RLF cause is randomAccessProblem or beamFailureRecoveryFailure (as in legacy).

7	The UE logs the available RSSI measurement in the RLF-Report. FFS in which case.

8	The UE should log the following RSSI values in the RLF-Report:

a.	For RLF, the latest measured RSSI of the NR-U channel of the last serving cell if measRSSI-ReportConfig is configured for the corresponding frequency.

b.	FFS: For HOF, the latest measured RSSI of the NR-U channel of the source cell, and the latest measured RSSI of the NR-U channel of the target cell, if measRSSI-ReportConfig is configured for the corresponding frequency.

In this paper, we would further discuss the details of MRO for NR-U.

2	Discussion

  - 2.1 Enhancements for RLF report

Considering mobility in NR-U, one possible case is that after the UE receives the HO command , the UE performs uplink LBT but the shared spectrum of the target PCell is occupied, so consistent uplink LBT failure may happen before or during RACH procedure with the target PCell, in such case, HO failure may happen when T304 expires. To enable network understand that HO failure is caused by LBT failure, RAN3#117-e meeting agreed to enhance RLF report i.e. adding an indication concerning HOF due to consistent LBT failure in the RLF report.

On the other hand, since RSSI is used to indicate the channle characteristics, in order to enable the network know channel status and optimize unlicensed radio resources, RAN3#117-e meeting also agreed to include the measured RSSI in the RLF report.

An LS from RAN3 is sent to RAN2 for consideration [4]: RAN3 discussed the benefits of enhancing the content of RLF report and RA report with new UE measurements to account for the impact of NR-U in MRO.

The following has been agreed:

- RLF Report needs to be enhanced by adding the latest measured RSSI, and an indication that handover failure occurred due to consistent LBT failures

- RA Report needs to be enhanced at least by adding an indication of consistent LBT failures per RA procedure (i.e., indicate when UE performs RA procedure due to consistent LBT failures).

RAN3 kindly asks RAN2 to enable the following:

  - addition in RLF report of the latest measured RSSI and an indication that handover failure occurred due to consistent LBT failures

  - addition in RA report of at least indications of consistent LBT failures per RA procedure.

From RAN2 point of view, an explicit indication concerning handover failure due to consistent LBT failure in the RLF report is beneficial for network MRO, for example, a new cause concerning that failure happens due to consistent LBT failure, or one-bit flag indicating failure due to consistent LBT failure, can be included in the RLF report.

Proposal 1: Include an explicit indication concerning handover failure due to consistent LBT failure in the RLF report.

For RSSI measurement in the RLF-Report, RAN2#122 meeting agreed that “for RLF, the latest measured RSSI of the NR-U channel of the last serving cell if measRSSI-ReportConfig is configured for the corresponding frequency”, but it is FFS that for HOF whether to report the latest measured RSSI of the NR-U channel of the source cell and the latest measured RSSI of the NR-U channel of the target cell if measRSSI-ReportConfig is configured for the corresponding frequency.

We think it is fine to follow the principle for legacy measResultLastServCell included in the RLF report, i.e. in case of HOF, the RSSI measurement results of the source PCell can be included in the RLF-Report if the measRSSI-ReportConfig is configured for that frequency.

Proposal 2: In case of HOF, include the RSSI measurement results of the source PCell in the RLF-Report if the measRSSI-ReportConfig is configured for that frequency.

RAN2#121 meeting agreed that “Log the last successful RA procedure related information in the RA report. Only some information to be logged for multiple successive RA procedures failed due to LBT issue”. The agreements can also be applied to RLF report. Furthermore, RAN2#122 meeting agreed to report number of preamble transmissions blocked by LBT per RA procedure.

Currently the UE switches BWP when the amount of LBT failures reaches lbt-FailureInstanceMaxCount, but the IE lbt-FailureInstanceMaxCount can’t be used to indicate the number of LBT failures experienced in one BWP because the LBT_COUNTER used in one BWP would be reset during RA procedure when the timer i.e. lbt-FailureDetectionTimer expires. Therefore, for other BWPs (i.e. except the last BWP) in which consistent LBT failure happens in the last RA procedure, besides basic BWP information which is used to identify the BWP (e.g. locationAndBandwidth, subcarrierSpacing, and absoluteFrequencyPointA), the number of LBT failures per BWP can be included in the RLF report, which may be bigger than the value indicated by the IE lbt-FailureInstanceMaxCount.

Proposal 3: The number of LBT failures per other BWPs (i.e. except the last BWP) in which consistent LBT failure happens of the last RA procedure can be included in the RLF report.

On the other hand, after the UE receives the HO command, it is possible that LBT with target PCell is successful before or during RACH procedure towards target PCell, but handover fails when T304 expires. Time information during handover procedure, e.g. time duration for UL LBT before per RACH attempt for the last BWP of the last RA procedure and the time elapsed since the last HO execution until successful LBT, is useful to decide how RLF report is used for MRO analysis, for example, if too long time which is close to timer period of T304 is spent for UL LBT, it may mean that the failure is mainly caused by channel occupancy rather than coverage issue even though LBT during handover procedure is successful, network may not perform a coverage optimization after receiving the failure information. Therefore, it is beneficial to include time information during handover procedure in the RLF report, e.g. time duration for UL LBT before per RACH attempt and the time elapsed since the last HO execution until successful LBT.

Proposal 4: Time information during handover procedure, e.g. time duration for UL LBT before per RACH attempt and the time elapsed since the last HO execution until successful LBT, can be included in the RLF report.

  - 2.2 Enhancements for RA report

Similar as enhancements for RLF report, to enable network better know whether RACH configuration is configured properly, it is beneficial to include number of LBT failures (i.e.  the number of LBT failures per other BWPs (i.e. except the last BWP) in which consistent LBT failure happens of the last RA procedure), and time information for UL LBT before per RACH attempt in the RA report.

Proposal 5: The number of LBT failures per other BWPs (i.e. except the last BWP) in which consistent LBT failure happens of the last RA procedure can be included in the RA report.

Proposal 6: Time duration for UL LBT before per RACH attempt can be included in the RA report.

  - 2.3 Enhancements for SHR

Considering SHR in NR-U system, there is a case that consistent LBT failures for at least one UL BWP are detected in the MAC layer of the source cell and/or target cell before successful handover, it shows that the unlicensed spectrum for the source cell and/or target cell are not available enough, even though final handover is successful, RLF/HOF may happen if consistent LBT failures occur in all the configured UL BWPs in the source cell and/or target cell. Therefore, consistent LBT failures in at least one UL BWP on the source cell and/or target cell can be considered as a triggering condition for generating a SHR.

Proposal 7: Consistent LBT failures in at least one UL BWP on the source cell and/or target cell can be considered as a triggering condition for generating a SHR in NR-U.

When at least one trigger condition is fulfilled, e.g. legacy SHR trigger condition or NR-U specific trigger condition e.g. consistent LBT failures occurring in at least one UL BWP on the source cell and/or target cell is satisfied, the UE would set the contents in the SHR, e.g. the flag for this triggering condition, information of the UL BWP(s) where consistent LBT failure occurs e.g. the identifier of the UL BWP, thus the network may modify LBT configurations correspondingly when it receives the successful HO related information.

Proposal 8: The identifier of the UL BWP where consistent LBT failure occurs can be included in the SHR.

Similar as enhancements for RLF report, to enable network better know whether RACH configuration towards the target cell is configured properly, it is beneficial to include number of UL LBT failures in source cell and/or target cell, and time information during handover procedure, e.g. time duration for UL LBT before per RACH attempt and the time elapsed since the last HO execution until successful LBT in the SHR.

Proposal 9: Number of UL LBT failures, and time information during handover procedure e.g. time duration for UL LBT before per RACH attempt and the time elapsed since the last HO execution until successful LBT can be included in the SHR.

3	Conclusion

In this contribution, MRO for NR-U is discussed. We have the following proposals:

Proposal 1: Include an explicit indication concerning handover failure due to consistent LBT failure in the RLF report.

Proposal 2: In case of HOF, include the RSSI measurement results of the source PCell in the RLF-Report if the measRSSI-ReportConfig is configured for that frequency.

Proposal 3: The number of LBT failures per other BWPs (i.e. except the last BWP) in which consistent LBT failure happens of the last RA procedure can be included in the RLF report.

Proposal 4: Time information during handover procedure, e.g. time duration for UL LBT before per RACH attempt and the time elapsed since the last HO execution until successful LBT, can be included in the RLF report.

Proposal 5: The number of LBT failures per other BWPs (i.e. except the last BWP) in which consistent LBT failure happens of the last RA procedure can be included in the RA report.

Proposal 6: Time duration for UL LBT before per RACH attempt can be included in the RA report.

Proposal 7: Consistent LBT failures in at least one UL BWP on the source cell and/or target cell can be considered as a triggering condition for generating a SHR in NR-U.

Proposal 8: The identifier of the UL BWP where consistent LBT failure occurs can be included in the SHR.

Proposal 9: Number of UL LBT failures, and time information during handover procedure e.g. time duration for UL LBT before per RACH attempt and the time elapsed since the last HO execution until successful LBT can be included in the SHR.

Reference

[1] Chairman notes for RAN2#119bis-e meeting

[2] Chairman notes for RAN2#121 meeting

[3] Chairman notes for RAN2#122 meeting

[4] R3-225241, LS on NR-U support for MRO, Ericsson