3GPP TSG-RAN WG2 Meeting #123	R2-2308019

Toulouse, FR, 21st – 25th August 2023

Agenda item:	 7.13.8

Source:		Lenovo

Title:		MRO for fast MCG link recovery

Document for:		Discussion and Decision

1	Introduction

In RAN2#120 meeting [1], agreements for MRO for fast MCG recovery were achieved:

Agreements

1	For fast MCG recovery MRO, prioritize NR-DC scenario. if time allows, study whether the same solution can be extended for others DC scenarios.

2	Consider at least below scenarios for fast MCG recovery MRO:

a.	T316 expiry

b.	SCG failure/deactivation during fast MCG recovery (i.e., running of T316). The “upon fast MCG recovery case” is FFS.

3	RLF report is enhanced to support fast MCG recovery MRO.

4	Fast MCG recovery failure cause shall be included for fast MCG recovery optimization. FFS details

In RAN2#122 meeting [2], further agreements were acheved:

Agreements:

1	RAN2 confirms scenario of near failure fast MCG recovery.

2	RAN2 confirms scenario f1, i.e., SCG fails or is deactivated before the UE sends the MCGFailureInformation. FFS RAN2 impact.

In this paper, we would further discuss the details of MRO for fast MCG link recovery.

2	Discussion

In R16, fast MCG link recovery procedure was introduced to inform the network about an MCG failure that the UE has experienced i.e. MCG radio link failure, in order to continue the RRC connection without RRC re-establishment. If T316 is configured, upon RLF happening in MCG, the UE transmits the MCGFailureInformation message to MN via SN while SCG is not suspended. The MN may decide to reconfigure or change or release MCG based on the received MCG failure information. The MN may provide the corresponding response message (e.g. RRC Reconfiguration message with reconfigurationwithSync for MCG, MobilityFromNRCommand, or RRC Release message) to the UE via SN. However, if SCG fails or is deactivated upon RLF happening in MCG, the UE performes RRC re-establishment.

Until now, RAN3 agreed belwo cases for MRO consideration:

-	Case 1: SCG fails when the UE is undergoing fast MCG recovery (i.e. SCG failure happens while T316 is running);

-	Case 2: the signalling delay is longer than the time the UE waits for the response (T316 expired);

-	Case 3: SCG fails or is deactivated before the UE sends the MCGFailureInformation.

RAN2#120 meeting agreed to consider at least below scenarios for fast MCG recovery MRO:

a.	T316 expiry (which is above Case 2 agreed in RAN3);

b.	SCG failure/deactivation during fast MCG recovery (i.e., running of T316) (which is above Case 1 agreed in RAN3).

Furthermore, RAN2#122 meeting agreed to consider the scenario that SCG fails or is deactivated before the UE sends the MCGFailureInformation, which is aligned with RAN3’s agreements (which is above Case 3 agreed in RAN3)

RAN2#120 meeting agreed to enahnce RLF report to support MRO for fast MCG recovery, and fast MCG recovery failure cause shall be included for fast MCG recovery optimization but the details are FFS.

For case b and case c, both MCG failure and SCG failure happen, obviously, both MCG failure related information and SCG failure related information should be included in the RLF report.

Proposal 1: The UE reports both MCG failure related information and SCG failure related information in the RLF report, for the case that SCG failure/deactivation during fast MCG recovery (i.e., running of T316), or, SCG fails or is deactivated before the UE sends the MCGFailureInformation.

Further, the fast MCG recovery failure cause of case a is T316 expiry, the failure cause of case b is SCG failure or SCG deactivation. It is beneficial for the UE to report the failure cause of fast MCG recovery failure in the RLF-Report to enable network to distinguish the reason of fast MCG link recovery failure for MRO. Furthermore, if the failure cause is SCG failure, the UE can also report the SCG failure type e.g. t310-Expiry, randomAccessProblem, rlc-MaxNumRetx and etc. Also, we can find that RAN3#119 meeting [3] agreed that “ It is beneficial for the UE to report at least PSCell where SCG failure happened, the cause of the fast MCG recovery failure (at least T316 expiry, SCG failure, SCG was deactivated or other cases that SCG is not available), and also if the problem is SCG failure, the SCG failure type (at least t310-Expiry, randomAccessProblem, rlc-MaxNumRetx)”, RAN2 can confirm these parameters agreed by RAN3.

Based on the reported fast MCG recovery failure information in the RLF report, network can modify corresponding parameters for MRO purpose. For example, the RLF-Report including fast MCG recovery failure related information (i.e. both MCG failure related information and SCG failure related information) may be transferred to MN and/or SN by the receiving node, then MN and/or SN can underatand why fast MCG link recovery fails e.g. based on the failure cause or failure type, and perform configuration update to aviod such a failure, for example, if failure is due to the timer T316 expires, MN may enlarge the duration of timer T316.

Proposal 2: The UE reports fast MCG recovery failure related information (i.e. both MCG failure related information and SCG failure related information) in the RLF report:

- cause of the fast MCG recovery failure, e.g. T316 expiry, SCG failure or SCG deactivation;

- failure type of the SCG failure, e.g. t310-Expiry, randomAccessProblem, or rlc-MaxNumRetx, when the cause of the fast MCG recovery failure is SCG failure.

3	Conclusion

In this contribution, MRO for fast MCG link recovery is further discussed. We have the following proposals:

Proposal 1: The UE reports both MCG failure related information and SCG failure related information in the RLF report, for the case that SCG failure/deactivation during fast MCG recovery (i.e., running of T316), or, SCG fails or is deactivated before the UE sends the MCGFailureInformation.

Proposal 2: The UE reports fast MCG recovery failure related information (i.e. both MCG failure related information and SCG failure related information) in the RLF report:

- cause of the fast MCG recovery failure, e.g. T316 expiry, SCG failure or SCG deactivation;

- failure type of the SCG failure, e.g. t310-Expiry, randomAccessProblem, or rlc-MaxNumRetx, when the cause of the fast MCG recovery failure is SCG failure.

Reference

[1] Chairman notes for RAN2#120 meeting

[2] Chairman notes for RAN2#122 meeting

[3] Chairman notes for RAN3#119 meeting