3GPP TSG-RAN WG2 #123	R2-2308021

Toulouse, France, 21st – 25th August 2023

Agenda item:	   7.16.2.2

Source:		Lenovo

Title:		Qualitative analysis on data collection requirements

Document for:		Discussion and Decision

# Introduction

It has been discussed in the previous RAN1 and RAN2 meetings that the data collection requirements could differ for different LCM purposes. Even though the exact quantitative requirements depend on RAN1 conclusion, RAN2 could still do some qualitative analysis to better understand the implication to different data collection methods.

RAN2 has agreed on the following understanding in the previous meeting, and this paper further analyses the related aspects.

RAN2#122 Agreements: RAN 2 assumes that for the data collection in some scenarios (e.g., internal data up to implementation or the existing data are enough), possibly no RAN2 specification effort is needed in some scenarios, e.g. (not exhaustive): - - For model inference of UE-sided model, input data for model inference is available inside the UE. - - For UE-side (real time) monitoring of UE-sided model, performance metrics are available inside the UE. UE can independently monitor a model's performance without any data input from NW. P4a: For the latency requirement of data collection, RAN2 assumes: - - for all types of offline model training (i.e., UE- /NW-/ two-sided model training), there is no latency requirement for data collection - - for model inference, when required data comes from other entities, there is a latency requirement for data collection - - for model monitoring, when required monitoring data (e.g., performance metric) comes from the other entities, there is a latency requirement for data collection. P6a: RAN2 assumes that the analysis/selection of the data collection frameworks should focus on the RRC_CONNECTED state (for both data generation and reporting). Analysis and potential enhancement on the non-connected state can be revisited when needed.

# Discussion

Issues related to data collection have been discussed in the email discussion [Post120][054][AIML18] Data Collection [1]. It seems common understanding that the requirements of data collection depend on the purpose of data collection. The four categories of data collection requirements seem agreeable to companies are:

a) the content of the data,

b) the data size,

c) latency and periodicity,

d) signalling and configuration aspects.

In our understanding, a) the content of data will eventually depend on RAN1. From RAN2 perspective, RAN2 can start with some high level assumption before RAN1 conclusion:

- For AI based CSI feedback optimization, in our understanding the LCM procedure would mostly rely on the L1 measurement or CSI measurement to be specific.

- For AI based beam management, similarly, the LCM procedure would rely on the L1 CSI measurement according to RAN1 discussion so far.

- For AI based positioning,

  - for AI model training, it would rely on the mapping of ground truth location and related radio measurement (including L1 and L3 measurements)

  - for AI model inference and other LCM steps, it may rely on the L1 and L3 measurements

For the content of data to be collected during LCM, RAN2 can start with the following assumption before RAN1 conclusion:

  - AI based CSI: L1 CSI measurement

  - AI based BM: L1 CSI measurement

  - AI based positioning: location information, L1 measurement, L3 measurement

P6a: RAN2 assumes that the analysis/selection of the data collection frameworks should focus on the RRC_CONNECTED state (for both data generation and reporting). Analysis and potential enhancement on the non-connected state can be revisited when needed.

Even though RAN2 assumes the data collection framework shall focus on RRC_CONNECTED state, we find it’s a bit different in case of AI based positioning since positioning in RRC Inactive state is supported in Rel17 and positioning in RRC Idle state is currently under Rel18 discussion. For AI based CSI/BM instead, the data collection in RRC Inactive/Idle states does not make much sense.

Data collection via LPP for positioning in RRC Inactive state is supported in Rel17.

Data collection for AI based CSI and BM is only performed in RRC connected state.

RAN2 considers the data collection for AI based positioning in RRC Inactive/Idle State.

With respect to b) data size, the exact data size depends on the actual AI algorithm/implementation and RAN1 also has a say on this.  Qualitatively, in our understanding the data size needed for AI training (especially offline training) is usually larger than the data size needed for AI inference and monitoring.  In addition, the AI monitoring usually rely on data collection in a time period to justify the performance of the AI model, thus overall data size needed for AI monitoring could be relatively larger than the data size needed for AI inference. Furthermore, depending on the purpose of AI monitoring the required monitoring period and overall data size would be different. For example, AI monitoring for slow reaction such as model update would usually require longer monitoring period and larger overall data size than AI monitoring for fast reaction such as model switching/fallback etc.

| Purpose of data collection | Dataset size requirement |
| --- | --- |
| Offline training | Large |
| Monitoring (slow reaction, e.g., for model update) | Medium large |
| Monitoring (fast reaction, e.g., for model switching/fallback) | Medium small |
| Inference | Small |

RAN2 considers the data size requirements for  AI operations as following: AI model offline training > Monitoring (slow reaction) > Monitoring (fast reaction) > Inference

In another perspective, in order to better support data collection for AI model training (especially offline training), RAN2 is suggested to study approaches that can transmit measurements related to AI model training in one package, instead of per measurement report manner as in legacy.

For AI training data collection, RAN2 studies approaches to support sending training data set (e.g., collection of measurements) in one package from UE to NW.

In terms of c) latency and periodicity, qualitatively speaking, AI inference usually happens most frequently compared to other AI operation. Because AI monitoring is performed based on the AI model performance in a period of time and AI training (especially offline training) is performed based on a series of AI monitoring (potentially from multiple UEs). Thus, similarly, we can qualitatively categorize the latency and periodicity requirements as in the table below.

| Purpose of data collection | Latency requirement |
| --- | --- |
| Offline training | Relaxed |
| Monitoring (slow reaction, e.g., for model update) | Low |
| Monitoring (fast reaction, e.g., for model switching/fallback) | Near-real-time |
| Inference | Time-critical |

RAN2 considers the latency/periodicity requirements for AI operations as following: AI model offline training > Monitoring (slow reaction) > Monitoring (fast reaction) > Inference.

Based on the above analysis, we propose to update the table endorsed from in R2-2302286 [2] by adding two columns as following, namely Applicable Use Cases and Applicable LCM steps. The applicable use cases and LCM steps for UAI is FFS because it really depends on what assistance information UE will report to the NW. Besides, it is unclear to us the usefulness of early measurement reporting.

RAN2 adopts the following modification of the endorsed table to capture the applicable use cases and LCM steps for each data collection method.

|  | RRC state to generate data | Contents to be collected | Irrelevant columns omitted | Model Offline Training Applicable Use Cases | Model Inference Applicable Use Cases | Model Monitoring Applicable Use Cases |
| --- | --- | --- | --- | --- | --- | --- |
| Logged MDT | RRC_IDLE/RRRC_INACTIVE | L3 cell/beam measurements, location info, sensor info, timing info | … | Positioning | NA | NA |
| Immediate MDT | RRC_CONNECTED | L3 cell/beam measurements, location info, sensor info | … | Positioning | NA | NA |
| L3 measurements | RRC_CONNECTED | L3 cell/beam measurements | … | Positioning | Positioning | Positioning |
| L1 measurement (CSI reporting) | RRC_CONNECTED | L1 CSI measurement | … | CSI, BM, Positioning | CSI, BM, Positioning | CSI, BM, Positioning |
| UAI | RRC_CONNECTED | Assistance information to show UE preference | … | FFS | FFS | FFS |
| Early measurements | RRC_IDLE/RRC_INACTIVE | L3 cell/beam measurements | … | FFS | FFS | FFS |
| LPP | RRC_CONNECTED/ RRC_INACTIVE | Location info, Sensor info, Positioning measurement | … | Positioning | Positioning | Positioning |

# 3	Conclusion

Based on the discussion above, we observe:

Observation 1	Data collection via LPP for positioning in RRC Inactive state is supported in Rel17.

Based on the discussion above, we propose:

Proposal 1	For the content of data to be collected during LCM, RAN2 can start with the following assumption before RAN1 conclusion:

a.	AI based CSI: L1 CSI measurement

b.	AI based BM: L1 CSI measurement

c.	AI based positioning: location information, L1 measurement, L3 measurement

Proposal 2	Data collection for AI based CSI and BM is only performed in RRC connected state.

Proposal 3	RAN2 considers the data collection for AI based positioning in RRC Inactive/Idle State.

Proposal 4	RAN2 considers the data size requirements for  AI operations as following: AI model offline training > Monitoring (slow reaction) > Monitoring (fast reaction) > Inference

Proposal 5	For AI training data collection, RAN2 studies approaches to support sending training data set (e.g., collection of measurements) in one package from UE to NW.

Proposal 6	RAN2 considers the latency/periodicity requirements for AI operations as following: AI model offline training > Monitoring (slow reaction) > Monitoring (fast reaction) > Inference.

Proposal 7	RAN2 adopts the following modification of the endorsed table to capture the applicable use cases and LCM steps for each data collection method.

|  | RRC state to generate data | Contents to be collected | Irrelevant columns omitted | Model Offline Training Applicable Use Cases | Model Inference Applicable Use Cases | Model Monitoring/Selection/etc. Applicable Use Cases |
| --- | --- | --- | --- | --- | --- | --- |
| Logged MDT | RRC_IDLE/RRRC_INACTIVE | L3 cell/beam measurements, location info, sensor info, timing info | … | Positioning | NA | NA |
| Immediate MDT | RRC_CONNECTED | L3 cell/beam measurements, location info, sensor info | … | Positioning | NA | NA |
| L3 measurements reporting | RRC_CONNECTED | L3 cell/beam measurements | … | Positioning | Positioning | Positioning |
| L1 measurement reporting (CSI reporting) | RRC_CONNECTED | L1 CSI measurement | … | CSI, BM, Positioning | CSI, BM, Positioning | CSI, BM, Positioning |
| UAI | RRC_CONNECTED | Assistance information to show UE preference | … | FFS | FFS | FFS |
| Early measurements reporting | RRC_IDLE/RRC_INACTIVE | L3 cell/beam measurements | … | FFS | FFS | FFS |
| LPP | RRC_CONNECTED/ RRC_INACTIVE | Location info, Sensor info, Positioning measurement | … | Positioning | Positioning | Positioning |

# 4	Reference

[1] R2-2301440	Outcome of [Post120][054][AIML18] Data Collection (Ericsson / vivo)

[2] R2-2302286 	Summary of [AT121][025]: Progress table of analyzing data collection framework (Apple)