3GPP TSG-RAN WG2 Meeting #123	R2-2308025

Toulouse, France, August 21-25, 2023

Agenda item:		7.5.4.1

Source:		Lenovo

Title:		Discussion on BSR enhancements for XR

Document for:		Discussion and Decision

- Introduction

- RAN#98-e has approved the XR WID in RP-223502 including BSR and delay report enhancement for XR capacity:

- Buffer Status Report (BSR) enhancements including at least new Buffer Status Table(s) (RAN2); - Delay reporting of buffered data in uplink (RAN2);

In this contribution we further discuss UE feedback enhancements e.g., BSR format and trigger.

- Discussion

In RAN2#121bis-e and RAN2#122 meeting following agreements for BSR enhancements have been reached:

- Support of new BSR table(s) is based on NW configuration and UE capability. FFS whether the UE capability can apply to non-XR UEs. - As a working assumption, at most one BS index or BS value is reported by an LCG. This assumption can be revisited if new BSR table design cannot achieve a target level of quantization error. FFS what this target level should be. - Design/configuration for new BSR table(s) should include support for narrower ranges (i.e. finer granularity) than the legacy. Details can be discussed after an agreement on how UE obtains new BSR table(s) (e.g. pre-definition vs RRC configuration) is made. - At least linear distribution is used for generating code points in new BSR table(s). FFS whether exponential distribution can be considered too. FFS if piecewise linear distribution is supported. - New BSR table(s) can be used by any UEs that support such a capability. However, design of the new BSR table(s) should be based on XR-specific use cases and requirements. - Network can configure which BSR table(s) an LCG is eligible to use. UE determines which BSR table (i.e. legacy or something else) the LCG should use. FFS details of this determination (e.g. based on buffer size) and how network knows which BSR table each LCG uses. - As working assumption (depending on how we create the new BSR table(s) and the MAC CE format), If more than one new BSR table are introduced, all of them have the same size BS field. FFS on the exact size. - Deprioritize Option 2c (static + dynamic BSR tables) and Option 2d (reference table + scaling factor). - Have more discussions on Option 2a (static BSR tables) vs Option 2b (RRC configured BSR tables). In next meeting, companies should explain how BSR table(s) are created and how many tables would be needed, and how the MAC CE structure will look like. Should also explain what is the expected quantization error. - UE calculates the remaining time based on the PDCP discard timer value. FFS if UE reports one or multiple values. FFS how this is modelled in PDCP specification. FFS which UEs support this. - When/if UE reports remaining time, the reference time for the remaining time is determined from the point of the first transmission of the information. FFS if intra-UE prioritization can impact this. - Support one static BSR table with 8 bits BS field for Rel-18 XR (for all cases). - We do not support additional piecewise linear BSR table in Rel-18. Can consider piecewise linearity when discussing how the BSR table values are defined.

BSR triggering and BSR MAC CE format enhancements

As mentioned above, IP packets belonging to the same PDU set are correlated to each other, and the end-user performance may depend on if all these IP packets belonging to a single PDU set are successfully delivered for some codecs. Providing the amount of data of the PDU set is beneficial for gNB to schedule the suitable UL grant size to schedule them as a whole. However, the legacy BSR procedure does not well support the PDU set level buffer size reporting.

- Current BSR format indicates all the available data volume with LCHs in an LCG. If there’s more than one PDU set available for transmission for an LCH, NW cannot identify the buffer size for each PDU set in the BSR and also cannot know the amount of data corresponding to the remaining delay budget information. RAN2 can discuss how to enhance the PDU set buffer size report format. For example, if an LCG only has one PDU set, the BSR includes the PDU set size for the PDU set available for transmission in Figure 1. In Figure 2, multiple PDU set buffer size can be included for one LCG. LCG index indicates buffer size of LCG is for which LCG, and PDU set bit indicates how many and which PDU set has buffer status.

Figure 1. An example of one PDU set in one LCG

Figure 2. An example of multiple PDU sets in one LCG

The enhanced BSR MAC CE format is only suitable for certain LCHs which are e.g. LCH carrying XR traffic. Therefore, NW should configure per LCH or LCG whether to use the enhanced BSR MAC CE format for buffer status and delay information reporting or whether the legacy BSR MAC CE should be used. By proper NW configuration it could be ensured that LCHs configured with the enhanced BSR MAC CE formats are grouped in the same LCG. Similarly, NW should configure whether UE /MAC should report delay information for a LCG or LCH. The delay information may be only required for certain LCH(s)/LCG(s) which carry specific XR-traffic. And in addition to a field of delay information corresponding to an LCG is needed in the BSR, another field corresponding to the LCG is needed to indicate the delay information of the LCG is present or not.

In case of padding enhanced BSR is triggered, if the padding bits is not sufficient to accommodate a BSR including delay information and buffer size information of one LCG or all LCGs, whether or how to prioritize which information for which LCG is needed to be discussed

There are 4 types of BSR trigger events currently specified:

1) UL data with an LCH in an LCG with higher priority becomes available or UL data with an LCH in an LCG becomes available again after no ULs data in an LCG is available;

2) number of padding bits is larger enough to include an BSR;

3) retxBSR-Timer expires and data is available in an LCG ;

4) periodicBSR-Timer expires.

The first 3 trigger conditions are not designed for periodic data. Due to the UL arrival jitter caused by variable coding delay depending on the variable packet size, it is difficult to set a suitable value for the periodicBSR-Timer to adapt to the periodical arrival considering also the jitter. When SDUs of a PDU set belonging to a LCH arrive and there are still PDUs of a previous PDU set pending for transmission in the buffer, no legacy BSR for the newly arrived PDU set can be triggered. Therefore, the BSR trigger for the PDU set may be delayed.

Proposal 1: RAN2 to discuss Long/short (truncated) BSR format enhancements to report multiple PDU set buffer sizes in one BSR MAC CE.

Proposal 2: RAN2 to discuss the Long/short truncated BSR format enhancements on how and whether to report the delay information and buffer size in one BSR MAC CE.

Proposal 3: NW configures UE to use a new enhanced BSR MAC CE format for specific logical channels/LCG, e.g. LCH carrying XR-traffic.

Proposal 4: NW configures whether UE /MAC should report delay information for a LCG or LCH

Proposal 5: UE will be configured, e.g. by RRC, with an enhanced BSR reporting procedure which is optimized for XR traffic in addition to the legacy BSR reporting procedure.

BSR report based on New Buffer Size Table

In the RAN2#122 meeting, RAN2 has agreed to introduce one new static BSR table with 8 bits BS field for Rel-18 XR to reduce the quantisation errors.

As a working assumption, UE always sends only one BSR. UE may use either the legacy BSR table or a new BSR table with smaller quantization error. UE chooses which BSR table to use based on its buffer size, e.g. use a new BSR table if its buffer size is within the range of the new BSR table or use the legacy BSR table instead. If UE selects a different BS table to report the BS index for different LCGs, the table information needs to be reported along with BS index. Additional overhead to indicate the BS table per LCG cannot avoided. For example, an BSR format to indicate the BS table information for each LCG. If the value of a field Ti corresponding to the LCG i is set to a specific value, the buffer size corresponding to LCG i is present at the second field which value is shown in the new BS table.

Figure 3. An example of BSR format to indicate BS table information

RAN2 can further evaluate whether the quantization errors based on the new BS table for Rel-18. UE selects a base buffer size and in addition indicates another relative buffer size if the remaining quantization error is larger than a threshold. The actual Buffer status of a PDU set is comprised of two parts, as shown in Figure 3. UE reports the two BSR components in the new BSR MAC CE so that the NW can derive the actual PDU set buffer size based on the two BS components for each LCG, i.e., the actual BS of PDU set = 1st BS + 2nd BS.

Based on the example of the legacy Table 6.1.3.1-2: Buffer size levels (in bytes) for 8-bit Buffer Size field, the quantization error can be reduced to about one hundred bytes. We observe this can offer sufficiently good performance based on legacy table.

| Original Quantization error | First Buffer Size | (BS Index) Second buffer size | Reduced error to (bytes) | Reduced error rate to (%) |
| --- | --- | --- | --- | --- |
| 2415 | 48676 | (88) ≤ 2533 | 118 | 4% |
| 4700 | 97221 | (98) ≤ 4751 | 51 | 1% |
| 530 | 58784 | (64) ≤ 560 | 30 | 5% |
| 899 | 117407 | (72) ≤ 926 | 27 | 3% |

Of course, if RAN2 intends to design a new fixed table to adapt the linear step to indicate this quantization error value can also work. One example, the range value is (10, 10000), if using BS index size is 8 bits, the linear step size = 40 bytes, if using BS index size is 6 bits, the linear step size = 156 bytes.

Proposal 6: RAN2 to use a table information field corresponding to a LCG field to indicate the buffer size corresponding to the LCG is in the new BS table.

Proposal 7: RAN2 can evaluate the quantization errors based on the new BS table, if the remaining quantization error is larger than a threshold after UE selects a base buffer status, UE selects a relative BS index to indicate the quantization error.

- Conclusion

In this contribution, we discuss UE feedback enhancements for XR capacity. We have the following proposals:

Proposal 1: RAN2 to discuss Long/short (truncated) BSR format enhancements to report multiple PDU set buffer sizes in one BSR MAC CE.

Proposal 2: RAN2 to discuss the Long/short truncated BSR format enhancements on how and whether to report the delay information and buffer size in one BSR MAC CE.

Proposal 3: NW configures UE to use a new enhanced BSR MAC CE format for specific logical channels/LCG, e.g. LCH carrying XR-traffic.

Proposal 4: NW configures whether UE /MAC should report delay information for a LCG or LCH

Proposal 5: UE will be configured, e.g. by RRC, with an enhanced BSR reporting procedure which is optimized for XR traffic in addition to the legacy BSR reporting procedure.

Proposal 6: RAN2 to use a table information field corresponding to a LCG field to indicate the buffer size corresponding to the LCG is in the new BS table.

Proposal 7: RAN2 can evaluate the quantization errors based on the new BS table, if the remaining quantization error is larger than a threshold after UE selects a base buffer status, UE selects a relative BS index to indicate the quantization error.

- References

RAN2#121bis chairman note.

R2-2303203, Discussion on UE Feedback enhancements, Lenovo