3GPP TSG-RAN WG2 Meeting #123	R2- 2308026

Toulouse, France, August 21-25, 2023

Agenda item:		7.3.4

Source:		Lenovo

Title:		Discuss on SSB-less SCell operation in NES

Document for:		Discussion and Decision

# Introduction

In RAN #99 meeting, 3GPP updated the work item “Network energy savings for NR” [1]. One of objectives is included:

- Specify SSB-less SCell operation for inter-band CA for FR1 and co-located cells, if found feasible by RAN4 study, where a UE measures SSB transmitted on PCell or another SCell for an SCell’s time/frequency synchronization (including downlink AGC), and L1/L3 measurements, including potential enhancement on SCell activation procedures if necessary [RAN4, RAN2]

In RAN4#107 meeting, SSB-less SCell operation was discussed. The following progresses on scenarios and conditions to be considered for SSB-Less SCell operation were achieved [2].

- Agreements

  - Continue RAN4 work on the following SSB-less SCell scenarios

    - Scenario 1: SCell without SSB transmission and with TRS transmission

    - Scenario 2a: SCell without SSB transmission and without any other DL transmissions, but with UL reception at the NW side

      - Note: No RAN1 impacts are expected, and no RAN4 requirements will be defined if the scenario is not supported from RAN1 specification perspective.

  - Deprioritize RAN4 work on the following SSB-less SCell scenario

    - Scenario 2: SCell without SSB transmission and without TRS transmission

  - Send LS to RAN1/2 to check on support of Scenario 2a from RAN1/2 specifications perspective

- Agreements

  - Further consider the following cases for requirements definition

    - Set 1: RTD ≤ 3us + X (X is FFS)

    - Set 2: 260ns < RTD < min(CP, 3us)

      - note: the SCS is the largest SCS across CCs

      - note: the CP corresponds to the largest SCS across CCs

    - Set 3: RTD ≤ 260ns

    - FFS whether all subsets are feasible from UE implementation perspective

While the LS was not sent because No consensus to send the LS. And the related proposal [3] was discussed in RAN#100. But no consensus about the proposal was achieved.

In this contribution, we discuss the related issues.

# Discussion

According to the RAN4 discussion on the scenarios to be considered for SSB-Less SCell operation for FR1 inter-band collocated CA, it could be that no DL transmission but with UL reception at the NW side on the SSB-less SCell e.g. Scenario 2a. From the perspective of UE, the UL transmission can be transmission on the CG resources, transmission on the DG resources, transmission of the DL feedback or SR on the configured PUCCH resources, the RACH procedure, or the SRS transmission. A downlink pathloss Reference including the RS needs to be configured for the UE calculating the UL transmission power. The RS resource is either on serving cell c or, if provided, on a serving cell indicated by a value of pathlossReferenceLinking. Some enhancement for PUSCH or PUCCH transmission may be needed to support UE measuring SSB transmitted on another SCell, for example, by extending the values of pathlossReferenceLinking. Currently, the PUCCH/PUSCH/SRS transmission is not precluded on the SSB-less SCell. Therefore, whether the related resources can be configured shall be discussed in RAN2. On the other side, considering the existing RACH procedure, it could be impossible to select a RACH resource without associated SSB on the cell. It means a RACH cannot be initiated on the SSB-less SCell. Correspondingly, the SSB-less SCell shall be in the same TAG as the reference serving cell if a RACH for TA cannot be initiated on the SSB-less SCell.

Proposal 1: RAN2 to discuss the impact on the potential UL resource configurations on the SSB-less SCell.

Proposal 2: RAN2 to discuss whether the SSB-less SCell is configured in the same TAG as the reference serving cell.

The SSB-less SCell activation related discussion has been kicked off in RAN4. But there is not a clear conclusion on the activation delay or the activation behavior. Therefore, RAN2 can discuss the potential enhancement on SCell activation procedures after RAN4 achieves a related conclusion.

Proposal 3: RAN2 to wait for RAN4 progress on potential enhancement on SCell activation procedures.

# Conclusion

In this contribution, we discuss the SSB-less SCell operation for inter-band CA for FR1 and co-located cells and provide the proposals:

Proposal 1: RAN2 to discuss the impact on the potential UL resource configurations on the SSB-less SCell.

Proposal 2: RAN2 to discuss whether the SSB-less SCell is configured in the same TAG as the reference serving cell.

Proposal 3: RAN2 to wait for RAN4 progress on potential enhancement on SCell activation procedures.

# References

RP-230566, WID revision: Network energy savings for NR, Huawei

RAN4#107 RRM session report.

RP-231397, Discussion on Rel-18 network energy saving, ZTE, Sanechips