3GPP TSG-RAN WG2 Meeting #123	R2- 2308027

Toulouse, France, August 21-25, 2023

Agenda item:		7.3.4

Source:		Lenovo

Title:		Cell selection/re-selection in NES

Document for:		Discussion and Decision

# Introduction

The following progresses on Cell Selection/Re-selection for NES techniques are achieved in RAN2 meetings [1] [2] [3].

Agreements on cell reselection

- Keep the terminology of "NES cell" in the TR. The definition of NES cell will be discussed in normative phase. Remove the FFS on definition (rapporteur to update this).

- For legacy UE barring mechanism, current TR is sufficient to conclude SI, and solution details should be discussed in normative phase. Remove the FFS on exact mechanism and spec impacts.

- From RAN2 perspective legacy devices and new NES UEs can be handled via cell selection/reselection techniques

Agreements:

- RAN2 confirms that non-NES UEs can access to NES cells if NES solution is backwards compatible

Agreements:

- We will define UE capabilities with signaling.  Details are FFS and will be discussed later during the WI phase.

- Separate camping restrictions for NES-capable and non-NES UEs will be defined.  FFS if it is a single bit or more.

And the potential solutions to prevent legacy UEs from camping and allowing the NES-capable UEs to prioritize/down-prioritize a specific NES cell or NES cells on a specific frequency are captured in the TR38.864, but the details are left to WI phase to discuss.

In this contribution, we further discuss the related issues.

# Discussion

Similar to LTE supporting an interference-aware small cell on/off mechanism in a heterogeneous network, it is beneficial to allow gNB to flexibly turn on/off transmission. For example, gNB is not allowed to transmit data or signal in DL slots/symbols that are indicated as transmission OFF, except for essential signals that must be transmitted in these DL slots/symbols. In LTE, the cell on/off can be realized by configuring a cell as an activated/deactivated cell for a UE which means the cell state can be identified by the UE.

Considering the network deployment and progress, there could be a possible scenario as shown in Fig. 1 that a NES cell in NES mode (or not) neighbours to the legacy cell and the NES UE attempts to camp on one of the cells which provide overlapped coverage to the UE. It was agreed that the network should be able to configure NES capable UEs to (de)prioritize NES cells. Then if the NES cell is in NES mode is aware at the UE side, the NES-capable UE can determine to prioritize/down-prioritize a specific NES cell by taking its traffic capability and the NES cell mode into account. The (de)prioritization of NES cells cannot be ensured only by the existing S criterion for cell selection and frequency priority for cell reselection. In order to support NES capable UEs to (de)prioritize NES cells in NES mode, the NES capable UEs shall at least be able to know which NES cell is in NES mode. For example, the NES cell broadcasts the related information.

Figure 1 NES UE at the edge of NES cell and legacy cell

Proposal 1: RAN2 to discuss how a RRC Idle/ Inactive NES capable UE can determine that a NES Cell is in NES mode currently (i.e., explicitly indicated by the cell or implicitly indicated by the configuration of Cell DRX/ DTX now).

Proposal 2: RAN2 to discuss which SIB is used to broadcast the NES mode related information.

As shown in Fig.1, there could be multiple cells including NES cell in NES mode and legacy cell that satisfy the cell reselection criteria. The UE shall perform ranking based on legacy cell reselection criteria. The NES cell is not turned off which means it would like to provide service to UEs, but maybe not to any UE. Then, if the NES UE can follow the energy saving technologies applied in the cell, it potentially provides benefits to the UE power saving e.g. less or long periodical reception. Therefore, an NES UE shall re-select the NES cell in priority if both NES cell and non-NES cell meet the cell reselection criteria.

Proposal 3: An NES UE shall re-select the NES cell in priority if both NES cell and non-NES cell meet the cell reselection criteria.

Another issue was discussed, and it was agreed that “Whether to bar legacy UEs is configurable by NES cells in Idle/Inactive mode and the network should be able to allow NES-capable UEs to camp on the NES cell”. That is to say the network shall be able to bar legacy UE and allow the access attempt of NES UEs only. The legacy UE could be unknown of the NES cell, for example, it is not necessary for the legacy UE to identify the type of the cell (e.g. NES cell) by which its access attempt is barred.

Observation 1: It could be unknown of the NES cell to the legacy UE, for example, it is not necessary for the legacy UE to identify the type of the cell (e.g. NES cell) by which its access attempt is barred.

And from the perspective of UE power saving, it’ll be better for the legacy UEs to be barred as early as possible. Therefore, the legacy “cellBarred” can be used to bar the access attempt of legacy UEs since anyway the legacy “cellBarred” is supported by legacy non-NES UE. On the other side, the network can set the “cellBarred” to “notBarred” and reuse the existing SIB1 barring mechanism to fulfill critical performance requirement if the cell would provide emergency call while the coverage is provided only by NES cell in NES mode. There could be no spec impact to the non-NES UEs.

Proposal 4: It is up to the network implementation to reuse the legacy “cellBarred” in MIB or SIB1 mechanism to restrict the access attempt of the non-NES UE.

Proposal 5: Reuse SIB1 mechanism to provide emergency call for non-NES UE if the coverage is provided only by the NES cell in NES mode.

Referring to the existing barring mechanisms for other case, e.g. RedCap, a dedicated IE can be defined to restrict the NES-capable UEs to camp on the NES cell. Although there could be several energy saving technologies, to simplify the implementation, we don’t think it is necessary to bar the access attempt per energy saving technology.

Proposal 6: A dedicated IE with a single bit to restrict the NES-capable UEs to camp can be introduced.

# Conclusion

In this contribution, we discuss the Cell Selection/Re-selection in Rel-18 NES and provide the proposals:

Proposal 1: RAN2 to discuss how a RRC Idle/ Inactive NES capable UE can determine that a NES Cell is in NES mode currently (i.e., explicitly indicated by the cell or implicitly indicated by the configuration ofCell DRX/ DTX now).

Proposal 2: RAN2 to discuss which SIB is used to broadcast the NES mode related.

Proposal 3: An NES UE shall re-select the NES cell in priority if both NES cell and non-NES cell meet the cell reselection criteria.

Proposal 4: It is up to the network implementation to reuse the legacy “cellBarred” in MIB or SIB1 mechanism to restrict the access attempt of the non-NES UE.

Proposal 5: Reuse SIB1 mechanism to provide emergency call for non-NES UE if the coverage is provided only by the NES cell in NES mode.

Proposal 6: A dedicated IE with a single bit to restrict the NES-capable UEs to camp can be introduced.

# References

RAN2#120 chairman notes.

RAN2#121 chairman notes.

RAN2#122 chairman notes.