3GPP TSG-RAN WG2 Meeting #123                                    R2-2308041

Toulouse, France, August 21-25, 2023

Agenda Item:	7.4.3

Source: 	OPPO

Title:  	Discussion on execution conditions for subsequent CPAC

Document for:	Discussion and Decision

Introduction

In last meeting, the following agreements have been reached for subsequent CPAC regarding to candidate cell configuration and execution condition:

For SN-initiated SCG selective activation, candidate SN generates execution conditions for subsequent CPC.

FFS if it shall be possible to do something like MN-initiated CPA/CPC where Candidate SN generate execution conditions for subsequent CPC

The UE shall skip the condition evaluation for a candidate which is a current PScell.

The reference configuration is provided to all candidates involved in preparation, FFS which node initially generates it. Assume it can be provided in MN initiated and in SN initiated procedures.

Will not spend specific efforts for supporting nested configurations for candidate cell configuration.

In this contribution, we would like to keep discussing the execution condition related issues for subsequent CPAC.

Discussion

Subsequent CPAC execution condition

For subsequent CPAC, we agreed to support both MN-initiated case and SN-initiated case. In legacy, for MN-initiated CPA/CPC, the execution condition of candidates referring to MCG measurement configuration and execution conditions based on A4 event are supported.

Assume to support the following scenarios of SCG selective activation:

    - SN initiated intra-SN SCG selective activation

    - MN initiated inter-SN SCG selective activation

    - SN initiated inter-SN SCG selective activation

For MN initiated inter-SN subsequent CPAC, it is agreed that source MN generates the execution conditions for the initial CPAC. Regarding to the subsequent CPC, whether source MN or candidate SN to generate the execution conditions is FFS.

For MN initiated inter-SN SCG selective activation, source MN generates the execution conditions for the initial CPAC.

- FFS on the following options for subsequent CPC:

- Option 1: Source MN generates the execution conditions for all subsequent CPC.

- Option 2: Candidate SN may generate execution conditions for subsequent CPC.

In our understanding, if we allow candidate SN to generate/modify the execution condition for MN initiated subsequent CPC procedure, the following issues may be introduced:

- Issue 1：Additional Xn signalling interactions for execution condition preparation

To prepare the execution conditions for subsequent CPC, each candidate is required to aware of the full candidate cell list, which may cause additional Xn signalling interactions. And the measurement configuration which provided by MN may also be required as a reference for candidate SCG to prepare the execution condition for subsequent CPC.

- Issue 2：Reconfiguration for all candidate cells is required if NW modify the candidate list

If candidate SN prepares the execution condition for all of other candidates during subsequent CPAC preparation, once NW modify the candidate cell list, i.e. add/release candidate cell, reconfiguration of execution condition for the new added or released candidate cell is required. As each candidate has prepared the execution conditions for all other candidates, reconfigurations for all candidate cells execution condition are required accordingly. On the other hand, if candidate cell prepares the execution conditions for all of other candidates after UE has accessed to the candidate cell, UE needs to postpone the condition evaluation for other candidate cells until reception of the execution condition reconfiguration message.

For MN-initiated subsequent CPAC, source MN generates the execution conditions for initial and subsequent CPC.

For MN-initiated subsequent CPAC, A3/A5 is not supported as the execution conditions for initial and subsequent CPC.

In previous meeting, a baseline procedure for selective activation of cell groups has been agreed as general guidance:

Baseline procedure to support subsequent secondary cell group change (FFS if UE keeps all configurations or if those are indicated by the network, FFS support of nested configs):

- a.	Step 1: when the execution condition of a CPC candidate PScell is met, a UE performs the execution of CPC towards this candidate PScell.

- b.	Step 2: After finishing the PSCell addition or change, the UE doesn’t release conditional configuration of other candidate PSCells for subsequent CPC, the UE continues evaluating the execution conditions of other candidate PScells.

- c.	Step 3: When the execution condition of a candidate PScell is met, the UE performs the execution of CPC towards this candidate PSCell.

For subsequent CPC defined in step 2, it is unclear how and when UE performs candidate cell evaluation of other candidate cells. In legacy CPA/CPC, UE starts to evaluate the execution condition(s) upon receiving the configuration for CPC/CPA, and stops evaluating the candidates once conditional PSCell change/addition is triggered. For the first CPC/CPA, the existing evaluation mechanism can be reused, while it is not feasible to cover subsequent CPC/CPA since no reconfiguration message is expected. Therefore, further study is needed to define when to start the evaluation on candidate cells for subsequent CPC/CPA. And the most straightforward way is to start evaluating other candidate cells directly after last PScell change/addition procedure.

As a baseline, UE starts the condition evaluation for subsequent CPC/CPA procedure upon PSCell change/addition/release completion.

How to configure the execution condition

As agreed in previous meeting, for SN-initiated subsequent CPC, candidate SN generates the execution conditions for subsequent CPC. That is, each candidate cell may generate the execution conditions for all other candidate cells for subsequent CPAC. To avoid frequently RRC reconfiguration for subsequent execution condition provision, one can prepare the execution conditions during the CPAC preparation phase. How the candidate SN generates and provides the execution conditions is FFS. The following two options can be considered for further discussion:

Option1: each candidate cell is associated with a list of execution conditions for subsequent CPC procedure. And both initial and subsequent execution conditions are included in conditionalReconfiguration message.

The signalling structure is given as an example below. In this option, an additional list of execution conditions for subsequent CPC is introduced and the list is configured together with the execution condition for initial CPAC procedure. For example, in the newly introduced list, the candidate cell#1 provides the execution conditions for other candidates, which is used for subsequent execution condition evaluation for other candidate cells when the candidate cell#1 becomes to the serving sell.

Figure 1: signalling structure of Option1

Option 2: the candidate cell generates the execution conditions for subsequent CPC and includes the execution conditions in the RRC reconfiguration of each candidate cell. The execution condition of initial CPAC is configured in legacy way.

In option2, legacy signalling structure can be reused, and the execution conditions provided by candidate cell for subsequent CPC procedure are included in each candidate cell configuration. For example, UE applies the candidate cell configuration during CPAC execution and updates/replaces the execution conditions associated with other candidate cells based on the configuration.

Figure 2: signalling structure of Option2

Both of the two options listed above can work, and option1 is a straightforward way to configure the execution for subsequent CPC, while option2 brings less spec impact. RAN2 can further study how to configure the execution condition for subsequent CPC

RAN2 further study how to configure the execution condition for subsequent CPC procedure:

- Option1: both initial and subsequent execution condition are included in conditionalReconfiguration message.

- Option2: the execution conditions for subsequent CPC are included in the RRC reconfiguration for each candidate cell.

Conclusion

Based on the discussion above, we have the following proposals:

Proposal 1	For MN-initiated subsequent CPAC, source MN generates the execution conditions for initial and subsequent CPC.

Proposal 2	For MN-initiated subsequent CPAC, A3/A5 is not supported as the execution conditions for initial and subsequent CPC.

Proposal 3	As a baseline, UE starts the condition evaluation for subsequent CPC/CPA procedure upon PSCell change/addition/release completion.

Proposal 4	RAN2 further study how to configure the execution condition for subsequent CPC procedure:

Option1: both initial and subsequent execution condition are included in conditionalReconfiguration message.

Option2: the execution conditions for subsequent CPC are included in the RRC reconfiguration for each candidate cell.