3GPP TSG-RAN WG2 Meeting #123	                              R2-2308052

Toulouse, France, August 21-25, 2023

Source: 			OPPO

Title: 	Further discussion on sidelink positioning

Agenda item:		7.2.2

Document for: 	Discussion and Decision

- Introduction

In the last RAN2 #122 meeting, agreements regarding sidelink positioning have been achieved as follows:

Agreement:

- SLPP over PC5-U/Uu will support reliable transport for at least unicast. FFS groupcast.

- SLPP carried over NAS is used between UE and LMF. FFS on how to manage the session/transaction.

- Anchor UE selection is supported by information about the candidate anchor UEs. At least the following list can be discussed for use in anchor UE selection:

- UE roles

- Supported positioning method

- In coverage or not

- RSRP

- LOS/NLOS

- Location

- PLMN

- A normative requirement on which anchor UEs to select (e.g., ranking) will not be specified. RAN2 impact of this information to be determined. FFS which information would be determined statically/dynamically

- SLPP can support multiple target UEs in the same session when LCS requests.

- RAN2 will not specify group management for multiple target UEs. RAN2 assumption is that a group ID will be provided from upper layers.

- FFS how session IDs are managed between multiple UEs.

In this paper, we would like to present our views on supporting the sidelink positioning in NR from the perspective of architecture, protocol stack, signaling procedure.

- Discussion

- 2.1 the protocol stack between UE and UE

According to TS 23.586 V18.0.0, RSPP (SLPP) messages are common for V2X capable UEs and 5G ProSe capable UEs, and PC5-U is used as the transport layer for RSPP messages. The RSPP message is carried using "Non-IP" PDCP SDU type for V2X capable UEs as defined in clause 6.1.1 of TS 23.287, and "Unstructured" PDCP SDU type for 5G ProSe capable UEs as defined in clause 6.1.2.2 of TS 23.304. For the figure to be captured in TS 38.305, we think it is better to show “Non-IP/Unstructrued” layer explicitly for the sake of the completeness of protocol stack, since non-IP payload shall be added with a non-IP Type header.

Observation 1: For the figure captured in 38305, it is better to show “Non-IP/Unstructrued” layer explicitly for the sake of the completeness of protocol stack.

Another issue is that whether V2X/ProSe layer is needed below the SLPP. In TR 23.700-86 i00, it is stated in the conclusion of KI#4 that:

For Key Issue #4 "Control of Operations for Ranging/Sidelink positioning", the followings are taken as conclusions: - A Ranging/SL Positioning layer is introduced on the Target UE/Reference UE/SL Positioning Server UE under Application layer and above AS layer to handle service request received from application layer and to control the Sidelink Positioning and Ranging operation: - Functionalities supported by the Ranging/SL Positioning layer include discovery of the UE(s) in proximity that can participate in Sidelink Positioning and Ranging service sessions and control signalling between UEs or among a group of UEs or between UE and LMF to manage and coordinate the Sidelink Positioning and Ranging operations. - The group management can be performed at application layer, and the application layer may provide group identifier information to the Ranging/SL Positioning layer. NOTE 1: Potential group management within RSPP layer is out-of-scope of SA2. - Information exchanged between Ranging/SL Positioning layer and lower layer includes one time or periodic ranging, ranging for distance or direction measurement or both. NOTE 2: Further information may be identified in the normative phase. - If PC5-U is used, Ranging/SL Positioning signaling is carried over V2X/ProSe Layer as payload. The existing V2X/ProSe transport mechanism is reused to handle the Layer-2 ID and Application Layer ID information. (omit the irrelevant text)

As highlighted above, SA2 has already made clear conclusion in TR stage, i.e., if PC5-U is used, Ranging/SL Positioning signaling is carried over V2X/ProSe Layer as payload. Therefore, in our understanding, for SLPP layer, V2X/ProSe layer is needed in PC5-U on top of Non-IP/Unstructured and below SLPP layer. For V2X UE, both SLPP layer and V2X layer are both carried in the non-IP payload with a non-IP Type header for SLPP. For ProSe UE, it is similar, but more work would be done in SA2/CT1 due to no design on unstructured header and payload for now.

Observation 2: SA2 has already made clear conclusion in TR stage, i.e., if PC5-U is used, Ranging/SL Positioning signaling is carried over V2X/ProSe Layer as payload.

Proposal 1: Regarding the protocol stack between UE and UE, RAN2 to take the following figure as baseline to capture in 38305, i.e., SLPP => V2X/ProSe layer => Non-IP/Unstructrued => AS layer.

- 2.2 Support of the multiple target UEs case

The purpose of Figure 6.20.3-1 included in the Section 6.20.3 Procedures of SL-MT-LR involving LMF in the TS 23.273 V18.2.0 is to enable an LCS Client or AF to obtain Ranging/Sidelink Positioning location results for a group of n UEs (n≥2), i.e. UE1, UE2, ..., UEn. And also in the step 16 for the procedures of SL-MT-LR involving LMF, ranging/sidelink positioning of UE1 and the other discovered UEs among UE2 , …, UEn are performed, while the other undiscovered UE will be positioned using the 5GC-MT-LR by the LMF interacting with GMLC. Finally, the LMF will return Sidelink positioning/ranging location results to the LCS Client or AF.

Obviously, such implementation does not enforce any RAN2 signaling procedure impact.

Proposal 2: RAN2 to not pursue further work for positioning of multiple target UE given the 5GC-MT-LR involving LMF procedure defined by SA2 .

- 2.3 Combination of Uu- and PC5-based positioning

As well known, the DL-PRS/SRSp propagation path between the UE and the gNB may be blocked or scattered to a large extent in the urban area, which causes NLOS complicated multipath and therefore degrades of the positioning accuracy. However, it should be noted that whether the DL-PRS/SRSp propagation path between a UE and a gNB in a particular scenario should be categorized to NLOS or LOS is only known at the receiver after the practical transmission of such reference signal has been done.

Observation 3: whether the DL-PRS/SRSp propagation path between a UE and a gNB in a particular scenario should be categorized to NLOS or LOS is only known at the receiver after the practical transmission of such reference signal has been done.

Such observation leads to two modes of adding PC5 based positioning for further calibration of the Uu based positioning result:

  - Option 1: UE/LMF autonomously triggers PC5 based positioning during the Uu positioning session, potentially based on pre-configured condition, e.g., detection of NLOS transmission of the DL-PRS/SRSp, low RSRPP of the received DL-PRS/SRSp, etc.

  - Option 2: LMF triggers PC5+Uu based positioning after the Uu positioning session is ended. The decision of triggering further PC5 positioning to calibrate the Uu based positioning could be a result of the bad Uu positioning result, i .e., low positioning accuracy or the integrity of the positioning result could not be guaranteed.

Obviously, Option 2 consumes more time than Option 1 to obtain a satisfied positioning result, due to the fact that new positioning session embracing PC5 based positioning signalling flow has to be triggered, and the Uu positioning capability and reference signal configuration needs to be transferred again between the UE and the gNB during such new positioning session.

Observation 4: Option 2 (LMF triggering PC5+Uu based positioning in a new positioning session) may consume more time than Option 1 (autonomously triggering PC5 based positioning based on some preconfigured condition) to obtain a satisfied positioning result, since another new positioning session has to be triggered for the option 2, and the Uu positioning capability and reference signal configuration may need to be transferred again between the UE and the gNB during such new positioning session.

According to the TS 23.273 section 6.20.1 and section 6.20.2, in signalling procedure defined by SA2, determination of whether or not to trigger the SL-MO-LR or SL-MT-LR is up to the target UE or the LMF implementation. For the SL-MO-LR, the target UE could autonomously trigger the SL positioning based on the previously collected sidelink positioning. On the other hand, for the SL-MT-LR, nothing is specified in the TS 23.273 as the reference for the LMF to trigger to send an SL-MT-LR request to the serving AMF as a supplementary services msg. We think that it is reasonable for RAN2 to specify the UE-triggering SL positioning criteria configured by LMF. Suppose the response time requirement is relatively loose (but not long enough to accommodate two continuous positioning sessions) but the positioning accuracy requirement is high, configuring such criteria for the target UE could benefit for the positioning accuracy.

Proposal 2: RAN2 to agree that UE-triggering SL positioning criteria e.g., detection of NLOS transmission of the DL-PRS/SRSp, low RSRPP of the received DL-PRS/SRSp, etc, could be configured by the LMF to UE to determine whether or not to trigger SL positioning to calibrate the ongoing Uu positioning session.

- 2.3 Consideration on anchor UE selection

According to the section 6.8 Procedure of Ranging/Sidelink Positioning control in the TS 23.586 v18.0.0, after receiving a Ranging/SL Positionig Service request, the target UE will perform UE discovery procedure of finding the suitable SL Reference UEs. Then, after the suitable SL Reference UEs have been discovered, capability exchange between the target UE and the discovered SL Reference UEs will be performed. Several Editor notes in the TS 23.586 indicate that the RSPP metadata information (e.g. the role(s) of the Discoveree UE) is included as the metadata in the Announcement/Solicitation/Response message, which value is determined by RAN WG2. So, in this section, we would like to present our view on which set of information should be included in the discovery message.

Primarily, the anchor UE role should be included in the discovery model A announcement msg transmitted by the anchor UE, or the required UE role, i.e. demand of the anchor UEs, should be included the discovery model B solicitation msg transmitted by the target UE. For example, when the discovery mode A is applied, the target UE will know whether or not candidate anchor UEs are in the neighborhood by monitoring the discovery announcing msg broadcasted by them with the anchor UE role indication included. Similarly, for the discovery model B, the candidate anchor UEs could choose whether or not to perform establishment of secure layer-2 link over PC5 towards the target UE, if the demand of the sidelink anchor UE is expressed in the discovery model B solicitation msg transmitted by a target UE. Such implementations are indicated as follows:

- An example of discovery mode A implementation

- An example of discovery mode B implementation

Figure 1: role indication to be included in the SL discovery msg for the SL positioning

Proposal 3: RAN2 to agree that the anchor UE role indication should be included in the discovery model A announcement message, and the required UE role, i.e. demand of the anchor UEs should be included in the discovery model B solicitation message transmitted by the target UE.

Also, note that according to the TS 23.586, UE-only Operation will be applied only when none of UE1/…/UEn are served by NG-RAN and the serving network does not support Ranging/SL Positioning. Otherwise, the LMF should be involved. Hence, in order for the UE to know whether or not to apply the UE-only Operation, the in-/out-of-coverage indication should be provided in the discovery mode A announcement msg or the discovery mode B solicitation msg transmitted by the anchor UE, as illustrated in the below figure.

- An example of discovery mode A implementation

- An example of discovery mode B implementation

Figure 2: in-/out-of-coverage to be included in the SL discovery msg for the SL positioning

Proposal 4: RAN2 to agree that the in-/out-of-coverage indication is provided in the discovery model A announcement message or the discovery model B solicitation response message transmitted by the candidate anchor UE.

Subsequently, if the location of the anchor UE is not known as a priori knowledge before entering into the positioning session, to obtain the information of the location of the anchor UE, further more positioning sessions need to be triggered during the positioning session of the target UE, which may postpone the time of obtaining the positioning of the target UE and even cannot fulfil the QoS requirement of response time. As a result, the anchor UE should expose whether or not its location information is already available in the transmitted announcement msg or the response msg.

Proposal 5: RAN2 to agree that the indication of the availability of the location of the anchor UE is provided in the discovery model A announcement message or the discovery model B solicitation response message transmitted by the candidate anchor UE.

Regarding the RSRP and the LOS/NLOS, such information could be obtained by the target UE itself by taking related measurement of the msg transmitted by the anchor UE, and as a result is not necessarily to be included in any msg of the discovery procedure.

Proposal 6: RAN2 to agree that the RSRP and the LOS/NOLS related information are not needed to be included in the discovery message for anchor UE selection.

In addition, so far, only two frequency bands (e.g., 5.9GHz band (n47) and 2.5GHz band(n38)) are allowed to distribute 5G NR SL services, while n38 is a licensed band (but RAN sharing is not allowed), n47 is non-operator managed, hence PLMN info seems not necessarily to be included in the Announcement message transmitted by the candiate anchor UE in discovery model A and the Solicitation msg transmitted by the target UE in the discovery model B.

Proposal 7: RAN2 to agree that the PLMN info should not be included in the Announcement message transmitted by the candidate anchor UE in discovery model A and the Solicitation msg transmitted by the target UE in the discovery model B.

- 2.4 Consideration on SL positioning server UE selection

According to the section 6.8 of TS 23.586 v18.0.0, for UE-only Operation, if UE1 does not support SL Positioning Server functionalities, a SL Positioning Server UE (either co-located with a SL Reference UE/Located UE, or operated by a sperate UE) is discovered (if not yet discovered in the step 2 (for anchor UE discovery)) and selected.

First of all, if in the step of discovery of anchor UE, the candidate anchor UEs could indicate it could play the role of the SL positioning server UE in the Solicitation response msg or the Announcement msg also, the target UE could collect the information and only needs to perform choice of the SL Positioning Server UE in the following step.

Observation 5: if in the step of discovery of anchor UE, the anchor UEs could indicate it could play the role of the SL positioning server UE in the Solicitation response msg or the Announcement msg, the target UE could collect the information and only needs to perform selection of the SL Positioning Server UE in the following step.

Proposal 8: RAN2 to agree that the UE also indicates whether or not it is capable of serving as the SL Positioning Server UE in the discovery model A announcement msg or the discovery model B response msg for the anchor UE discovery procedure.

However, if unfortunately, there is no UE indicating that it is capable of playing the role of SL Positioning Server UE in the step 2, and if the UE1 is not able to serve as the SL Positioning Server UE, then the UE1 needs to perform the SL Positioning server UE discovery procedure. An implementation example similar with the discovery of the candidate SL positioning server UE is indicated as follows:

- An example of discovery mode A implementation

- An example of discovery mode B implementation

Figure 3: SL role indication to be included in the SL discovery msg for the SL positioning

Furthermore, regarding the supported positioning method of the candidate SL Positioning server UEs, for the UE-only Operation, since it is the target UE receiving the Ranging/SL Positioning service Request, only the target UE knows the detailed positioning QoS requirement of the positioning service request, before making the final selection of the SL Positioning Server UE, hence the required positioning QoS requirement or the indication of the required SL positioning method(s) should be considered to be embed in the discovery messages for the purpose of making selection of the qualified SL Positioning server UE, as indicates as follows.

- An example of discovery mode A implementation of a candidate anchor UE

- An example of discovery mode B implementation of the target UE

- An example of discovery mode A implementation of the candidate SL positioning server UE

- An example of discovery mode B implementation of the candidate SL positioning server UE

Figure 4: SL positioning QoS or the positioning methods info to be included in the SL discovery msg for making selection of the SL Positioning server UE

Proposal 9: RAN2 to agree that the candidate anchor UE AND the candidate SL positioning server UE indicates the supported positioning method and/or the supported positioning QoS requirement(s) as the SL positioning server UEin the discovery model A announcement msg.

Proposal 10: RAN2 to agree that the required positioning QoS requirement or the indication of the required SL positioning method(s) for selection of the SL positioning server UE should be considered to be embed in the discovery model B solicitation message transmitted by the target UE for the purpose of making selection of the qualified SL Positioning server UE.

- 2.5 Consideration on assistance data and measurement transfer procedure

Regarding assistance data transfer, in the context of SL positioning, we think it could serve for three purposes. Firstly, if the SL-PRS configuration could be autonomously decided by SL-PRS transmitter UE, it may need to send the SL-PRS configuration to the location server UE firstly for further distribution to other SL-PRS monitor UE(s). Subsequently, the SL positioning server UE could also employ the assistance data transfer msg to distribute the SL-PRS configuration to the SL-PRS monitor UEs for performing further measurement.

In addition, the location information of anchor UEs could be also included in the assistance data transfer msg to be transmitted towards the location server UE or other UEs for the position calculation.

Overall, SL positioning assistance data transfer msg should include required assistance data for UEs to perform the necessary SL-PRS measurements. As a result, we propose RAN2 that SL positioning assistance data transfer msg could be used in following scenarios, optionally with the location information of the anchor UEs included:

- the SL-PRS transmitter UE sends out its SL-PRS configuration to the location server UE/LMF

- the location server UE/LMF transmits the SL-PRS configuration to be monitored to the SL-PRS monitor UEs

Observation 6: the SL positioning assistance data transfer msg should include required assistance data for UEs to perform the necessary SL-PRS measurements and/or the SL position calculation.

Proposal 11: RAN2 to agree that SL positioning assistance data transfer msg could be used in following scenarios, optionally with the location information of the anchor UEs to be included:

- the SL-PRS transmitter UE sends out its SL-PRS configuration to the location server UE/LMF

- the location server UE/LMF transmits the SL-PRS configuration to be monitored to the SL-PRS monitor UEs

Next, similar to the Uu based positioning, the SL Positioning Location Information Request should tell the SL-PRS monitor UE which kind of SL-PRS measurement result is needed, which is related to the positioning method chosen by the location server UE. Bearing in mind that SL-AOA, SL-TDOA and SL-RTT have been agreed in the WID as the candidate sidelink positioning method, we kindly propose RAN2 to at least include following three positioning method related IEs in the SLPP RequestLocationInformation msgs:

- SL-AOA-RequestLocationInformation

- SL-TDOA-RequestLocationInformation

- SL-RTT-RequestLocationInformation

Once the SL-PRS monitor UE receives the SLPP RequestLocationInformation msg, it can perform the SL-PRS measurement corresponding to the demanded positioning method and report the related positioning measurement result to the location server.

Proposal 12: RAN2 to agree that at least following three positioning method related IEs should be included in the SLPP RequestLocationInformation msgs:

- SL-AOA-RequestLocationInformation

- SL-TDOA-RequestLocationInformation

- SL-RTT-RequestLocationInformation

Subsequently, after performing the SL positioning measurement, the SL-PRS monitor UE or the UE taking the task of position calculation should send back the positioning measurement result or position estimates to the location server. As a result, we propose at least following IEs should be at least included in the SLPP ProvideLocationInformation msg:

- commonIEsProvideLocationInformation (including the position estimate result or the ranging result)

- SL-Multi-RTT-ProvideLocationInformation

- SL-TDOA-ProvideLocationInformation

- SL-AOA-ProvideLocationInformation

Proposal 13: RAN2 to agree at least following IEs should be included in the SLPP ProvideLocationInformation msg at least:

- commonIEsProvideLocationInformation (including the position estimate result or the ranging result)

- SL-Multi-RTT-ProvideLocationInformation

- SL-TDOA-ProvideLocationInformation

- SL-AOA-ProvideLocationInformation

- 2.6 Consideration on session ID and session management for the SL positioning

- 2.6.1 SLPP over NAS transport layer

Recall in the Uu-based positioning, the LPP session is initiated by sending out an LPP message for an initial LPP transaction j by either the target or the server, and is further terminated by a LPP message with a final transaction N. Each LPP session is associated with a session ID and the main purpose is to differentiate concurrent sessions (potentially with different positioning QoS requirement) between two endpoints. The session ID is actually achieved by taking advantage of routing ID transmitted in the NAS layer blow the LPP layer.

According to the TS 27.273 V18.0.0, for Ranging/Sidelink Positioning procedures, regardless of procedures of SL-MO-LR involving LMF, 5GC-MO-LR Procedure using SL positioning, procedures of SL-MT-LR involving LMF, 5GC-MT-LR procedure using SL positioning, the SL positioning could be triggered by The AMF invoking the Nlmf_Location_DetermineLocation service operation towards the LMF to request the current location of the UE, similar with the legacy Uu based positioning method. In the meanwhile, the routing ID, the LCS Correlation Identifier, is also assigned by the AMF and is included in the Nlmf_Location_DetermineLocation Request message. On the other hand, the anchor UE serves as the TRP for the SL positioning and hence should follow the NRPPa operation: only transaction ID is needed for the related msg. So we propose RAN2 to agree that the LCS Correlation Identifier is viewed as the session ID for the SL positioning involving the LMF and therefore no explicit session ID is needed to be included in the SLPP protocol.

Observation 7: according to the TS 27.273 V18.0.0, for Ranging/Sidelink Positioning procedures, regardless of procedures of SL-MO-LR involving LMF, 5GC-MO-LR Procedure using SL positioning, procedures of SL-MT-LR involving LMF, 5GC-MT-LR procedure using SL positioning, the SL positioning could be triggered by The AMF invoking the Nlmf_Location_DetermineLocation service operation towards the LMF to request the current location of the UE, similar with the legacy Uu based positioning method.

Proposal 14: RAN2 to agree that the LCS Correlation Identifier is viewed as the session ID for the SL positioning involving the LMF and therefore no explicit session ID is needed to be included in the SLPP protocol.

- 2.6.2 SLPP over PC5-U/Uu layer

Recall in the previous RAN2 meeting, we have agreed that SLPP carried over PC5-U will be applied for SL positioning msg transfer between UEs. This not only applies to the UE-only operation (OOC case) but also to the SL positioning involving LMF since it could be found from the TS 23.273 that UE1 will forward the assistance data received from the LMF to UE2/…/UEn and the UE1 will also performs the location information transfer procedure with UE2/…/n.

For the SL positioning involving LMF case, the UE-UE link only performs assistance data transfer and location information transfer procedure, which is similar with the NRPPa link, therefore the explicit session ID is not needed, transaction ID is sufficient.

But, for the UE-only operation, there is potentially one UE serving as the positioning server UE, and therefore the positioning session should be maintained for the link between the target UE and the positioning server UE, which follows the legacy operation of Uu based positioning. On the other hand, for the link between the target UE and the anchor UE, and between the location server UE and the anchor UE, the NRPPa protocol which only takes advantage of the transaction ID should be referred to

But the question remains whether to introduce the session ID in the SLPP protocol or the protocol layer below the SLPP? In our opinion, the legacy operation for the LPP is to embed the session ID in the NAS transport layer, the layer below the LPP. To pursue a unified design, we prefer to embodying the session ID in the protocol layer blow the SLPP also, e.g., the Prose/V2X layer.

Proposal 15: RAN2 to agree that the SLPP session ID should be optionally accommodated (only for the link between the target UE and the location server UE) in the protocol layer blow the SLPP, e.g., the Prose/V2X layer.

- 2.6.3 session management for the SL positioning

Naturally, regarding the positioning session management, since there is no difference between the anchor UE functionality and the TRP functionality in terms of SL-PRS transmission and SL-PRS measurement, whether or not the SL-PRS transmission/measurement should be perform and which kind of measurement result should be provided is totally up to the SLPP msg received and therefore they does not need to sense whether or not the positioning session is terminated, the session management could be transparent to anchor UEs.

Proposal 16: RAN2 to agree that the session management could be transparent to anchor UEs.

In addition, the session ID, for the UE-Only Operation, could be allocated by the UE1 (i.e., target UE) when it requests SL positioning server UE to participate in the Ranging/Sidelink positioning.

Proposal 17: for the UE-only operation, RAN2 to agree that the session ID could be allocated by the UE1 (i.e., target UE) when it requests the SL Positioning server UE to participate in the Ranging/Sidelink positioning.

- Conclusion and proposals

In this paper, following observations and proposals have been made by us:

Observation 1: For the figure captured in 38305, it is better to show “Non-IP/Unstructrued” layer explicitly for the sake of the completeness of protocol stack.

Observation 2: SA2 has already made clear conclusion in TR stage, i.e., if PC5-U is used, Ranging/SL Positioning signaling is carried over V2X/ProSe Layer as payload.

Proposal 1: Regarding the protocol stack between UE and UE, RAN2 to take the following figure as baseline to capture in 38305, i.e., SLPP => V2X/ProSe layer => Non-IP/Unstructrued => AS layer.

Observation 3: whether the DL-PRS/SRSp propagation path between a UE and a gNB in a particular scenario should be categorized to NLOS or LOS is only known at the receiver after the practical transmission of such reference signal has been done.

Observation 4: Option 2 (LMF triggering PC5+Uu based positioning in a new positioning session) may consume more time than Option 1 (autonomously triggering PC5 based positioning based on some preconfigured condition) to obtain a satisfied positioning result, since another new positioning session has to be triggered for the option 2, and the Uu positioning capability and reference signal configuration may need to be transferred again between the UE and the gNB during such new positioning session.

Proposal 2: RAN2 to agree that UE-triggering SL positioning criteria e.g., detection of NLOS transmission of the DL-PRS/SRSp, low RSRPP of the received DL-PRS/SRSp, etc, could be configured by the LMF to UE to determine whether or not to trigger SL positioning to calibrate the ongoing Uu positioning session.

Proposal 3: RAN2 to agree that the anchor UE role indication should be included in the discovery model A announcement message, and the required UE role, i.e. demand of the anchor UEs should be included in the discovery model B solicitation message transmitted by the target UE.

Proposal 4: RAN2 to agree that the in-/out-of-coverage indication is provided in the discovery model A announcement message or the discovery model B solicitation response message transmitted by the candidate anchor UE.

Proposal 5: RAN2 to agree that the indication of the availability of the location of the anchor UE is provided in the discovery model A announcement message or the discovery model B solicitation response message transmitted by the candidate anchor UE.

Proposal 6: RAN2 to agree that the RSRP and the LOS/NOLS related information are not needed to be included in the discovery message for anchor UE selection.

Proposal 7: RAN2 to agree that the PLMN info should not be included in the Announcement message transmitted by the candidate anchor UE in discovery model A and the Solicitation msg transmitted by the target UE in the discovery model B.

Observation 5: if in the step of discovery of anchor UE, the anchor UEs could indicate it could play the role of the SL positioning server UE in the Solicitation response msg or the Announcement msg, the target UE could collect the information and only needs to perform selection of the SL Positioning Server UE in the following step.

Proposal 8: RAN2 to agree that the UE also indicates whether or not it is capable of serving as the SL Positioning Server UE in the discovery model A announcement msg or the discovery model B response msg for the anchor UE discovery procedure.

Proposal 9: RAN2 to agree that the candidate anchor UE AND the candidate SL positioning server UE indicates the supported positioning method and/or the supported positioning QoS requirement(s) as the SL positioning server UEin the discovery model A announcement msg.

Proposal 10: RAN2 to agree that the required positioning QoS requirement or the indication of the required SL positioning method(s) for selection of the SL positioning server UE should be considered to be embed in the discovery model B solicitation message transmitted by the target UE for the purpose of making selection of the qualified SL Positioning server UE.

Observation 6: the SL positioning assistance data transfer msg should include required assistance data for UEs to perform the necessary SL-PRS measurements and/or the SL position calculation.

Proposal 11: RAN2 to agree that SL positioning assistance data transfer msg could be used in following scenarios, optionally with the location information of the anchor UEs to be included:

- the SL-PRS transmitter UE sends out its SL-PRS configuration to the location server UE/LMF

- the location server UE/LMF transmits the SL-PRS configuration to be monitored to the SL-PRS monitor UEs

Proposal 12: RAN2 to agree that at least following three positioning method related IEs should be included in the SLPP RequestLocationInformation msgs:

- SL-AOA-RequestLocationInformation

- SL-TDOA-RequestLocationInformation

- SL-RTT-RequestLocationInformation

Proposal 13: RAN2 to agree at least following IEs should be included in the SLPP ProvideLocationInformation msg at least:

- commonIEsProvideLocationInformation (including the position estimate result or the ranging result)

- SL-Multi-RTT-ProvideLocationInformation

- SL-TDOA-ProvideLocationInformation

- SL-AOA-ProvideLocationInformation

Observation 7: according to the TS 27.273 V18.0.0, for Ranging/Sidelink Positioning procedures, regardless of procedures of SL-MO-LR involving LMF, 5GC-MO-LR Procedure using SL positioning, procedures of SL-MT-LR involving LMF, 5GC-MT-LR procedure using SL positioning, the SL positioning could be triggered by The AMF invoking the Nlmf_Location_DetermineLocation service operation towards the LMF to request the current location of the UE, similar with the legacy Uu based positioning method.

Proposal 14: RAN2 to agree that the LCS Correlation Identifier is viewed as the session ID for the SL positioning involving the LMF and therefore no explicit session ID is needed to be included in the SLPP protocol.

Proposal 15: RAN2 to agree that the SLPP session ID should be optionally accommodated (only for the link between the target UE and the location server UE) in the protocol layer blow the SLPP, e.g., the Prose/V2X layer.

Proposal 16: RAN2 to agree that the session management could be transparent to anchor UEs.

Proposal 17: for the UE-only operation, RAN2 to agree that the session ID could be allocated by the UE1 (i.e., target UE) when it requests the SL Positioning server UE to participate in the Ranging/Sidelink positioning.