3GPP TSG-RAN WG2 Meeting #123	R2-2308053

Toulouse, France, August 21st – 25th, 2023

Source: 			OPPO

Title: 	Discussion on the reply LSs to SA2 on SL Positioning

Agenda item:		7.2.1

Document for: 	Discussion and Decision

# 1. Introduction

In R2-2307054 (S2-2305735), SA2 answers the questions asked by RAN2, and further asks two questions to RAN2 and RAN1, as shown below.

1. Overall Description: (omit SA2 answers part.) In addition, SA2 has the following questions regarding UE discovery and selection: Question 1: What are the criteria from RAN2 perspective for the selection of a Located UE and SL Positioning Server UE? And when does the selection take place? With reference to attached S2-2305750, SA2 has the following question for RAN1 and RAN2: Question 2: SA2 discussed whether or not relative velocity in the S2-2305750 would be feasible to specify in Rel-18, and would like to see if RAN2 and RAN1 has any feedback. SA2 would also welcome feedbacks on other results defined in S2-2305750. 2. Actions: To RAN2, RAN1 ACTION: Take SA2 answers into consideration and provide feedbacks to SA2 on the SA2 further questions.

In SA2 LS R2-2307056 (S2-2307553) entitled “LS on assistance information provided to UE”, SA2 has the following questions on assistance information provided to UE.

1. Overall Description: SA2 has defined procedures of SL-MO-LR and SL-MO-LR, which include - UE based Ranging/SL Positioning, for which result calculation is performed at UE - Network based Ranging/SL Positioning, for which result calculation is performed at LMF For UE based Ranging/SL Positioning, SA2 believes that LMF may provide some assistance information to UE to facilitate UE to perform result calculation. SA2 would check with RAN2 about the following questions: Q1: Whether it is needed to provide assistance information to UE for result calculation? Q2: If answer to Q1 is yes, whether the assistance information needs to be exposed to SA2 and is relevant to any SA2 procedure? Q3: If answer to Q2 is yes, what are the assistance information and how SA2 is expected to update the related procedure? 2. Actions: To RAN2 ACTION: Answer the above questions and feedback to SA2.

In this contribution, we would like to further address our views on these two LSs, and the draft reply LSs are attached as Annex.

# 2. Discussion

## 2.1 Discussion on the questions in R2-2307054 (S2-2305735)

For Question 1 regarding the criteria for the selection of a Located UE and SL Positioning Server UE, in the last meeting, RAN2 has an LS R2-2306842 to SA2 which addresses partly this question on anchor UE (i.e., SL Reference UE/Located UE) selection, however, the criteria for SL Positioning Server UE selection are not touched.

As discussed in our contribution R2-2308052 submitted in this meeting, the criteria discussed for use in anchor UE selection is used to evaluate a candidate anchor UE from SL-PRS transmission or measurement perspective, while SL Positioning Server UE needs to take responsibility of location calculation, so there would be additional criteria to be considered from location calculation perspective. In our understanding, when making selection of the qualified SL Positioning server UE, the supported positioning QoS requirement or the supported SL positioning method(s) for a candidate SL Positioning server UE should be considered from location calculation perspective.

- Different from the criteria for anchor UE selection, the criteria for SL Positioning Server UE selection needs to be considered from location calculation perspective, e.g., the supported positioning QoS requirement, or the supported SL positioning method(s) from location calculation perspective.

Besides, regarding when the anchor UE or SL Positioning server UE selection takes place, in our understanding, RAN2 spec would not have strict restrictions on the order of selection procedure in the whole SL positioning procedures. Considering that the selection procedure needs the exchange of information about the candidate anchor UE or SL Positioning server UE, the selection procedure is expected to be performed after SL positioning discovery procedure, and optionally after the SLPP capability exchange procedure.

- The selection procedure is expected to be performed after SL positioning discovery procedure, and optionally after the SLPP capability exchange procedure.

For Question 2 regarding the feasibility of relative velocity and its accuracy, in our understanding, we may rely on RAN1 to answer.

- Relying on RAN1 to answer Question 2 regarding the feasibility of relative velocity and its accuracy.

- RAN2 to consider the draft reply LS attached in Annex 1 as baseline.

## 2.4 Discussion on the questions in R2-2307056 (S2-2307553)

In this LS, SA2 asks Q1/2/3 regarding whether LMF needs to provide some assistance information to UE to facilitate UE to perform location calculation for UE-based Ranging/SL Positioning. In general, we think it is similar to the Uu positioning, LMF may provide assistance data, e.g., SL-PRS configuration, to assist UE to perform SL-PRS measurement, and provide assistance data to assist UE-based location calculation for UE based Ranging/SL Positioning, while this assistance information is defined within SLPP layer, therefore it is not relevant to SA2 and not needed to be exposed to SA2.

- Similar to the Uu positioning, LMF may provide SLPP assistance data to assist UE to perform SL-PRS measurement and/or to assist UE-based location calculation.

- SLPP assistance data is defined within SLPP layer, therefore it is not relevant to SA2 and not needed to be exposed to SA2.

- RAN2 to consider the draft reply LS attached in Annex 2 as baseline.

# 3. Conclusion

Based on the discussion above, we made the following observations:

- Different from the criteria for anchor UE selection, the criteria for SL Positioning Server UE selection needs to be considered from location calculation perspective, e.g., the supported positioning QoS requirement, or the supported SL positioning method(s) from location calculation perspective.

- The selection procedure is expected to be performed after SL positioning discovery procedure, and optionally after the SLPP capability exchange procedure.

- Relying on RAN1 to answer Question 2 regarding the feasibility of relative velocity and its accuracy.

- Similar to the Uu positioning, LMF may provide SLPP assistance data to assist UE to perform SL-PRS measurement and/or to assist UE-based location calculation.

- SLPP assistance data is defined within SLPP layer, therefore it is not relevant to SA2 and not needed to be exposed to SA2.

Based on the discussion above, we give the following proposals:

- RAN2 to consider the draft reply LS attached in Annex 1 as baseline.

- RAN2 to consider the draft reply LS attached in Annex 2 as baseline.

# 4. Reference

[1] R2-2307054, Reply LS to LS to SA2 on Sidelink positioning procedure (S2-2305735; contact: Xiaomi), SA2

[2] R2-2307056, LS on assistance information provided to UE (S2-2307553; contact: Xiaomi), SA2

[3] R2-2308052, Further discussion on sidelink positioning, OPPO

# Annex 1: Draft Reply LS on Reply LS to LS to SA2 on Sidelink positioning procedure

3GPP TSG-RAN WG2 Meeting #123	R2-23xxxxx

Toulouse, France, August 21st – 25th, 2023

Title:	[draft] Reply LS on Reply LS to LS to SA2 on Sidelink positioning procedure

Response to:	R2-2307054_S2-2305735

Release:	Rel-18

Work Items:	NR_pos_enh2, Ranging_SL

Source:	OPPO (to be RAN2)

To:	SA2

CC:	RAN1

Contact Person:

Name:	Yang Liu

E-mail Address:	liuyangbj@oppo.com

1. Overall Description:

RAN2 would like to thank SA2 for the Reply LS (R2-2307054_S2-2305735) on sidelink positioning procedure.  Regarding the further questions in the Reply LS, RAN2 would like to ask SA2 to take the following feedback into consideration:

Question 1: What are the criteria from RAN2 perspective for the selection of a Located UE and SL Positioning Server UE? And when does the selection take place?

RAN2 Answer:

From RAN2 perspective, SL Positioning Server UE needs to take responsibility of location calculation, there would be criteria to be considered from location calculation perspective when making selection of the qualified SL Positioning server UE, e.g.,

- the supported positioning QoS requirement or the supported SL positioning method(s) for a candidate SL Positioning server UE from location calculation perspective.

Besides, regarding when the anchor UE or SL Positioning server UE selection takes place, RAN2 spec would not have strict restrictions on the order of selection procedure in the whole SL positioning procedures. Considering that the selection procedure needs the exchange of information about the candidate anchor UE or SL Positioning server UE, the selection procedure is expected to be performed after SL positioning discovery procedure, and optionally after the SLPP capability exchange procedure.

Question 2: SA2 discussed whether or not relative velocity in the S2-2305750 would be feasible to specify in Rel-18, and would like to see if RAN2 and RAN1 has any feedback. SA2 would also welcome feedbacks on other results defined in S2-2305750.

RAN2 Answer: RAN2 relies on RAN1 to answer Question 2 regarding the feasibility of relative velocity and its accuracy.

2. Actions:

To SA2:

ACTION: RAN2 kindly ask SA2 to take above information into consideration.

3. Date of Next RAN WG2 Meetings:

TSG-RAN2 Meeting #123	21-25 August 2023		Toulouse, FR

TSG-RAN2 Meeting #123bis	9-13 October 2023		Xiamen, China

# Annex 2: Draft Reply LS on LS on assistance information provided to UE

3GPP TSG-RAN WG2 Meeting #123	R2-23xxxxx

Toulouse, France, August 21st – 25th, 2023

Title:	[draft] Reply LS on LS on assistance information provided to UE

Response to:	R2-2307056_S2-2307553

Release:	Rel-18

Work Items:	NR_pos_enh2, Ranging_SL

Source:	OPPO (to be RAN2)

To:	SA2

CC:

Contact Person:

Name:	Yang Liu

E-mail Address:	liuyangbj@oppo.com

1. Overall Description:

RAN2 would like to thank SA2 for the LS (R2-2307056_S2-2307553) on assistance information provided to UE from LMF. Regarding the questions in the LS, RAN2 would like to ask SA2 to take the following feedbacks into consideration:

For UE based Ranging/SL Positioning, SA2 believes that LMF may provide some assistance information to UE to facilitate UE to perform result calculation. SA2 would check with RAN2 about the following questions: Q1: Whether it is needed to provide assistance information to UE for result calculation? RAN2 Answer: From RAN2 perspective, similar to the Uu positioning, LMF may provide SLPP assistance data to assist UE to perform SL-PRS measurement and/or to assist UE-based location calculation. Q2: If answer to Q1 is yes, whether the assistance information needs to be exposed to SA2 and is relevant to any SA2 procedure? RAN2 Answer: SLPP assistance data is defined within SLPP layer, therefore from RAN2 perspective it is not relevant to SA2 and not needed to be exposed to SA2. Q3: If answer to Q2 is yes, what are the assistance information and how SA2 is expected to update the related procedure? RAN2 Answer: Not needed to answer.

2. Actions:

To SA2:

ACTION: RAN2 kindly ask SA2 to take above information into consideration.

3. Date of Next RAN WG2 Meetings:

TSG-RAN2 Meeting #123	21-25 August 2023		Toulouse, FR

TSG-RAN2 Meeting #123bis	9-13 October 2023		Xiamen, China