# 3GPP TSG-RAN2 Meeting #123	R2-2308064

# Toulouse, France, August 21-25, 2023

# Source: 			ZTE Corporation, Sanechips

# Title: 	Consideration on DC location reporting

# Agenda item:	6.1.3.1

# Document for: 	Discussion and Decision

- Introduction

RAN2 receives a new LS from RAN4 on DC location signalling enhancement. The content of the LS is as follows [1]:

Unlike the Rel-15 and Rel-16 DC reporting mechanisms where UE would directly signal its UL DC location to the network, the Rel-17 method is composed of a default UL DC location in conjunction with a frequency offset relative to the default DC location chosen by the UE which can be communicated to the network. The default DC location is determined by the UE reported configurations which can be based on “configured carriers”, “configured BWPs”, “active carrier(s)”, or “active BWP(s)”. The default DC location based on configured carriers/BWPs or active carrier(s) provides the benefits for simpler UE implementation and less signaling complexity/overhead, while it compromises the UL power saving and maximum output power performance, especially when only PCC is activated. On the other hand, when default DC location is based on activated BWP(s), it provides the benefits of power saving and better maximum output power performance (potentially lower MPR/A-MPR), especially when only PCC is activated. However, it also has the disadvantage of higher UE implementation complexity and signaling complexity/overhead as it potentially requires more configuration-dependent non-zero offset values, especially when the number of configured carriers increases. From the above assessment, the benefits for having default DC location based on either “configured carriers”, “configured BWPs”, or “active carrier(s)” when both PCC and SCC are activated and based on “active BWP” when only PCC is activated can be envisioned. However, the mixed configurations for default DC determination is currently not supported in Rel-17 DC location signaling design. RAN4 kindly asks RAN2 a way to leverage the benefits of DC location signaling for single UL CC in UL CA configuration when only PCC UL is activated in a complementary way. The possible usage is that when the capability is indicated, UE is allowed to either explicitly signal its DC location associated with each activated BWP, same as the Rel-15 DC reporting for single CC UL, or indicate default DC UE bandwidth based on activated BWP with the corresponding offsets. In that case, when only PCC UL is activated, the UE DC would either follow the explicitly signaled locations or activated BWP default DC and offset, while when more than one CCs are activated, the DC is based on the default DC location and the indicated frequency offset. It is noted that RAN4 couldn’t reach a consensus on from which release, either Rel-17 or Rel-18, a new scheme is applicable if introduced.

In this contribution, we’d like to share our views on RAN4’s request.

- Discussion

In the Rel-17 DC location reporting framework, the DC location is reported to network in the format of default UL DC location and frequency offset. And for default DC location derivation, the UE needs to indicate to network the choice of frequency component among ‘activated CC’, ‘configured CC’, ‘activated BWP’ and ‘configured BWP’.

#UplinkTxDirectCurrentMoreCarrierList-r17

UplinkTxDirectCurrentMoreCarrierList-r17 ::= SEQUENCE (SIZE (1..maxNrofCC-Group-r17)) OF CC-Group-r17

CC-Group-r17 ::= SEQUENCE {

servCellIndexLower-r17  ServCellIndex,

servCellIndexHigher-r17 ServCellIndex OPTIONAL,

defaultDC-Location-r17 DefaultDC-Location-r17,

offsetToDefault-r17 CHOICE{

offsetValue OffsetValue-r17,

offsetlist SEQUENCE (SIZE(1..maxNrofReqComDC-Location-r17)) OF OffsetValue-r17

} OPTIONAL

}

OffsetValue-r17::= SEQUENCE {

offsetValue-r17 INTEGER (-20000.. 20000),

shift7dot5kHz-r17 BOOLEAN

}

DefaultDC-Location-r17 ::= CHOICE {

ul FrequencyComponent-r17,

dl FrequencyComponent-r17,

ulAndDL FrequencyComponent-r17

}

FrequencyComponent-r17 ::= ENUMERATED {activeCarrier,configuredCarrier,activeBWP,configuredBWP}

According to the LS, the advantages, disadvantages and applicable scenario of the four frequency components are listed as below:

| Frequency component | Advantages | Disadvantages | Applicable scenario |
| --- | --- | --- | --- |
| ‘Configured carrier’; ‘Configured BWP’; ‘Active carrier’ | Simpler UE implementation; Less signalling complexity/overhead | Increased UL power consumption; Degraded maximum output power performance | When both PCC and SCC are activated |
| ‘Active BWP’ | Power saving; Better maximum output power performance | Higher UE implementation complexity; Higher signalling complexity/overhead | When only PCC is activated |

In our understanding, RAN4 expects the UE to directly report its DC location similar to Rel-15 or to indicate default DC location and offset based on the frequency component ‘active BWP’ when only PCC is activated, and expects the UE to signal the default DC location and offset based on the frequency component ‘configured carrier’, ‘configured BWP’ or ‘active carrier’ when both PCC and SCC are activated. However, in the current specification, for different CC activation combinations within one CC group, UE can only calculate default DC location based on the same frequency component.

Observation 1: According to the LS, the UE is expected by RAN4 to directly report its DC location or to indicate default DC location and offset based on the frequency component ‘active BWP’ when only PCC is activated.

Observation 2: In current specification, the frequency component for default DC location derivation is the same in different CC activation combinations within one CC group.

After reviewing the discussions for DC location reporting in Rel-17, we found the issue that whether the UE is allowed to calculate default DC location for different CC combinations based on different frequency components in a CC group has been discussed in RAN2#119-e meeting, and it has been agreed that it is not allowed. The corresponding agreement is as follows:

RAN2’s understanding is that UE can only calculate default DC location for different CC/BWP combinations in a CC group based on the same frequency component. However this restriction can be confirmed by RAN4 finally.

Observation 3: RAN2 has agreed in Rel-17 that UE can only calculate default DC location for different CC/BWP combinations in a CC group based on the same frequency component.

Then RAN2 sent the LS to RAN4 to confirm this restriction, and the feedback from RAN4 is positive.

R2-2209002 LS to RAN4 on DC location RAN2 LS out Rel-17 Question 1: RAN4 has concluded that frequency component type is same for both default DC locations for different CC groups. RAN2 also understands that the frequency component type is same for default DC locations for a given CC group independing on the different activated CC/BWP combination in a CC group RAN2 asks RAN4 to confirm our understanding. R4-2220814 LS on DC location reporting LS in Rel-17 Answer 1: RAN4 confirms that activation state of CC/BWP within a CC group will not impact the frequency component type.

Observation 4: In Rel-17, RAN4 has confirmed that the activation state of CC within a CC group would not impact the frequency component type.

In our understanding, the current Rel-17 DC location reporting scheme is feasible, and the request from RAN4 is only for signalling optimization. Considering that we have not seen enough benefits of redesigning signalling and the magnitude of the work is not small, we suggest to maintain the current signalling and send LS to inform RAN4 the conclusion.

Observation 5: The request from RAN4 is only for optimization, not for correction.

Proposal 1: We suggest to maintain the current signalling and send LS to inform RAN4 the conclusion.

However, we are also fine if majority companies prefer to follow RAN’4 request. Based on the LS, RAN4 could not reached a consensus that the new scheme is applicable from which release (Rel-17 or Rel-18). In our view, it is not a good idea to modify the Rel-17 signalling considering that the Rel-17 has been frozen. The enhancement on DC location reporting shall be considered from Rel-18.

Proposal 2: If majority companies prefer to follow RAN4’s request, the enhancement on DC location reporting shall be considered from Rel-18.

As mentioned in LS, the existing Rel-15 signalling is allowed to indicate the DC location directly. While, the Rel-15 mechanism is not suitable for intra-band CA scenario because the UE may shift DC location following with CC activation. Considering the benefits of Rel-15 and Rel-17, it is fine to allow the UE to use Rel-15 and Rel-17 DC location reporting scheme simultaneously. Specifically, for the case that only PCC is activated, Rel-15 DC location reporting scheme is used; for the case that both PCC and SCC are activated, Rel-17 DC location reporting scheme is used. Compared with redesigning new Rel-18 signalling, the combination scheme is simpler and has less impact on the specification.

Proposal 3: With the assumption of following RAN4’s request, allow the UE to report DC location information to network via Rel-15 and Rel-17 reporting signallings simultaneously.

According to the existing specification, which reporting DC location scheme would be used (Rel-15 or Rel-16 or Rel-17) is configured by network via different network indications, as follows:

CellGroupConfig ::= SEQUENCE {

****skip non-related part***

reportUplinkTxDirectCurrent ENUMERATED {true} OPTIONAL -- Cond BWP-Reconfig

reportUplinkTxDirectCurrentTwoCarrier-r16 ENUMERATED {true} OPTIONAL -- Need N

reportUplinkTxDirectCurrentMoreCarrier-r17 ReportUplinkTxDirectCurrentMoreCarrier-r17 OPTIONAL -- Need N

****skip non-related part***

}

In order to allow the UE to use Rel-15 and Rel-17 reporting scheme at the same time, two network indications need to be configured at the same time. Because if only Rel-17 network indication is configured, the UE is unclear that whether the network can handle the two reporting scheme simultaneously, which may lead to signalling wasted in some cases. In addition, considering that the size of Rel-15 network indication is only 1 bit, the signalling overhead for the two network indications is acceptable.

Proposal 4: With the assumption of following RAN4’s request, allow the Rel-15 and Rel-17 network indications to be configured at the same time. In this case, the reported Rel-15 signalling is applicable when only PCC is activated.

Finally, in order to indicate the network whether the UE supports DC location reporting enhancement or not, a new UE capability is needed.

Proposal 5: With the assumption of following RAN4’s request, a new UE capability is needed to indicate the network whether the UE supports DC location reporting enhancement or not.

- Conclusion and proposals

Observation 1: According to the LS, the UE is expected by RAN4 to directly report its DC location or to indicate default DC location and offset based on the frequency component ‘active BWP’ when only PCC is activated.

Observation 2: In current specification, the frequency component for default DC location derivation is the same in different CC activation combinations within one CC group.

Observation 3: RAN2 has agreed in Rel-17 that UE can only calculate default DC location for different CC/BWP combinations in a CC group based on the same frequency component.

Observation 4: In Rel-17, RAN4 has confirmed that the activation state of CC within a CC group would not impact the frequency component type.

Observation 5: The request from RAN4 is only for optimization, not for correction.

Proposal 1: We suggest to maintain the current signalling and send LS to inform RAN4 the conclusion.

Proposal 2: If majority companies prefer to follow RAN4’s request, the enhancement on DC location reporting shall be considered from Rel-18.

Proposal 3: With the assumption of following RAN4’s request, allow the UE to report DC location information to network via Rel-15 and Rel-17 reporting signallings simultaneously.

Proposal 4: With the assumption of following RAN4’s request, allow the Rel-15 and Rel-17 network indications to be configured at the same time. In this case, the reported Rel-15 signalling is applicable when only PCC is activated.

Proposal 5: With the assumption of following RAN4’s request, a new UE capability is needed to indicate the network whether the UE supports DC location reporting enhancement or not.

- Reference

- R2-2307046  LS on Rel-17 DC location signaling enhancement (R4-2310438; contact: Apple) Rel-17 NR_RF_FR2_req_enh2-Core From RAN4  To RAN2