3GPP TSG-RAN WG2 Meeting #123	R2-2308068

Toulouse, France, 21th – 25th Aug, 2023

Agenda item:	7.21.2

Source:	ZTE Corporation, Sanechips

Title:	Remaining CP issues for CE

Document for:	Discussion

- Introduction

In this contribution, we discuss the remaining CP issues on PRACH coverage enhancements and provide our views.

- Discussion

  - 2.1 Msg1-based SI request

For Msg1-based SI request, the following agreements were made in the last RAN2 meeting.

Agreements - MSG1 repetition can be applicable to the 4-step CBRA procedure initiated by Msg3-based SI request - FFS for MSG1 repetition can be applicable to the 4-step CBRA procedure initiated by Msg1-based SI request.

Based on RAN2 Post email discussion [1], companies hold different views on whether to support Msg1 repetition for Msg1-based SI request.

In our understanding, supporting Msg1 repetition for Msg1-based SI request is not very necessary. Firstly, it makes the configuration more complicated and occupies more RACH resources. Note that Msg1-based SI request is different from CFRA cases, since the UE is in idle or inactive state and the UE’s position is unknown to the network, the network may need to configure RACH resources to associate with multiple repetition numbers, similar to the initial random access. If multiple SIs are configured as On-demand SI, then the network has to reserve many RACH resources. Secondly, if a UE in good coverage triggers Msg1-based SI request at the same time, a UE in poor coverage can also acquire the requested SI message(s). So, we prefer not to support Msg1 repetition for Msg1-based SI request, and no RRC and MAC spec modification is needed.

Proposal 1: Msg1 repetition for Msg1-based SI request is not supported.

If companies want to support Msg1 repetition for Msg1-based SI request to improve the success rate of SI request for UEs in poor coverage, we suggest that for a specific scenario (e.g. NUL/SUL/RedCap), the different repetition numbers are associated with different preambles on the same RO, there is no need to configure separate ROs for different repetition numbers.

Proposal 2: If Msg1 repetition for Msg1-based SI request is supported, for a specific scenario (NUL/SUL/RedCap), different repetition numbers are associated with different preambles, not associated with separate ROs.

In addition, if Msg1 repetition for Msg1-based SI request is supported, one open issue discussed in [1] is the UE behavior when Msg1 repetition for Msg1-based SI request is not configured, for example, whether the UE in poor coverage can directly perform Msg3-based SI request with Msg1 repetition, if configured. In our understanding, this kind of configuration is valid, because the RACH resource pool configured for Msg3-based SI request is shared with initial access, from network perspective, it is possible the network enables Msg1 repetition for initial access, but without enabling Msg1 repetition for Msg1-based SI request (to save RACH resources). In this case, the UE should perform Msg1-based SI request without repetition instead of Msg3-based SI request.

If company wants the UE to directly trigger Msg3-based SI request with Msg1 repetition, this will have big impacts on RRC and MAC specs, because, even if in RRC spec, we can update the text procedure to ask the UE to first evaluate DL RSRP and prioritize “Msg3-based with repetition”, in MAC spec, since Msg3-based SI request is considered as a normal RACH case, the UE still needs to determine the applicable features and select RACH partition accordingly. So, when other feature has higher priority than Msg1 repetition feature, it is possible the UE will select a RACH partition which not associated with Msg1 repetition, for which the performance is same as triggering Msg1-based SI request. If we want to avoid this, we need to consider special treatment for Msg3-based SI request in MAC spec, this requires additional specification efforts.

So, we think if Msg1 repetition for Msg1-based SI request is supported, then lack of Msg1 repetition resources does not mean the UE can select Msg3-based SI request with Msg1/3 repetitions. The existing mechanism should be followed and switching from Msg1-based SI request to Msg3-based SI request should be disallowed.

Proposal 3: If Msg1 repetition for Msg1-based SI request is supported, in case the network does not provide RACH resources for Msg1-based SI request with repetition, but only provide RACH resources for Msg1-based SI request without repetition, the UE should perform Msg1-based SI request without repetition (same as legacy).

  - 2.2 UE capability

RAN1 will discuss the UE feature in this August meeting, and UE capability for Msg1 repetition may be discussed. But as we know, RAN2 discussed whether and how to define the Msg3 repetition capability in release 17. Therefore, from RAN2 perspective, we can also discuss Msg1 repetition related capability, which involves the following issues.

- Issue1: Whether to introduce UE capability for Msg1 repetition?

Similar to Msg3 repetition, we think reporting UE capability for Msg1 repetition is needed for SON/MDT purpose. The network can monitor the performance of Msg1 repetition and further optimize RACH configurations based on the information of collected UE capabilities. In addition, the UE capability can also be used in configuring the RACH resources in dedicated BWP.

Proposal 4: From RAN2 perspective, UE capability for Msg1 repetition is needed.

- Issue2: Whether to introduce separate UE capabilities for different repetition numbers?

From RAN2 perspective, we think it is unnecessary to introduce separate capabilities for different repetition numbers. Otherwise, it will cause market fragmentation and bring complexity to commercial configuration. For example, based on the cell coverage, the network may want to configure a RACH partition for “RedCap +Msg1 repetition 4”, but if some RedCap UEs only support repetition number 2 and some RedCap UEs only support repetition number 4 or 8, then the network will be forced to configure multiple RACH partitions for covering different UE types, e.g. “RedCap + Msg1 repetition 2”, “RedCap +Msg1 repetition 4” and “RedCap + Msg1 repetition 8”. This is complex and waste of RACH resources.

So, we haven’t seen clear motivation for introducing separate capabilities for different repetition numbers. For simplicity, we suggest to only introduce a single capability, and the presence of the capability indicates the UE supporting Msg1 repetition with repetition numbers {2, 4, 8}.

Proposal 5: From RAN2 perspective, there is no need to define separate UE capabilities for indicating the support of different Msg1 repetition numbers.

- Issue3: The relation between Msg1 repetition capability and Msg3 repetition capability.

Regarding the relation of Msg1 repetition capability and Msg3 repetition capability, there are the following two understandings.

- Understanding 1: Msg1 repetition and msg3 repetition are independent features. A UE supporting Msg1 repetition may not support Msg3 repetition, vice versa.

- Understanding 2: A UE supporting Msg1 repetition shall also support Msg3 repetition.

Technically, although Msg1 repetition and Msg3 repetition are designed for the same purpose (i.e. UL coverage enhancement), we don’t see the need to bind them together, it will also increase the UE implementation complexity, as the UE needs to pass the test for both functions in order to declare the support of Msg 1 repetition.

On the other hand, the network should have the flexibility to enable only one function (not both), so we suggest to define independent capabilities for Msg1 repetition and Msg3 repetition.

Proposal 6: From RAN2 perspective, UE capabilities for Msg1 repetition and Msg3 repetition are defined independently.

- Conclusion

Based on the above discussion, we propose the following observations and proposals:

Proposal 1: Msg1 repetition for Msg1-based SI request is not supported.

Proposal 2: If Msg1 repetition for Msg1-based SI request is supported, for a specific scenario (NUL/SUL/RedCap), different repetition numbers are associated with different preambles, not associated with separate ROs.

Proposal 3: If Msg1 repetition for Msg1-based SI request is supported, in case the network does not provide RACH resources for Msg1-based SI request with repetition, but only provide RACH resources for Msg1-based SI request without repetition, the UE should perform Msg1-based SI request without repetition (same as legacy).

Proposal 4: From RAN2 perspective, UE capability for Msg1 repetition is needed.

Proposal 5: From RAN2 perspective, there is no need to define separate UE capabilities for indicating the support of different Msg1 repetition numbers.

Proposal 6: From RAN2 perspective, UE capabilities for Msg1 repetition and Msg3 repetition are defined independently.

- Reference

- R2-2308663   Summary of [Post122][801][R18CEenh-CP] CP open issues    Huawei