3GPP TSG-RAN WG2 Meeting #123                                  R2-2308070

Toulouse, France, August 21-25, 2023

Source: 			ZTE Corporation, Sanechips

Title: 	Discussion on ATG

Agenda item:		7.25.1

Document for: 	Discussion and decision

- Introduction

RAN4 replied an LS to RAN2 on the applicability of SIB19 in the last meeting [1]. In the LS, RAN4 feedbacks the reference location accuracy issue and indicates to RAN2 that the mechanism of koffset in ATG system would be introduced. Besides, a new LS from RAN4 has been received, which requests RAN2 to introduce new signaling to indicate whether set1 or set2 requirements apply for a particular cell [2]. In this contribution, we’d like to share our views on the ATG based on the LS.

- Discussion

Air-to-ground (ATG) network is an in-flight connectivity technique, which establishes a radio link between the cell towers on the ground and the aircraft’s antenna(s) of ATG terminal, as shown in Fig.1. And the passengers in the aircraft can connect to ATG terminal. Considering the network deployment cost and limited ATG terminal quantity, a large cell size is required in ATG network. Based on TR 38.876, the ATG network shall be able to provide up to 300km cell coverage.

Fig.1 ATG network

In reality, the speed of the aircraft could be up to 1200km/h, and the normal commercial airplane altitude is about 10~12km. Considering the large transmission delay and serious Doppler effect, the time and frequency compensation are required in the ATG network, similar to NTN. In this way, ATG UE needs ATG BS location information to perform time and frequency pre-compensation. Due to large RTT, in order to avoid the out-of-order in the existing scheduling timeline, k-offset mechanism is introduced. In addition, RAN4 has agreed to define two sets of cell reselection requirements for ATG UE in RRC IDLE and INACTIVE states, thus, a flag which is used to indicate which requirement set would be used is needed.

Based on [3], RAN4 suggested us to reuse IE EphemerisInfo in SIB19 to broadcast location information, while other options are not precluded. In the next subject, we would discuss the feasibility of reusing EphemerisInfo in SIB19 first

## 2.1 The analysis of the solution reusing IE EphemerisInfo in SIB19

For reusing IE EphemerisInfo in SIB19, we need to first clarify whether the gNB would broadcast the ATG information and NTN information simultaneously. The SIB19 in NTN is broadcast by satellite, while the SIB19 in ATG network is broadcast by the ATG BS on the ground. Thus, it is impossible for a gNB to broadcast ATG information and NTN information simultaneously.

In the current specification, the IE EphemerisInfo in SIB19 is as follows:

SIB19-r17 ::= SEQUENCE {

ntn-Config-r17                     NTN-Config-r17                               OPTIONAL, -- Need R

t-Service-r17                      INTEGER (0..549755813887)                   OPTIONAL, -- Need R

referenceLocation-r17             ReferenceLocation-r17                       OPTIONAL, -- Need R

distanceThresh-r17                INTEGER(0..65525)                             OPTIONAL, -- Need R

ntn-NeighCellConfigList-r17      NTN-NeighCellConfigList-r17                OPTIONAL, -- Need R

lateNonCriticalExtension         OCTET STRING                                   OPTIONAL,

...

[[

ntn-NeighCellConfigListExt-v1720 NTN-NeighCellConfigList-r17 OPTIONAL -- Need R

]]

}

NTN-Config-r17 ::=             SEQUENCE {

epochTime-r17                    EpochTime-r17                                 OPTIONAL,  -- Need R

ntn-UlSyncValidityDuration-r17 ENUMERATED{ s5, s10, s15, s20, s25, s30, s35,

s40, s45, s50, s55, s60, s120, s180, s240, s900}     OPTIONAL,  -- Cond SIB19

cellSpecificKoffset-r17        INTEGER(1..1023)                             OPTIONAL,  -- Need R

kmac-r17                          INTEGER(1..512)                              OPTIONAL,  -- Need R

ta-Info-r17                      TA-Info-r17                                   OPTIONAL,  -- Need R

ntn-PolarizationDL-r17         ENUMERATED {rhcp,lhcp,linear}             OPTIONAL,  -- Need R

ntn-PolarizationUL-r17         ENUMERATED {rhcp,lhcp,linear}             OPTIONAL,  -- Need R

ephemerisInfo-r17               EphemerisInfo-r17                            OPTIONAL,  -- Need R

ta-Report-r17                    ENUMERATED {enabled}                        OPTIONAL,  -- Need R

...

}

EphemerisInfo-r17 ::=          CHOICE {

positionVelocity-r17           PositionVelocity-r17,

orbital-r17                    Orbital-r17

}

PositionVelocity-r17 ::=       SEQUENCE {

positionX-r17                  PositionStateVector-r17,

positionY-r17                  PositionStateVector-r17,

positionZ-r17                  PositionStateVector-r17,

velocityVX-r17                 VelocityStateVector-r17,

velocityVY-r17                 VelocityStateVector-r17,

velocityVZ-r17                 VelocityStateVector-r17

}

PositionStateVector-r17 ::= INTEGER (-33554432..33554431)

It is noted that IE EphmerisInfo is used to provide satellite ephemris information in the NTN, and the location information is given in the format of position and velocity state vector in ECEF (Earth-Centerred, Earth-Fixed) or in format of orbital parameters in ECI (Earth-Centered Inertial). For ATG, considering the ATG BS is fixed, it is more suitable to apply ECEF and three velocity state vectors in PositionVelocity-r17 need to be set to zero. However, this would increase unnecessary signaling overheads, because the velocity state vectors are mandatory.

In the current specification, the UE is required to maintain the SIB19 validity upon SIB19 acquisition, as follows:

##### 5.2.2.4.21 Actions upon reception of SIB19 Upon receiving SIB19, the UE in RRC_CONNECTED shall: 1> start or restart T430 for serving cell with the timer value set to ntn-UlSyncValidityDuration for the serving cell from the subframe indicated by epochTime for the serving cell; NOTE: UE should attempt to re-acquire SIB19 before the end of the duration indicated by ntn-UlSyncValidityDuration and epochTime by UE implementation.

In the current SIB19, considering that the serving satellite moves with respect to UE on Earth, satellite assistance information is only valid in the certain duration. However, for ATG BS location information, considering ATG BS is fixed, there is no need to maintain UL synchronization or Timer 340 to update location information. For k_offset in the SIB19, k_offset is cell-specific and no need to be updated either. For requirement set flag, in our view, the flag would not change frequently and we don’t need to use timer to maintain the validity of the flag. Thus, if SIB19 is reused, ntn-UlSyncValidityDuration is meaningless, but it is also mandatory in the SIB19, as follows:

| Conditional Presence | Explanation |
| --- | --- |
| SIB19 | The field is mandatory present for the serving cell in SIB19. The field is optionally present, Need R, otherwise. |

Besides, in our view, it is a little weird to use NTN-Config to indicate the ATG information. Thus, we suggest not to reuse IE EphemerisInfo-r17 and cellSpecificKoffset-r17 in SIB19 to indicate the ATG information.

Observation 1: EphemerisInfo-r17 and cellSpecificKoffset-r17 in SIB19 can be reused to indicate ATG-related information with the cost of increased signaling overheads and disabled SIB19 validity mechanism.

Proposal 1: Not to reuse IE EphemerisInfo-r17 and cellSpecificKoffset-r17 in SIB19 to indicate ATG-related information.

## 2.2 Other solutions

In our view, it is fine to introduce a new IE ATG-Config to carry ATG-related information. For location information, there are the following two options:

Option 1: Use position vectors in ECEF to indicate the location information of ATG BS;

Option 2: Use EllipsoidPoint to indicate the location information of ATG BS.

Ellipsoid-Point describes a point on the surface of the ellipsoid in format of longitude and latitude, and could be used to refer to a point on Earth's surface or close to Earth's surface [4]. However, the altitude of the point can not be provided. For ECEF (Earth-Centerred, Earth-Fixed), it is a three-dimensional coordinate, and the altitude information could be given. In our understanding, the vertical distance between ATG BS and ATG UE is important. Because the vertical distance would be 10~12km, if unclear of the exact vertical distance, the accuracy of BS location would be very low. Thus, option 1 is preferred.

Proposal 2: Introduce a new IE ATG-Config to carry ATG-related information.

Proposal 3: Use position vectors in the ECEF to indicate the location information of ATG BS.

For k_offset, existing cellSpecificKoffset-r17 for NTN could be taken as start point. However, the RTT of NTN is much larger than ATG. Thus, the upper boundary of k_offset shall be much smaller than NTN. The exact value of k_offset upper boundary shall be up to RAN4.

For the requirement set flag, we can take highSpeedMeasFlagFR2-r17 for reference, as follows:

highSpeedMeasFlagFR2-r17 ENUMERATED {set1, set2} OPTIONAL, -- Need R

In summary, the corresponding signaling is as follows:

ATG-Config-r18 ::=             SEQUENCE {

position-r18 ::=     SEQUENCE {

positionX-r18                  PositionStateVector-r17,

positionY-r18                  PositionStateVector-r17,

positionZ-r18                  PositionStateVector-r17,

}

cellSpecificKoffset-r18        INTEGER(1..FFS)                              OPTIONAL,  -- Need R

aTGRequirementFlag-r18          ENUMERATED {set1, set2}                     OPTIONAL,  --Need R

...

}

Proposal 4: For k_offset and requirement set flag in ATG, existing cellSpecificKoffset-r17 and highSpeedMeasFlagFR2-r17 can be taken as start point, but the upper boundary of k_offset shall be up to RAN4.

For the location of ATG-Config, we can reuse SIB19 or introduce a new SIB. In our view, there is no need to introduce a new SIB to broadcast a few parameters (ATG BS location information and koffset). Even through other ATG specific parameters would be introduced in the future, SIB19 can also carry them without the impact on NTN specific parameters. Thus, we prefer to reuse SIB19 to carry ATG-Config.

Proposal 5: Reuse SIB19 to carry ATG-Config.

## 2.3 Open issues

For the introduction of ATG, some general issues need to be clarified.

- Issue 1: Whether one Cell can serve for ATG UE and normal UE at the same time.

For ATG Cell, the direction for the beam is upward as shown in Fig.1, and ATG Cell only can serve for the UE located at several kilometers above the ground. Thus it is impossible for normal UE to detect the beams from ATG Cell. Also for normal Cell, the normal Cell is on the ground, and only serves for the UE on the ground. While, ATG UE only works in the air. Thus, it is impossible for ATG UE to detect the beams from normal Cell.

Observation 2: Normal UEs can not detect the beams from ATG Cell, and ATG UEs can not detect the beams from normal Cell.

- Issue 2: How does the ATG UE determine whether a Cell is ATG capable and whether the ATG Cell is allowed to access?

With the conclusion of issue 1, we think the cell is considered to be ATG capable if ATG UE in the air detects a cell on the ATG bands. And in order to indicate whether the ATG Cell is allowed to access, legacy IE CellBarred in MIB can be reused.

Proposal 6: If ATG UE in the air detects a cell on the ATG bands, the Cell is considered to be ATG capable.

Proposal 7: Reuse CellBarred in MIB to indicate to the ATG UE whether the ATG Cell is allowed to access or not.

- Issue 3: Whether to broadcast the location information and k_offset of the neighbor ATG Cells

During the RRC_Idle/Inactive state, the UE needs to perform measurement on the neighbour cells for cell selection and re-selection. However, due to the ATG Cell size is very large, a lot of neighbour cells exists and their information (e.g. frequency, SMTC) would be broadcast in the SIB 3~5. With the location information of the neighbor ATG Cells, the UE can know to perform measurement on which Cells according to its flight path, which avoids the unnecessary measurement on some far cells. Besides, the actual STMC of the neighbor cell may have offset compared with its broadcast value due to large transmission delay. The UE needs the location information of the neighbor ATG cells to adjust the SMTC configuration.

For k_offset, the first place to use this IE is in the transmission of Msg3. So we think the ATG BS doesn’t need to broadcast k_offset of the neighbor ATG Cells. The UE can receive SIB19 to obtain the k_offset upon camping the ATG Cell.

Proposal 8: ATG BS needs to broadcast the location information of the neighbor ATG Cells, but doesn’t need to broadcast k_offset of the neighbor ATG Cells.

- Conclusion and proposals

Observation 1: EphemerisInfo-r17 and cellSpecificKoffset-r17 in SIB19 can be reused to indicate ATG-related information with the cost of increased signaling overheads and disabled SIB19 validity mechanism.

Observation 2: Normal UEs can not detect the beams from ATG Cell, and ATG UEs can not detect the beams from normal Cell.

Proposal 1: Not to reuse IE EphemerisInfo-r17 and cellSpecificKoffset-r17 in SIB19 to indicate ATG-related information.

Proposal 2: Introduce a new IE ATG-Config to carry ATG-related information.

Proposal 3: Use position vectors in the ECEF to indicate the location information of ATG BS.

Proposal 4: For k_offset and requirement set flag in ATG, existing cellSpecificKoffset-r17 and highSpeedMeasFlagFR2-r17 can be taken as start point, but the upper boundary of k_offset shall be up to RAN4.

Proposal 5: Reuse SIB19 to carry ATG-Config.

Proposal 6: If ATG UE in the air detects a cell on the ATG bands, the Cell is considered to be ATG capable.

Proposal 7: Reuse CellBarred in MIB to indicate to the ATG UE whether the ATG Cell is allowed to access or not.

Proposal 8: ATG BS needs to broadcast the location information of the neighbor ATG Cells, but doesn’t need to broadcast k_offset of the neighbor ATG Cells.

- Reference

- R2-2307038  Reply LS on applicability of SIB19 for NR ATG (R4-2310058; contact: Qualcomm)  Rel-18 NR_ATG-Core From RAN4  To RAN2

- R2-2307040   LS on signaling for NR ATG (R4-2310152; contact: Ericsson)  Rel-18 NR_ATG-Core From RAN4 To RAN2

- R2-2302348  LS on applicability of SIB19 for NR ATG (R4-2303684; contact: Qualcomm)  Rel-18 NR_ATG-Core From RAN4 To RAN2

- TS 23.032    Universal Geographical Area Description (GAD)