3GPP TSG-RAN2 Meeting #123	R2-2308072

Toulouse, France, August 21-25, 2023

Source: 			ZTE Corporation, Sanechips

Title: 	Discussion on FR2 unknown SCell activation

Agenda item:	7.25.1

Document for: 	Discussion and Decision

- Introduction

Based on the discussion in RAN2#122, RAN2 reached a consensus that existing L3 measurement reporting framework can be reused. However, detailed signaling design and corresponding specification impact have not been discussed yet. Additionally, we have received a new LS from RAN4 which contains the new agreements for the unknown SCell measurement reporting and the request to support beam sweeping factor reduction [1]. Thus, in this paper, we first discuss FR2 unknown measurement reporting scheme and then share our views on the beam seeping factor reduction.

- Discussion

## 2.1 FR2 unknown SCell measurement reporting scheme

In the last meeting, RAN2 agreed that the unknown SCell measurement report is configured via MeasConfig. According to this agreement, to enable that the UE could report the available but not reported L3 measurement results upon the reception of SCell activation command, a simple and straightforward way is to introduce a new report type in the report configuration. The signaling is as follows:

ReportConfigNR ::=                          SEQUENCE {

reportType                                  CHOICE {

periodical                                  PeriodicalReportConfig,

eventTriggered                              EventTriggerConfig,

...,

reportCGI                                    ReportCGI,

reportSFTD                                   ReportSFTD-NR,

condTriggerConfig-r16                      CondTriggerConfig-r16,

cli-Periodical-r16                          CLI-PeriodicalReportConfig-r16,

cli-EventTriggered-r16                     CLI-EventTriggerConfig-r16,

rxTxPeriodical-r17                          RxTxPeriodical-r17,

reportOnActivation-r18                     ReportOnActivation-r18

}

}

Proposal 1: Introduce a new report Type in the IE reportConfigNR to enable that the UE could report the available but not reported L3 measurement results to network upon the reception of SCell activation command.

Regarding the reported measurement results, RAN4 only agreed that SSB beam index can be reported. However, it is unclear how many beams would be reported if the UE has available measurement results for several beams, and which quantity would used by the UE for sorting the beams, and besides SSB index, whether the beam results are needed or not. To control how to report measurement results, there are the following two solutions:

Solution 1:  The reporting rules are specified in the specification directly.

Solution 2:  Network controls how to report measurement results.

In legacy L3 RRM, how to report beam level measurement results is within network’s control. In our view, the same principle can be applied in the FR2 unknown SCell reporting scheme. Besides, with solution 2, UE can only report the measurement results that network is interested in, which reduces signaling overheads in special cases.

In the legacy, the network can configure the UE to only report the best N beams, the network can configure a sorting quality to the UE, and the network can also inform the UE whether beam results are needed or not. Similarly, the content of reporting configuration can include the follows:

- a maximum beam number that can be reported for an SCell;

- quantity indication (RSRP or RSRQ or SINR) used to sort the reported measurement results;

- a flag that indicates whether or not the UE shall include beam quality in the measurement report.

The corresponding signaling is as follows:

ReportOnActivation ::= SEQUENCE {

reportQuantityCell                  MeasReportQuantity,                            OPTIONAL,   -- Need R

reportQuantityRS-Indexes           MeasReportQuantity                             OPTIONAL,   -- Need R

maxNrofRS-IndexesToReport         INTEGER (1..maxNrofIndexesToReport)          OPTIONAL,   -- Need R

includeBeamMeasurements            BOOLEAN                                           OPTIONAL,   -- Need R

}

Proposal 2: Network can control how to report measurement results, the content of reporting configuration can include the follows:

- a maximum number of beams that can be reported for an SCell;

- quantity indication (RSRP or RSRQ or SINR) used to sort the reported measurement results;

- a flag that indicates whether or not the UE shall include beam quality in addition to beam index in the measurement report.

In the existing specification, the measurement reporting procedure is initiated when UE satisfies the triggering criterion of a measId. For FR2 unknown SCell activation, there are the following two options for measId to link SCell(s) with new report type:

Option 1: One measId only links one SCell with the new report type;

Option 2: One measId can link more-than-one SCells with the new report type.

For option 1, one measurement identity links to one measurement object, following legacy measurement configuration framework. However, if each SCell occupies one measurement identity for fast SCell activation, a lot of measurement identities are required, which seriously impacts UE measurement capability in some cases. Thus, we prefer option2.

Proposal 3: For fast FR2 unknown SCell activation, one measurement identity can link more-than-one SCells with the new report type.

Regarding how to link more-than-one SCells with new report type, we can take height-based reporting in LTE as reference. In the height-based reporting, the reference height threshold is contained in the MeasConfig, and triggering criteria is contained in the reportConfigEUTRA (event h1/h2). Similarly, for fast FR2 unknown SCell activation, the list of SCells which the network is expected to receive available but not reported measurement results upon SCell activation command is contained in the MeasConfig, and the triggering criteria is contained in the reportConfigNR. For the measurement object, the network can configure a measId to link new report type with a MO for serving cell, which reduces the signaling overheads. The corresponding signaling is as follows:

MeasConfig ::=             SEQUENCE {

measObjectToRemoveList              MeasObjectToRemoveList                 OPTIONAL,   -- Need N

measObjectToAddModList              MeasObjectToAddModList                 OPTIONAL,   -- Need N

reportConfigToRemoveList            ReportConfigToRemoveList               OPTIONAL,   -- Need N

reportConfigToAddModList            ReportConfigToAddModList               OPTIONAL,   -- Need N

measIdToRemoveList                   MeasIdToRemoveList                       OPTIONAL,   -- Need N

measIdToAddModList                   MeasIdToAddModList                       OPTIONAL,   -- Need N

s-MeasureConfig                       CHOICE {

ssb-RSRP                            RSRP-Range,

csi-RSRP                            RSRP-Range

}                                                                                   OPTIONAL,   -- Need M

quantityConfig                      QuantityConfig                             OPTIONAL,   -- Need M

measGapConfig                       MeasGapConfig                               OPTIONAL,   -- Need M

measGapSharingConfig                MeasGapSharingConfig                     OPTIONAL   -- Need M

...,

[[

interFrequencyConfig-NoGap-r16      ENUMERATED {true}                         OPTIONAL    -- Need R

]],

[[

sCellListForFastUnknownSCellAct-r18  SEQUENCE (SIZE (1..maxNrofSCells)) OF SCellIndex  OPTIONAL -Need R

]]

}

sCellListForFastUnknownSCellAct-r18 Indicates which SCell(s) that need to report available but not reported measurement results to network.

Proposal 4: Introduce an SCell list in the measConfig to indicate which SCells that need to report available but not reported measurement results to network upon the reception of SCell activation command.

Proposal 5: The network can configure a measId to associate the new report type with one serving cell measurement object .

## 2.2 Beam sweeping factor reduction in FR2 unknown SCell activation

According to the LS, RAN4 achieved the following agreements in the past meetings:

- For unknown FR2 SCell activation enhancement, introduce the UE capability to support Rx beam sweeping factor less than 8 for cell detection part and SSB based L1-RSRP measurement. - if UE has full set (N=8) of beam sweeping during AGC settling part during FR2 unknown SCell activation procedure. - Introduce beam sweeping factor capability of X1 for cell detection part (X1*Trs) and beam sweeping factor capability of X2 for SSB based L1-RSRP measurement - Beam sweeping factor capability X1/X2 are two independent capabilities - Candidates of X1 are {1,2,4,6} - Candidates of X2 are {0,1,2,3,4,5,6,7} - If X1 is absent, beam sweeping factor for cell detection part is 8 - If X2 is absent, beam sweeping factor for SSB-based L1 measurement is 8

In the legacy L1 measurement, the beam sweeping factor X1 and X2 are always 8. However, in Rel-18, RAN4 agreed to reduce the beam sweeping factor X1 and X2 in order to shorten unknown FR2 SCell activation delay. It is noted that the SCell activation enhancement in FR2-2 in Rel-18 is descoped, thus the new capability is only applicable in FR2-1. Besides, in our view, the granularity of new UE capability is per UE level.

Proposal 6: The new capability for beam sweeping factor reduction is only applicable in FR2-1.

Proposal 7: The granularity of beam sweeping factor reduction capability is per UE level.

The signaling and corresponding description are as follows:

MeasAndMobParametersCommon ::= SEQUENCE {

****skip non-related part***

beamSweepingFactorEnhForUnknownSCell-FR2-r18 SEQUENCE {

beamSweepFactorEnhForCellSearch-r18  ENUMERATED {0, 1, 2, 4,6};              OPTIONAL,

beamSweepFactorEnhForL1RSRP-SSB-r18  ENUMERATED {0,1,2,3,4,5,6,7};          OPTIONAL

}                                                                                            OPTIONAL

...

}

| Definitions for parameters | Per | M | FDD-TDD DIFF | FR1- FR2 DIFF |
| --- | --- | --- | --- | --- |
| beamSweepingFactorEnhForUnknownSCell-FR2-r18 Indicates the support of Rx beam sweeping factor less than 8 for cell detection part and SSB based L1-RSRP measurement in the FR2 unknown SCell activation. The capability comprises signaling of: -BeamSweepFactorForCellSearch-r18 indicates beam sweeping factor for cell search. If this field is absent, the beam sweeping factor for cell detection part is 8. -BeamSweepFactorForL1RSRP-SSB-r18 indicates beam sweeping factor for SSB based L1-RSRP. If this field is absent, the beam sweeping factor for SSB based L1-RSRP is 8. | UE | UE | FFS | FR2-1 only |

- Conclusion and proposals

Proposal 1: Introduce a new report Type in the IE reportConfigNR to enable that the UE could report the available but not reported L3 measurement results to network upon the reception of SCell activation command.

Proposal 2: Network can control how to report measurement results, the content of reporting configuration can include the follows:

- a maximum number of beams that can be reported for an SCell;

- quantity indication (RSRP or RSRQ or SINR) used to sort the reported measurement results;

- a flag that indicates whether or not the UE shall include beam quality in addition to beam index in the measurement report.

Proposal 3: For fast FR2 unknown SCell activation, one measurement identity can link more-than-one SCells with the new report type.

Proposal 4: Introduce an SCell list in the measConfig to indicate which SCells that need to report available but not reported measurement results to network upon the reception of SCell activation command.

Proposal 5: The network can configure a measId to associate the new report type with one serving cell measurement object .

Proposal 6: The new capability for beam sweeping factor reduction is only applicable in FR2-1.

Proposal 7: The granularity of beam sweeping factor reduction capability is per UE level.

- Reference

[1] R2-2304636  LS on FR2 unknown SCell activation enhancement (R4-2306321; contact: Apple)  Rel-18 NR_RRM_enh3  From RAN4 To RAN2