3GPP TSG RAN WG2 Meeting #123                                                           R2-2308077

# Toulouse, France, August 21-25, 2023

Agenda item:	7.18.2

Source:	Intel Corporation

Title:	MT SDT – CP Open Topics on Capabilities and Configurations (including CG-SDT)

Document for:	Discussion and decision

# Introduction

This document continues discussion on remaining open items to enable MT SDT mechanism focusing on control plane topics associated with UE capabilities and configuration (specific to MT-SDT and CG-SDT operation).

# Discussion

## UE Capabilities for MT-SDT

A new radio capability per UE will be defined to identify UEs supporting MT-SDT, however this new feature could be defined in different ways:

- MT-SDT capability is defined as independent/standalone feature to be used with any RB

- MT-SDT capability is defined as independent/standalone feature to be used with any DRB and another MT-SDT-SRB capability is defined when MT SDT is to be sent over SRB.

- MT-SDT capability is defined to be dependent to the support and/or configuration of Rel-17 SDT features, i.e. RA-SDT, CG-SDT and/or SRB-SDT

For sure, UE needs to inform the network when it supports MT-SDT feature. The open questions are:

- Q1) How to define a capability to support MT-SDT: (1) for any RB or (2) for DRB vs for SRB.

- Q2) What is the relation between the support of Rel-18 MT-SDT and Rel-17 SDT features (RA-SDT, CG-SDT and/or SRB-SDT), if any.

When UE initiates MT-SDT, UL/DL SDT is allowed during an ongoing SDT session. Therefore, it seems simpler to define the following UE behaviour:

- If UE support Rel-18 MT-SDT for DRB, the UE shall also support Rel-17 RA-SDT.

- If UE support Rel-18 MT-SDT for SRB2, the UE shall also support Rel-17 RA-SDT and Rel-17 SRB-SDT.

It is also important to keep in mind that it would be up to network decision whether to configure UE’s usage of resume initiated via MO-SDT or MT-SDT.

- A new optional radio capability signaling (e.g., MT-SDT-r18) is defined to indicate UE’s support of Rel-18 MT-SDT.

  - MT-SDT-r18 indicates whether the UE supports transmission of data over allowed radio bearers in RRC_INACTIVE state via Random Access procedure (i.e., RACH) or via configured grant type 1 (i.e., CG-SDT), as specified in TS 38.331 [9].

    - If a UE supports MT-SDT-r18, this UE shall also support Rel-17 RA-SDT (i.e., ra-SDT-r17).

- A separate capability (e.g., srb-MT-SDT-r18) is defined to indicate the support of MT-SDT over SRB2. I.e., two new radio capabilities would be defined e.g., MT-SDT-r18 and srb2-MT-SDT-r18.

  - If a UE supports srb2-MT-SDT-r18, this UE shall also support mt-SDT-r18 and Rel-17 SRB-SDT (i.e., srb-SDT-r17).

In RAN2#122 meeting, the following agreement was made on the usage of CG-SDT resources to initiate SDT that it is equally applicable for MT-SDT and MO-SDT:

- For both MO and MT-SDT, if the next CG-SDT resource is too far, then RACH resource can be selected first.   This is checked at the point of initial resource selection (e.g. CG SDT selection).   FFS what is too far and how this is configured.   Assumption is that we will continue this discussion in SDT session.  CONFIRM with main session [CB]

The simplest approach is to enable a new UE capability specific to this feature that it can be used only when MT-SDT or MG-SDT are configured.

- A new optional radio capability signaling (e.g., sdt-RACH-r18) is defined to indicate UE’s support to select RACH resources instead of configured grant type 1 resource when triggering resume for SDT or MT-SDT and next CG-SDT resource is too far as specified in TS 38.331.

  - If a UE supports sdt-RACH-r18, this UE shall also support Rel-17 CG-SDT (i.e., cg-SDT-r17).

- If Proposal 1 to Proposal 3 are agreed, to adopt as baseline the TP captured in the UE capabilities draft CRs for Rel-18 MT-SDT to TS 38.306 available in R2-2308082 and to TS 38.331 available in R2-2308083.

## Configuration for MT-SDT

RAN2 agreements relevant for the discussion of MT-SDT related configurations:

R2#122

- Allow support of only MT-SDT in a cell.  A separate SIB configuration will be introduced.  FFS what is put in there.

- A separate sdt-RSRP threshold for MT-SDT can be configured, at least in the case where MO-SDT is not configured in the cell.

- There is no need to define new Rel-18 CG configurations specific to MT-SDT.

- When the UE is configured with both MO and MT SDT the radio bearer configuration is common for both.

R2#121

- Upon receiving MT-SDT trigger, the UE shall initiate SDT procedure if the following checks are satisfied (all these same as Rel-17)

  - FFS 3a: Check for DVT (if UL data becomes available in UL)

  - 3b: Check for SDT RSRP threshold

  - 3c: Check for TA validation before selecting CG (if applicable)

  - 3d: Check for SSB level RSRP threshold for CG resource (if applicable)

- RBs configured for SDT are common for MO-SDT and MT-SDT

- When UE resumes for MT-SDT, UE resumes all RBs configured for SDT

R2#120

- It is possible for the network to configure only MT-SDT without MO-SDT RA resources and/or CG-SDT.

- MT-SDT is data that belongs to bearers that are configured for SDT. The network can only trigger MT-SDT if the data belongs to those bearers.  FFS whether the configuration is MO-SDT or MT-SDT specific

### Cell specific configuration for MT-SDT

Email discussion [Post122][312] discussed the running CR to 38.331 for MT-SDT the following new TP is proposed although it is unclear how to capture exactly previous agreement highlighted in blue above:

SIB1-v1800-IEs ::=               SEQUENCE {

mt-SDT-ConfigCommonSIB-r18   SDT-ConfigCommonSIB-r17  OPTIONAL,  -- Cond MT-SDT-Only

nonCriticalExtension         SEQUENCE {}              OPTIONAL

}

| Conditional Presence | Explanation |
| --- | --- |
| MT-SDT-Only | The field is optionally present, Need R, in a cell that supports MT-SDT only (i.e. it is present if the cell supports MT-SDT but does not support MO-SDT), otherwise it is absent Need R. |

In addition, RAN2 needs to also discuss the applicability/usage of other fiends currently included in sdt-ConfigCommon-r17 as follows:

SDT-ConfigCommonSIB-r17 ::=          SEQUENCE {

sdt-RSRP-Threshold-r17               RSRP-Range    OPTIONAL, -- Need R

sdt-LogicalChannelSR-DelayTimer-r17  ENUMERATED { sf20, sf40, sf64, sf128, sf512, sf1024,

sf2560, spare1}  OPTIONAL, -- Need R

sdt-DataVolumeThreshold-r17          ENUMERATED {byte32, byte100, byte200, byte400,

byte600, byte800, byte1000, byte2000, byte4000,

byte8000, byte9000, byte10000, byte12000,

byte24000, byte48000, byte96000},

t319a-r17                            ENUMERATED { ms100, ms200, ms300, ms400, ms600,

ms1000, ms2000,ms3000, ms4000, spare7, spare6,

spare5, spare4, spare3, spare2, spare1}

}

Considering previous RAN2 agreements and the MO-SDT and MT-SDT do not need to have different requirements on SDT RSRP threshold, duration of T319a or delay timer for SR, the values provided by Rel-17 MO-SDT could still be applicable. However, there are two potential concerns: 1) how the network can allow the initiation of SDT usage due to MT-SDT only (i.e., without allowing MO-SDT initiated) and 2) SDT data threshold is defined as mandatory present if SDT-ConfigCommonSIB-r17 is signaled. RAN2 only agreed that network should be able to configure only MT-SDT (without MO-SDT RA resources and/or CG-SDT), however scenario described by 1) was not explicitly discussed. One design approach is that when SDT is allowed, both MO-SDT and MT-SDT initiated are possible (dependent on whether UE was configured by network or not). This approach might be too restricted from network and operator point of view. Therefore, it seems preferable that from specification point of view, network has the flexibility to allow resume initiated by both or either of them. The simpler approach seems to define a MT-SDT specific common configuration and later discuss whether ASN.1 should be optimized to only use it when MT-SDT is only allowed, i.e. if MO-SDT and MT-SDT are both allowed, UE could use the common and applicable parameters from SDT-ConfigCommonSIB-r17.

In Rel-17 MO-SDT, a UE configured with SDT can initiate MO-SDT if SIB1 is included in SDT-ConfigCommonSIB-r17 and other SDT specific conditions are met. If SDT-ConfigCommonSIB-r17 is used for MT-SDT, it would be a problem if network only wants to allow resume due to MT-SDT (i.e., not MO-SDT).

During RAN2#122 meeting, there was a common understanding that the following agreement “network to configure only MT-SDT without MO-SDT RA resources and/or CG-SDT” applies to both UE-specific and cell-specific SDT configuration. For the cell-specific SDT configuration, this would mean that network should be able to allow UEs to initiate resume for MT-SDT only (i.e., without allowing the initiation of resume for MO-SDT).

The following options could be considered on how to enable the cell specific configuration for MT-SDT:

- MO-SDT cell specific configuration IE is reused. This would be as discussed during email discussion [Post122][312]:

SIB1-v1800-IEs ::=               SEQUENCE {

mt-SDT-ConfigCommonSIB-r18   SDT-ConfigCommonSIB-r17  OPTIONAL, -- Cond MT-SDT-Only

nonCriticalExtension         SEQUENCE {}              OPTIONAL

}

The new MT-SDT common SIB configuration (e.g., mt-SDT-ConfigCommonSIB-r18) could be defined reusing SDT-ConfigCommonSIB-r17 as captured in the TP proposed during email discussion [Post122][312]. However, MT-SDT does not require the usage of sdt-DataVolumeThreshold-r17 (which is currently defined as mandatory field in SDT-ConfigCommonSIB-r17.

- New MT-SDT cell specific configuration. A new MT-SDT common SIB configuration (e.g., mt-SDT-ConfigCommonSIB-r18) is defined including at least the following IEs from SDT-ConfigCommonSIB-r17: sdt-RSRP-Threshold, sdt-LogicalChannelSR-DelayTimer and t319a.

SIB1-v1800-IEs ::=               SEQUENCE {

mt-SDT-ConfigCommonSIB-r18   MT-SDT-ConfigCommonSIB-r18  OPTIONAL,

nonCriticalExtension         SEQUENCE {}              OPTIONAL

}

MT-SDT-ConfigCommonSIB-r18 ::=          SEQUENCE {

sdt-RSRP-Threshold-r18               RSRP-Range    OPTIONAL, -- Need R

sdt-LogicalChannelSR-DelayTimer-r18  ENUMERATED { sf20, sf40, sf64, sf128, sf512,

sf1024, sf2560, spare1}  OPTIONAL, -- Need R

t319a-r18                            ENUMERATED { ms100, ms200, ms300, ms400, ms600,

ms1000, ms2000,ms3000, ms4000, spare7, spare6,

spare5, spare4, spare3, spare2, spare1} – Need R

}

In addition, further clarification would need to capture in specification to guarantee that the new MT-SDT parameters are only broadcasted when MO-SDT is not allowed.

- When network allows initiation of resume due to MT-SDT and MO-SDT:

  - mt-SDT-ConfigCommonSIB-r18 is signalled but does not include Ies already included in SDT-ConfigCommonSIB-r17 to avoid unnecessary signaling.

  - UE initiating resume due to MT-SDT also applies the applicable Ies signal in SDT-ConfigCommonSIB-r17.

- When network only allows initiation of resume due to MT-SDT (i.e., not MO-SDT)

  - Network does not signal SDT-ConfigCommonSIB-r17 and all required common Ies are provided via the new mt-SDT-ConfigCommonSIB-r18.

Both approaches are feasible however the second one seems more optimum from signaling point of view while avoiding redundancy or providing MO-SDT specific parameters when cell only configures MT-SDT.

- A new MT-SDT common SIB configuration (e.g., mt-SDT-ConfigCommonSIB-r18) is defined including the following IEs: sdt-RSRP-Threshold, sdt-LogicalChannelSR-DelayTimer and t319a. The following behaviour is also captured in 38.331 specification:

  - When network allows initiation of resume due to MT-SDT and MO-SDT, mt-SDT-ConfigCommonSIB-r18 is signalled but does not include IEs already included in SDT-ConfigCommonSIB-r17 to avoid unnecessary signaling. I.e., UE initiating resume due to MT-SDT also applies the applicable Ies signal in SDT-ConfigCommonSIB-r17).

  - When network only allows initiation of resume due to MT-SDT (and not MO-SDT, i.e., network does not signal SDT-ConfigCommonSIB-r17), the required IEs are provided via the new mt-SDT-ConfigCommonSIB-r18.

### UE specific configuration for MT-SDT

Legacy define SDT configuration for MO-SDT provided via RRCRelease message as follows:

SDT-ConfigSIB-r17 ::=          SEQUENCE {

sdt-DRB-List-r17          SEQUENCE (SIZE (0..maxDRB)) OF DRB-Identity  OPTIONAL,  -- Need M

sdt-SRB2-Indication-r17   ENUMERATED {allowed}                         OPTIONAL,  -- Need R

sdt-MAC-PHY-CG-Config-r17           SetupRelease {SDT-CG-Config-r17}   OPTIONAL,  -- Need M

sdt-DRB-ContinueROHC-r17            ENUMERATED { cell, rna }           OPTIONAL   -- Need S

}

#### RB configuration for MT-SDT

RAN2 had agreed that MT-SDT could be configured by itself (i.e., without MO-SDT). In addition, in last meeting it was also agreed that when UE is configured with both MO-SDT and MT-SDT, the RB configuration is common.

- When the UE is configured with both MO and MT SDT, the radio bearer configuration is common for both.

The ASN.1 would need to allow both requirements above. Therefore, a simple flag indication that MO-SDT specific RB configuration is re-used for MT-SDT would not be sufficient.

Only re-using MO-SDT specific RB configuration for MT-SDT is not sufficient as UE should be able to be only configured with MT-SDT even though when a UE is configured with both MO-SDT and MT-SDT, the RB configuration is common for both.

On summary, some form of MT-SDT specific configuration needs to be defined in RRCRelease message. The following approaches could be discussed on how network indicates which RBs are configured for MT-SDT:

- New MT-SDT specific RB configuration. Which RB is configured for MT-SDT could be enabled as follow by providing different configuration specific for DRBs and SRB2 similarly as it was done in Rel-18. One open question would be whether we also need to have an indication specific to indicate whether ROHC can be continued per cell or RNA as it was done for Rel-17 SDT, as shown in highlighted line.

Mt-SDT-Config-rXY    SetupRelease { MT-SDT-Config-rXY }  OPTIONAL		-- Need M

MT-SDT-Config-rXY ::=        SEQUENCE {

mt-SDT-DRB-List-rXY   SEQUENCE (SIZE (0..maxDRB)) OF DRB-Identity  OPTIONAL, -- Need R

mt-SDT-SRB2-Indication-rXY    ENUMERATED {allowed}              OPTIONAL     -- Need R

}

- Definition of Rel-17 SDT specific RB configuration is changed to allow its usage for MT-SDT only and both MT-SDT and MO-SDT (with a new signaling indicating the allowed SDT operation). This approach requires that a Rel-18 supporting MT-SDT have slightly different understanding of Rel-17 SDT configuration. A new indication would indicate for each RBs whether this UE specific configuration is applicable only for MT-SDT or for both MT-SDT and MO-SDT. If no new indication is added, the default behaviour is aligned to legacy, i.e. applicable only to MO-SDT.

SDT-r18          ENUMERATED {mt-SDT, mo-mt-SDT}          OPTIONAL,

In addition, legacy definition of sdt-Config would require some update, e.g. as follows:

sdt-DRB-List Indicates the ID(s) of the DRB(s) that are configured for SDT. If size of the sequence is zero, then the UE assumes that none of the DRBs are configured for SDT. The network only configures MN terminated MCG bearers for SDT. If UE support MT-SDT, the configuration of SDT-r18 indicates whether the DRB is configured only for MT-SDT or for both MO-SDT and MT-SDT; and absent of configuration of SDT-r18 indicates that the DRB is configured only for MO-SDT.

Sdt-SRB2-Indication Indiates whether SRB2 is configured for SDT or not. If UE support MT-SDT, the configuration of SDT-r18 indicates whether the SRB2 is configured only for MT-SDT or for both MO-SDT and MT-SDT; and absent of configuration of SDT-r18 indicates that the SRB2 is configured only for MO-SDT.

Enabling only approach 1) has the disadvantage that some configuration would be repeated for those RBs configured for both MO-SDT and MT-SDT. From signaling point of view, approach 2) seems simple even though the definition of legacy UE-specific SDT configuration needs to be updated. We suggest discussing both options.

Adding MT-SDT specific RB configuration (as explained in approach 1)) is feasible although there might be redundant information with the MO-SDT specific one which could be avoid if the definition of Rel-17 SDT specific RB configuration is changed to allow MT-SDT only (as explained in approach 2). However, approach 2) would require changes on legacy MO-SDT definition when UE support MT-SDT.

- For the BR configuration associated to MT-SDT and provided in RRCRelease message, to discuss whether to define it via: approach (1) a new/independent MT-SDT specific RB configuration, or approach (2) extend the definition of Rel-17 SDT specific RB configurations to also allow its usage for MT-SDT only and both MT-SDT and MO-SDT (with a new signaling indicating the allowed SDT operation).

## Configuration for CG-SDT

RAN2 agreements relevant for the discussion when MT-SDT should not used UE’s configured CG-SDT resources:

R2#122

- In case CG-SDT resources cannot be used or are not available for MT-SDT, UE uses non-SDT RACH for RA-based MT-SDT.  FFS whether new triggers are defined

- For both MO and MT-SDT, if the next CG-SDT resource is too far, then RACH resource can be selected first.   This is checked at the point of initial resource selection (e.g. CG SDT selection).   FFS what is too far and how this is configured.   Assumption is that we will continue this discussion in SDT session.  CONFIRM with main session [CB]

Email discussions [Post122][312] discussed the running CR to 38.331 and captured the following new IE on this topic:

SDT-MAC-PHY-CG-Config-r17 ::=       SEQUENCE {

-- CG-SDT specific configuration

cg-SDT-ConfigLCH-RestrictionToAddModList-r17 SEQUENCE (SIZE(1..maxLC-ID)) OF

CG-SDT-ConfigLCH-Restriction-r17 OPTIONAL,   -- Need N

cg-SDT-ConfigLCH-RestrictionToReleaseList-r17 SEQUENCE (SIZE(1..maxLC-ID)) OF

LogicalChannelIdentity  OPTIONAL,   -- Need N

cg-SDT-ConfigInitialBWP-NUL-r17   SetupRelease {BWP-UplinkDedicatedSDT-r17}  OPTIONAL, -- Need M

cg-SDT-ConfigInitialBWP-SUL-r17   SetupRelease {BWP-UplinkDedicatedSDT-r17}  OPTIONAL, -- Need M

cg-SDT-ConfigInitialBWP-DL-r17    BWP-DownlinkDedicatedSDT-r17   OPTIONAL,   -- Need M

cg-SDT-TimeAlignmentTimer-r17     TimeAlignmentTimer             OPTIONAL,   -- Need M

cg-SDT-RSRP-ThresholdSSB-r17      RSRP-Range                     OPTIONAL,   -- Need M

cg-SDT-TA-ValidationConfig-r17    SetupRelease { CG-SDT-TA-ValidationConfig-r17 }  OPTIONAL,   -- Need M

cg-SDT-CS-RNTI-r17                RNTI-Value    OPTIONAL,   -- Need M

...,

[[

cg-SDT-MaxDurationToNext-CG-Occasion-r18 ENUMERATED { ms10, ms100, msxx, msxx, msxx, msxx,

msxx, spare1}           OPTIONAL    -- Need M

]]

}

cg-SDT-MaxDurationToNext-CG-Occasion The maximum duration until the next CG-SDT occasion as specified in TS 38.321 [3]. If configured, the CG-SDT resource can only be used for the initial CG-SDT transmission if the duration between the initiation of the SDT procedure and the next CG-SDT occasion is less than the value configured by this field.

During Email discussions [Post122][312], it was discussed whether this parameter should be configured or not per LCH. This seems reasonable as the actual periodicity value could be very different.

- To define cg-SDT-MaxDurationToNext-CG-Occasion-r18 per LCH.

During RAN2#122 main session, the related topic was discussed as part of TEI18 although the focus was on the extension of the value range for the CG-SDT periodicity, as shown below on the text included in the LS [1].

In Release 17, the maximum configurable CG-SDT periodicity is 640 ms. In RAN2#122, it was agreed that RAN2 intends to enable configuration of the CG periodicities to higher values from Rel-18. RAN2 made this agreement under the assumption that this extension would have no or low impact on RAN1 specifications. The intention is to extend the maximum CG-SDT periodicity up to a few minutes/hours (FFS the exact value).

The related discussions on TEI18 and MT-SDT are complementary and can continue in parallel although some coordination might be required on the value/range depending on the new values defined for CG-SDT periodicity.

MT-SDT is discussing the usage of CG-SDT resources when are “too far” for the initiation of any SDT (MO-SDT or MT-SDT) while TEI18 related discussion focuses on the maximum range of CG-SDT periodicity while. Both topics are inter-related and can continue their meeting discussion independently. If/when the range of CG-SDT periodicity is extended, it could be re-discussed if new values may be needed.

The value/range of cg-SDT-MaxDurationToNext-CG-Occasion-r18 should take into consideration Rel-17 CG-SDT periodicities which are summarized in below table captured in RAN1 spec.

| PUSCH configuration period Tcg (msec) | Association period (number of PUSCH configuration periods) |
| --- | --- |
| 5 | {1, 2, 4, 8,16, 32, 64, 128} |
| 8 | {1, 2, 4, 5, 8, 10, 16, 20, 40, 80} |
| 10 | {1, 2, 4, 8,16, 32, 64} |
| 16 | {1, 2, 4, 5, 8,10,20,40} |
| 20 | {1, 2, 4, 8,16, 32} |
| 32 | {1, 2, 4, 5, 10, 20} |
| 40 | {1, 2, 4, 8, 16} |
| 64 | {1, 2, 5, 10} |
| 80 | {1, 2, 4, 8} |
| 128 | {1, 5} |
| 160 | {1, 2, 4} |
| 320 | {1, 2} |
| 640 | {1} |

If the threshold is set to 10msec, any periodicity above of 10 msec may be restricted to use its CG-SDT resource (which could apply to 11 of the possible periodicity values). Similarly, for 100msec threshold, up to 4 of the possible periodicities may be restricted. It might be good to also consider at least one smaller and one larger value than 100 msec if the network wants to provide more specific settings on when this behaviour is allowed. For example, 40 and 300 msec.

- For the values of cg-SDT-MaxDurationToNext-CG-Occasion-r18, to consider also add one more value smaller and higher than 100 msec (e.g., 40 and 300 msec).

# Conclusion

The observations captured are the following:

Observation 1.	In Rel-17 MO-SDT, a UE configured with SDT can initiate MO-SDT if SIB1 is included in SDT-ConfigCommonSIB-r17 and other SDT specific conditions are met. If SDT-ConfigCommonSIB-r17 is used for MT-SDT, it would be a problem if network only wants to allow resume due to MT-SDT (i.e., not MO-SDT).

Observation 2.	During RAN2#122 meeting, there was a common understanding that the following agreement “network to configure only MT-SDT without MO-SDT RA resources and/or CG-SDT” applies to both UE-specific and cell-specific SDT configuration. For the cell-specific SDT configuration, this would mean that network should be able to allow UEs to initiate resume for MT-SDT only (i.e., without allowing the initiation of resume for MO-SDT).

Observation 3.	Only re-using MO-SDT specific RB configuration for MT-SDT is not sufficient as UE should be able to be only configured with MT-SDT even though when a UE is configured with both MO-SDT and MT-SDT, the RB configuration is common for both.

Observation 4.	Adding MT-SDT specific RB configuration (as explained in approach 1)) is feasible although there might be redundant information with the MO-SDT specific one which could be avoid if the definition of Rel-17 SDT specific RB configuration is changed to allow MT-SDT only (as explained in approach 2). However, approach 2) would require changes on legacy MO-SDT definition when UE support MT-SDT.

Observation 5.	MT-SDT is discussing the usage of CG-SDT resources when are “too far” for the initiation of any SDT (MO-SDT or MT-SDT) while TEI18 related discussion focuses on the maximum range of CG-SDT periodicity while. Both topics are inter-related and can continue their meeting discussion independently. If/when the range of CG-SDT periodicity is extended, it could be re-discussed if new values may be needed.

The proposals captured are the following:

Proposal 1.	A new optional radio capability signaling (e.g., MT-SDT-r18) is defined to indicate UE’s support of Rel-18 MT-SDT.

Proposal 1.1.	MT-SDT-r18 indicates whether the UE supports transmission of data over allowed radio bearers in RRC_INACTIVE state via Random Access procedure (i.e., RACH) or via configured grant type 1 (i.e., CG-SDT), as specified in TS 38.331 [9].

Proposal 1.1.1.	If a UE supports MT-SDT-r18, this UE shall also support Rel-17 RA-SDT (i.e., ra-SDT-r17).

Proposal 2.	A separate capability (e.g., srb-MT-SDT-r18) is defined to indicate the support of MT-SDT over SRB2. I.e., two new radio capabilities would be defined e.g., MT-SDT-r18 and srb2-MT-SDT-r18.

Proposal 2.1.	If a UE supports srb2-MT-SDT-r18, this UE shall also support mt-SDT-r18 and Rel-17 SRB-SDT (i.e., srb-SDT-r17).

Proposal 3.	A new optional radio capability signaling (e.g., sdt-RACH-r18) is defined to indicate UE’s support to select RACH resources instead of configured grant type 1 resource when triggering resume for SDT or MT-SDT and next CG-SDT resource is too far as specified in TS 38.331.

Proposal 3.1.	If a UE supports sdt-RACH-r18, this UE shall also support Rel-17 CG-SDT (i.e., cg-SDT-r17).

Proposal 4.	If Proposal 1 to Proposal 3 are agreed, to adopt as baseline the TP captured in the UE capabilities draft CRs for Rel-18 MT-SDT to TS 38.306 available in R2-2308082 and to TS 38.331 available in R2-2308083.

Proposal 5.	A new MT-SDT common SIB configuration (e.g., mt-SDT-ConfigCommonSIB-r18) is defined including the following IEs: sdt-RSRP-Threshold, sdt-LogicalChannelSR-DelayTimer and t319a. The following behaviour is also captured in 38.331 specification:

Proposal 5.1.	When network allows initiation of resume due to MT-SDT and MO-SDT, mt-SDT-ConfigCommonSIB-r18 is signalled but does not include IEs already included in SDT-ConfigCommonSIB-r17 to avoid unnecessary signaling. I.e., UE initiating resume due to MT-SDT also applies the applicable Ies signal in SDT-ConfigCommonSIB-r17).

Proposal 5.2.	When network only allows initiation of resume due to MT-SDT (and not MO-SDT, i.e., network does not signal SDT-ConfigCommonSIB-r17), the required IEs are provided via the new mt-SDT-ConfigCommonSIB-r18.

Proposal 6.	For the BR configuration associated to MT-SDT and provided in RRCRelease message, to discuss whether to define it via: approach (1) a new/independent MT-SDT specific RB configuration, or approach (2) extend the definition of Rel-17 SDT specific RB configurations to also allow its usage for MT-SDT only and both MT-SDT and MO-SDT (with a new signaling indicating the allowed SDT operation).

Proposal 7.	To define cg-SDT-MaxDurationToNext-CG-Occasion-r18 per LCH.

Proposal 8.	For the values of cg-SDT-MaxDurationToNext-CG-Occasion-r18, to consider also add one more value smaller and higher than 100 msec (e.g., 40 and 300 msec).

# Reference

- R2-2306904, LS on longer CG-SDT periodicities, TEI18, May 2023.