3GPP TSG RAN WG2 Meeting #123                     	                	                    R2-2308079

Toulouse, FR, 21st – 25th Aug, 2023

Agenda Item:	7.12.2.2

Source:	Intel Corporation

Title:	UE cell (re)selection towards mobile IAB cell

Document for:	Discussion and Decision

# Introduction

During RAN2 #122 meeting, following IDLE/INACTIVE state UE behaviour was agreed to help UE camping on and stick on a mobile IAB-cell when necessary:

RAN2 acknowledges following two problems to be addressed for idle/inactive UEs: - Problem 1: For a UE that is physically on a moving vehicle but not camped on its mobile IAB-cell yet (i.e. the UE is camped on a stationary cell), how to help such UE(s) to identify a neighbour mobile IAB-cell, prioritize mobile IAB-cell (frequency and cell) and to be “pulled” into this mobile IAB-cell, especially for inter-frequency scenario where the mobile IAB-cell’s frequency priority is low. - Problem 2: After the UE physically on a moving vehicle is camped on the mobile IAB cell, how to avoid it reselecting other non-mIAB-(stationary) cells. - Such UE may prioritize a highest ranked cell at a frequency, if it broadcasts a mIAB-cell type indicator in SIB1 for cell reselection. UE may use the SIB4 assistance information to identify the presence of such mobile IAB-cell(s), if broadcasted. A SIB4 assistance information may include mIAB-cell frequencies. FFS on stage-2/3 to clarify the UE in problem 1 and 2.

In this contribution, we continue discuss further details and specification impact about UE behavior on mobile IAB-cell prioritization.

# Discussion

## “physically onboard” UE Identification

As observed from the above agreement, only the UE that is physically on a moving vehicle and facing the problems as above may prioritize mIAB-cell or mIAB-cell’s frequency during cell (re)selection. It is important to have a standardised approach to ensure predictable reselection behaviour.  Identifying a physically onboard UE cannot be left to UE implementation.

A specified solution to identify such UEs that are physically onboard a moving vehicle is needed so that UEs that are physically onboard can select a mIAB cell while unnecessary cell (re)selection could be avoided by the other remaining UEs (i.e. UEs that are not physically on a moving vehicle).

As defined in TS 38.304 [1], the UE mobility state is determined by how frequent cell reselection does a UE perform during a time period.

Normal-mobility state criteria: - If number of cell reselections during time period TCRmax is less than NCR_M. Medium-mobility state criteria: - If number of cell reselections during time period TCRmax is greater than or equal to NCR_M but less than or equal to NCR_H. High-mobility state criteria: - If number of cell reselections during time period TCRmax is greater than NCR_H.

Considering a UE which is physically on board and moving together with mobile IAB-cell, if the UE is determined as medium/high-mobility state, it indicates that this UE is frequently switched between different cells rather than camping on the mobile IAB-cell on the vehicle, i.e. a UE facing Problem 1. On the other hand, if a UE is in normal-mobility state, it is camped on the right mIAB-cell when it is physically onboard the vehicle, i.e. a UE in Problem 2.

It should also be possible to differentiate UE that are onboard a moving vehicle that supports a mIAB cell from a UE that is in a vehicle that does not have a mIAB cell.  Otherwise, all UEs that meet the high mobility criteria will prioritise the mIAB frequency and attempt to find/camp on an mIAB cell that doesn’t exist, draining battery.

Observation 1: It should be possible to differentiate UEs that on board vehicles with mIAB cell (and should reselect the mIAB cell) from UEs that are on board vehicles that do not have a mIAB cell (and should not prioritise mIAB frequency or cell).

It was questioned that the mobility state determination may consume a considerable amount of time which is much larger than the time frame taken by entering a vehicle. According to TS 38.331 [2], TCRmax can be configured to minimum 30 secs and maximum 240 secs. Besides, NCR_M and NCR_H could be set within the range of 1 to 16. There’s no problem the UE still camps on a non-mIAB cell immediately after boarding while the vehicle is still stopped, since the original gNB-cell can still provide coverage for that UE. When the vehicle starts moving, while the UE hasn’t camped on the right mIAB-cell, the number of cell reselection could change frequently within a short time. In our understanding, with suitable configuration, mobility state of the UE can reflect the camping status of the UE correctly and to be used for cell reselection enhancements.

Another concern of using mobility state is that the number of cell reselection will also include the case where mobile IAB-DU performs DU migration or PCI change. However, compared with fast cell reselections among other non-mIAB cells of the physically onboard UE, the cell reselection caused by inter-CU migration and/or PCI change will be negligible.

Observation 2: It is possible to identify a physically onboard UE based on its mobility state for purpose of reselecting a mIAB cell.  A physically onboard normal-mobility UE is highly likely camped on a mobile IAB-cell.  A physically onboard UE not camping on a mIAB cell is highly likely to be in Medium or High-mobility states.

Observation 3: The cell reselection caused by inter-CU migration/PCI change is negligible compared to the number of cell reselection of a medium/high mobility state UE caused by camping on non-mIAB-cell.

## mIAB Cell reselection prioritization for a physically onboard UE

Following flow chart is summarized to illustrate the general procedure for a physically onboard UE’s cell (re)selection prioritization based on the agreement.

In this section, we mainly focus on the specification impact of prioritizing mobile IAB-cell during cell reselection when such UE is facing Problem 1) and 2).

Problem 1: Physically onboard UE that is not camping on mIAB cell

As agreed during RAN2 #122 meeting, the UE may use the SIB4 assistance information to identify the presence of such mobile IAB-cell(s), if broadcasted.

A similar concept was adopted in HSDN. In HSDN prioritisation mechanism, UE in high mobility state prioritises HSDN cells. For inter-frequency scenario, HSDN indication is provided in the HSDN cells, and frequency+PCI range is broadcasted in SIB4 for inter-frequency cells (interFreqNeighHSDN-CellList-r17).  The UE uses this list to consider the HSDN “cell” as high priority when in high mobility state.

Observation 4: HSDN prioritisation concept can be adapted to mIAB to help a physically onboard UE to prioritise and reselect an mIAB cell.

This concept can be adapted to prioritise an mIAB frequency and even a mIAB cell. A physically onboard UE that has not camped on an mIAB cell will be in high mobility state as it reselects neighbouring cells when the vehicle moves. There are two approaches for the UE to identify an mIAB cell during inter-frequency cell (re)selection.

Alternative 1: SIB4 only includes a list of frequency where mIAB cell exists.

If the UE in medium/high mobility state reads the presence of frequency list in SIB4, it may consider those frequencies as highest priority during inter-frequency cell (re)selection. After camping on such frequency, the UE may reselect the mIAB cell while it is still physically onboard based on the cell ranking, as the mIAB cell is most likely to be the cell with highest signaling strength. However, the UEs in medium/high mobility state on board a vehicle that does not have a mobile IAB may also prioritize the mIAB frequency, which may lead to an overload of that frequency or repeated reselection between mIAB frequency and next priority frequency, since the onboard vehicle doesn’t have a mobile IAB-cell onboard. Furthermore, the UE also needs to perform SIB1 reading when the highest ranked cell in the prioritized frequency does not broa dcast mIAB in SIB.

Alternative 2: SIB4 includes a list of frequency and PCI range of mIAB cell

Alternatively, for each frequency, the PCI range of the mIAB cell can also be included in the broadcast information of SIB4. This can further reduce UE’s measurement effort by reducing the number of cells (non-mIAB cells) that a UE needs to measure. The UE only needs to measure and identify the best cell among PCI range.  It can also prevent UEs that are in medium/high mobility state and onboard vehicles that does not have mIAB from unnecessarily prioritising the mIAB frequency and reading SIB1 of the highest ranked cell on mIAB frequency.

Observation 5: Mobility state of a UE along with the mIAB frequency and PCI can be used to identify whether it is physically onboard the vehicle that has an mIAB cell and that the UE should prioritise a mobile IAB-cell.

Observation 6: Not including the mIAB PCI range for inter-frequency carriers in SIB4 can lead to an overload on mIAB frequency or battery consumption for UEs that are in high mobility state on board vehicles that do not have mIAB cells.

Proposal 1: A SIB4 assistance information may also include mIAB-cell PCI range for inter-frequencies.

Problem 2: Physically onboard UE that is current camping on mIAB cell

Once camped on the mIAB cell, the UE will drop to normal mobility state as there is no further cell crossing while the UE is still physically onboard, irrespective of the vehicle speed. The physically onboard UE that is now camped on mIAB cell can further confirm it is camping on a mobile IAB-cell by reading SIB1 information that carries mobile-IAB type indication. The UE can keep prioritizing the camped mIAB-cell as highest priority if “mIAB-cell” type indicator is broadcasted in SIB1.

Proposal 2: RAN2 to confirm that 1 bit mobile-IAB cell type indication is introduced in SIB1, to help UE know it is now camping on a mobile IAB-cell and consider it as highest priority.

Proposal 3: A UE in high/medium mobility state considers the neighbouring mIAB-cell’s frequency (cell if PCI range is included in SIB4) as highest priority if “mIAB-cell presence” identification is included in SIB4. Such UE performs cell (re)selection towards the highest ranked cell at the prioritized frequency. A UE in normal mobility state considers the camped mIAB-cell as highest priority if “mIAB-cell” type indicator is included in SIB1.

A corresponding text proposal reflecting above IDLE/INACTIVE UE behaviors during cell (re)selection is attached below:

### 5.2.4 Cell Reselection evaluation process #### 5.2.4.1 Reselection priorities handling Absolute priorities of different NR frequencies or inter-RAT frequencies may be provided to the UE in the system information, in the RRCRelease message, or by inheriting from another RAT at inter-RAT cell (re)selection. In the case of system information, an NR frequency or inter-RAT frequency may be listed without providing a priority (i.e. the field cellReselectionPriority is absent for that frequency). If any fields with cellReselectionPriority or nsag-CellReselectionPriority are provided in dedicated signalling, the UE shall ignore any fields with cellReselectionPriority and nsag-CellReselectionPriority provided in system information. When UE is in camped normally state, if it supports slice-based cell reselection and has received the network slice(s) and NSAG information from NAS to be used for cell reselection, UE shall derive reselection priorities according to clause 5.2.4.11. If UE is in camped on any cell state, UE shall only apply the priorities provided by system information from current cell, and the UE preserves priorities provided by dedicated signalling and deprioritisationReq received in RRCRelease unless specified otherwise. When the UE in camped normally state, has only dedicated priorities other than for the current frequency, the UE shall consider the current frequency to be the lowest priority frequency (i.e. lower than any of the network configured values). When the HSDN capable UE is in High-mobility state, the UE shall always consider the HSDN cells to be the highest priority (i.e., higher than any other network configured priorities). When the HSDN capable UE is not in High-mobility state, the UE shall always consider HSDN cells to be the lowest priority (i.e., lower than any other network configured priorities). If the UE is configured to perform both NR sidelink communication and V2X sidelink communication, the UE may consider the frequency providing both NR sidelink communication configuration and V2X sidelink communication configuration to be the highest priority. If the UE is configured to perform NR sidelink communication and not perform V2X communication, the UE may consider the frequency providing NR sidelink communication configuration to be the highest priority. If the UE is configured to perform V2X sidelink communication and not perform NR sidelink communication, the UE may consider the frequency providing V2X sidelink communication configuration to be the highest priority. When the UE is not in normal-mobility state and discover a mobile IAB-cell as neighbouring cell in SIB4, the UE shall always consider the mobile IAB-cells to be the highest priority (i.e., higher than any other network configured priorities). When the UE is in normal-mobility state that currently camped on a mobile IAB-cell with mIAB-cell type indicator broadcasted in SIB1, the UE shall always consider this mobile IAB-cells to be the highest priority (i.e., higher than any other network configured priorities).

Proposal 4: RAN2 to consider the above TP to TS38.304 as stage-2 of UE cell (re)selection mIAB-cell prioritization.

## Priority between mIAB cell and other functionality

There are multiple functionalities that may require frequency prioritization during inter-frequency cell (re)selection, for example, MBS, NR sidelink, V2X sidelink and HSDN. In this section, we mainly discuss the frequency prioritization between mIAB’s frequency with other functionality frequencies.

According to TR 22.839 [3] Section 5.11.3, mobile IAB can also be deployed on a moving train.

Some big size vehicles, e.g. trains or trams, can require multiple 5G base station relays to provide good coverage to users/UEs located in different parts of the vehicle. In such cases, if active users/UEs move around the vehicle their connection can switch between different relays. In the example described hereafter, Tom is travelling on a high-speed train that provides 5G on board coverage (from his HLMN) by using relays installed 2-3 wagons apart, connected via wireless backhaul to the wide-area 5G macro RAN.

As captured in TS 38.304 Section 5.2.4.1 [1], HSDN cells are also considered as highest priority by HSDN capable UEs when it is in High-mobility state.

If UE is in camped on any cell state, UE shall only apply the priorities provided by system information from current cell, and the UE preserves priorities provided by dedicated signalling and deprioritisationReq received in RRCRelease unless specified otherwise. When the UE in camped normally state, has only dedicated priorities other than for the current frequency, the UE shall consider the current frequency to be the lowest priority frequency (i.e. lower than any of the network configured values). When the HSDN capable UE is in High-mobility state, the UE shall always consider the HSDN cells to be the highest priority (i.e., higher than any other network configured priorities). When the HSDN capable UE is not in High-mobility state, the UE shall always consider HSDN cells to be the lowest priority (i.e., lower than any other network configured priorities). If the UE is configured to perform both NR sidelink communication and V2X sidelink communication, the UE may consider the frequency providing both NR sidelink communication configuration and V2X sidelink communication configuration to be the highest priority. If the UE is configured to perform NR sidelink communication and not perform V2X communication, the UE may consider the frequency providing NR sidelink communication configuration to be the highest priority. If the UE is configured to perform V2X sidelink communication and not perform NR sidelink communication, the UE may consider the frequency providing V2X sidelink communication configuration to be the highest priority.

Hence, for inter-frequency cell (re)selection, it is questionable whether a HSDN capable UE on a moving high-speed train with mobile IAB-node deployed should consider a HSDN cell or a mobile IAB-cell’s frequency as highest priority if it is in High-mobility state.

HSDN cells are normally deployed along the railway, which is outside of the moving high-speed train. Compared with an onboard 5G coverage that provided by mIAB, the UE may receive lower signaling strength from HSDN cells. Furthermore, the UE may also needs to perform cell reselection across different HSDN cells once it is still onboard. For such UEs, camping on a mobile IAB-cell may provide better coverage than camping on HSDN cells.

Hence, during inter-frequency prioritization, the HSDN capable UE in High-mobility state may consider the frequency for mobile IAB-cells has higher priority than the frequency for HSDN cells. Furthermore, the UE may also consider MBS, NR sidelink, V2X sidelink functionalities does not replace cell reselection priorities caused by mIAB but that can be left to implementation.

Proposal 5: The UE does not consider MBS broadcast, NR sidelink communication, V2X sidelink communication or HSDN functionality to replace cell reselection priorities caused by mobile IAB.

# Conclusion

In this contribution, we first discuss how to identify a UE is physically onboard based on mobility state, and further discuss the detailed information encoded in SIB4 and SIB1 to assist UE find and stick on a mIAB cell. In the end, we discuss the priority rules between mIAB cells and other functionalities.

We have following observations and proposals:

Observation 1: It should be possible to differentiate UEs that on board vehicles with mIAB cell (and should reselect the mIAB cell) from UEs that are on board vehicles that do not have a mIAB cell (and should not prioritise mIAB frequency or cell).

Observation 2: It is possible to identify a physically onboard UE based on its mobility state for purpose of reselecting a mIAB cell.  A physically onboard normal-mobility UE is highly likely camped on a mobile IAB-cell.  A physically onboard UE not camping on a mIAB cell is highly likely to be in Medium or High-mobility states.

Observation 3: The cell reselection caused by inter-CU migration/PCI change is negligible compared to the number of cell reselection of a medium/high mobility state UE caused by camping on non-mIAB-cell.

Observation 4: HSDN prioritisation concept can be adapted to mIAB to help a physically onboard UE to prioritise and reselect an mIAB cell.

Observation 5: Mobility state of a UE along with the mIAB frequency and PCI can be used to identify whether it is physically onboard the vehicle that has an mIAB cell and that the UE should prioritise a mobile IAB-cell.

Observation 6: Not including the mIAB PCI range for inter-frequency carriers in SIB4 can lead to an overload on mIAB frequency or battery consumption for UEs that are in high mobility state on board vehicles that do not have mIAB cells.

Proposal 1: A SIB4 assistance information may also include mIAB-cell PCI range for inter-frequencies.

Proposal 2: RAN2 to confirm that 1 bit mobile-IAB cell type indication is introduced in SIB1, to help UE know it is now camping on a mobile IAB-cell and consider it as highest priority.

Proposal 3: A UE in high/medium mobility state considers the neighbouring mIAB-cell’s frequency (cell if PCI range is included in SIB4) as highest priority if “mIAB-cell presence” identification is included in SIB4. Such UE performs cell (re)selection towards the highest ranked cell at the prioritized frequency. A UE in normal mobility state considers the camped mIAB-cell as highest priority if “mIAB-cell” type indicator is included in SIB1.

Proposal 4: RAN2 to consider the above TP to TS38.304 as stage-2 of UE cell (re)selection mIAB-cell prioritization.

Proposal 5: The UE does not consider MBS broadcast, NR sidelink communication, V2X sidelink communication or HSDN functionality to replace cell reselection priorities caused by mobile IAB.

# References

- TS38.304, User Equipment (UE) procedures in Idle mode and RRC Inactive state

- TS38.331, NR; Radio Resource Control (RRC) Protocol Specification

- TR 22.839, Study on vehicle-mounted relays