3GPP TSG RAN WG2 Meeting #123                                           	R2-2308084

Toulouse, FR, 21st – 25th Aug, 2023

Agenda item:	7.15.3

Source:	Intel Corporation

Title:	On SL resource (re)selection and enhanced LCP

Document for:	Discussion

# Introduction

In the RAN2 meeting #121bis-e, the issue of whether a change to the LCP procedure is needed was discussed, given that COT information is shared. Following agreements related to SL LCP and COT were made in the RAN2 meeting #121bis-e [1][2]:

RAN2 #121bis-e - RAN2 will wait for more conclusion from RAN1 on the assistance information for COT sharing. - If the resource to be used is within a shared COT, and if PDU not generated before COT arrival, and there is data in buffer satisfying COT requirement, at least enhanced LCP should be allowed. FFS on the condition for UE to use enhanced LCP. FFS on spec impact. - If a UE decides to use the resource in a shared COT, and when enhanced LCP is decided to be used, for destination selection step in enhanced LCP, at least further restrict the destinations to be the candidates allowed by the COT (as defined by RAN1).

Subsequently, there was some discussion in RAN2#122 meeting and the following was agreed as a working assumption [3]:

Agreements on SL enhanced LCP

1: 	Working assumption: For shared COT, CAPC restriction is applicable to enhanced LCP according RAN1 agreement on CAPC requirement.

This document discusses way forward, considering the RAN1/2 discussion so far and presents our views.

# Discussion

## Conditions for Enhanced SL LCP procedure

In the RAN2 agreement referenced above, as a first step, it was agreed in RAN2 to restrict destinations to be candidates allowed by the COT in the destination selection step in SL LCP procedure e.g., the responding UE (which received shared COT information) can select the COT initiating UE as its destination for the upcoming transmission only if the MAC or PHY IDs match with the transmission through which the responding device receives the COT information. It was also agreed to use enhanced LCP if the resource to be used is within a shared COT, if PDU has not been generated before COT arrival, and if there is data in the buffer satisfying COT requirement.

However, it was left FFS on the solution selection itself, i.e. when enhanced LCP is used and the potential/associated spec impact. In the discussion during the previous meetings, there was some contention on the use of enhanced LCP with respect to shared COT, especially how/if legacy priority-based selection is considered. For example, the COT responding UE may have SL grant(s) within shared COT but may also have other high priority data associated with other destination UEs not allowed by the COT (as defined by RAN1). Per RAN1 agreement, the source and destination IDs may be included in the COT initiator’s SCI for the case of unicast link. However, limitation on additional UE identification included in the COT sharing information from the COT initiator is still FFS in RAN1.  On the aspect of whether the responding UE may have data corresponding to other destination UEs apart from the COT initiator UE, similar discussion on contents of COT container and included additional IDs was also brought up in RAN1. There are some companies in support of introducing additional IDs to maximize COT usage and allowing responding UE to transmit PSFCH to UEs other than the COT initiator UE. However, several companies discourage it due to overhead in SCI, and unclear benefits. Considering this progress/discussion so far, we believe there are three options that can be considered in RAN2

- Option 1: RAN2 waits for further progress and clarity in RAN1 on additional IDs in the COT container.

- Option 2: RAN2 also considers at least PDB in the destination selection step in enhanced LCP, if responding UE has data with higher priority in the buffer than SL data or MAC CEs towards COT initiating UE.

- Option 3: RAN2 agrees that it is up to UE implementation whether to use enhanced LCP or not.

In our view, given the limited time left for this WI, we think RAN2 may need to agree between option 2 and 3, i.e. whether the LCP enhancement to consider the PDB in the destination selection step is specified or is left to UE implementation.

- RAN2 is proposed to discuss and agree to include at least PDB in the destination selection step (in addition to shared COT) for the enhanced LCP.

## LCH Selection in Enhanced LCP

Another aspect which was discussed in RAN2 was whether LCP change should also consider the CAPC values (shared in COT information for the RB sets over which the COT has been initiated), in addition to the default priority, while prioritizing different logical channels. On this issue, RAN2 made a working assumption in the last meeting to apply the CAPC restriction for the enhanced LCP procedure (in line with RAN1 agreements). In our view, it seems to be a feasible next step in RAN2 to add the LCP restriction of UE not including MAC SDUs of logical channels having CAPC value higher than that indicated in the shared COT information. So, we propose to confirm this working assumption in RAN2.

- Confirm the working assumption to restrict the COT responding UE to select LCHs with equal or smaller CAPC value than that indicated in the shared COT information.

## Sidelink Resource reselection for MCSt

In the last RAN2 meeting, there was discussion on an LS from RAN1 regarding MCSt operation and RAN2 addressed RAN1 questions regarding different approaches of resource reselection for MCSt. There was also an FFS related to whether and how SL C-LBT failure triggers resource reselection specifically for the case of MCSt:

- RAN2 understands UE triggers a resource (re)selection when PSSCH transmission was not performed due to an LBT failure indication from L1. FFS on MCST case. Send LS to RAN1 to check if there is any concern.

The two approaches discussed basically revolve around whether the UE should be allowed to trigger SL C-LBT when receiving LBT failure indication from L1 in case of MCSt transmissions. Note that RAN2 already agreed to support this behaviour for the non-MCSt case. In our understanding, MCSt design is specifically tailored to reduce type 1 LBT overhead and allow multiple transmissions without having to trigger SL-LBT failure repeatedly. Keeping this in mind, we think that for the MCSt case, UE shall not trigger resource reselection for each PSSCH transmission due to LBT failure.

- For MCSt case, RAN2 to agree that the UE shall not trigger resource reselection for each PSSCH transmission due to LBT failure.

# Conclusion

This contributions discusses the open issues related to shared COT and SL LCP enhancements dur to SL LBT operation and makes the following observations and proposals:

- The impact of assistance information to the COT sharing procedure is not clear.

- RAN1 discussion on additional IDs is ongoing and no consensus has been reached yet.

- RAN2 is proposed to discuss and agree to include at least PDB in the destination selection step (in addition to shared COT) for the enhanced LCP.

- Confirm the working assumption to restrict the COT responding UE to select LCHs with equal or smaller CAPC value than that indicated in the shared COT information.

- For MCSt case, RAN2 to agree that the UE shall not trigger resource reselection for each PSSCH transmission due to LBT failure.

# Reference

- R2-2304234, Summary on [AT121bis-e][509][V2X/SL] The need of Assistance Information (Xiaomi), RAN2 121bis-e

- R2-2304205, Report from session on LTE V2X and NR SL, RAN2 121bis-e.

[3]	RAN2#122 Chairman Notes