3GPP TSG-RAN WG2 Meeting #123 	R2-2308088

Toulouse, France, August 21-25, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.822 | CR |  | rev | - | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Running UE capability CR on 38.306 for Rel-18 R1 R4 feature lists |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Intel Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_netcon_repeater, NR_DSS_enh, NR_MC_enh, NR_BWP_wor, TEI18 |  |  |  |  |  | Date: |  |  | 2023-08-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Capture further Release-18 UE capabilities based on the RAN1 UE feature list (R1-2306223). All the entries that are not concluded in the feature lists from RAN1 feature lists and those that are highlighted (or has pre-requisite with features that are highlighted) in R1 feature list are not considered as part of this CR. |  |  |  |  |  |  |  |  |
| Summary of change: |  | New Release-18 capabilities from RAN1 are added based on the latest RAN1 feature lists. The following RAN1 and 4 feature lists and the endorsed CRs are included: - R1-2306223 Rel18 RAN1 UE feature List |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | New RAN1 related UE capabilities will not be captured in specifications |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS38.331 CRx,TS38.306 CRx |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

START OF CHANGE

# 7	Release 18 UE feature list

## 7.1	Layer-1 UE features

### 7.1.0	General

Tables 7.1.1-1 to 7.1.x-1 provide the list of Layer-1 features, as shown in [xx] and the corresponding UE capability field name, as specified in TS 38.331 [2].

### 7.1.x	NR_NCR

Table 7.1.1-1: Layer-1 feature list for NR_NCR

| Features | Index | Feature group | Components | Prerequisite feature groups | Field name in TS 38.331 [2] | Parent IE in TS 38.331 [2] | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 43. NR_netcon_repeater | 43-1 | Basic NCR support | 1. Support of fixed beam for C-link/backhaul link 2. Support of TDMed UL transmission of C-link and backhaul link 3. Support of ON-OFF operation for NCR-Fwd based on access link beam indication 4. Support of TDD UL/DL determination for backhaul/access link based on TDD UL/DL configuration of C-link 5. Support of Tx/Rx timing determination for backhaul/access link based on Tx/Rx timing of C-link 6. Support of beam correspondence of the DL/UL of the access link at NCR-Fwd 7.Support periodic beam indication for access link 8. Priority flag for periodic indication |  |  |  | No | No | Yes | A NCR-MT that includes ncr-NodeIndication in RRC Setup Complete must support FG 43-1 | Optional without capability signaling |
| 43. NR_netcon_repeater | 43-5 | Simultaneous UL transmission of backhaul link and C-link | 1. Simultaneous UL transmission of backhaul link and C-link | 43-1 | ncr-Simultaneous-UL-BackhaulAndC-Link-r18 | Phy-ParametersCommon | No | No | No |  | Optional with capability signaling |

### 7.1.x	NR_MC_enh

| Features | Index | Feature group | Components | Prerequisite feature groups | Field name in TS 38.331 [2] | Parent IE in TS 38.331 [2] | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 49. NR_MC_enh | 49-4a | Nominal RBG size of Configuration 3 for FDRA type 0 for DCI format 1_3 | 1) Support of nominal RBG size of Configuration 3 for FDRA type 0 for DCI format 1_3 | At least one of {49-1, 49-1b} Editor’s NOTE: These features are yet to be added to 822 | nominalRBG-SizeOfConfig-3-FDRA-Type-0-DCI-1-3-r18 | Phy-ParametersCommon | No | No | No |  | Optional with capability signaling |
| 49. NR_MC_enh | 49-4b | Nominal RBG size of Configuration 3 for FDRA type 0 for DCI format 0_3 | 1) Support of nominal RBG size of Configuration 3 for FDRA type 0 for DCI format 0_3 | At least one of {49-2, 49-2b} Editor’s NOTE: These features are yet to be added to 822 | nominalRBG-SizeOfConfig-3-FDRA-Type-0-DCI-0-3-r18 | Phy-ParametersCommon | No | No | No |  | Optional with capability signaling |
| 49. NR_MC_enh | 49-4c | Configurable Type-1A fields for DCI format 0_3/1_3 | 1) Support Type-1A for ‘Antenna port(s)’ field for DCI format 1_3 2) Support Type-1A for ‘Antenna port(s)’, ‘Precoding information and number of layers’ and ‘SRS resource indicator’ fields for DCI format 0_3 | At least one of {49-1, 49-1b, 49-2, 49-2b} Editor’s NOTE: These features are yet to be added to 822 | configurableType-1A-FieldsForDCI-0-3-And-1-3-r18 | Phy-ParametersCommon | No | No | No |  | Optional with capability signaling |
| 49. NR_MC_enh | 49-4d | FDRA Type 1 granularity of 2, 4, 8, or 16 consecutive RBs based RIV for DCI format 1_3/0_3 | 1) Support of FDRA Type 1 granularity of 2, 4, 8, or 16 consecutive RBs based RIV for DCI format 0_3 2) Support of FDRA Type 1 granularity of 2, 4, 8, or 16 consecutive RBs based RIV for DCI format 1_3 | At least one of {49-1, 49-1b, 49-2, 49-2b} Editor’s NOTE: These features are yet to be added to 822 | frda-Type-1-Gty-2-4-8-16-RBs-RIV-DCI-1-3-And-0-3-r18 | Phy-ParametersCommon | No | No | No |  | Optional with capability signaling |

### 7.1.x	NR_DSS_enh

| Features | Index | Feature group | Components | Prerequisite feature groups | Field name in TS 38.331 [2] | Parent IE in TS 38.331 [2] | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 52. NR_DSS_enh | 52-2 | Two LTE-CRS overlapping rate matching patterns within a part of NR carrier using 15 kHz overlapping with a LTE carrier (regardless of support or configuration of multi-TRP) | 1) Support of two LTE-CRS overlapping rate matching patterns configured by lte-CRS-PatternList3-r18 and lte-CRS-PatternList4-r18 within a part of NR carrier using 15 kHz overlapping with a LTE carrier (regardless of support or configuration of multi-TRP) for the case when crs-RateMatchPerCoresetPoolIndex is not configured 2) Maximum number of LTE-CRS rate matching patterns in total within a NR carrier using 15 kHz SCS: {2,3,4,5,6} 3) Maximum number of LTE-CRS non-overlapping rate matching patterns within a NR carrier using 15 kHz SCS: {1,2,3} | 5-28 | twoRateMatchingEUTRA-CRS-patterns-3-4-r18 { maxNumberPatterns-r18, maxNumberNon-OverlapPatterns-r18} | BandNR | N/A | FR1 only | N/A | If a UE supports FG52-2 and FG14-1, FG14-1 is reported for list1/2 and FG52-2 is reported for list3/4 | Optional with capability singaling |
| 52. NR_DSS_enh | 52-2a | Two LTE-CRS overlapping rate matching patterns with two different values of coresetPoolIndex within a part of NR carrier using 15 kHz overlapping with a LTE carrier | Support of two LTE-CRS overlapping rate matching patterns configured by lte-CRS-PatternList3-r18 and lte-CRS-PatternList4-r18 with two different values of coresetPoolIndex within a part of NR carrier using 15 kHz overlapping with a LTE carrier for the case when crs-RateMatchPerCoresetPoolIndex is configured | 52-2, 16-2a | overlapRateMatchingEUTRA-CRS-patterns-3-4-diff-CS-Pool-r18 | BandNR | N/A | FR1 only | N/A |  | Optional with capability singaling |

### 7.1.x	NR_BWP_wor

| Features | Index | Feature group | Components | Prerequisite feature groups | Field name in TS 38.331 [2] | Parent IE in TS 38.331 [2] | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 53. NR_BWP_wor | 53-3 | Support RLM/BM/BFD measurements based on NCD-SSB within active BWP | 1. UE performs RLM/BM/BFD measurements based on NCD-SSB, where the NCD-SSB is within the active DL BWP. 2. Bandwidth of UE-specific RRC configured BWP may not include bandwidth of the CORESET#0 (if CORESET#0 is present) and CD-SSB for PCell/PSCell (if configured) and bandwidth of the UE-specific RRC configured BWP may not include CD-SSB for Scell 3. NCD-SSB within the active DL BWP can be used as the QCL source for other reference signal. |  | rlm-BM-BFD-NCD-SSB-BWP-MeasActive-BWP-r18 | BandNR | No | No | n/a | This FG is not applicable to RedCap UEs. | Optional with capability signalling |

### 7.1.x	TEI18

| Features | Index | Feature group | Components | Prerequisite feature groups | Field name in TS 38.331 [2] | Parent IE in TS 38.331 [2] | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 55. TEI18 | 55-1 | additionalSR-Periodicities-r18 | Indicates whether the UE supports the following SR periodicities in the periodicityAndOffset parameter as specified in TS 38.331: -5sl for 30 kHz SCS -5sl and 10sl for 120 kHz SCS |  | additionalSR-Periodicities-r18 | Phy-ParametersCommon | No | No | N/A |  | Optional with capability signaling |
| 55. TEI18 | 55-3 | Multiple PUSCHs scheduling by single DCI for non-consecutive slots in FR1 | 1. Multi-PUSCH scheduling by single DCI format 0_1 for the operation with non-contiguous allocation | 10-17 | multiPUSCH-SingleDCI-NonConsSlots-r18 | BandNR | n/a | n/a | n/a |  | Optional with capability signaling |

## 7.2	Layer-2 and Layer-3 features

Editor’s NOTE: Placeholder

### 7.2.0	General

Tables 7.2.1-1 to 7.2.x-1] provide the list of Layer-2 and Layer-3 features, and the corresponding UE capability field name, as specified in TS 38.331 [2].

### 7.2.1	NR_xx

Table 7.2.1-1: Layer-2 and Layer-3 feature list for NR_xx

| Features | Index | Feature group | Components | Prerequisite feature groups | Field name in TS 38.331 [2] | Parent IE in TS 38.331 [2] | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

## 7.3	RF and RRM Features

Editor’s NOTE: Placeholder

### 7.3.0	General

Tables 7.3.1-1 to 7.3.xx-1 provide the list of RF and RRM features, as shown in [18], and the corresponding UE capability field name, as specified in TS 38.331 [2].

### 7.3.1	NR_xx

| Features | Index | Feature group | Components | Prerequisite feature groups | Field name in TS 38.331 [2] | Parent IE in TS 38.331 [2] | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |