3GPP TSG-RAN WG2 Meeting #123 	R2-2308126

Toulouse, France, 21st – 25th May, 2023

Agenda Item:	7.2.4

Source:	Spreadtrum Communications

Title:	Discussion on LPHAP

Document for:	Discussion and Decision

1 Introduction

In last three RAN2 meetings, the following agreements of LPHAP have been reached:

Agreements:

When configured with SRS configuration along with SRS validity area, if the UE reselects to another cell within the SRS validity area during SRS transmission, the UE continues the SRS transmission, subject to validation for SRS transmission.

Wait for RAN1 progress for the validation of SRS transmission with issues such as interference, timing advance and spatial relation information, etc.

Agreements:

RAN2 assume when the UE reselects out of the positioning validity area during SRS transmission, the UE may send an RRC message to the network for SRS configuration request.

LS to RAN3 to confirm this.

Agreement:

Wait for RedCap progress on extending eDRX cycle (from RAN2 perspective).

Agreement:

The SRS validity area configuration contains a list of cells in which it is valid.  FFS validity timer or if we would depend only on explicit release by the network.

Agreements:

RRCRelease can be used to provide SRS configuration with validity area for use in RRC_INACTIVE.

Agreement:

SRS configuration request can be indicated via Msg3/MsgA transmission.  FFS if the request is in the RRC message or an accompanying MAC CE.

Agreement:

Send a reply LS to SA2 to indicate that from the perspective of RAN2, “low power or high accuracy” positioning is out of the Rel-18 WI scope.

Agreements:

Define an SRS for positioning validity-area specific TA timer (e.g., with larger values) for a UE in RRC_INACTIVE state.

- The UE starts/restarts the area-specific TA timer when it receives the TA command.

- The UE stops the SRS transmission when the area-specific TA timer expires.

- The UE stops the area-specific TA timer when it reselects to a cell out of the SRS validity area.

- Other stop/restart conditions can be discussed.

Agreements:

RAN2 consider that the LMF should determine the area-specific SRS configuration.  Details are up to RAN3.

LS to RAN3 to indicate this conclusion, including RAN1 to prompt them for parameters.  To be included in the LS from [AT122][415].

RAN2 will introduce an activation indication and/or request for preconfigured SRS using at least Msg3/MsgA; FFS if Msg1 would be supported also.  FFS RRC signalling or MAC CE for the Msg3/MsgA case, as for the configuration request.  This agreement does not imply that the UE will be allowed to transmit autonomously.

In this contribution, we discuss the question of the power saving enhancement and express our opinions.

2 Discussion

2.1 PRS and DRX alignment

Considering PRS align with DRX enhancement, there are two solutions:

Option 1: PRS align with fixed DRX

Option 2: DRX align with fixed PRS

(e)DRX is configured considering all service requirements of the UE, but positioning only considers positioning service. In additional, in Rel-17, there are on-demand PRS procedure to modify PRS configuration considering UE or LMF requirement. And in option 1, UE/gNB/AMF can transfer DRX configuration to LMF to adjust PRS configuration. And in option 2, gNB/AMF can adjust DRX configuration according to received PRS configuration from LMF.

Both options are feasible. Considering spec impact and reusing legacy on-demand PRS procedures, option 1 is preferred.

Proposal 1: PRS alignment with fixed DRX is preferred by reusing UE-initiated on demand PRS procedure.

2.2 SRS configuration enhancement

Considering SRS configuration enhancement, there are two types of enhancements. One is SRS configuration with validity area. And the other is pre-configured SRS configuration.

For SRS configuration with validity area mechanism, there are two potential options to obtain SRS configuration:

Option 1: One dedicated SRS configuration is associated with each cell.

One is that LMF requests the neighbor cell to reserve the SRS configuration. For instance, LMF request serving cell and neighbor cell to reserve the SRS configuration and transfer the SRS configuration to the UE. Another is that gNB requests the neighbor cell to reserve the SRS configuration. And in the last meeting, SA2 has concluded that LMF sends LPHAP indication to RAN in the NRPPa message. After receiving LPHAP indication, RAN decides to configure SRS configuration with validity area. And the serving cell determines the SRS configuration and requests the neighbor cell to reserve SRS configuration by Xn message. And part will affect the spec of RAN3 and requires the participation of RAN3. And then serving cell can provide the area specific SRS configuration to UE and LMF.

Option 2: share common SRS configuration among the cells within validity area. The serving cell can provide the determined SRS configuration to neighbor cell to reserved SRS resources. And then SRS configuration can be delivered to UE with validity area. for option 2, SRS configuration update may be need to refresh some dynamic parameters such as spatial relation, pathloss reference. This part dependents on RAN1 progress.

For option 1, if each cell has a dedicated SRS configuration within validity area, and when UE reselects to other cell, the network needs to be notified by UE. and it will consume the UE power, which is against the LPHAP enhancement purposes. And RAN1 also have discussed the power control, spatial relation and TA timer issues, which is critical for validity area SRS configuration. From RAN1 agreements [1], we can see that above three issues can be solved even through the SRS configuration is common within a validity area.

RAN1 #113 Agreement For the power control of an SRS for positioning configuration in multiple cells for a UE in RRC_INACTIVE state, when pathloss RS is provided in the configuration, support: - Alt. 2-1 (modified): Reuse the configuration of pathloss RS in Rel-17; - FFS: A CD SSB or non-CD SSB can be configured as pathloss RS - If the UE determines that the pathloss RS cannot be accurately measured, pathloss may be calculated based on the RS resources obtained from SS/PBCH block of the new camping cell that the UE uses to obtain MIB. Agreement For the spatial relation of an SRS for positioning configuration in multiple cells for UEs in RRC_INACTIVE state, when the spatial relation information is provided in the configuration, support: - Alt. 1-1: Reuse the configuration of spatial relation information in Rel-17. - When the UE determines that the configured RS for the spatial relation information cannot be accurately measured, the UE suspends the transmission of the SRS for positioning resource. Agreement For the determination of UL timing to transmit SRS for positioning by UEs in RRC_INACTIVE state within the SRS positioning validity area, support the following to determine a valid TA: - The DL reference timing follows the DL timing of current camping cell. - By default, UE maintains the TA from the last serving cell. - UE can adjust its UL timing according to the change in DL reference timing. - If configured by the network, subject to UE capability, UE autonomously adjusts the TA, when cell-reselection happens. - Send LS to RAN4 asking about feasibility and necessary conditions (e.g. whether the above behaviour applies when the DL reference timing difference between the last camping cell and current camping cell exceeds a threshold and how UE adjusts it, or additional RRM procedure to obtain the timing difference).

Proposal 2: To determine the SRS configuration with validity area mechanism, a common SRS configuration among the cells within validity area can be configured by LMF.

And for Pre-configured SRS, the motivation of having pre-configured SRS is to reduce UE’s wake up duration for receiving SRS configuration in RRC_INACTIVE state. For that, UE-specific SRS configuration can be delivered to UE via dedicated signalling (e.g., RRCRelease or RRCReconfiguration) before UE goes to RRC_INACTIVE state.

Proposal 3: Pre-configured SRS can be sent to UE by dedicated RRC signalling (e.g. RRCRelease or RRCReconfiguration.

3 Conclusions

In this contribution, we discuss the positioning enhancements of Rel-18 LPHAP techniques. And the proposals are given as follows:

Proposal 1: PRS alignment with fixed DRX is preferred by reusing UE-initiated on demand PRS procedure.

Proposal 2: To determine the SRS configuration with validity area mechanism, a common SRS configuration among the cells within validity area can be configured by LMF.

Proposal 3: Pre-configured SRS can be sent to UE by dedicated RRC signalling (e.g. RRCRelease or RRCReconfiguration.

4 References

[1] Chair's notes RAN1#113 v20. 2023. 5