3GPP TSG-RAN WG2 Meeting #123                                        R2-2308137

Toulouse, France, Aug 21th – Aug 25th, 2023

Source:         ZTE Corporation

Title:            Discussion on BW aggregation and RedCap positioning

Agenda item:    7.2.5

Document for:	Discussion and Decision

- Introduction

WID[1] on Expanded and Improved NR Positioning was approved for Rel-18 where items to specify bandwidth aggregation techniques and RedCap positioning are as follows:

- Specify support of positioning for UEs with Reduced Capabilities (RedCap UEs) - Specify support of Frequency Hopping (FH) beyond maximum RedCap UE bandwidth for reception of DL PRS and transmission of UL SRS for positioning [RAN1, RAN2]. - Specify bandwidth aggregation for positioning measurements across up to three intra-band contiguous carriers [RAN1, RAN2, RAN4]. - Specify signalling and procedures to support aggregation of PRS/SRS (respectively) resources across PFLs/carriers (respectively) for positioning measurements under the assumption that the signals over aggregated resources are transmitted and received (respectively) using a single RF chain (same antenna) [RAN1, RAN2]. - NOTE: The support of bandwidth aggregation for positioning measurements applies only to timing related measurements (e.g., RSTD, RTOA, and UE/gNB Rx-Tx time difference).

In this contribution, we provide our views on the configuration of bandwidth aggregation and RedCap positioning from RAN2 perspective.

- Discussion

  - Bandwidth aggregation

    - DL-PRS aggregation for on-demand procedure

During RAN1#112-bis-e[2] meeting, the following conclusion is achieved by RAN1:

Conclusion The details for on-demand PRS on PRS bandwidth aggregation are up to RAN2 and RAN3.

When PRS bandwidth aggregation is supported from network side, the following agreement in RAN1#112[3] has been made:

Agreement - Support LMF-initiated and UE-initiated on-demand PRS request for PRS bandwidth aggregation - FFS details - Support preconfigured on-demand PRS across PFLs for PRS bandwidth aggregations - FFS details

For UE initiated on-demand PRS, Rel-17 supports preconfigured on-demand PRS where maximum 8 PRS configurations can be configured by LMF to UE as shown in the following box abstracted from 37.355. Then UE can further request the preferred configuration ID(s).

NR-On-Demand-DL-PRS-Configurations-r17 ::= SEQUENCE { on-demand-dl-prs-configuration-list-r17 SEQUENCE (SIZE (1..maxOD-DL-PRS-Configs-r17)) OF On-Demand-DL-PRS-Configuration-r17, ... } On-Demand-DL-PRS-Configuration-r17 ::= SEQUENCE { dl-prs-configuration-id-r17 DL-PRS-Configuration-ID-r17, nr-DL-PRS-PositioningFrequencyLayer-r17 NR-DL-PRS-PositioningFrequencyLayer-r16, nr-DL-PRS-Info-r17 NR-DL-PRS-Info-r16, ... }

Alternatively, as shown in the following box abstracted from 37.355, UE can directly request the preferred PRS parameters for each PFL including the preferred PRS bandwidth, repetition factor, comb size, etc.

NR-On-Demand-DL-PRS-Information-r17 ::= SEQUENCE (SIZE (1..nrMaxFreqLayers-r16)) OF NR-On-Demand-DL-PRS-PerFreqLayer-r17 NR-On-Demand-DL-PRS-PerFreqLayer-r17 ::= SEQUENCE { dl-prs-FrequencyRangeReq-r17 ENUMERATED { fr1, fr2, ...}, dl-prs-ResourceSetPeriodicityReq-r17 ENUMERATED { p4, p5, p8, p10, p16, p20, p32, p40, p64, p80, p160, p320, p640, p1280, p2560, p5120, p10240, p20480, p40960, p81920, ...} OPTIONAL, dl-prs-ResourceBandwidthReq-r17 INTEGER (1..63) OPTIONAL, dl-prs-ResourceRepetitionFactorReq-r17 ENUMERATED {n2, n4, n6, n8, n16, n32, ...} OPTIONAL, dl-prs-NumSymbolsReq-r17 ENUMERATED {n2, n4, n6, n12, ...} OPTIONAL, dl-prs-CombSizeN-Req-r17 ENUMERATED {n2, n4, n6, n12, ...} OPTIONAL, dl-prs-QCL-InformationReqTRPlist-r17 DL-PRS-QCL-InformationReqTRPlist-r17 OPTIONAL, ... } dl-prs-ResourceBandwidthReq This field specifies the requested number of PRBs allocated for the DL-PRS Resource (allocated DL-PRS bandwidth) in multiples of 4 PRBs. Integer value 1 corresponds to 24 PRBs, value 2 corresponds to 28 PRBs, value 3 corresponds to 32 PRBs and so on.

Based on the agreement, in the case there is preconfigured on-demand PRS, LMF should inform UE whether/which PFLs are linked/aggregated. Since one On-Demand-DL-PRS-Configuration is associated with one PFL, that is, LMF should inform UE whether/which On-Demand-DL-PRS-Configuration IDs are linked/aggregated.

As shown Step 1 in Figure 1, the grouping information should be informed to UE together with the Rel-17 on-demand PRS configurations. In the figure, it is assumed four on-demand PRS configurations are divided into two groups where configuration#1 and #2 are linked, and #3 and #4 are linked for aggregation.

In step 2, after UE receives the on-demand PRS configurations and the grouping information, UE can request the preferred on-demand PRS configuration ID(s) if the UE does not request PRS aggregation measurement, or can request the preferred group ID(s) if the UE needs PRS aggregation measurement for higher positioning accuracy.

Alternatively, in step 2, UE can directly request the preferred PRS bandwidth value regardless of on-demand PRS configuration where the case of the requested bandwidth larger than the maximum value of the single PFL, e.g. >272 PRBs implies that UE requests PRS bandwidth aggregation measurement for higher positioning accuracy.

Figure 1. On-demand PRS for PRS bandwidth aggregation

For LMF-initiated on-demand PRS, it only supports LMF directly requests preferred PRS parameters including bandwidth. Similar as Option 2 in Figure 1, Rel-18 can be enhanced to support that the requested bandwidth is larger than the maximum value of the single PFL, which implies that LMF requests PRS bandwidth aggregation. Since it depends on RAN3’s design, we suggest to send LS to RAN3 on LMF-initiated on-demand PRS request.

Proposal 1: For UE-initiated on-demand PRS request for PRS bandwidth aggregation, support

- On-demand PRS configuration can include the group information where the linked on-demand PRS configuration IDs are in the same group for aggregation

- UE can request on-demand PRS configuration ID(s) or group ID(s)

- UE can request PRS bandwidth larger than that of a single PFL implying PRS bandwidth aggregation request

Proposal 2: For LMF-initiated on-demand PRS request for PRS bandwidth aggregation, support

- LMF can request PRS bandwidth larger than that of a single PFL implying PRS bandwidth aggregation request

- Send an LS to RAN3

    - DL-PRS aggregation signaling design

In RAN1#112 meeting, the following agreement for DL-PRS aggregation is achieved:

Agreement Support joint measurement and report for the PRS resources aggregated across the PFLs for DL-TDOA and multi-RTT positioning methods - In a measurement report element, single RSTD or single UE Rx-Tx time difference is reported for the PRS resources across aggregated PFLs - FFS: RSRP, RSRPP - FFS: In a measurement report, PFL aggregation indication is supported to indicate whether/which PFLs are aggregated for the PRS measurement - FFS whether to use PRS assistance data or use location information request message to indicate UE to perform joint measurement across aggregated PFLs - FFS RSTD reference configuration or report should be enhanced

In the RAN1#112-bis meeting, the following agreement for DL-PRS aggregation is achieved:

Agreement For PRS resources aggregated across PFLs for DL-TDOA and multi-RTT positioning methods, use similar signaling as the existing Rel-16/Rel-17 DL PRS measurement of single PFL with the necessary update. - FFS: In a measurement report element, single RSRP or single RSRPP is reported - In a measurement report element, PFL aggregation indication is supported to indicate whether/which measurement is aggregated - Support new signaling in location information request message to indicate UE whether to perform joint measurement across aggregated PFLs - Single RSTD reference in assistance data and measurement report is used for PRS bandwidth aggregation measurement - FFS RSTD reference is aggregated or not

In the latest RAN1#113[4] meeting, the following agreement for DL-PRS aggregation is achieved:

Agreement For PRS bandwidth aggregation across PFLs, support - Option 2: Per TRP basis and per PRS resource set basis. - For each TRP, support new signaling to indicate which PRS resource sets across PFLs are linked. - It is assumed that the PRS resources across the linked PRS resource sets are linked if the conditions are satisfied. For the non-linked PRS resource sets, no aggregation is assumed even if the conditions are satisfied. Agreement For PRS bandwidth aggregation across PFLs, in a measurement report element, support - Single RSRP or single RSRPP - FFS: the single RSRP/RSRPP is based on aggregated PRS resources across aggregated PFLs - The aggregated reference RSTD - The used PRS resource set IDs for the aggregated measurement which are shared for RSRP/RSRPP and/or timing measurement results

Provide assistance data

In DL assistance data, a TRP can be associated with at most 8 PRS resource sets, in which, up to 2 sets are within 1 PFL. The aggregation indication is per TRP per PRS resource sets among two or three PFLs. Since that the agreement says a new signaling of linking resource sets is agreed, in order to facilitate the signaling design, we can enumerate the following cases with the following basic assumption:

Assumption: a TRP is associated with 4 PFLs and 8 PRS resource sets, in which PRS resource 1 and 2 belong to PFL1, PRS resource set 3 and 4 belong to PFL2, PRS resource set 5 and 6 belong to PFL3, PRS resource set 7 and 8 belong to PFL4.

- Case 1: One TRP is associated with a single combination of aggregated PFLs, e.g., PFL1 and PFL2 are linked;

  - Case 1-1: only one pair of PRS resource sets is linked in the single combination of aggregated PFLs, e.g., PRS resource set 1 and PRS resource set 3 are linked;

- Figure 2. example of case 1-1

  - Case 1-2: up to two pair of PRS resource sets are linked in the single combination of aggregated PFLs, e.g., PRS resource set 1 and PRS resource set 3 are linked, PRS resource set 2 and PRS resource set 4 are linked;

- Figure 3. example of case 1-2

- Case 2: One TRP is associated with more than one combinations of aggregated PFLs, e.g., PFL1 and PFL2 are linked, PFL1, PFL3 and PFL4 are linked;

  - Case 2-1: Multiple pairs of PRS resource sets are linked, in which, one PRS resource set ID is contained in only one combination of aggregated PFLs. E.g., PRS resource set 1 and PRS resource set 3 are linked for PFL1&PFL2, PRS resource set 2, PRS resource set 4 and PRS resource set 6 are linked for PFL1&PFL2&PFL3;

- Figure 4. example of case 2-1

  - Case 2-2: Multiple pairs of PRS resource sets are linked, in which, one PRS resource set ID is contained in more than one combinations of aggregated PFLs. E.g., PRS resource set 1 and PRS resource set 3 are linked for PFL1&PFL2, PRS resource set 1, PRS resource set 4 and PRS resource set 6 are linked for PFL1&PFL2&PFL3.

- Figure 5. example of case 2-2

- RAN1 does not define whether it is allowed to let one TRP have multiple linkages. Therefore, it is not sure whether all the above cases are valid or not, it depends on which level of flexibility the feature wants to achieve. In addition, linked PRS resource sets can be seen to be transmitted from a same TRP RF chain. For Case 2-2, if a PRS resource set of a PFL is linked twice by different PFLs, how can the TRP send the PRS resource set using aggregation? So we suggest to ask RAN1 for the confirmation.

- Observation 1: There can be one or more PFL aggregation linkages per TRP, within each PFL aggregation linkage, there can be one or more pairs of PRS resource sets to be linked.

- Proposal 3: Send LS to RAN1 to confirm which of the following case(s) should be supported in Rel-18:

- Case 1: One TRP is associated with a single combination of aggregated PFLs

- Case 1-1: only one pair of PRS resource sets is linked in the single combination of aggregated PFLs

- Case 1-2: up to two pair of PRS resource sets are linked in the single combination of aggregated PFLs

- Case 2: One TRP is associated with more than one combinations of aggregated PFLs

- Case 2-1: Multiple pairs of PRS resource sets are linked, in which, one PRS resource set ID is contained in only one combination of aggregated PFLs

- Case 2-2: Multiple pairs of PRS resource sets are linked, in which, one PRS resource set ID is contained in more than one combinations of aggregated PFLs.

- The PRS resource set ID is uniquely defined in one TRP, and there is no PFL ID in the current LPP spec. So the PFL link should be represented as PRS resource set ID link in a TRP. Since there may be multiple PFL links and multiple PRS resource set links per TRP(if confirmed by RAN1), one example of signaling design is as follows, which is to introduce a common IE DL-PRS-AggreIndicationList to be quoted at other places. The common IE indicates the linkage relationship of PRS resource sets in a TRP.

- – DL-PRS-AggreIndicationList The IE dl-PRS-AggreIndicationList provides the aggregation relationship of DL-PRS resource sets within a TRP. -- ASN1START DL-PRS-AggreIndicationList-r18 ::= SEQUENCE (SIZE(1..maxAggrePerTrp-r17)) OF DL-PRS-AggreIndication-r18 DL-PRS-AggreIndication-r18 ::= SEQUENCE (SIZE(2..3)) OF NR-DL-PRS-ResourceSetID-r16 -- ASN1STOP - NR-DL-PRS-Info field descriptions. - DL-PRS-AggreIndicationList This field indicates the combination(s) of aggregated PRS resource sets within a TRP.. - DL-PRS-AggreIndication This field indicates each combination of aggregated PRS resource sets in the TRP. Each combination of aggregated PRS resource sets is consist of two or three NR-DL-PRS-ResourceSetID, which indicates two or three frequency layers (where the NR-DL-PRS-ResourceSetID belongs to) of the TRP are aggregated.. - – NR-DL-PRS-AssistanceData <omitted> NR-DL-PRS-AssistanceDataPerTRP-r16 ::= SEQUENCE { dl-PRS-ID-r16 INTEGER (0..255), nr-PhysCellID-r16 NR-PhysCellID-r16 OPTIONAL, -- Need ON nr-CellGlobalID-r16 NCGI-r15 OPTIONAL, -- Need ON nr-ARFCN-r16 ARFCN-ValueNR-r15 OPTIONAL, -- Need ON nr-DL-PRS-SFN0-Offset-r16 NR-DL-PRS-SFN0-Offset-r16, nr-DL-PRS-ExpectedRSTD-r16 INTEGER (-3841..3841), nr-DL-PRS-ExpectedRSTD-Uncertainty-r16 INTEGER (0..246), nr-DL-PRS-Info-r16 NR-DL-PRS-Info-r16, ..., [[ prs-OnlyTP-r16 ENUMERATED { true } OPTIONAL -- Need ON ]], [[ nr-DL-PRS-ExpectedAoD-or-AoA-r17 NR-DL-PRS-ExpectedAoD-or-AoA-r17 OPTIONAL -- Need ON ]] [[ dl-PRS-AggreIndicationList-r18 DL-PRS-AggreIndicationList-r18 OPTIONAL -- Need ON ]] }

- The one or more pair of linked PRS resource set within one aggregated PFLs can be combined freely(as the example signaling above), or can be restricted with rule. The rule can be: the first PRS resource sets in those aggregated PFLs are assumed to be linked, the second PRS resource sets in those aggregated PFLs are assumed to be linked. In this way, the linkage signaling can be simplified. Only a PFL ID linage IE configured per TRP will be introduced (in this case, PFL ID should be firstly introduced).

- Proposal 4: In ProvideAssistanceData for each TRP, introduce one of the following linage signaling:

- one or more lists of DL-PRS resource set IDs which are aggregated; or

- one or more lists of PFL ID which are aggregated, and the first PRS resource sets in those aggregated PFLs are assumed to be linked, the second PRS resource sets in those aggregated PFLs are assumed to be linked.

- Note: This can be done after receiving the RAN1’s reply LS.

- Request location information

In request location information message, for each TRP, the LMF should indicate UE whether to perform joint measurement across aggregated PFLs. The signaling structure of the request message in Request location information should be the same as that of the linkage provision message in provide assistance data.

measurement report

Currently for multi-RTT, each Rx-Tx time difference measurement is associated with a TRP ID and optionally a DL-PRS resource set ID. The aggrement says an indication should be introduced in each NR-Multi-RTT-MeasElement indicating the aggregation status of the measurement element. Therefore, we can introduce a new list of DL-PRS resource set ID to indicate the Rx-Tx time difference measurement is performed using the aggregation of the DL-PRS resource sets. The new list can be optional. If the new list of DL-PRS resource set ID is present, the original DL-PRS resource set ID should be absent.

Currently for each measurement report in DL-TDOA, there is one reference TRP and several neighbor TRPs, each measurement element is associated with a neighbor TRP. RAN1 has agreed that RSTD reference is aggregated, so a new list of DL-PRS resource set ID should be introduced to indicate aggregation per reference TRP and per RSTD measurement element(per neighbor TRP). As for the condition, it is not clear whether a measurement of a reference TRP with aggregation can make RSTD with a measurement of a neighbor TRP without a aggregation, and vise versa. We suggest to confirm with RAN1 since this has impact to the condition design.

Proposal 5: For multi-RTT reporting, support to introduce a new list of DL-PRS resource set ID to indicate the Rx-Tx time difference measurement is performed using the aggregation of the DL-PRS resource sets.

Proposal 6: For DL-TDOA reporting, send LS to RAN1 to ask whether a measurement of a reference TRP with aggregation can make RSTD with a measurement of a neighbor TRP without a aggregation, and vise versa.

    - Reporting granularity of DL-PRS aggregation

RAN4’s newest LS R4-2310166 indicates the additional reporting granularity for DL-PRS aggregation as follows:

Agreements: For FR1 the additional reporting granularity values are 0.5 Tc, 1 Tc and 2 Tc. For FR2 the additional reporting granularity values are 0.25 Tc and 0.5 Tc. The above reporting granularity values apply to both UE and gNB positioning measurements. Send LS to RAN2 and RAN3 (and CC to RAN1) to define signaling for UE and gNB positioning measurement reporting respectively. RAN4 respectfully asks RAN2 and RAN3 to take the above information into account and define signaling support for the additional reporting granularity corresponding to k=-1 and k=-2.

RAN4 discussed the smaller report granularity when PRS/SRS is aggregated to be a larger bandwidth. The reporting granularity is included in NR-DL-TDOA-ReportConfig-r16 and NR-Multi-RTT-ReportConfig-r16, in which the resolution step is 2k×Tc. The example signaling is given as follows:

For DL-TDOA: NR-DL-TDOA-ReportConfig-r16 ::= SEQUENCE { maxDL-PRS-RSTD-MeasurementsPerTRPPair-r16 INTEGER (1..4) OPTIONAL, -- Need ON timingReportingGranularityFactor-r16 INTEGER (0..5) OPTIONAL, -- Need ON ..., [[ measureSameDL-PRS-ResourceWithDifferentRxTEGs-r17 ENUMERATED { n0, n2, n3, n4, n6, n8, ... } OPTIONAL, -- Need ON reducedDL-PRS-ProcessingSamples-r17 ENUMERATED { requested, ... } OPTIONAL, -- Need ON lowerRxBeamSweepingFactor-FR2-r17 ENUMERATED { requested } OPTIONAL, -- Need ON ]], [[ AdditionalTimingReportingGranularityFactor-r18 INTEGER (-2..5) OPTIONAL -- Need ON ]] } AdditionalTimingReportingGranularityFactor This field specifies the recommended reporting granularity for the DL RSTD measurements. Value (-2..5) corresponds to K=-2..K=5 used for nr-RSTD and nr-RSTD-ResultDiff in NR-DL-TDOA-MeasElement. The UE may select a different granularity value for nr-RSTD and nr-RSTD-ResultDiff. If this field is present, the target device shall ignore the timingReportingGranularityFactor.. For multi-RTT: NR-Multi-RTT-ReportConfig-r16 ::= SEQUENCE { maxDL-PRS-RxTxTimeDiffMeasPerTRP-r16 INTEGER (1..4) OPTIONAL, -- Need ON timingReportingGranularityFactor-r16 INTEGER (0..5) OPTIONAL, -- Need ON [[ AdditionalTimingReportingGranularityFactor-r18 INTEGER (-2..5) OPTIONAL -- Need ON ]] } AdditionalTimingReportingGranularityFactor This field specifies the recommended reporting granularity for the UE Rx-Tx time difference measurements. Value (-2..5) corresponds to K=-2..K=5 used for nr-UE-RxTxTimeDiff and nr-UE-RxTxTimeDiffAdditional in NR-Multi-RTT-MeasElement. The UE may select a different granularity value for nr-UE-RxTxTimeDiff and nr-UE-RxTxTimeDiffAdditional. If this field is present, the target device shall ignore the timingReportingGranularityFactor..

Proposal 7: Agree the proposed TP to capture the additional reporting granularity into LPP spec.

    - SRS aggregation in RRC_CONNECTED

RAN1#113 has agreed the following for SRS aggregation:

Agreement For SRS bandwidth aggregation across two or three carriers, support Option 2: Per SRS resource set basis. Support new signaling to indicate which SRS resource sets across carriers are linked. It is assumed that the SRS resources across the linked SRS resource sets are linked if the conditions are satisfied. For the non-linked SRS resource sets, no aggregation is assumed even if the conditions are satisfied.

In RRC_CONNECTED state, SRS is currently configured per UL BWP per serving cell, and SRS resource set ID is global for a single UL BWP. If SRS resource set is going to be linked for 2 or 3 carriers, the linkage signaling in the SRS resource set should include SRS resource set ID to be aggregated, UL BWP ID and serving cell ID where the aggregated SRS resource set is configured in.

Proposal 8: For SRS aggregation in RRC_CONNECTED, support to introduce (1)SRS resource set ID to be aggregated, (2)UL BWP ID and (3)serving cell ID in the SRS resource set.

    - SRS aggregation in RRC_INACTIVE

In Rel-17, semi-persistent and periodic SRS for positioning are supported for UE in RRC inactive mode. The positioning SRS configured by RRCRelease signaling, and the details are shown in following according to 38.331, where SRS configuration for NUL or SUL can be within or outside the initial BWP. It is noted that the SRS is assumed within the same serving cell of the initial BWP.

SRS-PosRRC-InactiveConfig-r17 ::=       SEQUENCE {

srs-PosConfigNUL-r17                    SRS-PosConfig-r17                                                   OPTIONAL,    -- Need R

srs-PosConfigSUL-r17                    SRS-PosConfig-r17                                                   OPTIONAL,    -- Need R

bwp-NUL-r17                             BWP                                                                 OPTIONAL,    -- Need S

bwp-SUL-r17                             BWP                                                                 OPTIONAL,    -- Need S

inactivePosSRS-TimeAlignmentTimer-r17   TimeAlignmentTimer                                                  OPTIONAL,    -- Need M

inactivePosSRS-RSRP-changeThreshold-r17 RSRP-ChangeThreshold-r17                                            OPTIONAL     -- Need M

}

RAN1#113 has the following agreement for SRS bandwidth aggregation in UE in RRC_INACTIVE state:

Agreement To support intra-band contiguous SRS bandwidth aggregation for UE in RRC_INACTIVE state, frequency information (e.g. point A, offset to carrier) of one or two additional carriers with respective SRS configurations should be provided to the UE, where the newly introduced carrier(s) and the carrier of the initial BWP should be intra-band contiguous carriers.

To support SRS carrier aggregation in RRC_INACTIVE state, one or two additional SRS configuration should be introduced and informed by RRCRelease signaling,each one is associated with a carrier other than the carriers of the initial BWP. Because intra-band contiguous carrier aggregation is only supported among NUL carriers, the newly introduced SRS configuration only for NUL is sufficient. An example signaling design is as follows:

SRS-PosRRC-Inactive-r17 ::= OCTET STRING (CONTAINING SRS-PosRRC-InactiveConfig-r17)

SRS-PosRRC-InactiveConfig-r17 ::=       SEQUENCE {

srs-PosConfigNUL-r17                    SRS-PosConfig-r17                                                   OPTIONAL,    -- Need R

srs-PosConfigSUL-r17                    SRS-PosConfig-r17                                                   OPTIONAL,    -- Need R

bwp-NUL-r17                             BWP                                                                 OPTIONAL,    -- Need S

bwp-SUL-r17                             BWP                                                                 OPTIONAL,    -- Need S

inactivePosSRS-TimeAlignmentTimer-r17   TimeAlignmentTimer                                                  OPTIONAL,    -- Need M

inactivePosSRS-RSRP-ChangeThreshold-r17 RSRP-ChangeThreshold-r17                                            OPTIONAL     -- Need M

}

AggreSRS-PosRRC-InactiveConfig-r18 ::=  SEQUENCE (SIZE(1..2)) OF AdditionalAggreSRS-PosRRC-Inactive-r18                                OPTIONAL,-- Need N

AdditionalAggreSRS-PosRRC-Inactive-r18::=       SEQUENCE {

AggreSrs-PosConfig-r18             SRS-PosConfig-r17               OPTIONAL,    -- Need R

AggreCellFreq-r18                    SEQUENCE {

offsetToPointAforSRSAggre                      INTEGER (0..2199)

offsetToCarrierforSRSAggre                    INTEGER (0..2199)

}                                                                          OPTIONAL,    -- Need M

}

Proposal 9: For SRS aggregation in RRC_INACTIVE, support to introduce several additional SRS configurations in RRCRelease signaling, each additional SRS configuration is associated with one carrier other than the carriers of the initial BWP.

    - MAC CE for SP-SRS aggregation

RAN1 sends RAN2 a newest LS R1-2306214[5], indicating that:

Working assumption: For semi-persistent positioning SRS for bandwidth aggregation, a single MAC CE can activate or deactivate: SRS resource set(s) in one or two or three of three aggregated carriers SRS resource set(s) in one or two of two aggregated carriers. Note: the single spatial relation is indicated by the MAC CE for each of two or three aggregated SRS resources. ACTION: RAN1 respectfully asks RAN2 to check the feasibility of the working assumption and inform RAN1 of RAN2’s conclusion on the feasibility.

RRC has provided the linkage relationship in advance. This MAC CE is to indicate the aggregation status.

Here the MAC CE is required to indicate several states for three aggregated carriers: 2 of the three carriers, 3 of the three carriers. Since the basic assumption is that the aggregation should be intra-band continuous carriers, as the following figure, 2 of the three aggregated carriers should only have 2 choices. So the total states that the MAC CE is required to indicate is 3.

Figure 6. 2 of the three carriers and 3 of the three carriers

However the legacy SP Positioning SRS Activation/Deactivation MAC CE only have a “R” field which is 1 bit, so the legacy SP Positioning SRS Activation/Deactivation MAC CE can not be used for aggregation purpose. Based on this, a new MAC CE should be introduced to indicate the carrier aggregation. The UE behaviour should be:

- If UE receives the legacy MAC CE containing the SRS resource set that has linkage via RRC signaling, UE will assume the SRS resource set should be transmitted as non-aggregated way (single carrier);

- If UE receives the new MAC CE containing the SRS resource set that has linkage via RRC signaling, UE will send the aggregated SRS resource sets as indicated by RRC and the MAC CE.

Accordingly, RAN2 should response to RAN1 that the feasibility of this working assumption can be assumed and a new MAC CE will be introduced.

Proposal 10: Introduce a new MAC CE to indicate activation or deactivation of a SP SRS bandwidth aggregation. Reply LS to RAN1 that the working assumption can be confirmed with validation.

    - Relationship with communication CA

In RAN1#112bis-e meeting, the following agreement was made.

Agreement At least from UE capability perspective, the UE support of positioning SRS bandwidth aggregation in RRC_CONNECTED state is decoupled from the UE support of communication CA.

For UE capability, it may be easy to introduce a new BandCombinationList-PosCA-r18, where only one band for intra-band contiguous CA is included in each BandCombination. For RRC configuration of SRS in RRC_CONNECTED, we think the existing RRC structure can be reused.

Proposal 11: For positioning SRS bandwidth aggregation in RRC_CONNECTED state which can be decoupled from communication CA, support to introduce a new UE capability BandCombinationList-PosCA-r18.

  - RedCap positioning

    - DL-PRS hopping

RAN1#112 has confirmed to support frequency hopping of positioning PRS as follows:

Agreement For positioning for RedCap UEs with DL PRS Rx Hopping, the UE hops within a DL PRS resource - FFS: whether there is specification update needed for RAN1 - FFS: remaining details

- RAN1 confirms that hopping is performed at receiver side. For PRS, the gNB sends PRS with a normal bandwidth, meanwhile at UE side, the frequency domain resource of receiving one PRS is divided into several parts, each part corresponds to a frequency hop, and several hops are received in different time periods, respectively. There are 2 cases of PRS repetition:

- If a PRS is inter-slot repeated with a repetition factor, UE should receive different frequency parts per slot per repetition instance;

- If a PRS is intra-slot configured and PRS symbol is larger than PRS comb size, the PRS is actually per comb-size-symbol(s) repeated within a slot, UE should receive different frequency parts per per comb-size-symbol(s) of a PRS resource within a slot.

- For the above 2 cases, RedCap UE can use on-demand PRS request to request hopping. Currently on-demand PRS request contains PRS bandwidth, PRS repetition factor, PRS symbol and PRS comb size. We think these IEs (and the value ranges of these IEs) are enough for RedCap UE to indicate NW of PRS reception hopping, for example, since NW knows the RedCap UE identity and the maximum bandwidth it supports (according to the reported UE capability), if UE requests a PRS bandwidth larger than its UE capability, it implies UE requests a Rx PRS hopping, the requested PRS repetition factor implies the number of inter-slot Rx hopping, and the requested PRS comb size and PRS symbols imply the number of intra-slot Rx hopping. If UE requests a PRS bandwidth equal or smaller than its UE capability, it implies UE does not request a Rx PRS hopping.

- Proposal 12: RedCap UE can use on-demand PRS procedure to request Rx PRS hopping without additional specification impact in RAN2, i,e., if UE requests a PRS bandwidth larger than its UE capability, it implies UE requests a Rx PRS hopping.

Since RAN1 agreed on the per-PRS resource Rx hopping, we suggest to introduce one or more of the following parameters per PRS resource in the DL-PRS configuration in the assistance data, in order to indicate UE on how to receive the PRS resource using hopping.

- Number of hops for a DL PRS resource.

- Number of RBs per hop.

- Number of overlapped RB between hops. This parameter is used for phase adjustment between adjacent hops. It will affect positioning accuracy.

Proposal 13: For Rx PRS hopping, support to introduce one or more of the following parameters per PRS resource in the DL-PRS configuration in the assistance data:

- Number of hops

- Number of RBs per hop

- Number of overlapped RB between hops

The DL-PRS configuration in the assistance data can be via broadcast or dedicate LPP signaling.

When PRS configuration is broadcasted with DL-PRS frequency hopping for all UEs, but the RedCap UE’s location accuracy requirement is not high, the RedCap UE does not have to perform hopping and combine the measurement of multiple hops. In this case, the RedCap UE should report which hop(s) the UE is used for the measurement.

When PRS configuration with DL-PRS frequency hopping is given to the RedCap UE via LPP dedicate signaling, the RedCap UE may not be able to perform hopping and combine the measurement of multiple hops all the time due to collision. Therefore, a fallback behaviour is needed to let UE report the corresponding hop indication for each measurement result.

Since there is no hop ID, we suggest UE to report hops using the corresponding frequency range.

RAN1 has agreed the following in RAN1#113:

Agreement For DL Rx hopping or UL Tx hopping, support the UE or gNB to report the following: A single measurement based on receiving multiple hops of the DL PRS or UL SRS for positioning One measurement where a measurement is associated with one received hop FFS: indication of how many received hops / which received hops where used in the measurement report. Note: no new measurement definition is introduced in RAN1 FFS: conditions when the above measurements are reported, and whether the above measurements can be reported together

Expect reporting measurement result with all hops and reporting measurement result with only one hop, there is still FFS on whether to let UE report which received hops were used in the measurement report. RAN2 should wait for RAN1’s conclusion on the FFS before designing the report signaling.

Proposal 14: RedCap UE should report the corresponding hop indication for measurement result, wherein the hop indication includes the frequency range.

    - UL-SRS hopping

RAN1#113 has agreed the following:

Agreement SRS Tx Frequency hopping is supported for both RRC_CONNECTED and RRC_INACTIVE state.

For positioning SRS hopping in RRC_CONNECTED, RAN1’s consensus is not to use BWP switching to perform hopping because of latency consideration. From signaling configuration view, the SRS for RedCap UE Tx hopping can be configured as a new SRS configuration within the legacy SRS-Config IE. The new SRS configuration should have the new bandwidth which is larger than the current BWP, and the new SRS configuration should indicate UE’s hopping behaviour.

For positioning SRS hopping in RRC_INACTIVE, a new SRS configuration can be introduced in RRC Release with separate BWP configuration.

Proposal 15: For RRC_CONNECTED state, support to configure a new SRS configuration within the legacy SRS-Config IE to indicate SRS hopping for RedCap UE;

For RRC_INACTIVE state, support to configure a new SRS configuration in RRC_Release to indicate SRS hopping for RedCap UE.

- Conclusion

- In this section we provide the following proposals:

- For bandwidth aggregation:

- DL-PRS aggregation- On-demand:

Proposal 1: For UE-initiated on-demand PRS request for PRS bandwidth aggregation, support

- On-demand PRS configuration can include the group information where the linked on-demand PRS configuration IDs are in the same group for aggregation

- UE can request on-demand PRS configuration ID(s) or group ID(s)

- UE can request PRS bandwidth larger than that of a single PFL implying PRS bandwidth aggregation request

Proposal 2: For LMF-initiated on-demand PRS request for PRS bandwidth aggregation, support

- LMF can request PRS bandwidth larger than that of a single PFL implying PRS bandwidth aggregation request

- Send an LS to RAN3

- DL-PRS aggregation- signaling design:

- Observation 1: There can be one or more PFL aggregation linkages per TRP, within each PFL aggregation linkage, there can be one or more pairs of PRS resource sets to be linked.

- Proposal 3: Send LS to RAN1 to confirm which of the following case(s) should be supported in Rel-18:

- Case 1: One TRP is associated with a single combination of aggregated PFLs

- Case 1-1: only one pair of PRS resource sets is linked in the single combination of aggregated PFLs

- Case 1-2: up to two pair of PRS resource sets are linked in the single combination of aggregated PFLs

- Case 2: One TRP is associated with more than one combinations of aggregated PFLs

- Case 2-1: Multiple pairs of PRS resource sets are linked, in which, one PRS resource set ID is contained in only one combination of aggregated PFLs

- Case 2-2: Multiple pairs of PRS resource sets are linked, in which, one PRS resource set ID is contained in more than one combinations of aggregated PFLs.

- Proposal 4: In ProvideAssistanceData for each TRP, introduce one of the following linage signaling:

- one or more lists of DL-PRS resource set ID which are aggregated; or

- one or more lists of PFL ID which are aggregated, and the first PRS resource sets in those aggregated PFLs are assumed to be linked, the second PRS resource sets in those aggregated PFLs are assumed to be linked.

- Note: This can be done after receiving the RAN1’s reply LS.

Proposal 5: For multi-RTT reporting, support to introduce a new list of DL-PRS resource set ID to indicate the Rx-Tx time difference measurement is performed using the aggregation of the DL-PRS resource sets.

Proposal 6: For DL-TDOA reporting, send LS to RAN1 to ask whether a measurement of a reference TRP with aggregation can make RSTD with a measurement of a neighbor TRP without a aggregation, and vise versa.

DL-PRS aggregation- Additional reporting granularity:

Proposal 7: Agree the proposed TP to capture the additional reporting granularity into LPP spec.

SRS aggregation：

Proposal 8: For SRS aggregation in RRC_CONNECTED, support to introduce (1)SRS resource set ID to be aggregated, (2)UL BWP ID and (3)serving cell ID in the SRS resource set.

Proposal 9: For SRS aggregation in RRC_INACTIVE, support to introduce several additional SRS configurations in RRCRelease signaling, each additional SRS configuration is associated with one carrier other than the carriers of the initial BWP.

Proposal 10: Introduce a new MAC CE to indicate activation or deactivation of a SP SRS bandwidth aggregation. Reply LS to RAN1 that the working assumption can be confirmed with validation.

Proposal 11: For positioning SRS bandwidth aggregation in RRC_CONNECTED state which can be decoupled from communication CA, support to introduce a new UE capability BandCombinationList-PosCA-r18.

- For RedCap:

- DL-PRS hopping：

- Proposal 12: RedCap UE can use on-demand PRS procedure to request Rx PRS hopping without additional specification impact in RAN2, i,e., if UE requests a PRS bandwidth larger than its UE capability, it implies UE requests a Rx PRS hopping.

Proposal 13: For Rx PRS hopping, support to introduce one or more of the following parameters per PRS resource in the DL-PRS configuration in the assistance data:

- Number of hops

- Number of RBs per hop

- Number of overlapped RB between hops

Proposal 14: RedCap UE should report the corresponding hop indication for measurement result, wherein the hop indication includes the frequency range.

- SRS hopping：

Proposal 15: For RRC_CONNECTED state, support to configure a new SRS configuration within the legacy SRS-Config IE to indicate SRS hopping for RedCap UE;

For RRC_INACTIVE state, support to configure a new SRS configuration in RRC_Release to indicate SRS hopping for RedCap UE.

- References

- RP-2301460, Revised WID on Expanded and Improved NR Positioning, 2023-06.

- Report of 3GPP TSG RAN WG1 meeting #112-bis-e, 2023-04.

- Report of 3GPP TSG RAN WG1 meeting #112, 2023-03.

- Report of 3GPP TSG RAN WG1 meeting #113, 2023-05.

- R1-2306214, LS to RAN2 on SRS bandwidth aggregation for positioning, RAN1, 2023-05.