3GPP TSG-RAN WG2 Meeting #123                                  	    R2-2308150

Toulouse, France, 21- 25 August, 2023

Agenda Item:	7.16.2.1

Source: 	MediaTek Inc.

Title:  	AI/ML LCM Dependency on gNB Configuration

Document for:	Discussion and decision

Introduction

In this contribution, we examine the TP that documents the progress of RAN1 studies along with the agreed consensus within RAN1. We investigate instances where the AI/ML algorithm depends on the gNB configuration and discuss its potential impacts on LCM.

Discussion

AI/ML Algorithm Dependency on gNB Configuration

AI/ML functionality-based LCM and Model-ID based LCM

For AI/ML functionality identification and functionality-based LCM of UE-side models and/or UE-part of two-sided models, functionality refers to an AI/ML-enabled Feature/FG enabled by configuration(s), where configuration(s) is(are) supported based on conditions indicated by UE capability. Correspondingly, functionality-based LCM operates based on, at least, one configuration of AI/ML-enabled Feature/FG or specific configurations of an AI/ML-enabled Feature/FG. For AI/ML model identification and model-ID-based LCM of UE-side models and/or UE-part of two-sided models, model-ID-based LCM operates based on identified models, where a model may be associated with specific configurations/conditions associated with UE capability of an AI/ML-enabled Feature/FG and additional conditions (e.g., scenarios, sites, and datasets) as determined/identified between UE-side and NW-side.

Observation 1: Each AI/ML functionality is associated with one, or a specific set of, configurations. Similarly, every AI/ML model corresponds with specific configurations or conditions - such as scenarios, sites, and datasets.

Observation 2: It is crucial to ensure alignment between the UE side and the network side regarding the specific configurations associated with the AI/ML functionality or model. This alignment can be achieved through mechanisms like UE capability reporting or functionality/model identification.

Generalization performance evaluation

CSI Feedback Enhancement In order to study the verification of generalization, the following aspects are encouraged to be reported: - The configuration(s)/scenario(s) for training dataset, including potentially the mixed training dataset from multiple configurations/scenarios - The configuration(s)/scenario(s) for testing/inference - The detailed list of configuration(s) and/or scenario(s) The following cases are considered for verifying the generalization performance of an AI/ML model over various scenarios/configurations: - Case 1: The AI/ML model is trained based on training dataset from one Scenario#A/Configuration#A, and then the AI/ML model performs inference/test on a dataset from the same Scenario#A/Configuration#A - Case 2: The AI/ML model is trained based on training dataset from one Scenario#A/Configuration#A, and then the AI/ML model performs inference/test on a different dataset than Scenario#A/Configuration#A, e.g., Scenario#B/Configuration#B, Scenario#A/Configuration#B - Case 3: The AI/ML model is trained based on training dataset constructed by mixing datasets from multiple scenarios/configurations including Scenario#A/Configuration#A and a different dataset than Scenario#A/Configuration#A, e.g., Scenario#B/Configuration#B, Scenario#A/Configuration#B, and then the AI/ML model performs inference/test on a dataset from a single Scenario/Configuration from the multiple scenarios/configurations, e.g., Scenario#A/Configuration#A, Scenario#B/Configuration#B, Scenario#A/Configuration#B. - Note: Companies to report the ratio for dataset mixing - Note: number of the multiple scenarios/configurations can be larger than two To verify the generalization performance of an AI/ML model over various scenarios, the set of scenarios are considered focusing on one or more of the following aspects: - Various deployment scenarios (e.g., UMa, UMi, InH) - Various outdoor/indoor UE distributions for UMa/UMi (e.g., 10:0, 8:2, 5:5, 2:8, 0:10) - Various carrier frequencies (e.g., 2GHz, 3.5GHz) - Other aspects of scenarios are not precluded, e.g., various antenna spacing, various antenna virtualization (TxRU mapping), various ISDs, various UE speeds, etc. - Companies to report the selected scenarios for generalization verification To verify the generalization/scalability performance of an AI/ML model over various configurations (e.g., which may potentially lead to different dimensions of model input/output), the set of configurations are considered focusing on one or more of the following aspects: - Various bandwidths (e.g., 10MHz, 20MHz) and/or frequency granularities, (e.g., size of subband) - Various sizes of CSI feedback payloads, FFS candidate payload number - Various antenna port layouts, e.g., (N1/N2/P) and/or antenna port numbers (e.g., 32 ports, 16 ports) - Various UE speeds (e.g., 10km/h, 30km/h, 60km/h, 120km/h, etc.) for CSI prediction sub use case - Other aspects of configurations are not precluded, e.g., various numerologies, various rank numbers/layers, etc. - Companies to report the selected configurations for generalization verification - Companies are encouraged to report the method to achieve generalization over various configurations to achieve scalability of the AI/ML input/output, including pre-processing, post-processing, etc Beam Management In the context of model generalization, scenarios may mean various deployment scenarios, various outdoor/indoor UE distributions, various UE mobility assumptions. Similarly, configurations may mean various UE parameters, various gNB settings, Various Set B of beam(pairs). The selected scenarios/configurations for generalization verification may consider the AI model inference node (e.g., @UE or @gNB) and use case (e.g., BM-Case1, or BM-Case2). Specifically, conside the following: - Scenarios: - Various deployment scenarios, e.g., UMa, UMi and others; e.g., 200m ISD or 500m ISD and others; e.g., same deployment, different cells with different configuration/assumption; e.g., gNB height and UE height; - Various outdoor/indoor UE distributions, e.g., 100%/0%, 20%/80%, and others - Various UE mobility, e.g., 3km/h, 30km/h, 60km/h and others - Configurations (parameters and settings): - Various UE parameters, e.g., number of UE Rx beams (including number of panels and UE antenna array dimensions) - Various gNB settings, e.g., DL Tx beam codebook (including various Set A of beam(pairs) and gNB antenna array dimensions) - Various Set B of beam (pairs) - T1 for measurement /T2 for prediction for BM-Case2 - Other scenarios/configurations(parameters and settings) are not precluded and can be reported Companies to report the selected scenarios/configurations for generalization verification. Note: other approaches for achieving good generalization performance for AI/ML-based schemes are not precluded. The following cases are considered for verifying the generalization performance of an AI/ML model over various scenarios/configurations as a starting point: - Case 1: The AI/ML model is trained based on training dataset from one Scenario#A/Configuration#A, and then the AI/ML model performs inference/test on a dataset from the same Scenario#A/Configuration#A - Case 2: The AI/ML model is trained based on training dataset from one Scenario#A/Configuration#A, and then the AI/ML model performs inference/test on a different dataset than Scenario#A/Configuration#A, e.g., Scenario#B/Configuration#B, Scenario#A/Configuration#B - Case 3: The AI/ML model is trained based on training dataset constructed by mixing datasets from multiple scenarios/configurations including Scenario#A/Configuration#A and a different dataset than Scenario#A/Configuration#A, e.g., Scenario#B/Configuration#B, Scenario#A/Configuration#B, and then the AI/ML model performs inference/test on a dataset from a single Scenario/Configuration from the multiple scenarios/configurations, e.g., Scenario#A/Configuration#A, Scenario#B/Configuration#B, Scenario#A/Configuration#B. - Notes: Companies to report the ratio for dataset mixing. Number of the multiple scenarios/configurations can be larger than two. - The following case for generalization verification, can be optionally considered by companies: - Case 2A: The AI/ML model is trained based on training dataset from one Scenario#A/Configuration#A, and then the AI/ML model is updated based on a fine-tuning dataset different than Scenario#A/Configuration#A, e.g., Scenario#B/Configuration#B, Scenario#A/Configuration#B. After that, the AI/ML model is tested on a different dataset than Scenario#A/Configuration#A, e.g., subject to Scenario#B/Configuration#B, Scenario#A/Configuration#B. - Companies to report the fine-tuning dataset setting (e.g., size of dataset) and the improvement of performance Positioning Accuracy Enhancement To investigate the model generalization capability, at least the following aspect(s) are considered for the evaluation for AI/ML based positioning: - Different drops: Training dataset from drops {A0, A1,…, AN-1}, test dataset from unseen drop(s) (i.e., different drop(s) than any in {A0, A1,…, AN-1}). Here N>=1. - Clutter parameters, e.g., training dataset from one clutter parameter (e.g., {40%, 2m, 2m}), test dataset from a different clutter parameter (e.g., {60%, 6m, 2m}); - Network synchronization error, e.g., training dataset without network synchronization error, test dataset with network synchronization error; - UE/gNB RX and TX timing error: The baseline non-AI/ML method may enable the Rel-17 enhancement features (e.g., UE Rx TEG, UE RxTx TEG). - InF scenarios, e.g., training dataset from one InF scenario (e.g., InF-DH), test dataset from a different InF scenario (e.g., InF-HH) - If an InF scenario different from InF-DH is evaluated for the model generalization capability, the selected parameters (e.g., clutter parameters) are compliant with TR 38.901 Table 7.2-4 (Evaluation parameters for InF). Note: In TR 38.857 Table 6.1-1 (Parameters common to InF scenarios), InF-SH scenario uses the clutter parameter {20%, 2m, 10m} which is compliant with TR 38.901. - Other aspects are not excluded. Companies can evaluate the impact of at least the following issues related to measurements on the positioning accuracy of the AI/ML model. The simulation assumptions reflecting these issues are up to companies. - SNR mismatch (i.e., SNR when training data are collected is different from SNR when model inference is performed). - Time varying changes (e.g., mobility of clutter objects in the environment) - Channel estimation error For AI/ML assisted approach, for a given AI/ML model design (e.g., input, output, single-TRP vs multi-TRP), identify the generalization aspects where model fine-tuning/mixed training dataset/model switching is necessary.

Observation 3: The generalization performance of the AI/ML model is evaluated over various of configurations/scenarios for each use case.

Observation 4: The term 'scenarios' could signify a range of conditions, including various deployment scenarios, different distributions of outdoor or indoor UE, a variety of UE mobility levels, or a range of carrier frequencies. Other aspects of scenarios are not excluded.

Observation 5: Configurations might stand for parameters such as different UE settings, an assortment of gNB settings, a variety of bandwidths (like 10MHz, 20MHz), diverse antenna port layouts (for instance, N1/N2/P) or different numbers of antenna ports (such as 32 ports, 16 ports), among others. Various use cases might emphasize different elements of the scenarios and configurations.

Data Collection

CSI Feedback Enhancement UE side data collection: - Enhancement of CSI-RS configuration to enable higher accuracy measurement. - Assistance information for UE data collection for categorizing the data in forms of ID for the purpose of differentiating characteristics of data due to specific configuration, scenarios, site etc. - The provision of assistance information needs to consider feasibility of disclosing proprietary information to the other side. - Signaling for triggering the data collection NW side data collection: - Enhancement of SRS and/or CSI-RS measurement and/or CSI reporting to enable higher accuracy measurement. - Contents of the ground-truth CSI including: - Data sample type, e.g., precoding matrix, channel matrix etc. - Data sample format: scaler quantization and/or codebook-based quantization (e.g., e-type II like). - Assistance information (e.g., time stamps, and/or cell ID, Assistance information for Network data collection for categorizing the data in forms of ID for the purpose of differentiating characteristics of data due to specific configuration, scenarios, site etc., and data quality indicator) - Latency requirement for data collection - Signaling for triggering the data collection - Ground-truth CSI format for model training, including scalar or codebook-based quantization for ground-truth CSI. The number of layers for which the ground truth data is collected, and whether UE or NW determine the number of layers for ground-truth CSI data collection, are considered. Beam Management At UE side for UE-side AI/ML model: - UE reporting to NW supported/preferred configurations of DL RS transmission. - Trigger to initiate data collection considering: - Option 1: data collection initiated/triggered by configuration from NW. - Option 2: request from UE for data collection. - Signalling/configuration/measurement/report for data collection, e.g., signaling aspects related to assistance information (if supported), Reference signals, configuration related to Set A and/or Set B, information on association/mapping of Set A and Set B - Assistance information from Network to UE (if supported) At NW side: - Mechanism related to the reporting. - Additional information for content of the reporting. - Reporting overhead reduction. - Signalling/configuration/measurement/report for data collection Positioning Accuracy Enhancement Training data collection for AI/ML based positioning: - Associated information of training data: - Ground truth label at least for model training; report from the label data generation entity - Measurement (corresponding to model input) at least for model training; report from the measurement data generation entity. - Quality indicator for and/or associated with ground truth label and/or measurement at least for model training; report from the label and/or the measurement data generation entity and/or as request from a different (e.g., data collection, etc.) entity. - RS configuration(s) at least for deriving measurement - Request from data generation entity (UE/PRU/TRP) to LMF and/or as LMF assistance signalling to UE/PRU/TRP. - Note: there may not be any enhancements on top of existing RS configuration(s) or any new RS configuration(s) for positioning measurement. - Time stamp at least for and/or associated with training data for model training; report from data generation entity together with training data and/or as LMF assistance signalling. - Separate time stamp for measurement and ground truth label, when measurement and ground truth label are generated by different entities. - Note: there may not be any enhancements on top of time stamp in existing positioning measurement report or any new time stamp report for positioning measurement - Assistance signalling and procedure to facilitate generating/collecting training data: - Potential determination of the UE/PRU/TRP which can provide the training data - Configuration of reference signal (for measurement and/or label) - Signalling other than above 2 for data collection, e.g., requested quality of training data

Observation 5： The collected data needs to be categorized for the purpose of differentiating characteristics of data due to specific configuration, scenarios, site.

Potential Impact to LCM

Data Collection

In our related contribution [1], we outline the importance of making the OTT server aware of the RAN configuration for UE-side data collection. As per Observations 1-5, different datasets used to train various AI/ML models are associated with specific combinations of configurations, conditions, and scenarios, and each has unique characteristics. Consequently, different datasets meant for disparate configurations, conditions, and scenarios should be distinct. This is why in RAN1's agreement to utilize assistance information for data collection. This enables the categorization of data into IDs to differentiate data characteristics resulting from specific configurations, scenarios, sites, etc. This principle applies to both UE-side and network-side data collection and model training.

Furthermore, configurations should take into account both UE and gNB settings. For instance, in beam management, various UE parameters such as the number of UE Rx beams (including the number of panels and UE antenna array dimensions) and various gNB settings like DL Tx beam codebook (including various Set A of beam(pairs) and gNB antenna array dimensions) should be considered. For CSI feedback enhancement, disparate antenna spacing, antenna virtualization, antenna port layouts (e.g., N1/N2/P), and/or antenna port numbers (e.g., 32 ports, 16 ports) may be considered.

Given the scenario, configuration, and site variability, if the AI/ML model lacks adequate generalization performance, a model trained based on a dataset collected under one gNB/UE setting may not be applicable to another gNB/UE setting. Therefore, any change in the gNB/UE setting might need to be communicated to the entities collecting and storing the dataset, allowing the categorization of datasets from different gNB/UE settings as unique.

When collecting data on the UE side, categorizing or assistance information associated with RAN configurations, conditions, and scenarios should be coupled with datasets exhibiting different characteristics and made available to the UE side.

Proposal 1: RAN2 assumes that categorizing or assistance information related to RAN configurations, conditions, scenarios is associated with datasets with distinct characteristics and made available to the entities that collect and store the dataset.

Model training and generalization verification

During offline training, the AI/ML model is trained based on a dataset that correlates with specific configurations, scenarios, and sites. It guarantees that the generated AI/ML model is suited for these particular configurations, scenarios, and sites. Ideally, the AI/ML model should exhibit excellent generalization performance, implying its applicability to other configurations, scenarios, and sites. To evaluate an AI/ML model's generalization performance, the model is tested across a variety of configurations, scenarios, and sites. If the AI/ML algorithm demonstrates a performance gain in these settings, the model's applicability to these configurations, scenarios, and sites is verified.

Once deployed, the AI/ML model is considered suitable for all configurations, scenarios, and sites related to the training dataset and the dataset with verified generalization. Hence, these applicable configurations, scenarios, and sites linked to the AI/ML model should be treated as integral parts of the model's meta-information and synchronized through model/functionality identification.

Proposal 2: RAN2 assumes that the configurations, scenarios and sites associated with the dataset for model training and the dataset that showed verified generalization performance can be perceived as part of the meta-information of the AI/ML model.

Model Delivery/Transfer

In our related contribution [2], we present two methodologies for initiating model delivery/transfer. In the proactive model transfer/delivery approach, AI/ML models are pre-downloaded to the UE, enabling a model switch when changes in scenarios, configurations, or sites happen. Conversely, in the reactive model transfer/delivery method, an AI/ML model gets downloaded in real-time when modifications of scenarios, configurations, or sites are detected.

| Case | Model delivery/transfer | Model storage location | Training location |
| --- | --- | --- | --- |
| y | model delivery (if needed) over-the-top | Outside 3gpp Network | UE-side / NW-side / neutral site |
| z1 | model transfer in proprietary format | 3GPP Network | UE-side / neutral site |
| z2 | model transfer in proprietary format | 3GPP Network | NW-side |
| z3 | model transfer in open format | 3GPP Network | UE-side / neutral site |
| z4 | model transfer in open format of a known model structure at UE | 3GPP Network | NW-side |
| z5 | model transfer in open format of an unknown model structure at UE | 3GPP Network | NW-side |

If reactive model transfer/delivery is considered, the challenge within case y requires ensuring the OTT server is aware of the associated radio access network configuration, situation, and site. This allows it to promptly deliver the suitable model to the UE. As for case 'z1' and 'z2', since the model is trained by the OTT server and held within the 3GPP network, akin to case 'y', the OTT server should be informed of the related configuration, situation, and site and store the trained model in the appropriate gNB location within the 3GPP network. The coordination of configuration, scenario, and site-related information between the OTT server and the 3GPP network remains unclear. Given that the model resides within the 3GPP network, the network must be knowledgeable about the AI/ML model's applicable configuration, scenario, and site.

Proposal 3: RAN2 assumes that entities initiating the reactive model transfer/delivery procedure are aware of the radio access network configurations, conditions, and scenarios related to the AI/ML model/functionality.

Functionality-based LCM/Model-ID based LCM

For model-ID based LCM, models are identified at network, and network/ the UE may activate/deactivate/select/switch individual AI/ML models via model ID. Each model is associated with specific configurations/conditions.

Functionality-based LCM operates based on, at least, one configuration or specific configurations of an AI/ML-enabled Feature/FG. In functionality-based LCM, it allows the possibility that AI/ML models may not be identified at the Network, and UE may perform model-level LCM without network intervention. In such cases, this necessitates the UE's awareness of the specific configuration tied to the identified functionality.

Proposal 4: RAN2 assumes that the entities overseeing model monitoring, as well as model activation, deactivation, selection, or switching, are aware of the RAN configurations, conditions, and scenarios associated with the AI/ML model or functionality.

Conclusion

Observations:

Observation 1: Each AI/ML functionality is associated with one, or a specific set of, configurations. Similarly, every AI/ML model corresponds with specific configurations or conditions - such as scenarios, sites, and datasets.

Observation 2: It is crucial to ensure alignment between the UE side and the network side regarding the specific configurations associated with the AI/ML functionality or model. This alignment can be achieved through mechanisms like UE capability reporting or functionality/model identification.

Observation 3: The generalization performance of the AI/ML model is evaluated over various of configurations/scenarios for each use case.

Observation 4: The term 'scenarios' could signify a range of conditions, including various deployment scenarios, different distributions of outdoor or indoor UE, a variety of UE mobility levels, or a range of carrier frequencies. Other aspects of scenarios are not excluded.

Observation 5: Configurations might stand for parameters such as different UE settings, an assortment of gNB settings, a variety of bandwidths (like 10MHz, 20MHz), diverse antenna port layouts (for instance, N1/N2/P) or different numbers of antenna ports (such as 32 ports, 16 ports), among others. Various use cases might emphasize different elements of the scenarios and configurations.

Proposals:

Proposal 1: RAN2 assumes that categorizing or assistance information related to RAN configurations, conditions, scenarios is associated with datasets with distinct characteristics and made available to the entities that collect and store the dataset.

Proposal 2: RAN2 assumes that the configurations, scenarios and sites associated with the dataset for model training and the dataset that showed verified generalization performance can be perceived as part of the meta-information of the AI/ML model.

Proposal 3: RAN2 assumes that entities initiating the reactive model transfer/delivery procedure are aware of the radio access network configurations, conditions, and scenarios related to the AI/ML model/functionality.

Proposal 4: RAN2 assumes that the entities overseeing model monitoring, as well as model activation, deactivation, selection, or switching, are aware of the RAN configurations, conditions, and scenarios associated with the AI/ML model or functionality.

Reference

[1] R2-2308151, Data Collection for Model Training at UE Side, Mediatek Inc.

[2] R2-2308178, Discussion on AI/ML Model Transfer/Delivery, Mediatek Inc.