3GPP TSG-RAN WG2 Meeting #123			  			                            R2-2308154

Toulouse, France, August 21 – 25, 2023                                                                         (Resubmission of R2-2305512)

Agenda Item:	7.3.6

Source: 	Sony

Title:	Skip monitoring of CSI-RS during non-active periods

Document for:	Discussion

# Introduction

RAN1#112bis-e agreement: From RAN1 point of view, Rel-18 UE supporting cell DTX does not expect to receive and/or process the following signals/channels from the gNB, during non-active periods of cell DTX. The list of signals/channels may be updated based on RAN2/RAN4 input and other signals/channels are not precluded from further discussions. - Periodic/Semi-persistent CSI-RS configured in CSI report configuration in CSI-ReportConfig with reportQuantity including RI (for CSI reporting) - FFS: - PDCCH in USS - UE behavior for retransmission - if some specific RNTI scrambled PDCCH in USS will be excluded from cell DTX operation - PDCCH in Type-3 CSS - UE behavior for retransmission - if some specific RNTI scrambled PDCCH in Type-3 CSS will be excluded from cell DTX operation - PRS - CSI-RS configured by measObjectNR (for RRM) - CSI-RS associated with RadioLinkMonitoringConfig and BeamFailureDectection (for RLM and BFD) - Periodic CSI-RS configured with trs-Info ‘true’ (for tracking) - Periodic/Semi-persistent CSI-RS (for BM) - FFS on how to differentiate (if needed) with other CSI-RS used for CSI reports for BM - FFS: Whether the same or different UE behavior is applicable with or without C-DRX - FFS: Whether the list of impacted signals/channels can be configurable - FFS: Whether there will be exception case(s) for UE receiving and/or processing listed signals/channels during non-active periods of DTX - FFS: RAN1 to consider impact on system if the channels/signals are not transmitted during non-active period

In this contribution, we take a notice of FFS in RAN1 on receiving CSI-RS config and that if CSI-RS is not received by the UE from a target cell then it will impair UE’s measurement results. But RAN2 had a very simple solution for handling measurement inaccuracy in LTE eICIC, so we think there should be no issue for blanking CSI-RS.

# Discussion

  - Measurement accuracy issue

As per the latest RAN1 agreements on the UE behaviour during cell DTX non-active period, a UE is not expected to receive or process some of the signals/channels during non-active period of cell DTX, and for example, CSI-RS configured by measObjectNR is among those signals but it is still FFS.

For a UE who is going to perform the RRM measurements, before the handover is trigger or after receiving CHO configurations as confirmed in RAN2#121bis-e meeting, either a missing transmission from a neighbour cell or missing the reception within a UE may result in measurement result inaccuracy. If non-active period overlaps with the RRM measurement periodicity of CSI-RS, UE may miss one or some of the measurement opportunities. As a result, a UE may handover to an improper cell or miss the chance to handover to a suitable cell. Depending on the setting of the filter coefficient in the L3 filtering, the missing measurement opportunities may have different impacts to the measurement accuracy problem. We notice that a similar issue of measurement inaccuracy in LTE eICIC was addressed by simply informing the UE about the pattern where CSI-RS will not be transmitted by a candidate cell which will be measured by the UE. UE will then adjust measurement samples accordingly. We therefore don’t see it as a blocking issue from RAN2 point of view if CSI-RS is not received during non-active period

Proposal 1: UE may skip CSI-RS for RRM measurements during non-active periods.

# Conclusion

We propose RAN2 to discuss and agree on the following proposal:

Proposal 1: UE may skip CSI-RS for RRM measurements during non-active periods.