3GPP TSG RAN WG2 Meeting #123               	R2-2308156

# Toulouse, France, 21 – 25 August 2023

Agenda Item: 	 7.5.2

Source:		Sony

Title:			Considerations on XR UL PDU set information (for UL scheduling)

Document for:	Discussion & Decision

# Introduction

In RAN2 the study part of XR awareness was concluded at RAN2#121 meeting [1], and an updated WID, [2 was approved at RANP#99 [3], including updates to the XR capacity objectives, see below. A revised version of TR 38.835 has also been approved at the RAN plenary meeting.[4].

Specify the enhancements related to power saving: - DRX support of XR frame rates corresponding to non-integer periodicities (through at least semi-static mechanisms e.g. RRC signalling) (RAN2). Specify the enhancements related to capacity: - Multiple Configured Grant (CG) PUSCH transmission occasions in a period of a single CG PUSCH configuration (RAN1, RAN2); - Dynamic indication of unused CG PUSCH occasion(s) based on Uplink Control Information (UCI) by the UE (RAN1, RAN2); - Buffer Status Report (BSR) enhancements including at least new Buffer Status Table(s) (RAN2); - Delay reporting of buffered data in uplink (RAN2); - Discard operation of PDU Sets for DL and UL (RAN2, RAN3); Specify the enhancements for XR Awareness: - Signalling by CN of semi-static information per QoS flow (e.g. PDU set QoS parameters), dynamic information per PDU set (PDU Set information and Identification) and End of Data Burst indication (RAN3, RAN2); - Impact of identifying by UE of PDU Sets, Data bursts and PSI, as needed (RAN2); - Provisioning by UE of XR traffic assistance information e.g. periodicity, UL traffic arrival information (RAN2, RAN3); - Support signalling the congestion information from RAN to the CN in alignment with SA2 (RAN3);

In the meeting, RAN2#121bis-e [5]. Which was the first meeting for the work item, XR awareness was discussed with few agreements,

	3.	UE can report jitter information associated to UL XR traffic. How UE derives this jitter is left up to implementation (similarly as it is captured by SA2 for the jitter associated with the periodicity in DL. FFS what exactly is reported to the RAN (aim to have similar information as for DL). FFS on UL traffic data arrival reporting.

	FFS on whether EoDB signalling is needed.

Furthermore, for XR Capacity enhancements more agreements were made, but for BSR, only BSR table was discussed.

Support of new BSR table(s) is based on NW configuration and UE capability. FFS whether the UE capability can apply to non-XR Ues.

AT-meeting email [212] (QC) to discuss pros and cons of all solutions on the table. Aim to preclude least-supported solution(s). Determine the support level of solutions.

Bulk agreements

1.  As a working assumption, at most one BS index or BS value is reported by an LCG. This assumption can be revisited if new BSR table design cannot achieve a target level of quantization error. FFS what this target level should be.

3. Design/configuration for new BSR table(s) should include support for narrower ranges (i.e. finer granularity) than the legacy. Details can be discussed after an agreement on how UE obtains new BSR table(s) (e.g. pre-definition vs RRC configuration) is made.

5. At least linear distribution is used for generating code points in new BSR table(s).  FFS whether exponential distribution can be considered too.  FFS if piecewise linear distribution is supported.

8. New BSR table(s) can be used by any Ues that support such a capability. However, design of the new BSR table(s) should be based on XR-specific use cases and requirements.

Online

6.  Network can configure which BSR table(s) an LCG is eligible to use. UE determines which BSR table (i.e. legacy or something else) the LCG should use. FFS details of this determination (e.g. based on buffer size) and how network knows which BSR table each LCG uses.

4. As working assumption (depending on how we create the new BSR table(s) and the MAC CE format), If more than one new BSR table are introduced, all of them have the same size BS field. FFS on the exact size.

2a. Deprioritize Option 2c (static + dynamic BSR tables) and Option 2d (reference table + scaling factor).

2b. Have more discussions on Option 2a (static BSR tables) vs Option 2b (RRC configured BSR tables). In next meeting, companies should explain how BSR table(s) are created and how many tables would be needed, and how the MAC CE structure will look like. Should also explain what is the expected quantization error.

At RAN2#122 [6], the following agreements were made related to XR awareness

1. 	UE reports to RAN the range of jitter in its UL traffic, defined in the similar way as the one for N6 jitter.

2: Reference time is defined in similar way as BAT (Burst Arrival Time) at UE side.

2. 	UL assistance information (burst arrival time, UL jitter, FFS on periodicity) is reported per QoS flow. Network can configure for which QoS flow UE should report assistance information.

3	RRC UAI framework is updated for Rel-18 to support signalling UL assistance information agreed so far for XR (Jitter, burst arrival time, FFS on periodicity).

1:  Do not use PIN delay budget request for jitter reporting for XR services.

3: Reuse existing mechanisms (e.g. (Padding) BSR with BS value equal to zero) as implicit End of Data Burst (EoDB) indicator for the RAN.

1: On the UL, the identification of PDU sets, data bursts and PSI is left to UE implementation. This doesn’t mean UE cannot use information provided by upper layers, but RAN2 does not intend to specify how.

Use the text: “RAN2 would like to thank SA4 for their LS on N6 PDU Set Identification and inform that RAN can benefit from getting XR related information about End of Burst (1 bit). RAN2 have not identified any other dynamic information that is useful.”

At RAN2#122 [6], the following agreements were made related to capacity.

1: UE calculates the remaining time based on the PDCP discard timer value. FFS if UE reports one or multiple values. FFS how this is modelled in PDCP specification. FFS which Ues support this.

When/if UE reports remaining time, the reference time for the remaining time is determined from the point of the first transmission of the information. FFS if intra-UE prioritization can impact this.

Support one static BSR table with 8 bits BS field for Rel-18 XR (for all cases).

We do not support additional piecewise linear BSR table in Rel-18. Can consider piecewise linearity when discussing how the BSR table values are defined.

In this contribution, we discuss potential solutions for XR awareness related to information the UE could provide to the base station for UL traffic in e.g., BSR.

# Discussions on PDU set information for UL scheduling

## Background

In XR studies in 3GPP, lots of emphasis has been put on handling of downlink traffic e.g., on how to improve UE power consumption and Radio Network capacity handling, by introducing knowledge of XR traffic characteristics from application layer via Core Network QoS handling down to Radio network and base station. However, little discussions have been made related to uplink traffic.

The difference between DL and UL is that in the DL, there is a defined interface between core network user plane (UPF) and RAN base station, and it is in the UPF where packets that are arriving from outside 3GPP network are assigned to QoS flow and PDU packets, and then encapsulated with a GTP-U header to provide information to the base station or Quality of Service parameters (5QI, but also for XR, PDU set parameters). Hence there is sufficient information available to the base station scheduler to schedule DL packet.

In the UE (for uplink), there is no such corresponding interface between the UE and the application layer, and hence it has mainly been left for UE implementation to handle potential knowledge related to UL traffic. In this case, the UE sends Buffer Status Report to the gNB to help UL resource allocation and this report can be further enhanced for better resource allocation and utilization.

## Discussion on PDU set information in the UL

In some XR scenarios, UL video traffic may be the main source of traffic, e.g., in use cases where the user (UE) is streaming XR related video in the uplink to XR server in the network or at the EDGE. For such scenarios there is less understanding and knowledge in the system about what radio resources would be needed to transmit the video frames in the uplink. Compared to the downlink case, for where it is discussed to provide the base station with XR related information (QoS config and GTP-U header information) in order to handle scheduling of radio resources, whilst no such corresponding information is specified to be provided from the XR application in the UE (for Uplink), down to the modem for the uplink case, even though UE implementations might be doing it but no clear motivation is provided in standards to provide this information. The only information and knowledge of the uplink traffic is what the UE has in it the transmit buffer information and which can be indicated via BSR to base station for getting scheduling of uplink (PUSCH) resources.

A video transmission, particularly a video transmission with high quality, may consist of a number of video frames of a relatively large size, which would typically be divided into sub-packets or PDUs, belonging to a PDU set. Each sub-packet is transmitted in a wireless channel from base-station to UE for downlink, and vice versa for uplink, in one or more “chunks”.

For downlink the latency is low since the base station is able to schedule radio resources direct when a PDU needs to be delivered, but for uplink there is an additional roundtrip latency that the UE first need to request radio resources from the base station and then the base station respond with a grant to the UE to use it. For uplink to utilize radio resources with low latency and efficiently, a mechanism is needed where the resources for the UL transmissions can be more efficiently predicted and scheduled and where resource reservation can be more intelligently managed and planned within a limited time budget.

Such mechanism could include information from the application layer to the UE modem about size of subsequent “chunks”/PDUs of data belonging to the same PDU set, but also to give a prediction of the size of the subsequent PDU set to be transmitted at the following occasion according to the defined frame rate e.g., at a periodicity of 16.67 ms (60 FPS). See Figure 1. UL Information

Figure 1. UL Information

For each frame (e.g., generated at 60 FPS), knowledge of when in time and the size of the whole PDU set that will be transmitted in the uplink should be delivered to or detected at the modem so that for each chunk of data delivered to, there is an indication of remaining data (“Intra PDU set remains”) and provide this information to the scheduler in RAN/gNB. The benefit of this approach is that instead of allocating a fixed size of periodic configured grant (CG) resources, gNB/UE may allocate variable size resources depending on the size of actual data in a PDU set or chunk of data while taking into account the time budget. So, if a PDU set is transmitted over e.g., 4 occasions of CG resources of equal size, this approach instead allows 4 occasions of variable size depending on the size of PDUs in a PDU set or chunks of data which might consume less resources than fixed size allocations and help avoiding resource wastage and increase system capacity.

Observation 1. It would be beneficial for the gNB to have as much information as possible about the PDU set to be scheduled for transmission in the UL.

Alternatively, if CG configuration has e.g., 4 occasions within a period, but only two occasions can be used/occupied, this approach allows a UE to indicate to the network/gNB that it would only need two occasions with certainty. Based on this indication, gNB can decrease/increase/keep the number of occasions in time-domain with a period or decrease/increase/keep the number Resource Blocks in freq-domain by reconfiguring the CG parameters via RRC signalling. Hence, this would help gNB to avoid resource wastage and increase the system capacity.

Observation 2: For fixed CG allocation, information could be used by gNB to avoid having CG resources that may not be used.

For dynamic grant (DG) scheduling, this approach will give the gNB scheduler an estimate of how much data is expected from the UE in the near future, so that gNB can schedule to the UE dynamically taking into account the time budget (or remaining time). Hence, this would help to deliver the XR data on time to the network and as a result would minimize dropping the current XR PDU set.

Observation 3: For dynamic grant scheduling such information will get the gNB an estimate of expected data in near future, within the PDU set.

At last meeting [6] it was agreed that

- 1: On the UL, the identification of PDU sets, data bursts and PSI is left to UE implementation. This doesn’t mean UE cannot use information provided by upper layers, but RAN2 does not intend to specify how.

This means that it is left to UE implementation to extract information from allocation part to be used in the modem part, but that relevant information can be used and provided to the network/gNB.

Observation 4: UE modem is able to extract UL PDU set information and this can be used in order to inform the scheduler in RAN/gNB to enhance resource scheduling.

Also it was agreed that, the RRC UAI framework is updated for Rel-18 to support signalling UL assistance information agreed so far for XR (Jitter, burst arrival time, FFS on periodicity).

The use of UE Assistance signaling, may be less desirable for intra and inter-PDU set information updates, due to more overhead and slower signaling using RRC, giving there may be a need to use L2/MAC signaling e.g. using BSR.

Proposal 1: Further discuss the usage of complementary use of BSR for UL PDU set information.

There is an FFS on periodicity, but we think that also information about subsequent PDU can be of interest.

Observation 5. Information or prediction about subsequent PDU set in the UL can also be relevant for the gNB to be aware of.

And furthermore, Hence, prediction of the next PDU set size (Inter PDU set predictability) can alternatively and preferably be indicated in the BSR. This next PDU set size can be predicted based on PDU statistics or known sequence of data chunks of the traffic for that specific service. It can also be predicted by the application layer based on data present in its buffer waiting for transfer to the modem.

Proposal 2: Discuss what additional parameters can be relevant to be provided to the gNB for uplink grant scheduling e.g., in the BSR related to prediction of subsequent PDU set.

# Summary

In this contribution, we have discussed our view on potential enhancements for PDU set awareness for UL traffic in NR. Our observation and proposals are listed below:

Observation 1. It would be beneficial for the gNB to have as much information as possible about the PDU set to be scheduled for transmission in the UL.

Observation 2: For fixed CG allocation, information could be used by gNB to avoid having CG resources that may not be used.

Observation 3: For dynamic grant scheduling such information will get the gNB an estimate of expected data in near future, within the PDU set.

Observation 4: UE modem is able to extract UL PDU set information and this can be used in order to inform the scheduler in RAN/gNB to enhance resource scheduling.

Proposal 1: Further discuss the usage of complementary use of BSR for UL PDU set information.

Observation 5. Information or prediction about subsequent PDU set in the UL can also be relevant for the gNB to be aware of.

Proposal 2: Discuss what additional parameters can be relevant to be provided to the gNB for uplink grant scheduling e.g., in the BSR related to prediction of subsequent PDU set.

# References

- Minutes of meeting RAN2#121

- RP-230786, Updated WID on XR Enhancements for NR

- draft_MeetingReport_RAN_99_230323_eom

- RP-230307 TR 38.835 v2.0.0 on Study on XR enhancements for NR,

- Minutes RAN2#121bis-e

- Minutes RAN2#122