3GPP TSG RAN WG2 Meeting #123               	R2-2308168

# Toulouse, France, 21 – 25 August 2023

Agenda Item: 	7.22.2

Source:		Sony

Title:			Considerations on LP-WUR in RRC Idle/Inactive mode

Document for:	Discussion & Decision

# Introduction

At the RAN#97 meeting, an updated SID for Low Power Wake-up Signal and Receiver was approved including the following objectives [1].

The study item includes the following objectives: - Identify evaluation methodology (including the use cases) & KPIs [RAN1] - Primarily target low-power WUS/WUR for power-sensitive, small form-factor devices including IoT use cases (such as industrial sensors, controllers) and wearables - Other use cases are not precluded - Study and evaluate low-power wake-up receiver architectures [RAN1, RAN4] - Study and evaluate wake-up signal designs to support wake-up receivers [RAN1, RAN4] - Study and evaluate L1 procedures and higher layer protocol changes needed to support the wake-up signals [RAN2, RAN1] - Study potential UE power saving gains compared to the existing Rel-15/16/17 UE power saving mechanisms, the coverage availability, as well as latency impact of low-power WUR/WUS. System impact, such as network power consumption, coexistence with non-low-power-WUR UEs, network coverage/capacity/resource overhead should be included in the study [RAN1] Note: The need for RAN2 evaluation will be triggered by RAN1 when necessary. -

The RAN1 group has already started working on this study and have made a number of agreements [5], [7].

At RAN2#121bis e- meeting, the first meeting to start discussing Low Power Wake-up Signal and Receiver, the following agreements were made [2]:

- Aim to do every Q: Collect RAN2 text proposals in a single document during the following meeting(s) and send the document to RAN1 as the input to the TR 38.869.

- Confirm that we follow R1 and include RRC idle/inactive/connected.

- Ultra-deep-sleep = R2 understands for now that this is a power saving state (introduced by R1) to denote a state when the Main Receiver (MR) may sleep/turn off.

- In scope: Use LPWUS with Idle / Inactive UE camping with reception of paging and other necessary transmissions (from serving cell), reusing if possible/reasonable concepts from earlier releases, where the LPWUS either wakes the UE to receive by MR, or it conveys information by itself, or both.

At RAN2#122 meeting, the following agreements were made [6]:

RAN2 expect that different coverage LR/MR may have RAN2 impact, e.g. UE need to stop using LP WUS when moving out of LR coverage, other aspects FFS. What to cover (if anything) in TS 38.304 is FFS.

For UE in RRC_IDLE/RRC_INACTIVE state, it is FFS to what extent the network is or need to be aware of which receiver the UE uses MR/LR or both (for paging reception etc). A potential drawback of not knowing could be increased LP WUS load, a potential drawback of awareness is increased signalling.

RAN2 assumes that UE uses LP WUS when pre-configured condition(s) are fulfilled. 
(Other control methods not precluded)

RAN2 assumes that using subgrouping for LP-WUS could be beneficial to reduce false alarms rate (depend on L1 capacity to carry payload).

In this contribution, we discuss our view on some RAN2 related aspects related to Paging monitoring, Sub-grouping, Latency, RRM, Mobility and the new Ultra-deep sleep state/mode.

# Discussions on RAN2 aspects for LP-WUR in RRC Idle/Inactive

During IDLE/INACTIVE mode, an NR UE needs to monitor the channel for potential paging and will then also perform measurements and check the signal strength of the serving cell and if necessary, neighbour cells for cell re-selection. Between paging monitoring and measurement activities, the UE can enter sleep state/mode.

In RRC-Connected mode, the UE has an established connection, but have possibilities be inactive for shorted periods when there is no traffic and can then be configured with a Connected mode DRX schemes in order for the UE to save power.

- In Rel-15, Wake- up signal for NB-IoT and for BL UE or UE in enhanced coverage was introduced.

Furthermore in Rel-16, NB-IoT UEs, BL UEs or UEs in enhanced coverage can use group (G) WUS, when configured in the cell, to reduce the power consumption related to paging monitoring [3].

Multiple WUS groups, possibly distributed over multiple WUS resources, can be configured in the cell; and the UE selects one WUS group based on its UE paging probability information and /or its UE NAS identity. The timing between WUS and the paging occasion (PO) is illustrated in Figure 10.1.4-1. The timing between GWUS and the paging occasion (PO) is illustrated in Figure 10.1.4-2.

Figure 10.1.4-1: Illustration of WUS timing

Figure 10.1.4-2: Illustration of GWUS timing for NB-IoT UEs

In NR Rel-17 paging monitoring enhancements were introduced to save power by reducing potential false paging triggering.

- UE power saving for paging monitoring: in order to reduce UE power consumption due to false paging alarms, the group of UEs monitoring the same PO can be further divided into multiple subgroups. With subgrouping, a UE shall monitor PDCCH in its PO for paging if the subgroup to which the UE belongs is paged as indicated via associated PEI. If a UE cannot find its subgroup ID with the PEI configurations in a cell or if the UE is unable to monitor the associated PEI occasion corresponding to its PO, it shall monitor the paging in its PO. [4].

Observation 1: Principles for Wake-up signalling and paging monitoring have been discussed for both LTE/NB-IoT as well as NR in earlier releases.

Proposal 1: Re-use principles and higher layer procedures for Wake-up signalling and paging monitoring agreed for both LTE/NB-IoT as well as NR in earlier releases, Rel-15/16/17 as the baseline.

## Paging monitoring

For legacy UEs in RRC_Idle/RRC_Inactive state, the UE should periodically wake up to perform paging monitoring according to a configured DRX cycle.

To reduce the power consumption of paging monitoring, PEI (Paging Early Indication) was introduced in Rel-17. One PEI is associated with one PO, which is used to indicate UEs whether they need to monitor the following PO. A UE supporting PEI can monitor PEI associated with its PO. However, in Rel-17, it would still be the main receiver (MR), that wake-up to do the PEI monitoring, giving that there will be some power consumption related to this wake-up. This particularly for use-cases such as wearables and industrial sensors with limited available energy resources.

Observation 2: Using the Main Radio (MR) for PEI or paging monitoring will lead to non-neglectable power consumption.

Introducing a Low-Power Wake-up radio that can monitor the Wake-up signal, could reduce the number of times for the Main Receiver to wake-up and hence reduce power consumption for the Main Receiver (MR) or UE in total.

RAN1 is studying power saving gain for different solutions, but RAN2 should investigate signalling support for introducing Wake-up signalling WUS using a Low Power Wake-up radio LP-WUR.

Proposal 2. RAN2 to study signalling for signalling support for introducing Wake-up signalling WUS using a Low Power Wake-up radio LP-WUR.

## Sub grouping

In earlier release starting form Rel-15, subgrouping have been defined for both LTE, Nb-IoT, and NR (Rel-17).

The concept of subgrouping may also be applicable for Rel-18 WUS/LP-WUR.

RAN2 concluded that subgrouping could be beneficial, but no agreements were made to include it, only considerations on L1 aspects related to capacity and payload.

Those aspects are indeed needed to be discussed in RAN1, but RAN1 has made a few number of agreements related to subgrouping in RAN#112bis-e [5].

Agreement

- For IDLE/INACTIVE mode study at least following candidates for content of LP-WUS

    - information on which user(s) is/are targeted by the LP-WUS

      - e.g. UE-group, -subgroup or -ID

    - FFS: cell information

    - FFS: SI change and ETWS/CMAS information, tracking area information, and RAN area information

- For CONNECTED mode, study at least following candidates for content of LP-WUS

    - information on which user(s) is/are targeted by the LP-WUS

      - e.g UE-group, -subgroup or -ID

    - indication to wake-up to PDCCH monitoring.

- Other information candidates are not precluded

- Study pros and cons of including above information to LP-WUS.

- Note: the information may be explicitly or implicitly indicated.

Proposal 3: RAN2 to discuss whether to introduce at least subgrouping for LP-WUS to reduce false wake-up.

## Latency

When operating based on a duty-cycled scheme, the average communication delay from when data is available at the gNB until paging/data is correctly received by the UE depends on the sleep duration in the duty-cycle length, contributing error events such as false-alarm or miss-detection, and the transition time before the main receiver is ready for PDCCH reception

The average communication delay in the LP-WUS/WUR scheme would be longer than the baseline scheme, i.e., Rel. 15/Rel. 16 scheme, if both are configured with the same duty-cycle length to fulfill a certain delay requirement. This is since the main radio goes to ultra-deep sleep state to further save power and therefore a longer transition time is needed to wake-up the main radio from ultra-deep sleep.

To address this, the operation of the LP-WUR can be configured as an adaptive operation where the LP-WUR can be adaptively operated according to a duty-cycled or an always-on scheme.

Observation 3 The long transition time to wake-up the main radio from ultra-sleep time together with sleep time of the duty-cycle can prevent some UEs from meeting the delay requirement.

Proposal 4 – Support an adaptive configuration where the UE, depending on its delay requirement, can operate based on an always-on or a duty-cycle scheme.

## RRM and Mobility using LP-WUR (incl coverage)

During paging monitoring, the UE is also performing RRM measurements. A UE supporting LP-WUR may be mobile/moving and the UE needs to continuously perform measurement at least to decide whether it is in the coverage of LP-WUS. Even if the UE would be stationary regular measurements are still needed, since the serving cell could be temporarily blocked, but an suitable neigho

Observation 4: the UE needs to continuously perform measurement at least to decide whether it is in the coverage of LP-WUS.

RAN2 made the conclusion that MR and LR may have different coverage, and that the UE needs to stop using LP-WuRx when moving out of LR coverage.

The question is to what extent the network needs to be aware whether the UE actually is able to use the LP-WuRx or not. If the UE could be moving in and out from LP area, without the gNB/network is not aware, it would not have the knowledge on whether using LP-WUS is obsolete or not.

Observation 5: If the UE could be moving in and out from LP area, without the gNB/network is not aware, it would not have the knowledge on whether using LP-WUS is obsolete or not.

So, further discussion is needed whether the network needs to be information when the UE is moving between inside and outside LP coverage.

Proposal 5: Further discussion is needed whether the network needs to be information when the UE is moving between inside and outside LP coverage.

## Low-power Reference Signal (LP-RS)

By using the  LP-WUR to do serving cell measurements may depending on the design of the radio, require a new Low-power Reference Signal (LP-RS) which the LP-WUR can use to detect and measure on.

Observation 5: There may be a need to define a new Low-power Reference Signal (LP-RS) which the LP-WUR can use to detect and measure on.

- The gNBs periodically broadcast a LP-RS which can be decoded by a LP-WuRx. This signal needs to be designed to have the same characteristics as the LP-WUS, i.e., has a lower complexity than the existing reference and synchronization signals. A typical ultra-low power signal has a very simple modulation and coding technique. It can be a PN sequence modulated by on-off keying modulation and for instance Manchester coding or spreading and can be decoded by a simple receiver consisting of an envelope detector and a correlator. Both LP-RS and LP-WUS are designed with the above design characteristics.

- Time and frequency allocation:

  - The LP-RS is accommodated in certain frequency resources with a BW in the range of 2 to 5 MHz which is smaller than the maximum bandwidth of a legacy RedCap UE or a legacy eMBB UE. It is allocated within the same BWP as the LP-WUS is accommodated. The BWP for transmission of LP-RS and LP-WUS can be a part of initial BWP or can be a BWP dedicated for LP-RS and LP-WUS transmission.

  - The LP-RS periodicity is configurable and can be configured to have the same periodicity as the SSB periodicity. Its transmission, as shown in Figure 1, is however with some certain delay, td, from the start of the SSB transmission.

Figure 1 - LP-RS and its relation to SSB transmission

The time delay td is selected long enough to be able to accommodate LP-WUS and a transition time for a main radio to wake-up. With this configuration both latency for waking up a device and UE power consumption are reduced.

Proposal 6: Introduce a new low-power reference signal (LP-RS) designed and dedicated for measurement and synchronization purposes.

To introduce a new LP-RS or LP-SS would mainly impact RAN1, but also RAN4. This is currently discussed in RAN1 [5], so further alignment and feedback can be expected from them related to measurement and synchronization.

In order to extend the usage of the LP-RS, not only synchronization and measurements on serving cell is beneficial, but it would be beneficial to also being able to perform measurements for cell reselection.

In cellular based wireless connectivity and when in IDLE/INACTIVE mode, however, the UEs needs to perform some certain measurements prior to performing communication to make sure the signal strength of the serving cell is at adequate level. If the signal strength of the serving cell does not fulfill a certain requirement, the UE enters cell-reselection evaluation procedure where the evaluation identifies the cell that the UE should camp on.

- Currently, the signals available for cell re-selection evaluation procedure, i.e., the synchronization signal or reference signals of the neighbor cells, can only be measured by the main radio.

- More importantly, the UE needs to receive and decode MIB, SIB1 together with SIB2-5 in order to get proper parameters to perform correct cell selection and reselection decisions.

  - MIB defines scheduling of SIB 1 and if cell is barred,

  - SIB1 defines the scheduling of other system information including periodicity.

  - SIB2 contains cell re-selection information common for intra-frequency, inter-frequency and/ or inter-RAT cell re-selection.

  - SIB4 contains information relevant only for inter-frequency cell re-selection.

  - SIB5 contains information relevant only for inter-RAT cell re-selection.

UEs, configured with DRX operation (Idle and Inactive mode DRX), often need to perform the neighbor cell measurement and cell re-selection evaluation when the signal strength of the serving cell is not sufficient. The measurement rule in TS 38.304 section 5.2.4.2 for Intra frequency cell reselection purpose is:

- if the serving cell fulfils Srxlev > SIntraSearchP and Squal > SIntraSearchQ, the UE may choose not to perform intra-frequency measurements.

- Otherwise, the UE shall perform intra-frequency measurements.

Note: More information on the required measurement rate and rules for neighbor cell measurement is specified in 38.133, section 4.2.

This means that the UE (with mobility), even if it is equipped with an extra LP-WuRx, needs to turn on its main radio to perform cell re-selection evaluation every time it evaluates the cell reselection criterions,  and it may also have to receive relevant system information, every time it moves to a new cell. This makes the power saving resulting from using LP-WuRX, even if LP-RS is introduced, limited (especially for UE´s that are not stationary). We, therefore think there is a need to also support cell re-selection for UEs with LP-WuRx.

RAN1 is currently discussing the usage of Low power synchronization or Refence signal for mobility and cell reselection purpose, so further alignment is needed with RAN1 [5].

Proposal 7: RAN2 to discuss the usage of LP-RS also for mobility and cell reselection purpose.

Proposal 8: RAN2 to align with discussion and agreements made in RAN1.

## Ultra-deep sleep

Definition of ultra-deep-sleep, and network awareness, enter into/wake up from.

In the last RAN2 meeting, the first meeting to start discussing Low power Wake-up Radio, RAN2 concluded the following: “Ultra-deep-sleep = R2 understands for now that this is a power saving state (introduced by R1) to denote a state when the Main Receiver (MR) may sleep/turn off.”

RAN1 has agreed on ultra-deep sleep, but RAN2 has not discussed how it will affect the RRC state machine and to what extent the RAN and CN networks are aware of or have the understanding of any Ultra-deep sleep state/mode where the Main Radio (MR) is turned off. Basically, it would be similar as UE power save state or MICO mode.

Proposal 9: RAN2 to discuss if/how the Ultra deep sleep state/mode it will affect the RRC state machine and to what extent the RAN and CN networks are aware of this new state/mode and what signaling that may be affected.

# Summary

In this contribution, we have discussed our views on aspects related to Paging monitoring, Sub-grouping, Latency, RRM, Mobility and the new Ultra-deep sleep state/mode

Observation 1: Principles for Wake-up signalling and paging monitoring have been discussed for both LTE/NB-IoT as well as NR.

Proposal 1: Re-use principle and higher layer procedure for Wake-up signalling and paging monitoring agreed for both LTE/NB-IoT as well as NR in earlier releases, Rel-15/16/17 as the baseline.

Observation 2: Using the Main Radio (MR) for PEI or paging monitoring will lead to non-neglectable power consumption.

Proposal 2. RAN2 to study signalling for signalling support for introducing Wake-up signalling WUS using a Low Power Wake-up radio LP-WUR.

Proposal 3: RAN2 to discuss whether to introduce WUS/LP-WUR to reduce falsa alarm paging wake-up.

Observation 3 The long transition time to wake-up the main radio from ultra-sleep time together with sleep time of the duty-cycle can prevent some UEs from meeting the delay requirement.

Proposal 4 – Support an adaptive configuration where the UE, depending on its delay requirement, can operate based on an always-on or a duty-cycle scheme.

Observation 4: the UE needs to continuously perform measurement at least to decide whether it is in the coverage of LP-WUS.

Observation 5: If the UE could be moving in and out from LP area, without the gNB/network is not aware, it would not have the knowledge on whether using LP-WUS is obsolete or not.

Proposal 5: Further discussion is needed whether the network needs to be information when the UE is moving between inside and outside LP coverage.

Observation 6: There may be a need to define a new Low-power Reference Signal (LP-RS) which the LP-WUR can use to detect and measure on.

Proposal 6: Introduce a new low-power reference signal (LP-RS) designed and dedicated for measurement and synchronization purposes.

Proposal 7: RAN2 to discuss the usage of LP-RS also for mobility and cell reselection purpose.

Proposal 8 RAN2 to align with discussions and agreements made in RAN1.

Proposal 9 RAN2 to discuss if/how the Ultra deep sleep state/mode it will affect the RRC state machine and to what extent the RAN and CN networks are aware of this new state/mode and what signaling that may be affected.

# References

- RP-222644, Revised SID: Study on low-power Wake-up Signal and Receiver for NR, vivo

- Minutes RAN2#121bis

- 36.300, TS (E-UTRA) and Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Overall description; Stage 2 (Release 16)

- 38.300, TS NR; NR and NG-RAN Overall Description; Stage 2, (Release 17)

- Minutes RAN1#112bis-e

- Minutes  RAN2#122

- RAN#113