3GPP TSG-RAN WG2 Meeting #123											     R2-2308169

Toulouse, France, August 21 - 25, 2023

Agenda Item:	7.18.3

Source: 	SONY, Nokia, Nokia Shanghai Bell, Samsung

Title:	Beam failure recovery for Rel-18 SDT

Document for:	Discussion and Decision

# Introduction

In the past few meetings, RAN2 has been discussing the Mobile Terminated-Small Data Transmission (MT-SDT) for Rel-18 UEs in RRC_INACTIVE state. One of the open issues is how to handle the beam failure during ongoing MT-SDT procedures. The following contribution was discussed and noted in the last RAN2#122 meeting [2]:

R2-2305527	Remaining procedures for MT-SDT	Sony	discussion	Rel-18	 NR_MT_SDT-Core

Proposal 3: For beam failure recovery of DL/UL SDT in Rel-18, a UE should trigger RACH procedure if the RSRP of the current beam is below a certain pre-configured threshold.

-	Intel thinks that the topic is not part of the SDT WI and if there is something broken it should be Rel-17 CR or TEI18 if there is a new enhancement.

-	Nokia thinks that we already support this CG SDT in Rel-17 and it would be good for RA SDT.    Samsung has the same view as Nokia and we missed it in Rel-17 and are ok to support in R18.

-	ZTE asks if it will delay cell reselection if the beam is bad should UE select to a better cell or continue looking for a better beam in the same cell.    Nokia explains that this is not really related to cell reselection and it will not delay it.

-	LG and Qualcomm asks if there would be impact to another group in RAN1 as we haven’t discuss whether we have the beam failure procedure for SDT.   Nokia explains that this is already for CG SDT and there would be no RAN1 impacts.

=>	Noted

In this contribution, we discuss and propose a beam failure recovery procedure for Rel-18 SDT. In addition, we show that there is no impact on other WGs for this proposal, but a small impact on the current RAN2 spec based on legacy Rel-17 (only TS 38.321), as captured on the appendix A.

## Beam failure recovery for Rel-18 SDT

SDT allows transmission and reception of small data whilst UE remains in RRC_INACTIVE state. When SDT is started, the SDT procedure applies a maximum duration to complete the data. The maximum duration is implemented by “SDT failure detection timer” which is configured by the network. If this timer expires, the UE goes to IDLE state.

One issue is that if there is a beam failure during ongoing SDT, the transmission and reception at the UE will not succeed, and the UE will wait until SDT failure detection timer expires, and then the UE goes to IDLE. This provides few disadvantages:

- Other urgent transmissions and receptions can not be done during waiting time.

- Power consumtion issue as UE waits until timer expires.

- At the end, the UE transitions to IDLE state resulting more delays.

In Rel-17, during an ongoing CG-SDT, in case of beam failure where there is no SSB (amongst the SSBs for which CG resources are configured) with RSRP value above a preconfigured threshold and data for SDT RBs is available for transmission, UE triggers RA procedure to recover from the beam failure. However, a similar recovery procedure has not been specified for RA-SDT.

Furthermore, for Rel-18, if there is an ongoing MT-SDT and if there is a beam failure where the RSRP value of the current beam/SSB (i.e., beam/SSB selected in the last random access procedure during the ongoing MT-SDT procedure) is less than a preconfigured threshold, the UE should have a mechanism to recover the beam failure.

Therefore, we propose that, during ongoing RA based MO-SDT or MT-SDT procedure if the RSRP value of the current beam is less than a pre-configured threshold, a UE should trigger RACH procedure to recover the beam failure.

Proposal 1: For beam failure recovery in Rel-18 SDT, during ongoing RA based MO-SDT or MT-SDT procedure (performed over RACH) if the RSRP value of the current SSB (i.e., SSB selected in the last random access procedure during the ongoing SDT procedure) is less than a pre-configured threshold, a UE should trigger RACH procedure similar to CG-SDT procedure in Rel-17 SDT.

An early impact analysis of the above proposal based on Rel-17 spec is captured on the appendix A. It is only identified a small impact on RAN2 spec (the changes shown on TS 38.321), and there is no impact on other working groups (WGs).

## Conclusions

In this contribution, we have discussed how to handle the beam failure during ongoing RA-SDT or MT-SDT procedure. We have the following proposal:

Proposal 1: For beam failure recovery in Rel-18 SDT, during ongoing RA based MO-SDT or MT-SDT procedure (performed over RACH) if the RSRP value of the current SSB (i.e., SSB selected in the last random access procedure during the ongoing SDT procedure) is less than a pre-configured threshold, a UE should trigger RACH procedure similar to CG-SDT procedure in Rel-17 SDT.

## References

[1] RP-213583, “New WI: Mobile Terminated-Small Data Transmission (MT-SDT) for NR,” RAN#94e.

[2] R2-2306543, Report from Session on NES, UAV, Rel-15-17 UP, Rel-17 Small Data, IIoT/URLLC, and RACH partitioning, RAN2#122.

Appendix A: Impact of the current spec based on Rel-17 TS 38.321:

================================NEXT CHANGE========================================

## 5.27	Small Data Transmission

### 5.27.1	General

The MAC entity may be configured by RRC with SDT and the SDT procedure may be initiated by RRC layer. The SDT procedure can be performed either by Random Access procedure with 2-step RA type or 4-step RA type (i.e., RA-SDT) or by configured grant Type 1 (i.e., CG-SDT).

RRC configures the following parameters for SDT procedure:

-	sdt-DataVolumeThreshold: data volume threshold for the UE to determine whether to perform SDT procedure;

-	sdt-RSRP-Threshold: RSRP threshold for UE to determine whether to perform SDT procedure;

-	cg-SDT-RSRP-ThresholdSSB: an RSRP threshold configured for SSB selection for CG-SDT.

The MAC entity shall, if initiated by the upper layers for SDT procedure:

1>	if the data volume of the pending UL data across all RBs configured for SDT is less than or equal to sdt-DataVolumeThreshold; and

NOTE 1:	For SDT procedure, the MAC entity also considers the suspended RBs configured with SDT for data volume calculation. It is up to the UE's implementation how the UE calculates the data volume for the suspended RBs. Size of the CCCH message is not considered for data volume calculation

1>	if the RSRP of the downlink pathloss reference is higher than sdt-RSRP-Threshold; or

1>	if sdt-RSRP-Threshold is not configured:

2>	if the Serving Cell is configured with supplementary uplink as specified in TS 38.331 [5]; and

2>	if the RSRP of the downlink pathloss reference is less than rsrp-ThresholdSSB-SUL:

3>	select the SUL carrier.

2>	else:

3>	select the NUL carrier.

2>	if CG-SDT is configured on the selected UL carrier, and TA for CG-SDT is valid according to clause 5.27.2 in the first available CG occasion for initial CG-SDT transmission with CCCH message according to clause 5.8.2; and

2>	if, for each RB having data available for transmission, configuredGrantType1Allowed, if configured for CG-SDT, is configured with value true for the corresponding logical channel; and

2>	if at least one SSB configured for CG-SDT with SS-RSRP above cg-SDT-RSRP-ThresholdSSB is available:

3>	indicate to the upper layers that the conditions for initiating SDT procedure are fulfilled;

3>	perform CG-SDT procedure on the selected UL carrier according to clause 5.8.2.

2>	else if a set of Random Access resources for RA-SDT is configured and can be selected according to clause 5.1.1b on the selected UL carrier on the BWP configured by initialUplinkBWP-RedCap, if configured for a RedCap UE; otherwise, on the BWP configured by initialUplinkBWP:

3>	if cg-SDT-TimeAlignmentTimer is running, consider cg-SDT-TimeAlignmentTimer as expired and perform the corresponding actions in clause 5.2;

3>	indicate to the upper layers that the conditions for initiating SDT procedure are fulfilled.

2>	else:

3>	indicate to the upper layers that the conditions for initiating SDT procedure are not fulfilled.

1>	else:

2>	indicate to the upper layers that the conditions for initiating SDT procedure are not fulfilled.

If RA-SDT is selected above and after the Random Access procedure is successfully completed (see clause 5.1.6), the UE monitors PDCCH addressed to C-RNTI received in random access response until the RA-SDT procedure is terminated. If CG-SDT is selected above and after the initial transmission for CG-SDT is performed, the UE monitors PDCCH addressed to C-RNTI as stored in UE Inactive AS context as specified in TS 38.331 [5] and CS-RNTI until the CG-SDT procedure is terminated.

NOTE 2:	When the UE determines if there is an SSB with SS-RSRP above cg-SDT-RSRP-ThresholdSSB, the UE uses the latest unfiltered L1-RSRP measurement.

The MAC entity shall:

1>	if RA-SDT procedure initiated for MO-SDT is ongoing and

1>	if SS-RSRP of the SSB selected in the last successfully completed random access procedure during the ongoing RA-SDT procedure is less than rsrp-ThresholdSSB; and

1>	if there is data available for transmission for at least one RB configured for SDT:

2>	initiate a Random Access procedure (see clause 5.1).

1>	else if RA-SDT procedure for MT-SDT is ongoing; and

1>	if SS-RSRP of the SSB selected in the last successfully completed random access procedure during the ongoing RA-SDT procedure is less than rsrp-ThresholdSSB:

2>	initiate a Random Access procedure (see clause 5.1).

===============================CHANGE ENDS=========================================