3GPP TSG-RAN WG2 #123	R2-2308181
Toulouse, France, August 21-25, 2023

Agenda Item:	7.3.4

Source:	OPPO

Title:	Discussion on cell selection/reselection

Document for:	Discussion, Decision

Introduction

This paper will discuss cell selection/reselection, which is one objective of NES WID.

- Specify mechanism(s) to prevent legacy UEs camping on cells adopting the Rel-18 NES techniques, if necessary [RAN2]

Discussion

In RAN2#122, RAN2 agrees to introduce separate camping restrictions for NES-capable and non-NES UEs.

Agreements:

- We will define UE capabilities with signaling.  Details are FFS and will be discussed later during the WI phase.

- Separate camping restrictions for NES-capable and non-NES UEs will be defined.  FFS if it is a single bit or more.

Since the legacy UE can only identify the current IE, we need to use the current IE to prevent the legacy UEs from accessing the NES cell. During the SI phase, the below solutions are raised for this purpose.

For backward compatibility, there is a need to allow NES cells to prevent legacy UEs from camping. NES cells should be able to configure whether to prevent legacy UEs, while allowing NES-capable UEs to camp on. Possible solutions may include but not limited to:

-	Use IntraFreqExcludedCellList/InterFreqExcludedCellList

-	Use the cellBarred or cell reservation fields in MIB/SIB

According to the existing spec, the intra-/inter frequency excluded cell list only applies to the cell reselection procedure while cellBarred/cell reservation is applicable for both cell selection and reselection procedure. To simplify the spec, a unified mechanism is preferred to prevent the legacy UEs from selecting/reselecting an unsuitable cell, i.e. use the cellBarred or cell reservation fields in MIB/SIB to control the legacy UEs accessing an NES cell. As cellBarred is the normal way to control legacy UEs, it seems the simplest choice is to reuse cellBarred for the NES case. For example, if any one of the NES techniques the network utilises would degrade the legacy UE’s performance, the network would set cellBarred as Barred and thus prevent the legacy UEs from accessing this NES cell.

Use cellBarred in MIB to control the legacy UE accessing an NES cell or preventing from an NES cell.

New camping restrictions for NES-capable UEs should be introduced to control NES-capable UEs separately. One open issue is whether to have more than one bit (e.g. a barring per feature) for controlling. In our view, one bit is sufficient since whether to block the NES-capable UE from a certain NES cell reflects an appeal of the network itself (e.g. energy saving or load balance).

One single bit is introduced for controlling all NES-capable UEs to access a cell, i.e. all NES-capable UEs are uniformly allowed or prevented from accessing a specific NES cell.

The new and single barring bit (e.g. cellBarredNES) can be introduced in SIB1 to control the NES-capable UEs accessing a certain NES cell. Accordingly, the controlling can be implemented as below,

- If cellBarred in MIB is indicated as barred, the legacy UEs are barred from this cell. Otherwise, the legacy UEs can access this cell.

- If cellBarred in MIB is indicated as barred and cellBarredNES is indicated in SIB1 as not barred, the NES-capable UE can access this cell.

- If cellBarred in MIB is indicated as barred and cellBarredNES is indicated in SIB1 as barred, the NES-capable UE cannot access this cell.

- If cellBarred in MIB is indicated as barred and cellBarredNES is not present in SIB1, the NES-capable UE cannot access this cell.

A new barring indication is introduced in SIB1 to control NES-capable UEs accessing an NES cell. In detail, once cellBarred in MIB is indicated as Barred, the NES-capable UEs need to further check the new barring indication in SIB1.

Conclusion

We have the following proposals:

Proposal 1	Use cellBarred in MIB to control the legacy UE accessing an NES cell or preventing from an NES cell.

Proposal 2	One single bit is introduced for controlling all NES-capable UEs to access a cell, i.e. all NES-capable UEs are uniformly allowed or prevented from accessing a specific NES cell.

Proposal 3	A new barring indication is introduced in SIB1 to control NES-capable UEs accessing an NES cell. In detail, once cellBarred in MIB is indicated as Barred, the NES-capable UEs need to further check the new barring indication in SIB1.

Reference

[1] RP-223540 New WID: Network energy savings for NR

[2] TR 38.864 Study on network energy savings for NR