3GPP TSG-RAN WG2 #123	R2-2308183
Toulouse, France, August 21-25, 2023

Agenda Item:	7.5.2

Source:	OPPO

Title:	Discussion on XR awareness

Document for:	Discussion, Decision

Introduction

This paper will discuss XR awareness.

Discussion

In Rel-16 URLLC, TSCAI is introduced to describe TSC traffic characteristics. The TSCAI is provided per QoS flow to the RAN, which contains the flow direction, periodicity and burst arrival time. Knowing that the TSC traffic characteristics are useful for RAN on more efficient scheduling for the QoS flows with periodic and deterministic traffic characteristics.

Similar to TSC, Rel-18 XR also has periodic and low latency requirements. Per the SA2’s progress, NG-RAN may obtain via TSCAI the XR-related traffic characteristics which contain the UL and/or DL Periodicity as well as N6 jitter information.

Table 1: TSC Assistance Information (TSCAI) in TS 23.501

| Assistance Information | Description |
| --- | --- |
| Flow Direction | The direction of the TSC flow (uplink or downlink). |
| Periodicity | It refers to the time period between start of two data bursts. |
| Burst Arrival Time (optional) | The latest possible time when the first packet of the data burst arrives at either the ingress of the RAN (downlink flow direction) or the egress of the UE (uplink flow direction). |
| Survival Time (optional) | Survival Time, as defined in TS 22.261 [2], refers to the time period an application can survive without any data burst. |
| Burst Arrival Time Window (BAT Window) (optional) (NOTE 1) (NOTE 2) | Indicates the acceptable earliest and latest arrival time of the first packet of the data burst at either the ingress of the RAN (downlink flow direction) or the egress of the UE (uplink flow direction). |
| Capability for BAT adaptation (optional) (NOTE 1) | Indicates that the AF will adjust the burst sending time according to the network provided Burst Arrival Time offset (see clause 5.27.2.5). |
| N6 Jitter Information (optional) (NOTE 3) | Jitter information associated with the Periodicity in downlink (see clause 5.378.1). |
| Periodicity Range (optional) (NOTE 4) | It indicates that the AF will adjust the periodicity and provides the acceptable range (which is formulated as lower bound and upper bound of the Periodicity) or acceptable Periodicity value(s) (which is formulated as a list of values for the Periodicity). |
| NOTE 1: Only one of the parameters (BAT Window or Capability for BAT adaptation) can be provided. NOTE 2: The parameter can only be provided together with Burst Arrival Time. NOTE 3: Only one of the parameters Burst Arrival Time or N6 Jitter Information may be provided for a given Traffic Flow. NOTE 4: The Periodicity Range can only be provided together with Periodicity when Burst Arrival Time and Burst Arrival Time Window are present. |  |

- TSCAI is also used to provide the gNB with the XR traffic characteristics which contain UL/DL periodicity and N6 jitter information.

Combined with the SA2’s progress above, RAN2 achieves the below agreements on semi-static information provided by UE to RAN in RAN2#122.

- UE reports to RAN the range of jitter in its UL traffic, defined in the similar way as the one for N6 jitter.

- Reference time is defined in similar way as BAT (Burst Arrival Time) at UE side.

- UL assistance information (burst arrival time, UL jitter, FFS on periodicity) is reported per QoS flow. Network can configure for which QoS flow UE should report assistance information.

- RRC UAI framework is updated for Rel-18 to support signalling UL assistance information agreed so far for XR (Jitter, burst arrival time, FFS on periodicity).

- Do not use PIN delay budget request for jitter reporting for XR services.

Regarding the jitter range, TR 38.838 assumes a baseline value range of [-4, 4] ms on the XR evaluation. As RAN3 has not decided on the N6 jitter range till now, RAN2 could consider [-4, 4] ms as a starting point to design the UL jitter range.

As a baseline, the range of UL jitter information would be [-4, 4] ms.

We understand the no necessity for the provision of UL periodicity from the UE to the gNB due to the reasons below.

- SA2 already agrees to provide the UL and/or DL Periodicity to NG-RAN by the SMF as below.

Although the UL periodicity is designed to provide for DRX, it can be easily extended for CG usage. We assume the gNB would use UL periodicity provided by the CN for capacity enhancement and power saving.

- As designed, what TSCAI provided would be semi-static, thus, the periodicity information would not be changed dynamically. Even If there is the case that the periodicity adjusts, no problem exists. It is because UE AL and AF have enough time to exchange the adjustment and then let the CN inform the adjusted periodicity information via TSCAI.

- If we allow the UE also to report the UL periodicity information, there would be a case that the UL periodicity provided by the CN and UE are different. Further discussion is needed on how to fix this case.

No need to report the UL periodicity from the UE to the gNB.

As agreed, the XR-related assistance information would be signalled using the RRC UAI framework. The most straightforward way is to update the existing UAI-related configurations to report UL jitter information as well as the burst arrival time, i.e. contain UL jitter/BAT in UEAssistanceInformation as well as new triggering condition on the UL jitter/BAT in OtherConfig. Usually, to avoid excessive reporting signalling for the semi-static information, a prohibition timer can be configured. For example, if the UE did not transmit a UAI with the UL jitter/BAT before, the UE initiates the UAI transmission to provide the UL jitter/BAT report. If the current UL jitter/BAT is different from the one indicated in the UE’s last UAI transmission and the prohibit timer expires, the UE initiates the UAI transmission to report the updated UL jitter/BAT.

OtherConfig contains a new triggering condition to report the UL jitter/BAT, which can be a prohibition timer.

If the UE did not transmit a UAI with the UL jitter/BAT before, the UE initiates the UAI transmission to provide the UL jitter/BAT report.

If the current UL jitter/BAT is different from the one indicated in the UE’s last UAI transmission and the prohibit timer expires, the UE initiates the UAI transmission to report the updated UL jitter/BAT.

Conclusion

We have the following observations:

Observation 1	TSCAI is also used to provide the gNB with the XR traffic characteristics which contain UL/DL periodicity and N6 jitter information.

We have the following proposals:

Proposal 1	As a baseline, the range of UL jitter information would be [-4, 4] ms.

Proposal 2	No need to report the UL periodicity from the UE to the gNB.

Proposal 3	OtherConfig contains a new triggering condition to report the UL jitter/BAT, which can be a prohibition timer.

Proposal 4	If the UE did not transmit a UAI with the UL jitter/BAT before, the UE initiates the UAI transmission to provide the UL jitter/BAT report.

Proposal 5	If the current UL jitter/BAT is different from the one indicated in the UE’s last UAI transmission and the prohibit timer expires, the UE initiates the UAI transmission to report the updated UL jitter/BAT.

Reference

[1] RP-230786 Updated WID on XR Enhancements for NR, Nokia, Qualcomm

[2] TR 23.700-60 Study on XR (Extended Reality) and media services

[3] R2-2302309 TR 38.835 v1.0.2, Nokia (Rapporteur)

[4] TS 23.501 v18.2.2