3GPP TSG-RAN WG2 #123	R2-2308185
Toulouse, France, August 21-25, 2023

Agenda Item:	7.5.4.1

Source:	OPPO

Title:	Discussion on BSR enhancement for XR

Document for:	Discussion, Decision

Introduction

This paper will discuss the BSR enhancement.

Discussion

Delay status reporting

In RAN2#120, RAN2 agrees to report delay information which is associated with data volume.

- RAN2 will introduce data volume information associated with delay information (e.g. remaining time) in a MAC CE. FFS if this is extension of BSR or new format. FFS how to do that (e.g. what exactly is reported) and how to ensure this information is up-to-date e.g. considering UL scheduling delay.

In RAN2#122, RAN2 further agrees that the remaining time is derived from the PDCP discard timer value.

- 1: UE calculates the remaining time based on the PDCP discard timer value. FFS if UE reports one or multiple values. FFS how this is modelled in PDCP specification. FFS which UEs support this.

In reality, one UE can have multiple services on-going but only the XR services have the requirements to report the delay information. To reduce the unnecessary signalling overhead of delay information reporting, the gNB can indicate which LCH/LCG is allowed to report such information.

The gNB configures which LCH/LCG is allowed to report the remaining time.

We assume that the remaining time is reported in the unit of the PDU set, since QoS-related characteristics (e.g. PSDB) and the required AS operation are designed per PDU set basis. Accordingly, the remaining time would be the difference value between the value configured for the PDCP discard timer and the buffering time of a PDU set. Since the PDUs of a PDU set would arrive at different times and all PDUs of a PDUs are operated as a whole, the remaining time to be reported would be derived by the discardTimer associated with the first SDU of the PDU set.

If a UE intends to report the remaining time for a PDU set, only one value is to be reported, which equals the remaining time of discardTimer associated with the first SDU of this PDU set.

Also, RAN2 agrees that the reference time for the remaining time relates to the point of the first transmission of the information. What is unclear is whether the intra-UE prioritization impacts this agreement.

- When/if UE reports remaining time, the reference time for the remaining time is determined from the point of the first transmission of the information. FFS if intra-UE prioritization can impact this.

In the intra-UE prioritization procedure, the MAC layer triggers a new transmission for the deprioritized MAC PDU when criteria fulfil. In this case, the MAC layer directly delivers the deprioritized MAC PDU to the identified HARQ process instead of invoking the Multiplexing and assembly entity to regenerate a new MAC PDU. If the remaining time to be reported is already contained in the deprioritized MAC PDU, the MAC layer has no chance to update the remaining time contained, even if it is the time is outdated. Similar issues also exist in e.g. BSR/PHR and no enhancement is agreed upon before. Thus, we prefer no impact on the determination of reference time for the remaining time due to intra-UE prioritization.

No impact on the determination of reference time for the remaining time due to intra-UE prioritization.

As a reporting content, the delay information is introduced from Rel-18 XR and no existing MAC CE contains such information before, it would be good to have a new MAC CE to contain the remaining time associated with the PDU set. Otherwise, timely scheduling is hard to implement and the benefit of introducing the latency report will be weakened. Typically, how much data volume is buffered at the UE side depends on several factors, e.g. UL delay budget, scheduling strategies and radio link quality. One LCH/LCG would buffer different data volumes (e.g. (less than) one PDU set, or more than one) in different cases. When the UE intends to report the delay information, one or more PDU sets may remain in the UE’s buffer associated with the LCH/LCG configured for delay information reporting. Potential alternatives on how to report such delay status are shown below.

- Alt 1: Only one remaining time value is reported for the associated LCH/LCG. The remaining time is the shortest value of the remaining times of the PDU sets in the associated LCH/LCG.

- Alt 2: Multiple remaining time values are reported for the associated LCH/LCG. Each remaining time value is associated with an individual PDU set which is buffered in the associated LCH/LCG.

In our view, Alt 1 is preferred since 1) The most important thing is to let the gNB knows the more urgent requirement, and 2) Alt 2 has an issue with signalling overhead.

Introduce a new MAC CE to contain the remaining time of the associated LCH/LCG.

Only one remaining time value is reported for the associated LCH/LCG. The remaining time is the shortest value of the remaining times of the PDU sets in the associated LCH/LCG.

Regarding the triggering event of delay status reporting, although either criterion below can work and provide its value, Alt b is preferred to let the gNB knows the urgent information as soon as possible.

- Alt a: The remaining time reporting is triggered when a fresh PDU set is available.

- Alt b: The remaining time reporting is triggered when the remaining time of a PDU set approaches the threshold.

The remaining time reporting is triggered when the remaining time of a PDU set approaches the threshold.

New BSR

In the current system, the BS information buffered in the uplink is reported by BSR MAC CE. The Index field in the current BSR format is used to represent the BS value, which is a quantization indication of the BS of an LCG in the UE’s buffer. The Index field indicates a range of buffer size, i.e. the actual buffer size is between the pre-defined maximum and minimum values. The larger the Index is, the larger the quantization error is introduced. The quantization error is tolerant in legacy but cannot be neglected for Rel-18 XR since the XR service has a low latency requirement as well as large and various packet sizes.

In the previous meetings, RAN2 agrees with the following on the BS reporting.

- 1. 	As a working assumption, at most one BS index or BS value is reported by an LCG. This assumption can be revisited if new BSR table design cannot achieve a target level of quantization error. FFS what this target level should be.

- 3.	Design/configuration for new BSR table(s) should include support for narrower ranges (i.e. finer granularity) than the legacy. Details can be discussed after an agreement on how UE obtains new BSR table(s) (e.g. pre-definition vs RRC configuration) is made.

- 8.	New BSR table(s) can be used by any UEs that support such a capability. However, design of the new BSR table(s) should be based on XR-specific use cases and requirements.

- 6. 	Network can configure which BSR table(s) an LCG is eligible to use. UE determines which BSR table (i.e. legacy or something else) the LCG should use. FFS details of this determination (e.g. based on buffer size) and how network knows which BSR table each LCG uses.

- 2a. 	Deprioritize Option 2c (static + dynamic BSR tables) and Option 2d (reference table + scaling factor).

- 2b. 	Have more discussions on Option 2a (static BSR tables) vs Option 2b (RRC configured BSR tables). In next meeting, companies should explain how BSR table(s) are created and how many tables would be needed, and how the MAC CE structure will look like. Should also explain what is the expected quantization error.

- Support one static BSR table with 8 bits BS field for Rel-18 XR (for all cases).

- We do not support additional piecewise linear BSR table in Rel-18. Can consider piecewise linearity when discussing how the BSR table values are defined.

In summary, a new static BS table with an 8-bit BS field and finer granularity than the legacy is introduced for Rel-18 XR.

According to the current spec, the existing BS tables cover the BS range from 0 to infinite, i.e. the 8-bit BS table indicates the amount of available data from 0 to (large than) 81M bytes. In short, the legacy BS table is sufficient to indicate any amount of available data with the drawback of large quantization error for the larger BS value. As the XR service has a large packet size as well as the new BS table tends to match the XR requirements with finer granularity, we understand the new BS table only needs to cover the upper BS value range in the legacy 8-bit BS table.

The new 8-bit BS table covers the upper BS value range in the legacy 8-bit BS table.

As assumed, the new BS table has a different BS range from the legacy BS tables. When the UE reports the data volume of the XR service, the UE needs to decide which BS table to be selected. Normally, the BS table determination depends on the data volume the UE buffered and the BS range the BS table covered. In one implementation, for the LCG configured to use both the new and legacy BS table, the UE selects the new BS table for encoding if the data volume the UE buffered is larger than the minimum value of the new BS table. Otherwise, the UE selects the legacy BS table. Accordingly, the UE needs to indicate to the gNB which BS table is selected. Thus, a new MAC CE associated with a new eLCID is introduced to contain the BS information of the XR service as well as a new field indicating the selected BS table. FFS on whether it is the same MAC CE of the remaining time reporting.

If an LCG is eligible to use both the legacy and new BS tables, the UE chooses the new BS table for encoding if the data volume the UE buffered is larger than the minimum value of the new BS table.

A new MAC CE associated with a new eLCID is introduced to contain the BS information of the XR service as well as a new per-LCG field indicating the selected BS table.

Conclusion

We have the following proposals:

Proposal 1	The gNB configures which LCH/LCG is allowed to report the remaining time.

Proposal 2	If a UE intends to report the remaining time for a PDU set, only one value is to be reported, which equals the remaining time of discardTimer associated with the first SDU of this PDU set.

Proposal 3	No impact on the determination of reference time for the remaining time due to intra-UE prioritization.

Proposal 4	Introduce a new MAC CE to contain the remaining time of the associated LCH/LCG.

Proposal 5	Only one remaining time value is reported for the associated LCH/LCG. The remaining time is the shortest value of the remaining times of the PDU sets in the associated LCH/LCG.

Proposal 6	The remaining time reporting is triggered when the remaining time of a PDU set approaches the threshold.

Proposal 7	The new 8-bit BS table covers the upper BS value range in the legacy 8-bit BS table.

Proposal 8	If an LCG is eligible to use both the legacy and new BS tables, the UE chooses the new BS table for encoding if the data volume the UE buffered is larger than the minimum value of the new BS table.

Proposal 9	A new MAC CE associated with a new eLCID is introduced to contain the BS information of the XR service as well as a new per-LCG field indicating the selected BS table.

Reference

[1] RP-230786 Updated WID on XR Enhancements for NR, Nokia, Qualcomm

[2] TR 23.700-60 Study on XR (Extended Reality) and media services

[3] R2-2302309 TR 38.835 v1.0.2, Nokia (Rapporteur)

[4] TS 23.501 v18.2.2