3GPP TSG-RAN WG2 Meeting #123                                                              R2-2308226

# Toulouse, France, August 21 – 25, 2023

Agenda item:	7.4.4

Source:	Samsung

Title:	Considerations on CHO with CPA/CPC

Document for:	Discussion & Decision

Introduction

After the email discussion post RAN2#122 meeting[1], there still left several open issues needs further discussion. And our considerations of these open issues are given in this contribution.

Discussion

- 2.1 Issue#3

FFS whether the legacy CHO recovery mechanism applies to the configuration for CHO with candidate SCG(s).

In Rel-17, CHO recovery mechanism is applied for PCell RLF or CHO failure and the same scenarios might occur in CHO+CPAC, so legacy CHO recovery mechanism also can be applied to avoid performing RRC Re-establishment procedure.

After the PCell failure, the UE will start the recovery procedure via the cell selection. However, different from Rel-17, the recovery procedure depends on the configurations at the UE side:

- Case 1: The UE is configured with CHO-only configuration and without CHO+CPAC configuration for the selected PCell

The UE will follow Rel-17 CHO recovery mechanism.

- Case 2: The UE is configured with CHO+CPAC for the selected PCell

The selected cell can be the candidate PCell of at least one CHO+CPAC configuration. However, one candidate PCell may have different candidate PSCells, the UE cannot know which MCG configuration is used for the recovery. RAN2 needs further discussions on how to perform the recovery.

- Case 3: The UE is configured with both CHO-only configuration and CHO+CPAC configuration for the selected PCell

The UE will follow Rel-17 CHO recovery mechanism via CHO-only configuration.

Proposal 1: If the selected cell after MCG failure is candidate PCell of configured CHO-only configuration, the UE can follow Rel-17 CHO recovery procedure by using the CHO-only configuration.

Proposal 2: RAN2 is kindly asked to discuss the recovery scheme if the selected cell after MCG failure is candidate PCell of multiple CHO+CPAC configurations, and if the CHO-only configuration for such cell is not configured.

- 2.2 Issue#4

FFS whether to stop conditional reconfiguration evaluation for CHO with Candidate SCG(s) upon initiating SCG failure information procedure.

According to TS 37.340, in Rel-17, UE will stop evaluating the CPA/CPC execution condition for SCGFailure.

In case of CPA/CPC, upon transmission of the SCGFailureInformation message to the MN, the UE stops evaluating the CPA/CPC execution condition. The UE is not required to continue measurements for candidate PSCell(s) for execution condition upon transmission of the SCGFailureInformation message to the MN.

Therefore in CHO+CPAC, upon SCG failure, UE stops conditional reconfiguration evaluation of CPAC. Thus the conditional reconfiguration evaluation for CHO+CPAC will never be fulfilled as CPAC evaluation is stopped, and the UE can wait for the reconfiguration from gNB.

Proposal3: Upon initiating SCG failure information procedure, conditional reconfiguration evaluation for CHO+CPAC should be stopped, and the UE can wait for the reconfiguration from gNB.

However, if CHO-only configuration is given, the UE could continue CHO execution condition evaluation to perform CHO since the CHO-only configuration can help UE recovery the bearers severed by the failed SCG.

Proposal4: Upon initiating SCG failure information procedure, the UE can continue the CHO evaluation if CHO-only configuration is given.

- 2.3 Issue#5

FFS whether to extend maxNrofCondCells-r16 for CHO with candidate SCG(s).

According to CondReconfigToAddModList IE in TS 38.331, CondReconfigId-r16 corresponds to one candidate PCell for CHO with SCG, and the value of CondReconfigId-r16 is from 1 to maxNrofCondCells-r16, the value of which is 8. However, in Rel-18, for CHO+CPAC, the CondReconfigId-r16 corresponds to one candidate PCell and one candidate PSCell. For each candidate PCell might be associated with multiple candidate PSCells, in this sense, the value of maxNrofCondCells-r16 needs to be expanded properly to ensure enough number of candidate PCells and the associated candidate PSCells for UE to select, and the expanded value is FFS.

Proposal5:  The value of maxNrofCondCells-r16 for CHO+CPAC needs to be extended, and the extended value is FFS.

- 2.4 Issue#6

FFS how to ensure the total number of the candidate PCells and the candidate PSCells from each candidate MN and the candidate SN is within the maximum limitation.

S-MN can determine the maximum number of candidate PCells and that of candidate PSCell associated to each candidate PSCell within the maximum limitation, and the maximum number of prepared candidate PSCells could be provided to candidate T-MN in in HO Request message for each candidate PCell which are in the scope of RAN3..

Proposal 6: RAN2 should determine the maximum limitation of the total number of the candidate PCells and the candidate PSCells. How to provide the maximum number of the candidate PSCells associated with the candidate PCell can be left to RAN3.

- 2.5 Issue#7

FFS whether to support condEventA3 or condEventA5 for the execution conditions for candidate PSCells for CHO with candidate SCG(s).

In CHO+CPAC, if event A4 is supported, CHO and CPAC execution condition seems to be fulfilled simultaneously much easier without the comparison of serving PSCell’s quality. However, in order to select a suitable PSCell and avoid the throughput drop due to the PSCell change failure after CHO with CPAC execution, the event A3/A5 based condition could be considered.

So event A3/A5 should not be excluded for the execution conditions for candidate PSCells for CHO with candidate SCG(s).

Proposal 7: Event A3/A5 should not be excluded for the execution conditions for candidate PSCells for CHO with candidate SCG(s).

- 2.7 Issue#9 and Issue#10

FFS which node (source MN or candidate MN) to recommend the candidate PSCells.

FFS whether to support recommendation of the candidate PSCells based on measurement results.

During CHO+CPAC preparation, the candidate T-MN will determine whether candidate SCG is needed or not, so it is reasonable that the candidate T-MN recommend the candidate PSCells to the candidate T-SN.

Proposal 8:  The candidate T-MN recommends the candidate PSCells to the candidate T-SN.

There might be several solutions for the candidate T-MN to provide the recommendation to the candidate T-SN, and one solution might be measurement-based. If measurement-based solution is supported, the candidate T-MN should get the measurement results from the source MN, and then provides to the candidate T-SN for candidate PSCell recommendation.

Proposal 9: If measurement-based candidate PSCell recommendation is supported, the source MN should provide the measurement results to the candidate T-MN, and the candidate T-MN sends it to the candidate T-SN.

- 2.9 Issue#11

FFS if to stop evaluating the execution conditions once PSCell change is triggered.

Once PSCell is changed, the configuration for the candidate SCGs included in the CHO+CPAC configuration may be invalid if delta configuration is applied. Therefore, the evaluation of the execution condition for CHO with candidate SCGs should be stopped.

Proposal10：The evaluation of the execution conditions is stopped once PSCell change is triggered.

Conclusion

Based on the discussion, we have the following proposals.

Proposal 1: If the selected cell after MCG failure is candidate PCell of configured CHO-only configuration, the UE can follow Rel-17 CHO recovery procedure by using the CHO-only configuration.

Proposal 2: RAN2 is kindly asked to discuss the recovery scheme if the selected cell after MCG failure is candidate PCell of multiple CHO+CPAC configurations, and if the CHO-only configuration for such cell is not configured.

Proposal3: Upon initiating SCG failure information procedure, conditional reconfiguration evaluation for CHO+CPAC should be stopped, and the UE can wait for the reconfiguration from gNB.

Proposal4: Upon initiating SCG failure information procedure, the UE can continue the CHO evaluation if CHO-only configuration is given.

Proposal5:  The value of maxNrofCondCells-r16 for CHO+CPAC needs to be extended, and the extended value is FFS.

Proposal 6: RAN2 should determine the maximum limitation of the total number of the candidate PCells and the candidate PSCells. How to provide the maximum number of the candidate PSCells associated with the candidate PCell can be left to RAN3.

Proposal7: Event A3/A5 should not be excluded for the execution conditions for candidate PSCells for CHO with candidate SCG(s).

Proposal8:  The candidate T-MN recommends the candidate PSCells to the candidate T-SN.

Proposal9: If measurement-based candidate PSCell recommendation is supported, the source MN should provide the measurement results to the candidate T-MN, and the candidate T-MN sends it to the candidate T-SN.

Proposal10：The evaluation of the execution conditions is stopped once PSCell change is triggered.

Reference

[1] R2-230xxxx Report of [Post122][057][Mob18] 38.331 Running CR for CHO with candidate SCGs