3GPP TSG-RAN WG2 Meeting #123	R2-2308234

Toulouse, France, 21 – 25 August 2023

Agenda item:	7.14.4

Source:	Nokia, Nokia Shanghai Bell

Title:	Detailed handling of QoE configuration and reporting in NR-DC

WID/SID:	NR_QoE_enh-Core - Release 18

Document for:	Discussion and Decision

# 1	Introduction

To support QoE in NR-DC, RAN2 and RAN3 has progressed with running CRs reflecting agreed principles.

This contribution continues discussion on detailed configuration and reporting principles to support QoE Measurements Collection in DC.

# 2	Discussion

## 2.1	MN in control of QoE measurement configurations

Within the developed framework for DC, the QoE configuration id does not need any specific differentiation (between the MN and SN) over RRC. However, the coordination between MN and SN should ensure the MN- and SN-triggered configurations respect some maximum number of the application layer configurations that the UE can support. In Rel-17 the maximum number of QoE configuration that the UE can handle (in Standalone scenario) is 16. For Dual Connectivity, a question arises whether the MN and SN should share the existing range:

Proposal 1: FFS if the existing maximum number of application layer measurement configurations (i.e., 16) needs extension for DC.

## 2.2	SRB use for QoE measurement reporting

Any QoE configuration can be uniquely identified by RRC ID, but it remains unclear how the UE should apply a reporting leg for a given QoE configuration. RAN2 agreed to use new SRB5 for reporting QoE to SN. One possible behaviour) would be that the QoE reports are sent to the configured node. I.e., if the QoE configuration is provided over SRB1 (by MN), it implies the UE reports available QoE measurements over SRB4 (as in Rel-17). If the QoE configuration is provided over SRB3 (by SN), it implies the UE reports available QoE measurements over SRB5 (by SN).

However, it cannot be a default behaviour for the QoE configuration generated by SN but delivered by MN. In this case the SN originates the configuration, but asks MN to configure the UE, and in result the SN can remain the consumer of the QoE reports (if it asks MN to configure the UE for reporting directly to SN):

When SN indicates its interest in configuring m-based QoE a measurement to a UE:

  - The SN can indicate to the MN that the reports are to be sent via the SRB5.

  - The SN can request the use of the SRB4 for reporting, which the MN can confirm or reject. FFS whether the indication is explicit or implicit. (RAN3 agreement)

For this scenario, the configuration of a reporting SRB for a QoE configuration is necessary. With the below undertaken agreement in RAN2 on optional SRB configuration, we think the optionality might need more precise conditions to be defined.

The network can optionally explicitly indicate the SRB for the QoE reporting if both SRB4 and SRB5 are configured. FFS on the granularity, e.g. per QoE config or otherwise (RAN2 agreement)

For clean feature design, we believe any QoE configuration should be always configured with an applicable SRB, if provided for the first time. If RRC reconfiguration of an existing QoE configuration happens, such indication of the SRB might be absent, as the UE would know and be already instructed.

Proposal 2: If provided for the first time, a QoE configuration is always configured with an applicable SRB (either SRB4 or SRB5).

This would imply the SRB configuration granularity, identified as open point (above), should be resolved as follows:

Proposal 3: Granularity of SRB configuration (SRB4 or SRB5) for the QoE reporting is per RRC ID(s).

## 2.3	Reporting leg switch

Based on RAN3 agreement to introduce an explicit indicator for ‘QoE reporting leg switch’, we note that SRB switch can happen for an ongoing QoE configuration. Upon the indication reception, the UE has to switch QoE measurements reporting from SRB4 (to MN) to SRB5 (to SN) or vice versa. Following the granularity of SRB configuration (per QoE configuration per RRC ID), for consistency we propose the reporting leg switch is also configured per RRC ID:

Proposal 4: Explicit indicator on ‘Reporting leg switch’ is configured per RRC ID, and only for impacted RRC ID(s).

Such explicit indicator passed to the UE would require the UE to check the indicator itself vs. Reporting possibilities (configured SRBs by the Network). The UE needs to receive a valid SRB configuration to proceed with reporting switch.  Whether SRB4/SRB5 configuration allows the reporting change should be NW responsibility. Hence, from the RRC signaling point of view, and associated UE procedures, we believe the explicit indicator on reporting leg switch requires that the Network ensures appropriate SRBs configuration:

Proposal 5: If the ‘Reporting leg switch’ is configured, the configuring node triggers appropriate SRB configuration (SRB4 in MN or SRB5 in SN), if not available.

## 2.4	SCG deactivation

To enable reasonable UE battery consumption, Rel-17 enabled a generic activation/deactivation mechanism of SCG. This introduces a specific case to handle QoE data collection continuity in DC. i.e. for a UE involved in QoE and operating in DC scenario, MN can decide to deactivate SCG. This may imply that reporting for and to the SN may get interrupted (if no dedicated handling for QoE reports is ensured). A rescue mechanism for impacted data due to SCG deactivation assumes RLC leg involvement and UEAssistanceInformation use:

38.331 5.7.4: UE Assistance Information: 1> if the SCG is deactivated, and, 1> the UE has uplink data to send for an SCG RLC entity while the UE previously did not have any uplink data to send for any SCG RLC entity: 2> initiate transmission of the UEAssistanceInformation message in accordance with 5.7.4.3 to indicate that the UE has uplink data to send for a DRB whose DRB-Identity is not included in any RLC-BearerConfig in the CellGroupConfig associated with the MCG. … 1> if transmission of the UEAssistanceInformation message is initiated to provide an indication that the UE has uplink data related to a deactivated SCG according to 5.7.4.2: 2> include uplinkData in the UEAssistanceInformation message.

UEAssistanceInformation-v17xy-IEs ::= SEQUENCE { uplinkData-r17 ENUMERATED { true } OPTIONAL, nonCriticalExtension SEQUENCE {} OPTIONAL }

Since the SCG is intentionally to be deactivated, the direct SRB5 between the SN and the UE cannot be used for QoE report transmission. For QoE configuration which is configured to be transmitted via SRB5, the need to continue with QoE reports transmission may vary depending on QoE type. For a RVQoE and QoE-container based reports, there might be different treatment needed upon the SCG gets deactivated by NW. For instance, the UE could be enabled to directly switch the QoE reports transmission to MN via SRB4 or discard the reports. To enable differentiated treatment, we propose:

Proposal 6: In case of SCG deactivation, the gNB provides additional RRC configuration to UE, to indicate how to deal with the QoE reporting configured to be transmitted via SRB5.

FFS if what options are allowed for RVQoE and container-based QoE configuration update if QoE data is available for transmission:

a. discard reports,

b. switch reporting leg (from SRB5 to SRB4),

c. trigger UEAssistanceInformation to reactivate SCG.

## 2.5	UL message for reporting SN reports to MN.

Another remaining aspect concerns reporting message for QoE data for SN-associated QoE reports (e.g. RV-QoE) via SRB4 (to MN). If SN-triggered or SN-associated QoE session will involve MN node to collect the reports and the SN-relevant reports need to be delivered to MN, it is not defined what message can be used in this scenario. We believe that even the reuse of the existing in Rel-17 messaging and parameters for DC impose certain difficulties and a need for adoption or coordination between two nodes (MN and SN). Therefore, we propose to adopt existing means as much as possible:

Proposal 7: MeasurementReportAppLayer message is reused UL RRC message for reporting SN-associated QoE reports via SRB4 (to MN).

## 2.6	QoE Pause

To deal with RAN overload, Rel-17 introduced QoE Pause/Resume mechanism. If the network is overloaded it can use ‘pauseReporting’ RRC indication to temporarily stop QoE reports from being sent from the UE to the network. Handling of the suspended reports relies on buffering in the UE Access Stratum, until the gNB resumes the QoE reporting.

In DC, the temporary suspension of the QoE reporting may take different meaning. After configuring the UE to suspend the QoE report transmission, the UE does not know whether the suspension concerns any QoE reporting (e.g., pause per UE configuration) or to MN or SN only (e.g., pause per node).

Observation 1: ‘pauseReporting’ RRC indication from MN does not clarify whether the UE can transfer the QoE reports to the SN.

The agreed indicator on ‘Reporting leg switch’, as well as the discussed SCG deactivation impact on for QoE reporting may have commonality with overload handling. In particular, to remain consistent with already agreed enhancements for Rel-18 (e.g., reporting leg switch, SRB indication, SN-triggered configuration through MN), we believe the ‘pauseReporting’ in DC needs to enable more granular handling than per UE only. So that partial offloading of the QoE reports should be possible. This can be realised, if the ‘pauseReporting’ supports suspending QoE reporting only to the impacted node.

Proposal 8: ‘pauseReporting’ in DC can be configured per leg.

# 3	Conclusion

This document has made the following proposals:

Proposal 1: FFS if the existing maximum number of application layer measurement configurations (i.e., 16) needs extension for DC.

Proposal 2: If provided for the first time, a QoE configuration is always configured with an applicable SRB (either SRB4 or SRB5).

Proposal 3: Granularity of SRB configuration (SRB4 or SRB5) for the QoE reporting is per RRC ID(s).

Proposal 4: Explicit indicator on ‘Reporting leg switch’ is configured per RRC ID, and only for impacted RRC ID(s).

Proposal 5: If the ‘Reporting leg switch’ is configured, the configuring node triggers appropriate SRB configuration (SRB4 in MN or SRB5 in SN), if not available.

Proposal 6: In case of SCG deactivation, the gNB provides additional RRC configuration to UE, to indicate how to deal with the QoE reporting configured to be transmitted via SRB5.

FFS if what options are allowed for RVQoE and container-based QoE configuration update if QoE data is available for transmission:

a. discard reports,

b. switch reporting leg (from SRB5 to SRB4),

c. trigger UEAssistanceInformation to reactivate SCG.

Proposal 7: MeasurementReportAppLayer message is reused UL RRC message for reporting SN-associated QoE reports via SRB4 (to MN).

Proposal 8: ‘pauseReporting’ in DC can be configured per leg.