3GPP TSG-RAN WG2 #123	R2-2308243

France Toulouse,  21st Aug – 25th Aug 2023

Source:		NEC

Agenda item:		7.17.2	Procedures for MUSIM temporary capability restriction

Title:		Discussion on early capability restriction indication

Document for:		Discussion and decision

# 1. Introduction

At RAN2 #121bis meeting, RAN2 agreed on the support of early indication of temporary capability restriction during RRC resume or setup procedure [1]:

Support “early indication” from UE to network during RRC connection setup/resume procedure.

FFS how to indicate this and in which message. The indication will tell network that UE capabilities are temporarily restricted.

FFS on details (i.e. when UE can indicate this, what does it indicate, how does it relate to UAI, etc.)

In this paper, we provide our view on how to support “early indication” of temporary capability restriction during RRC resume/setup/re-establishment procedure.

# 2. Discussion

To avoid configuring the MUSIM UE with exceeded capability during RRC resume/setup procedure, RAN2 agree to support “early indication” to the network to indicate that the UE capability is temporarily restricted, such that the gNB would configure the UE with minimum configuration and configuration of reporting preference on temporary capability restriction, and then UE indicates preference on temporary capability restriction to the network by UAI message.

For RRC setup procedure, since the RRCSetup message only configures SRB1 and some basic physical layer configurations, the “early indication” can be sent by either Msg3 or Msg5. But for RRC resume procedure, RRCResume message may configure the UE with configurations with greater RF capabilities requirement, it would be late to indicate by Msg5. However, if one bit flag in RRCResumeRequest or RRCResumeRequest1 is introduced, the precious 1 spare bit should be used. This should be avoided and the 1 spare bit should be saved for more important use case.

Observation: sending the early indication in RRCResumeRequest message would use the precious 1 spare bit, and sending the early indication in RRCResumeComplete message is too late.

There are some other alternative options.

- Option 1. By sending CCCH message using dedicated LCID

For this option, if the UE determines that its capability is temporarily restricted, dedicated LCID would be used to transmit the CCCH message.

- Option 2. By initiating RA procedure using dedicated resource,

For this option, the RA procedure can be initiated using dedicated random access resource or UL BWP.

Comparing these two options, Option1 is preferred for us because it does not require additional radio resource. For example, LCID 37 and LCID 38 can be used. In Rel-17, dedicated LCIDs (35 and 36) are used for indication of Redcap UE. Since a MUSIM UE could not be a Redcap UE, so there is no issue to define dedicated LCID for CCCH message transmission for MUSIM usage.

As said above, for RRC setup procedure, it is OK to send the early indication in RRCSetupComplete message. But considering that it is better to have aligned solution, dedicate LCID for CCCH messages should also be applied for RRC setup case.

Proposal 1: During RRC setup and RRC resume procedure, UE indicates its capability is temporary restricted by sending CCCH message using dedicated LCID.

Another similar case is RRC re-establishment, the source cell may not support Rel-18 MUSIM but the target cell supports, so the target cell is not aware of that the UE capability is temporary restricted. Since the RRCreestablishment message and RRCReconfiguration message may be sent in the same MAC PDU, and the RRCReconfiguration message may configure the UE with exceeded capability requirement, it is beneficial for the UE to also send early indication of temporary capability restriction for this case. And the same solution as RRC resume can be used.

Proposal 2: During RRC re-establishment procedure, UE indicates its capability is temporary restricted by sending CCCH message using dedicated LCID.

If the network does not support early indication, CCCH message transmitted using dedicated LCID would be considered as erroneous data by the network. To avoid this, network should indicate in system information if the UE is allowed to send early indication of temporary capability restriction, and UE sends early indication only if the network indicates that it is allowed in system information.

Proposal 3: UE sends indication of temporary capability restriction during RRC setup/resume/re-establishment procedure only if the network indicates that it is allowed in system information.

3. Conclusion

In this paper, we provide our view on how to support early indication during RRC setup, resume or re-establishment procedure.

Observation: sending the early indication in RRCResumeRequest message would use the precious 1 spare bit, and sending the early indication in RRCResumeComplete message is too late.

Proposal 1: During RRC setup and RRC resume procedure, UE indicates its capability is temporary restricted by sending CCCH message using dedicated LCID.

Proposal 2: During RRC re-establishment procedure, UE indicates its capability is temporary restricted by sending CCCH message using dedicated LCID.

Proposal 3: UE sends indication of temporary capability restriction during RRC setup/resume/re-establishment procedure only if the network indicates that it is allowed in system information.

# Reference

[1] Meeting report of RAN2 #121bis.