3GPP TSG-RAN WG2 #123	Tdoc R2-2308251

Toulouse, France, Aug 21st – 25th, 2023

Agenda Item:	6.1.3

Source:	Ericsson

Title:	NSAG validity when TAI list is omitted

Document for:	Discussion

# 1	Introduction

In this document, we identified a possible issue in the specification of NAS-AS interaction on slice-based cell re-selection. In the NAS signalling providing the NSAG information, the TA list is optional. If absent, the NSAG is valid in all TAs of the PLMN.

One could argue this is not crystal clear from RAN2 specifications, and we would like RAN2 to discuss whether a clarification is needed.

# 2	Discussion

SA2 stage 2 TS 23.501 specifies the option to have the NSAG valid in all cells/TAs of the PLMN, as highlighted below:

5.15.14	Network Slice AS Groups support

The NG-RAN may support Network Slice AS Groups (NSAGs) which are used as specified in TS 38.300 [27], TS 38.331 [28], TS 38.321 [143] and TS 38.304 [50]. A Network Slice AS Group is an identifier of a group of network slices which are associated with it. A Network Slice AS Group association with a group of network slices may be valid in one or more Tracking Areas. An S-NSSAI can be associated with at most one NSAG values for Random Access and at most one NSAG value for Cell Reselection within a Tracking Area. An S-NSSAI can be associated with different NSAG values in different Tracking Areas.

The NG-RAN provides (and updates) the AMF with the values of the NSAG(s) an S-NSSAI is associated with in a TA using the NG Set Up and RAN Configuration Update procedures (see TS 38.413 [34]). The AMF in turn provides this information to the NSSF. In deployments where the total number of groups does not exceed the number of groups associated with the NSAG size limit defined in TS 38.331 [28]), all the NSAGs configured in the NG-RAN may be unique per PLMN or SNPN. If the UE has indicated that the UE supports NSAG in the 5GMM Core Network Capability (see clause 5.4.4a), the AMF may, with or without NSSF assistance, configure the UE with NSAG Information for one or more S-NSSAIs in the Configured NSSAI, by including this NSAG Information in the Registration Accept message or the UE Configuration Command message. The UE uses the NSAG Information as defined in clause 5.3.4.3.1. The AMF shall indicate in the NSAG Information in which TA a specific NSAG association to S-NSSAI(s) is valid if the AMF provides in the UE configuration a NSAG value which is used in different TAs with a different association with NSSAIs. The configuration the AMF provides includes at least the NSAGs for the UE for the TAs of the Registration Area. If the AMF does not include the list of TAIs in association with an NSAG in the NSAG Information, the NSAG is valid in the Registered PLMN and equivalent PLMNs, or SNPN.

This is captured in TS 24.501 for CN to UE NAS signalling, and provided to AS (RRC):

4.6.2.6	Provision of NSAG information to lower layers

The NSAG information provided by the network and stored in the UE includes a list of NSAGs each of which contains:

a)	an NSAG ID;

b)	a list of S-NSSAI(s), which are associated with the NSAG and shall be part of the configured NSSAI;

c)	a priority value that is associated with the NSAG; and

NOTE 1:	The AMF can take local configuration, UE 5GMM capabilities, subscribed S-NSSAIs, HPLMN, etc. to determine the NSAG priority information for the associated NSAG to a UE.

d)	optionally a list of TAIs in which the NSAG is valid. If it is not provided by the network, the NSAG is valid in the PLMN or SNPN which has sent the NSAG information and its equivalent PLMN(s)..

NOTE 2:	If the NSAG for the PLMN and its equivalent PLMN(s) have different associations with S-NSSAIs, then the AMF includes a list of TAIs in the NSAG information.

In RRC signalling (for size optimization in system information and dedicated signalling), if the tracking area code is omitted, UE shall assume the tracking area code of the serving cell.
From 38.331:

#### –	NSAG-IdentityInfo

The IE NSAG-IdentityInfo is used to identify an NSAG (TS 23.501 [32]) for slice-based cell reselection.

NSAG-IdentityInfo information element

-- ASN1START

-- TAG-NSAG-IDENTITYINFO-START

NSAG-IdentityInfo-r17 ::=            SEQUENCE {

nsag-ID-r17                          NSAG-ID-r17,

trackingAreaCode-r17                 TrackingAreaCode               OPTIONAL      -- Need R

}

-- TAG-NSAG-IDENTITYINFO-STOP

-- ASN1STOP

NSAG-IdentityInfo field descriptions

trackingAreaCode If absent, UE assumes the trackingAreaCode of the serving cell.

In TS 38.304, we have procedure text to describe the UE behaviour, see below.

From TS 38.304:

5.2.4.11	Reselection priorities for slice-based cell reselection

The UE derives reselection priorities for slice-based cell reselection by using:

-	NAS provided NSAG information, only for NSAG(s) associated with the network slice(s) provided by NAS for cell reselection (see TS 23.501 [10], TS 24.501 [14]),

-	sliceInfoList and/or sliceInfoListDedicated per frequency with nsag-CellReselectionPriority per NSAG, if provided in system information and/or dedicated signalling (see TS 38.331 [3]),

-	cellReselectionPriority per frequency provided in system information and/or dedicated signalling (see TS 38.331 [3]).

The UE considers an NR frequency to support all slices of an NSAG if

-	the nsag-ID and TA of the NSAG as provided by NAS are indicated for the NR frequency (see TS 38.331[3]). If FreqPriorityListDedicatedSlicing is configured, UE only considers the NSAG-frequency pairs indicated in FreqPriorityListDedicatedSlicing for slice-based cell reselection.

The UE considers a cell on an NR frequency to support all slices of an NSAG if

-	the nsag-ID and TA of the NSAG as provided by NAS are indicated for the NR frequency in dedicated signalling but not in SIB16 (see TS 38.331 [3]); or

-	the nsag-ID and TA of the NSAG as provided by NAS are indicated for the NR frequency in SIB16 (see TS 38.331 [3]); and

-	the cell is either listed in the sliceAllowedCellListNR (if provided in the sliceInfoList) or the cell is not listed in the sliceExcludedCellListNR (if provided in the sliceInfoList); or

-	Neither sliceAllowedCellListNR nor sliceExcludedCellListNR is configured in the sliceInfoList.

<cut>

From this text in TS 25.304 (“…the nsag-ID and TA…”) alone, one could argue it is not crystal clear that the TAI list might be omitted, and hence the NSAG is valid not only in the current TA, but in the whole PLMN.

RAN2 to confirm that in case TAI list is not provided for an NSAG, the NSAG is valid in the whole PLMN.

We provided a draft CR to clarify this in [5]

RAN2 to agree the CR to 38.304 provided in [5].

# Conclusion

Based on the discussion in the previous sections we propose the following:

Proposal 1	RAN2 to confirm that in case TAI list is not provided for an NSAG, the NSAG is valid in the whole PLMN.

Proposal 2	RAN2 to agree the CR to 38.304 provided in [5].

# References

TS 38.331, v17.5.0

TS 38.304, v17.5.0

TS 24.501, v17.11.1

TS 23.501, v17.9.0

R2-2308252, NSAG validity when TAI list is omitted, Source: Ericsson

# Annex

3GPP TSG-RAN WG2 Meeting #123	R2-2308252

# Toulouse, France, Aug 21st – 25th, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.304 | CR | 351 | rev | - | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | NSAG validity when TAI list is omitted |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_Slice-Core |  |  |  |  |  | Date: |  |  | 2023-08-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In case TAI list is absent for an NSAG in NAS signalling (TS 24.501), the NSAG is valid in the whole PLMN. In RRC signalling (SIB and dedicated signalling), if the TAC is absent, the UE assumes the TAC of the current cell. Existing text in TS38.304 does not clearly describe the resulting UE behaviour |  |  |  |  |  |  |  |  |
| Summary of change: |  | Text on how UE examines information recieved in RRC signaling and NAS signalling is clarified by more clear reference to “NSAG information provided by NAS”. Impact Analysis Impacted 5G architecture options: NR SA Impacted functionality: Slice-based cell re-selection Inter-operability: CR only impact UE behaviour, hence no inter-operability issues. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unclear text will remain in specification, and might result in terroneous UE implementation (UE is prevented from re-selecting neighbouring cell across TA border). |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.4.11 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | N | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | N | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | N | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

#### 5.2.4.11	Reselection priorities for slice-based cell reselection

The UE derives reselection priorities for slice-based cell reselection by using:

-	NAS provided NSAG information, only for NSAG(s) associated with the network slice(s) provided by NAS for cell reselection (see TS 23.501 [10], TS 24.501 [14]),

-	sliceInfoList and/or sliceInfoListDedicated per frequency with nsag-CellReselectionPriority per NSAG, if provided in system information and/or dedicated signalling (see TS 38.331 [3]),

-	cellReselectionPriority per frequency provided in system information and/or dedicated signalling (see TS 38.331 [3]).

The UE considers an NR frequency to support all slices of an NSAG if

-	the nsag-ID and TA of the NSAG indicated for the NR frequency (see TS 38.331[3]) are included in the NSAG information provided by NAS. If FreqPriorityListDedicatedSlicing is configured, UE only considers the NSAG-frequency pairs indicated in FreqPriorityListDedicatedSlicing for slice-based cell reselection.

The UE considers a cell on an NR frequency to support all slices of an NSAG if

-	the nsag-ID and TA of the NSAG indicated for the NR frequency in dedicated signalling but not in SIB16 (see TS 38.331 [3]) are included in the NSAG information provided by NAS; or

-	the nsag-ID and TA of the NSAG indicated for the NR frequency in SIB16 (see TS 38.331 [3]) are included in the NSAG information provided by NAS ; and

-	the cell is either listed in the sliceAllowedCellListNR (if provided in the sliceInfoList) or the cell is not listed in the sliceExcludedCellListNR (if provided in the sliceInfoList); or

-	Neither sliceAllowedCellListNR nor sliceExcludedCellListNR is configured in the sliceInfoList.

The UE shall derive reselection priorities for slice-based cell reselection according to the following rules:

-	Frequencies that support at least one prioritized NSAG received from NAS have higher reselection priority than frequencies that support none of the NSAG(s) received from NAS.

-	Frequencies that support at least one NSAG provided by NAS are prioritised in the order of the NAS-provided priority for the NSAG with highest priority supported on the frequency.

-	Among the frequencies (one or multiple) that support the highest prioritised NSAG(s) with the same NAS-provided priorities, the frequencies are prioritized in the order of their highest nsag-CellReselectionPriority given for these NSAG(s). If no nsag-CellReselectionPriority is given for a NSAG at a frequency, the lowest priority value is used (i.e, lower than any of the network configured values for these frequencies).

-	Frequencies that support none of the NSAG(s) provided by NAS are prioritized in the order of their cellReselectionPriority.

For a UE performing slice-based cell reselection, if the highest ranked cell or best cell in a frequency fulfils the inter- freqeuency cell reselection criteria (see clause 5.2.4.5) based on reselection priority for the frequency and NSAG derived according to this clause or fulfils intra-frequency and equal priority inter-frequency cell reselection criteria (see clause 5.2.4.6), but this cell does not support the NSAG according to this clause:

-	if this cell supports any other NSAG(s) according to this clause, the UE shall re-derive a reselection priority for the frequency by considering the NSAG(s) supported by this cell (rather than those of the corresponding NR frequency);

-	Otherwise, the UE shall re-derive a reselection priority for the frequency as if none of the NSAG(s) provided by NAS is supported.

This re-derived reselection priority is used for a maximum of 300 seconds, or until new network slice(s) and/or NSAG information are received from NAS. UE shall ensure the cell reselection criteria above are fulfilled based on the newly derived priorities.