3GPP TSG-RAN WG2 Meeting #123											R2-2308279

Toulouse, France, August 21-25, 2023

Agenda item:		7.3.2

Title: 	The remaining issues on cell DTX and DRX

Source: 			ZTE Corporation, Sanechips

Document for: 	Discussion and Decision

- Introduction

The agreements about the transmission or reception of signals during Cell DTX/DRX non-active time could be summarized as below:

Agreements for SIB/paging

- There will be no impact to RACH, paging, and SIBs in idle/inactive for both gNB and Rel-18 and legacy UEs

- Rel-18 NES capable CONNECTED UE(s) can perform RACH and receive SIBs in non-active duration of cell DTX and/or DRX (i.e., same behavior for cell DTX and cell DRX).  No further enhancements for CBRA and CFRA will be pursued.

Agreements for RACH

1  UE monitors PDCCH for RAR during Cell DTX non-active time. The ra-ResponseWindow could be started as legacy.

2	UE monitors PDCCH for msg4 during Cell DTX non-active time. The ra-ContentionResolutionTimer could be started as legacy.

Agreements for SPS and CG

- As baseline, UE doesn’t monitor SPS occasions during Cell DTX non-active period. As baseline, gNB is assumed to be not transmitting PDSCH to that UE on such SPS occasions during the Cell DTX non-active period

- As baseline, UE does not transmit on CG occasions during Cell DRX non-active periods

Agreements for SR

- As baseline, UE does not transmit SR occasions overlapping with Cell DRX non-active periods, e.g. SR transmissions are dropped during the non-active period

FFS: whether we will allow to configure the UE per SR configuration with whether SR can be transmitted during Cell DRX non-active period to to support high priority traffic

- (for the SRs that will be dropped) If SR is not to be transmitted on an PUCCH occasion during Cell DRX non-active time, the UE keep the SR pending, i.e., the UE delays the SR transmission till the Cell DRX active period without triggering RACH.  For the FFS case there may be some exceptions.

Agreements for dynamic scheduling

- The understanding for the gNB scheduling behaviour for new transmissions during Cell DTX non-active period is that the gNB does not schedule UE-specific dynamic grants/assignments, even if the UE is in C-DRX Active Time.   UE doesn’t monitor PDCCH for dynamic grants/assignments for new transmissions during Cell DTX non-active period, even if the UE is in C-DRX Active time.   FFS how to deal with any exceptions (e.g. SR if agreed and RACH).

FFS how to deal with retransmissions

- When an DG grant is received, by the gNB during cell DRX/DTX, the UE follows the grant assignment (i.e. like in legacy).  This includes DL HARQ feedback.

Agreements for retransmission

- Working assumption:  When the retransmission timer is running (if C-DRX is configured), the UE is expected to monitor PDCCH, like in legacy.  It is up to the network whether it schedules retransmissions out of the Cell DTX active period, i.e., when the DRX retransmission timer is running, the UE should monitor PDCCH regardless of the Cell DTX.

In addition, the agreements about the Cell DTX/DRX configuration could be summarized as below:

Agreements

- A periodic cell DTX/DRX configuration is explicitly signalled to the UEs.

- A periodic cell DTX/DRX pattern is configured by UE specific RRC signalling.

- The Cell DTX/DRX configuration contains at least: periodicity, start slot/offset, on duration.

- As a baseline Cell DTX/DRX is activated/deactivated implicitly by RRC signalling, i.e. activated immediately once configured by RRC and deactivated once the RRC configuration is released.

- From RAN2 point of view, majority companies see a benefit with L1 signalling for Cell DTX/DRX activation/deactivation, send a LS to RAN1 (email 308) with our preference and ask about feasibility and design details.   Ask about feasibility and reliability of using L1 signaling.  Clarify that the question is about activation/deactivation copy the agreement from last meeting that we are focusing on single configuration.  Extract a few key benefits of dynamic signaling from email discussion and online discussions

In this paper, we will further clarify the details on cell DTX/DRX mechanism and the remaining problems.

- Discussion

  - 2.1 gNB and UE behavior

For SPS/CG, it has been agreed the initial transmission isn’t allowed during Cell DRX non-active periods as above, but the retransmission of SPS/CG isn’t clear. For dynamic scheduling of retransmission, if configuredGrantTimer expires, UE could consider the successful transmission/reception for the corresponding HARQ process. According to the above agreements, if UE transmits on CG during Cell DRX active periods or receives on SPS during Cell DTX active periods, it is up to the network whether to schedule retransmissions during Cell DTX non-active periods. In particular, the network could select to schedule retransmission in sake of the reliable transmission, or to enter into sleep in sake of NES gain. For UE, the DRX retransmission timer could keep running, and UE could monitor PDCCH for retransmission during Cell DTX non-active periods. If UE couldn’t receive PDCCH for retransmission, the successful transmission/reception is assumed. As a result, configuredGrantTimer could keep running during Cell DTX non-active periods.

Proposal 1: configuredGrantTimer could keep running during Cell DTX non-active periods.

For automatic retransmission on CG, if cg-RetransmissionTimer expires, UE could retransmit on CG. However, if UE performs automatic retransmission on CG, it requires the network to keep reception during Cell DRX non-active periods, and limits the NES gain. Hence, the automatic retransmission on CG couldn’t be allowed during Cell DRX non-active periods, and it is up to the network whether to schedule retransmission during Cell DTX non-active periods. In other words, cg-RetransmissionTimer (if configured) could stop during Cell DTX non-active periods.

Proposal 2: cg-RetransmissionTimer (if configured) could stop during Cell DTX non-active periods.

For SR, in the current specification, the SR resource could be bound with the logical channel, thus, gNB and UE could know the priority of SR resource. In order to minimize the impacts on high priority traffic, gNB could guarantee those SR occasions overlapping with Cell DRX non-active periods are not for high priority traffic via the appropriate Cell DRX configuration, hence, the exceptional SR configuration where SR can be transmitted during Cell DRX non-active period isn’t significant. Besides, a single Cell DRX pattern is simpler than a Cell DRX pattern + an exceptional SR configuration both for gNB and UE. Hence, there is no need to configure an exceptional SR configuration.

Proposal 3: There is no need to configure an exceptional SR configuration where SR can be transmitted during Cell DRX non-active period.

For the dynamical scheduling of initial transmission, according to the above agreements, during Cell DTX non-active period, initial transmission isn’t allowed, and UE doesn’t monitor PDCCH for dynamic grants for initial transmissions. Hence, UE special drx-InactivityTimer could stop during Cell DTX non-active periods.

Proposal 4: UE special drx-InactivityTimer could stop during Cell DTX non-active periods.

  - 2.2 configuration of Cell DTX/DRX

As above, when gNB determines to enable the DTX/DRX configuration, the premise is that UE performance won’t deteriorate greatly. For the delay sensitive traffic, it can be left to gNB’s suitable configuration to avoid the overlapping between the SPS/CG/SR occasions and the Cell DTX/DRX non-active period. And this strategy is feasible for gNB to minimize the impacts on UE performance. Since symbol-level SPS/CG/SR occasions are configured for delay sensitive traffic, the muting on those occasions also should be conducted at the symbol-level. Therefore, in order to implement this strategy, the DTX/DRX configuration should support symbol-level granularity.

Proposal 5: The cell DTX/DRX configuration should support symbol-level granularity.

As a result, the Nth Cell DTX/DRX non-active period occurs in the symbol, where:

symbol number = (start time in symbol + N× periodicity) modulo (total symbol number)

Proposal 6: the Nth Cell DTX/DRX non-active period occurs in the symbol, where:

symbol number = (start time in symbol + N× periodicity) modulo (total symbol number)

In RAN1#113[1], some agreements about common DCI are reached as below:

Agreement RAN1 supports the group common L1 signaling using PDCCH for cell DTX/DRX activation and deactivation without HARQ feedback - Send an LS to RAN2 to consider the additional support of a MAC CE based indication - Subject to UE capability Agreement - DCI format for group common L1 signaling using PDCCH for cell DTX/DRX activation and deactivation (downselect just one among alternatives) - Alt 1) DCI Format 2_6 (power saving information outside DRX Active Time) - FFS: Monitoring within DRX active time - FFS: Field content - Alt 2) Based on new DCI format 2_X - Field content format - Block number 1, block number 2, …, block number N - For each block should at least support the following: - DTX configuration activation/deactivation - DRX configuration activation/deactivation - FFS: other field details, mapping of UE and each blocks - DCI size indicated by higher layers - FFS: RNTI - FFS: application delay, timers for activation/deactivation - FFS: handling of multiple cells including when UE supports different number of cells - FFS: details on PDCCH monitoring aspects, including but not limited to: - Search Space - PDCCH monitoring occasion - slots to monitor (during cell DTX/DRX non-active periods, and active periods) - BD/CE aspects - FFS: UE behavior upon reception of the group common PDCCH (during cell DTX/DRX non-active periods, and active periods), including fallback behavior (if any) Agreement For the group common L1 signaling using PDCCH for cell DTX/DRX activation and deactivation - Alt 2) Based on new DCI format 2_X - DCI size budget is not increased - Number of required BDs is not increased - FFS: PDCCH monitoring configuration for the new DCI format is identical to PDCCH monitoring configuration for DCI format 2_6 if the UE monitors both DCI formats - FFS: New RNTI is used

As above, a new DCI format 2_X is introduced. According to RAN1’s discussion, this group common DCI could supply the enough coverage, thus, even cell edge UE is able to receive the group common DCI successfully. And the group common DCI could be transmitted within or out of cell DTX/DRX non-active period. In a word, new DCI format 2_X could provide great support for the cell DTX/DRX activation and deactivation. Moreover, new DCI format 2_X lacks the HARQ feedback. In order to guarantee the reliable reception, a MAC CE is suggested in RAN1. However, the benefit of this MAC CE is not clear. Firstly, if the network determines to wake up, the deactivation of the Cell DTX/DRX is always for all UE whose Cell DTX/DRX is activated, and the deactivation of the Cell DTX/DRX one by one via MAC CE may decrease the system performance. Secondly, the MAC CE is only scheduled during the UE DRX active period and cell DTX active period, this limits the transmission of deactivation MAC CE. In final, since new DCI format 2_X is enough, we suggest not to introduce an additional MAC CE for cell DTX/DRX activation and deactivation.

Proposal 7: A MAC CE for cell DTX/DRX activation and deactivation is not introduced.

  - 2.3 Alignment of Cell DTX/DRX with UE C-DRX

In SI phase, RAN2 has a consensus that the alignment of UE C-DRX with Cell DTX/DRX is beneficial. In WI phase, the understanding on how to align Cell DTX/DRX and UE C-DRX should be clarified clearly.

Firstly, the parameters of UE C-DRX, such as starting time, period, and on duration, are typically configured by the gNB implementation. gNB may consider many factors such as UE power consumption, traffic type, load balancing, and interference coordination when configuring these parameters. For example, gNB could configure a long C-DRX period for a UE with the delay tolerate traffic. gNB could stagger the starting time of C-DRX to decrease the interference between UEs. Those functions doesn’t conflict with the Cell DTX. Hence, if the Cell DTX is enabled, the impacts on those functions should be minimized.

Observation 1: gNB may consider many factors when configuring the parameters of UE C-DRX. The enabled Cell DTX should minimize the impacts on other functions.

Secondly, in order to minimize the impacts on UE performance, it is optimal that on-duration of each UE’ C-DRX falls within Cell DTX active period, and Cell DTX non-active period falls within the common part of C-DRX non-active period among UE. From UE side, UE could follow C-DRX. In this way, there is no specification impact. But, it tightly restricts the configuration of Cell DTX non-active and active period and requires UE’ C-DRX pattern to be aligned regardless whether Cell DTX is enable. And it could be implemented only under some special cases, for example, UE C-DRX non-active period among UEs could be aligned. Even, we couldn’t exclude the case that there is no common part of C-DRX non-active period among UEs. Hence, it will greatly decrease the NES gain for gNB.

Observation 2: The restriction that the on-duration of C-DRX falls within Cell DTX active period will greatly decrease the NES gain for gNB.

Moreover, the arrival of data to the gNB is always dynamic, thus, it is very common that there is no scheduling for UE during on duration time of C-DRX due to absence of arrival date. Actually, when there is no data transmission, gNB can enter into sleep mode regardless of the active period of C-DRX. Hence, the Cell DTX active period shouldn’t be limited by UE C-DRX, and it is up to gNB implementation.

Proposal 8: Cell DTX configuration is up to gNB implementation and gNB implementation shouldn’t be restricted.

As above, on-duration of C-DRX may partially overlap with Cell DTX active period. In order to saving power consumption, UE could align with Cell DTX, when gNB enters into sleep mode, UE could stop monitoring. Hence, the straightforward way is that the aligned active period means a union of Cell DTX active period and UE CDRX active period.

Proposal 9: The UE’s aligned active period is an overlapping between Cell DTX active period and UE DRX CDRX active period.

- Conclusion and proposals

Based on the analysis in previous sections, the following observations and proposals are given:

Proposal 1: configuredGrantTimer could keep running during Cell DTX non-active periods.

Proposal 2: cg-RetransmissionTimer (if configured) could stop during Cell DTX non-active periods.

Proposal 3: There is no need to configure an exceptional SR configuration where SR can be transmitted during Cell DRX non-active period.

Proposal 4: UE special drx-InactivityTimer could stop during Cell DTX non-active periods.

Proposal 5: The cell DTX/DRX configuration should support symbol-level granularity.

Proposal 6: the Nth Cell DTX/DRX non-active period occurs in the symbol, where:

symbol number = (start time in symbol + N× periodicity) modulo (total symbol number)

Proposal 7: A MAC CE for cell DTX/DRX activation and deactivation is not introduced.

Observation 1: gNB may consider many factors when configuring the parameters of UE C-DRX. The enabled Cell DTX should minimize the impacts on other functions.

Observation 2: The restriction that the on-duration of C-DRX falls within Cell DTX active period will greatly decrease the NES gain for gNB.

Proposal 8: Cell DTX configuration is up to gNB implementation and gNB implementation shouldn’t be restricted.

Proposal 9: The UE’s aligned active period is an overlapping between Cell DTX active period and UE DRX CDRX active period.

- References

[1] RAN1#113, chairman notes