3GPP TSG-RAN WG2 #123	R2- 2308284

Toulouse, FR, 21 August – 25 August, 2023

Agenda item:	7.2.2

Source:	Robert Bosch GmbH

Title:	Discussion on sidelink positioning

Document for:	Agreement

# Introduction

In this contribution, we discuss remaining sidelink positioning issues with respect to priority, discovery, cross-PLMN, handover, delays and relative positioning.

# Discussion

## SL positioning priority and parameters

### SLPP priority and SL Positioning QoS

The “Study on Architecture Enhancement to support Ranging based services and sidelink positioning” [1] §6.13.2 discusses the inclusion of the following parameters:

The Ranging/SL positioning service request includes the following parameters: - Reference UE ID - Target UE ID - Result content: distance, angle or both - Required QoS

Additionally, the range, as described in [1] §5.4.2.4, is also used for positioning QoS, and is sent together with the PQI value also in TS 23.287 [1]:

### 5.4.2.4 Range The Range value indicates the applicability of the PC5 QoS parameters in PC5 communication, i.e. when the receiving UEs are not within the Range specified distance from the transmitting UE, the communication is best effort. Lower layer (PHY/MAC layer) may use the Range to determine the corresponding packet handling, e.g. HARQ as defined in TS 38.300 [11], to achieve the QoS guarantee indicated by PC5 QoS parameters. Range is in the unit of meters. The UE is configured with the maximum Range value it can use for a particular V2X service. A V2X service may request a different range value, and the V2X layer ensures that it does not exceed the maximum Range value. Range is only used for groupcast mode communication over PC5 reference point.

Finally, in RAN1#112 meeting [2], RAN1 made the following agreement.

Agreement: From RAN1 perspective, priority value for SL PRS should be provided by higher layers from Tx UE perspective

The existing PC5 QoS profile should consider positioning QoS and associated priority for SL-PRS and SLPP transport. The extended V2X PQI handling needs to include the positioning QoS parameters and requirements which should characterise the SL-PRS and, additionally, the SLPP transport. This includes latency of positioning, accuracy and precision of the measurements (i.e., affecting number of measurements, used bandwidth, priority of the signalling) and how critical the positioning is to the application (i.e., also affects the priority).

At least in a shared resource pool for sidelink, there is a potential conflict for resources between the positioning signalling, SL-PRS transmission occasions, and sidelink data. This conflict should be solved by using priorities. Currently, there is to this date no mechanism to assign priorities as a function of the positioning QoS.

In RAN1#112 agreement [2], the priority value (from RAN1 perspective) should be coming from higher layers. The higher layer should convey at least the priority of the SL positioning for SL-PRS design including setting SL-PRS pattern, comb structure, PRS frequency, etc. Another straightforward priority value should be for SLPP transport, which should follow exactly existing PQI for PC5 data.

When sending a positioning request, the initiating entity provides the PQI value together with the request, e.g., each location service request should consider identifying the positioning QoS based on the originating application and/or service. The priority is then used for the resource allocation and SL-PRS design additionally. In this case, two possible options are envisaged:

- The higher layers send a PQI value that correspond to either Data QoS, SLPP transport QoS or ranging QoS, where priority for Data, SLPP transport QoS or ranging QoS are different.

- The higher layers send a PQI value (e.g., with indication) that corresponds to PC5 Data QoS (as in Rel-16) and another same PQI value for both SLPP transport QoS and associated ranging QoS.

The latter, Option 2, is more reasonable since the Ranging/SL Positioning priority should be equivalent to that of the associated SLPP transport priority. Herewith, SL-PRS priority is set to any of them, either ranging/SL positioning or SLPP transport. However, if the first option is considered (i.e., SLPP transport QoS and ranging QoS are different for the same service) then the SL-PRS priority should follow the values of the Ranging/SL Positioning QoS to convey exactly the ranging and positioning measurement requirements.

In RAN2#122, some companies provided their views regarding priority of SL-PRS. The possible options where considered, whether SL-PRS priority is based on:

- Option 1: Ranging/SL Positioning QoS;

- Option 2: SLPP transport QoS;

In our understanding, option 1 is more realistic for all cases and should be considered in Rel-18.

Observation 1: The existing PC5 QoS should consider positioning QoS and the possible priorities associated to positioning QoS for SL-PRS and SLPP Transport.

Observation 2: In a shared resource pool for sidelink, there is a potential conflict for resources between the positioning signalling and the data.

Observation 3: SLPP transport QoS and the Ranging/SL Positioning QoS are different, SLPP priority should follow Ranging/SL Positioning QoS if identified.

Proposal 1: Decide whether SL-PRS priority is based only on ranging/SL positioning and inform SA2 about the decision.

### PC5 QoS flow mapping to SLRBs

For PC5 QoS flow mapping to SLRBs (i.e., in Rel-16), the SDAP sublayer was considered. Additionally, if PC5 QoS flow mapping is extended to data, SLPP transport and/or ranging, RAN2 needs to consider any possible impact and whether SDAP sublayer will be impacted. This is needed for both shared and dedicated resource pool of SL positioning.

Observation 4: In a shared or dedicated resource pool, the extended PC5 QoS flow needs to be mapped to SLRB.

Proposal 2: Discuss whether mapping between the SL QoS flows (extended for Data, SLPP transport and/or ranging) and SLRBs is similar to SL in Rel-16. FFS: impacts on SDAP sublayer.

### SL-PRS periodicity

In RAN1#113 meeting, RAN1 discussed the SL-PRS resource allocation and agreed the following:

Agreement For Scheme 2, in a dedicated resource pool, - Multiple L1 SL-PRS priority are allowed in a resource pool - A SL PRS resource within the resource selection window is used as a candidate resource - with regards the reservation interval of SL-PRS, it is provided by UE’s higher layers with values TBD. The set of values is (pre-)configured. - Use the periodicities available for legacy SL communication and the ones defined for DL-PRS as a starting point. - with regards to the resource (re)-selection procedure - support re-evaluation & pre-emption for SL-PRS using the Rel-16 re-evaluation and pre-emption respectively as a starting point.

One positioning procedure may be comprised of multiple PRS slots, for instance in order to average the measurements and increase the precision (e.g. reduce standard deviation).

In the case of semi-persistent PRS frequency, an entity is responsible for setting the frequency for a defined period. In the current RAN1 discussions, it is mentioned that the PRS frequency is set by higher layers without giving more details. Additionally, for SL communications, the periodicity is set mainly based on the (remaining) PDB. However, for SL positioning procedure, the selection of periodicities should be different, e.g., based on the required accuracy and/or other QoS requirements.

Different applications may have different positioning requirements depending on external parameters (e.g., UE speed, the application nature, etc.), i.e., setting different QoS requirements for positioning. Depending on MO-LR and MT-LR sidelink positioning procedures, the LMF/server UE could translate Application layer requirements, e.g., update rate, accuracy, and precision, into an appropriate PRS period and possible positioning QoS.

Observation 5: SL-PRS reservation period can be related to the QoS requirements of applications triggering positioning procedure.

Observation 6: Based on MO-LR and MT-LR sidelink positioning procedures, the requesting entity may set the SL-PRS period together with the QoS parameters.

Proposal 3: Discuss possible RAN2 impact for setting the PRS period including whether LMF/server UE is involved.

Proposal 4: Discuss possible RAN2 impact for mapping between positioning QoS requirements and the PRS period.

## Remaining details on discovery procedure and anchor / server UE selection

RAN2 has been discussing the development of procedures for SL positioning including anchor and target UE roles and possible discovery procedure.

In RAN2#121-bis [3]:

RAN2 Agreement:

Discovery procedure is included in the sidelink positioning procedure at least for out of coverage scenario.

RAN2 Agreement:

RAN2 confirms that discovery messages will be used to carry information for targeted discovery and candidate selection of SL positioning UEs, including at least the indication of anchor UE, target UE. and server UE roles.  FFS how much information is indicated about anchor UEs (e.g., knowledge of location).

The UE role information is indicated in the discovery SLPP metafield.  FFS if this applies to both discovery modes and which messages.

### SLP in local coordinates: absolute positioning with located anchor UEs

In [4], multiple solutions for ranging in coverage, partial coverage or out of coverage are proposed. However, the only solutions allowing to make use of the absolute positions together with ranging are withing the “in coverage” case (solution #12, solution #19). Solution #34 mentions absolute positions in out of coverage mode but focusses on vetting and does not provide any information on how to use the mentioned absolute position.

In RAN2, there is to this date no solution allowing to combine absolute and relative positions. In sidelink positioning, both ranging and angle measurements (AoA) are available and allow to compute relative polar coordinates. A method called free stationing could be used to position a point with ranging and angle measurement to two located anchor UEs.

In out of coverage mode, the server UE is responsible for coordinating the positioning process. If it does not have an absolute position available (non-RAT dependent positioning or non a priori known position like for RSU or gNB), it is not able to provide absolute positioning for the target UE, even when an absolute position for a located anchor UE is known.

The available absolute position and the sidelink relative positioning capabilities could be used to provide an absolute position for the target UE. In realistic scenarios, RSU and/or fixed Anchors with known positions may be allocated along a line, e.g., along highways, tunnels, railways, etc. In order to overcome the dilution of positioning due to the effect of this linear allocation of Anchors, the free stationing can be a supportive for positioning calculations, which may rely on at least two anchors at a time to conclude the measurements.

Observation 7: Absolute coordinates of located anchor UEs are needed to compute absolute coordinates of the target UE.

Proposal 5: RAN2 to support absolute coordinates of located anchor UEs in discovery meta-field within signalling to LMF / Server UE in order to calculate absolute coordinates of target UE.

### Further Details on Discovery and Signaling Aspects

During RAN2#121 [5], the following steps were agreed as a baseline for sidelink positioning procedure:

Agreement:

With respect to the overall signaling procedure for PC5-only positioning (including at least IC and OOC; FFS if there are differences for PC), it is proposed to agree that the sidelink positioning procedure comprises the following series of steps as a baseline, between the LMF/positioning server UE/NG-RAN/candidate Anchor UE(s) and Target UE(s):

- Triggering event

- Sidelink positioning capability exchange

3.	Sidelink positioning assistance data transfer

4.	SL Positioning Request Location Information

5.	Measurement of SL-PRS

6.	Location calculation

7.	SL Positioning Provide Location Information

Some steps may have dependencies on SA2 and can be revisited in this light.  The order is subject to further discussion.  FFS if discovery and selection of anchor UEs and/or server UE are part of the positioning layer in RAN2 scope.

LS to SA2 to ask for confirmation and guidance on the SA2 aspects.

In this section, we are focusing on SL positioning/ranging in Out of coverage scenario to be defined in stage 2, where a server UE is also considered. Based on the agreed series of steps for SL positioning procedure, Figure 1 suggests a simplistic out-of-coverage SL procedure introducing Server UE, which can be separated or co-located with target UE.

In the signal flow in Figure 1, and after location service request is triggered, SL positioning capability is exchanged between server UE/Target UE and Target UE/Anchor UE(s). In this proposal, the Anchor UE(s) selection can be either in server UE or Target UE. Additionally, the SL-PRS is exchanged between Anchor UE(s) and target UE. Finally, the measurement data and positioning information are provided from Anchor UE(s) to target UE/Server UE and from Target UE to Server UE, where the final positioning calculation is conducted in the server UE.

Figure 1 UE sidelink positioning for out of coverage scenario with server UE (Optional)

Observation 8: In case of out of coverage, server UE can be captured as an optional entity in stage 2, where possible co-location with target UE should be noted.

Proposal 6: RAN2 to confirm the signaling flow for UE based sidelink positioning for out of coverage with server UE (optional, and possibly co-located with target UE) as captured in Figures 1.

### Further Details on Anchor UE selection

For candidate anchor UE selection, RAN2 #122 agreed at least to consider the following list, where an exact selection criterion (e.g., ranking) will not be discussed.

RAN2 Agreement:

Anchor UE selection is supported by information about the candidate anchor UEs.  At least the following list can be discussed for use in anchor UE selection:

1.	UE roles

2.	Supported positioning method

3.	In coverage or not

4.	RSRP

5.	LOS/NLOS

6.	Location

7.	PLMN

A normative requirement on which anchor UEs to select (e.g., ranking) will not be specified.

RAN2 impact of this information to be determined.

FFS which information would be determined statically/dynamically.

In case a server UE/LMF is involved for initial anchor UE selection after discovery, the following 2 options are envisaged (for possible down selections):

- Option 1: The LMF/server UE based approach: where the LMF/server UE is sending the candidate Anchors list for the Target to select from. The exact selection criteria is (as in RAN2#122 agreement above) is also sent by LMF/server UE based.

- Option 2: The LMF/server UE assisted approach: where the LMF/server UE may provide selection criteria (based on the 7 criteria in RAN2#122 agreement above) to the target UE for it to make its final selection accordingly.

In case no server UE/LMF is involved for initial anchor selection after discovery, the following 3rd option can be added as follows:

- Option 3: autonomous selection approach: the target is autonomously selecting the Anchors where the selection criteria is listed (as in RAN2#122 agreement above) [No need for specifying support for server UE/LMF]

Observation 9: If sever UE/LMF is considered, it may support anchor UE(s) after discovery.

Proposal 7: Based on LMF/server UE assumption, RAN2 to down select between the following options for anchor UE selection after discovery:

- Option 1: The LMF/server UE based approach: where the LMF/server UE is sending the candidate Anchors list for the Target to select from. The exact selection criteria is (as in RAN2#122 agreement above) is also sent by LMF/server UE based.

- Option 2: The LMF/server UE assisted approach: where the LMF/server UE may provide selection criteria (based on the 7 criteria in RAN2#122 agreement above) to the target UE for it to make its final selection accordingly.

- Option 3: autonomous selection approach: the target is autonomously selecting the Anchors where the selection criteria is listed (as in RAN2#122 agreement above) [No need for specifying support for server UE/LMF]

### Further Details on server UE and its selection

RAN2 agreed in #121 [5] meeting on the following functionalities of server UE:

Agreement:

RAN2 confirm that for cases without LMF involvement, besides method determination, assistant data distribution and anchor UE selection (agreed in RAN2), the SL positioning server UE may perform SL-PRS configuration coordination and location calculation.

Additionally, section 6.8 in TS 23.586 [6] dictates the following:

SL Positioning Server UE: A UE offering method determination, assistant data distribution and/or location calculation functionalities for Sidelink Positioning and Ranging based service. It interacts with other UEs over PC5 as necessary in order to determine Ranging/SL Position method, distribute assistant data and calculate the location of the Target UE. Target UE or SL Reference UE can act as SL Positioning Server UE if any of the functionalities is supported.

In RAN2#122, two different understanding were considered for this agreement (which have been captured in the Summary of AI 7.2.2):

- Understanding 1: different functionalities of server UE can be distributed over different UEs, all these UEs are considered as location server UE.

- Understanding 2: all the functionalities of server UE are located in the same UE, which is known as server UE.

In our view, understanding two is easier to consider in Rel-18 given that a server UE is considered for (at least) out-of-coverage scenarios. In this case, a separate entity that hold all functionalities of the server UE need to be defined (also in stage 2 specifications). Additionally, the second understanding is not misaligned with SA2 definition above so far. Wherein this server UE entity can be, at any time, the target UE itself or any of the Anchor UE(s) or a third-party UE (not directly involved in SL-PRS measurements, e.g., a RSU in V2X use case). Additionally, RAN2 may discuss the case when server UE is not selected initially and whether it is the target UE or Anchor UE based on the location service request, e.g., based on MO-RT or MT-RT, etc..

Observation 10: understanding 2 (all the functionalities of server UE are located in the same UE) is easier to conclude in Rel-18

Proposal 8: RAN2 to confirm that all the functionalities of server UE are located in the same UE, which can be the Targe UE, Anchor UE, or a third-party UE.

## Sidelink positioning in case of different serving PLMNs

In RAN2#122, it was agreed to include the PLMN ID for Anchor UE selection. However, it was also discussed whether the serving PLMN of candidate Anchor UE(s) has to be the same serving PLMN of Target UE (see Figure 2 for more details). In our view, cross-PLMN (e.g., where anchor and target UE are served with different PLMNs) needs to be handled in this release. One solution is to consider anchor UE(s) served by another PLMN than the target UE treated as out of coverage UE(s) of the target UE network, and subsequently in partial coverage of the PLMN of the target UE. SA2 also studied a possible solution when Anchor UE(s) is in out coverage of the target UE which can be utilized in this case.

Figure 2: illustration of the situation where SL positioning is performed within UEs that are connected to different LMFs

Observation 11: cross-PLMN sidelink positioning may impact RAN2 SL positioning procedure.

Proposal 9: RAN2 to discuss how to handle possible case when anchor UE(s) is(are) served by other PLMN as the target UE.

## Sidelink positioning during handover

A case similar as cross-PLMN that needs to be handled in RAN2 is SL positioning during a handover situation. In this, due to high mobility of target or anchor UE(s), there is always a possibility that one UE will be out coverage during SL procedure. Figure 3 depicts the situation of anchor UE performing handover during the SL positioning procedure.

Figure 3: a) one target UE and two anchor UEs in the coverage of an LMF during an SL positioning procedure; b) one target UE and two anchor UEs in the coverage of two different LMFs after HO took place during the SL positioning procedure

Since the UE(s) are all in proximity of each other within a valid range (could be, e.g., a communication range or any other valid positioning range), SL procedure may be completed while one or more UE is (are) switching to out of coverage (or, accordingly, partial coverage).

The nodes are involved in a sidelink positioning procedure, preferably a session-based one.

Therefore, the possible options for SL positioning procedure in this case, i.e., when one (or more) UE is (are) performing handover could be one or more of the following options:

- consider switching the SL positioning procedure to out of coverage when all UEs perform handover.

- consider switching the SL positioning procedure to partial coverage or out of coverage when only target UE performs handover.

- consider switching the SL positioning procedure to partial coverage or out of coverage when only anchor UEs perform handover.

Observation 12: the current procedure does not cover the handover case

Proposal 10: RAN2 to discuss handover during SL positioning procedure.

## Further SLPP signalling related timing and delays

There are some timing-related factors that affect the relevance of the estimation, e.g., when the target UE has moved between the positioning request and the positioning provision. This is especially true for high mobility UEs such as cars. RAN2 already mentioned to SA2 that relative velocity between two UEs can be estimated; however, it is not yet decided if it is a Rel-18 feature.

In addition to relative velocity, the direction of the mobility could also impact the measurements. Herewith, RAN2 should consider supporting the exchange of UE mobility characteristics (i.e. UE speed, direction). These high mobility nodes also have the advantage of having known speed and heading thanks to cooperative awareness, and also sometimes known predicted trajectories. Combined with precise timestamping of measurements and a priori estimation of delays, the LMF or server UE may be able to compensate the mobility of nodes during the SLPP.

The mobility affecting the measurements is exactly the displacement of the UEs (e.g., relative to each other) during the measurement/positioning procedure delays. Knowing/exchanging the exact timestamps should support compensating such a displacement error in positioning. Therefore, RAN2 need to discuss exchanging other assistance timing information for increasing positioning accuracy, e.g., timestamping of SLPP measurements and additional reporting delays.

Observation 13: the accuracy of the estimated position may drastically be affected by delays for high mobility nodes.

Observation 14: Exchanging of time-stamping and additional reporting delay should be used to compensate any uncounted displacement due to mobility

Proposal 11: RAN2 to consider supporting the exchange of assistance mobility information for the target and anchor UEs.

Proposal 12: RAN2 to consider supporting the exchange of assistance timing information, e.g., precise timestamping of SLPP measurements, additional delays (e.g., network delays).

## Open topics in sidelink relative positioning

### Further SLPP signalling related to per panel reporting

During RAN1#113, per panel reporting has been discussed: Agreement For provision of assistance information for sidelink positioning, the ARP location information can be provided to LMF or UE. - FFS: which UEs can receive the location information (note: which may be decided by other WGs) - FFS: details on the location information, e.g., relative location information - Note: different ARPs have their own location information Agreement For per ARP measurement - The ARP ID of an ARP used for reception can be reported along with SL positioning measurement in measurement report. The ARP ID is used to uniquely identify an ARP associated with a UE - FFS: UE can indicate whether different ARPs for Rx and Tx are used for UE Rx-Tx time difference, if the UE optionally reports the Tx time information - FFS: ARP ID of an ARP used for transmission, and details if supported

From RAN2 point of view, it is important to identify which UE can receive the (assistance) location information and what are the details of (assistance) location information.

For the study of different positioning methods in RAN1, the following aspects are considered: at least for angle-based positioning methods, antenna configuration consideration(s) using practical UE capabilities should be assumed. Wherein during SL positioning study item, the orientation of the antenna and the location of the panel were discussed, without conclusion in RAN1. However, antenna and/or ARP orientation can be very beneficial is shared, at least, as an assistance information (e.g., leaving further utilization to the UE implementation). Therefore, in Rel-18 SL positioning, further assistance antenna information should be included in the reporting of RAN2, including the orientation and allocation information of the ARP.

Observation 15: RAN1 did not identify the details on the (assistance) location information and which UE can receive the location information.

Observation 16: in the case of vehicle devices, distances between panels can be of several meters, highly impacting the estimation of the distance if not taken into account.

Proposal 13: RAN2 to consider defining the per panel report (ARP) including the panel used (ARP ID) for measurements, e.g., Tx-Rx time difference and/or angle of arrival,

Proposal 14: RAN2 to consider defining the per panel report (ARP) including possible assistance location information per APR, such as: the panel location in the device and panel orientation.

Proposal 15: RAN2 to consider whether the ARP assistance information report being received by all UEs performing sidelink positioning measurements.

### Positioning Filtering of Higher Accuracy

Multilateration (spatial) and multi-iteration (temporal) are two related concepts that are used in positioning systems to determine the location of an object or device. Multilateration refers to the process of determining the location of an object or device based on the distance between it and multiple reference points or sensors. This distance can be measured using various techniques, such as time-of-flight, angle-of-arrival, or phase differences. By combining the distances from multiple reference points, the position of the object or device can be triangulated. Multi-iteration, on the other hand, refers to the process of iteratively refining the position estimate of an object or device based on the measurement data from multiple reference points. This process involves iteratively adjusting the position estimate until it converges to the true position of the object or device.

Multilateration and mult-iteration enable accurate and efficient determination of the location of objects or devices. However, it’s important to note that both methods have limitations and can be affected by various factors, such as signal strength, multipath effects, and sensor noise.

Observation 17: multilateration and multi-iteration can be used to enhance the UE positioning accuracy.

Multilateration and/or multi-iteration can be done when Server UE requests relative positioning between target and one or more anchor UEs, especially using located anchor UEs. They both yield multiple measurements that need to be processed in order produce on estimate. To do so, higher layer filtering of measurements can be done at server UE.

Proposal 16: RAN2 to discuss whether a protocol of multiple measurements should be specified or left to the UE implementation. FFS: if specified, whether the higher layer filtering should be discussed in RAN2 or other WGs.

# Conclusion

Based on the discussion in this paper, we recommend RAN2 to discuss and adopt the following proposals:

Proposal 1: Decide whether SL-PRS priority is based only on ranging/SL positioning and inform SA2 about the decision.

Proposal 2: Discuss whether mapping between the SL QoS flows (extended for Data, SLPP transport and/or ranging) and SLRBs is similar to SL in Rel-16. FFS: impacts on SDAP sublayer.

Proposal 3: Discuss possible RAN2 impact for setting the PRS period including whether LMF/server UE is involved.

Proposal 4: Discuss possible RAN2 impact for mapping between positioning QoS requirements and the PRS period.

Proposal 5: RAN2 to support absolute coordinates of located anchor UEs in discovery meta-field within signalling to LMF / Server UE in order to calculate absolute coordinates of target UE.

Proposal 6: RAN2 to confirm the signaling flow for UE based sidelink positioning for out of coverage with server UE (optional, and possibly co-located with target UE) as captured in Figures 1.

Proposal 7: Based on LMF/server UE assumption, RAN2 to down select between the following options for anchor UE selection after discovery:

Proposal 8: RAN2 to confirm that all the functionalities of server UE are located in the same UE, which can be the Targe UE, Anchor UE, or a third-party UE.

Proposal 9: RAN2 to discuss how to handle possible case when anchor UE(s) is(are) served by other PLMN as the target UE.

Proposal 10: RAN2 to discuss handover during SL positioning procedure.

Proposal 13: RAN2 to consider defining the per panel report (ARP) including the panel used (ARP ID) for measurements, e.g., Tx-Rx time difference and/or angle of arrival.

Proposal 14: RAN2 to consider defining the per panel report (ARP) including possible assistance location information per APR, such as: the panel location in the device and panel orientation.

Proposal 15: RAN2 to consider whether the ARP assistance information report being received by all UEs performing sidelink positioning measurements.

Proposal 16: RAN2 to discuss whether a protocol of multiple measurements should be specified or left to the UE implementation. FFS: if specified, whether the higher layer filtering should be discussed in RAN2 or other WGs.

# References

| [1] | TS 23.287 Architecture enhancements for 5G System (5GS) to support Vehicle-to-Everything (V2X) services. |
| --- | --- |
| [2] | R1-2302256 Report of RAN1#112 meeting. |
| [3] | R2-2304204 Report from session on positioning and sidelink relay, RAN2 #121bis-e. |
| [4] | TR 23.700-86 Study on Architecture Enhancement to support Ranging based services and sidelink positioning. |
| [5] | R2-2301904 Report from session on positioning and sidelink relay, RAN2 #121. |
| [6] | TS 23.586 Architectural Enhancements to support Ranging based services and Sidelink Positioning. |
| [7] | Architectural Enhancements to support Ranging based services and Sidelink Positioning. |