3GPP TSG RAN WG2 Meeting #123                                      	R2-2308285

Toulouse, France, August 21-25, 2023

Agenda item:	7.6.4

Source:	Samsung

Title:	Enhancements to discontinuous coverage

WID/SID:	IoT_NTN_enh-Core

Document for:	Discussion and Decision

# Introduction

In RAN2#121bis-e meeting, RAN2 handled proposals in the email discussion “[offline 115] Discontinuous coverage enhancements” [1] and agreed the following [2]:

Agreements via email – from offline 115: - RAN2 will not introduce any enhancement to allow a UE in RRC Connected to stay in RRC_CONNECTED during/after a coverage gap (e.g. suspend RLM/RLF, activation time in RRC Reconfiguration, CHO enhancement). Agreements online: - RAN2 to introduce enhancement to RRC Release using one of the following options (FFS which one): - Explicit RRC Release using a new RRC Release cause - UE Autonomous release (e.g. timer based or upon detection of coverage gap)

Moreover, in RAN2#122 meeting, RAN2 postponed discussion on the following proposals in [3]:

*** RRC connection release enhancements *** From R2-2305959: Proposal 2a: A new release reason, e.g., ‘Release due to discontinuous coverage’ as that introduced in RAN3, can be introduced in RRC release message for indicating UE to stop the subsequent AS layer processes after it is released to idle mode. … From R2-2304812: Proposal 1: UE can indicate “RRC_IDLE” to the Network as the preferred RRC state when it is going to enter a coverage gap and start a timer, similar as the mechanism designed by R17 MUSIM. *** UE behaviour when in discontinuous coverage / possible RLF enhancements *** From R2-2305171: Proposal 3: It is beneficial in terms of UE power saving if the UE can enter RRC_IDLE upon detecting a coverage gap without executing the RLF procedure. From R2-2304812: Proposal 3：After RLF is triggered, UE enters RRC_IDLE if the remaining serving time is less than a threshold in discontinuous coverage scenario.

Additionally, RAN2 included the above proposals in a post RAN2#122 email discussion [4]:

[Post122][113][IoT NTN Enh] Discontinuous coverage (Huawei)

Scope: Discuss possible enhancements for discontinuous coverage (e.g. paging enhancements, RRC connection release enhancements, UE behaviour when in discontinuous coverage)

Intended outcome: Summary of the email discussion

Deadline:  August 10th 10:00 UTC

In this contribution, we discuss some of the remaining proposals on discontinuous coverage enhancement based on the outcomes of the email discussion “[Post122][113][IoT NTN Enh] Discontinuous coverage” [4].

# Discussion

## Out-of-coverage information for discontinuous coverage

In RAN2#121 the following proposal was proposed from [5]:

Proposal 2c): Additional measurement assistance information (e.g., PCI or serving frequency) to help UE accelerate measurements and re-gain uplink sync more efficiently is left for FFS.

One of the papers proposing this have the following proposals [6]:

Proposal 1: Measurement assistance information, i.e., PCI or carrier frequency, is provided in SIB32 to facilitate measurements and a faster uplink sync and ensure minimal disruption to paging reception after an NTN coverage gap.

For Rel-17 IoT NTN maintenance during RAN2#121 there were discussions on discontinuous coverage, more specifically the problem of what the UE shall measure and what part of AS functions that are allowed to be shut down while the UE is in discontinuous coverage [7]. As a result, the compromise was that the changes in idle mode specifications from 36.304 V17.3.0 to V17.4.0 was that only the AS functions related to NTN was powered down compared to all AS functions being powered down:

----------------- 36.304 V17.4.0 -----------------

If SystemInformationBlockType32 has been received and if the UE has determined that it is out of coverage using available satellite assistance information (e.g. ephemeris parameters and coverage parameters in current or previously received SystemInformationBlockType32, SystemInformationBlockType31, t-Service in SystemInformationBlockType3 or other parameters), the AS configuration (e.g. priorities provided by dedicated signalling and logged measurements) is kept, but the UE need not perform any idle mode tasks related to NTN. It is up to UE implementation to handle running timers. The detection of out of coverage using satellite assistance information is up to UE implementation and once in NTN coverage the UE shall perform all idle mode tasks related to NTN.

----------------- 36.304 V17.4.0 -----------------

The issue with the current specifications is that the notion of idle mode tasks related to NTN is not well-defined and can be enhanced in Release 18.

Observation 1: The notion of “idle mode tasks related to NTN” can be enhanced in Release 18.

The enhancements that can be introduced is that it can be defined to what entity or frequencies that these idle mode tasks would be related to. This could enable a network to better control how a UE may operate in a network which contains discontinuous coverage network elements. If for instance a specific frequency, that may be important to measure regardless of discontinuous coverage, this can be configured by a network. Thus we propose the following regarding measurement assistance information:

Proposal 1: Measurement assistance information can be provided to control how the UE shall perform idle mode tasks (i.e. whether a UE can power down specific frequencies during discontinuous coverage) in a discontinuous coverage NTN.

## Establishing a connection with discontinuous coverage network

In discontinuous coverage scenarios, the satellite’s coverage may only be visible to a UE during so-called satellite’s flyover time (i.e. coverage period). The signalling between the UE and network is impacted in case of insufficient flyover duration needed to complete a given RRC procedure (or a new connection establishment).

In this purpose, if a UE attempts to initiate a RRC procedure but the remaining time of the satellite coverage is insufficient to complete this procedure, and then the UE may decide to not perform this procedure or simply postpone triggering it until the next available satellite’s coverage period. This way, the UE would avoid the problem of power and resource wastage due to triggering a RRC procedure (or a new connection establishment) that is destined to fail under insufficient satellite’s coverage period.

This understanding was confirmed by the majority view in the following proposal, which was discussed in RAN2#121 [5] and RAN2#121bis [2] meetings:

For earth-moving cells, some assistance information (similar to NR-NTN) will be broadcast in SIB31 to assist the UE to verify if the remaining time of current cell’s coverage is sufficient to accommodate a new connection establishment.

Observation 2: The UE may avoid performing a given RRC procedure in the case of insufficient satellite coverage time to complete this procedure.

Moreover, in the ‘[Post122][113][IoT NTN Enh] Discontinuous coverage’ email discussion, some companies supported that the UE moves to RRC_IDLE when RLF is triggered upon detecting a discontinuous coverage. This is considering that the UE will not have enough time to complete the RRC re-establishment procedure due to the discontinuous coverage [4].

Proposal 3: UE may directly go to RRC_IDLE after RLF is triggered, if there is not enough time for the UE to finish the procedure of RRC re-establishement due to the discontinuous coverage. FFS whether a NOTE is needed for this.

In our view, this UE behavior could also extend to the triggering of any other RRC procedures that would not have enough time to be completed due to the discontinues coverage case. Hence, we propose that:

Proposal 2(a): The UE verifies whether it has sufficient coverage time to complete a given RRC procedure (or a new connection establishment).

Proposal 2(b): The UE initiates a given RRC procedure if it has sufficient coverage time to complete this procedure. Otherwise, the UE will not initiate the procedure or will wait for the next available satellite coverage period.

## RRC release for discontinuous coverage

- In RAN2#121bis-e the following was agreed [2]:

- RAN2 to introduce enhancement to RRC Release using one of the following options (FFS which one): - Explicit RRC Release using a new RRC Release cause - UE Autonomous release (e.g. timer based or upon detection of coverage gap)

- Observation 3: RAN2 is considering two options for RRC connection enhancements; (1) explicit RRC Release using a new cause, and (2) UE Autonomous release upon the UE detection of a coverage gap.

- Releasing a UE using RRC Release with a new RRC Release cause is being proposed due to RAN3 agreeing that a new release cause for discontinuous coverage is to be introduced. This is a reasonable option to be introduced for instance to ensure that certain actions are performed by the UE to reduce power consumption.

- For the UE autonomous release option, the suggested approach is that when a UE decides that it is about to enter a discontinuous coverage, the UE autonomously moves to RRC_IDLE state (or UE releases itself).

In the ‘[Post122][113][IoT NTN Enh] Discontinuous coverage’ email discussion, the two options were discussed the following is the outcome [4]:

- Option 1: Explicit RRC Release using a new RRC Release cause (6/15): - To make the UE and NW consistently understand the discontinuous coverage. - An indication from AS to NAS is also needed. - Option 2: UE Autonomous release (9/15) - To solve the DC state mismatch between UE and network - UE releases the RRC connection upon entering an unreachability period. - MUSIM like mechanism can be applied here, i.e., UE send an leave indication to NW for RRC release. - No enhancement is needed. […] Proposal 2: UE performs autonomous RRC release based on a timer (e.g., similar as MUSIM mechanism) or upon detection of the coverage gap (e.g., stop AS operation related to NTN). No further enhancement is needed

- Considering that both the UE and network are aware on the start and duration of discontinuous coverage, hence, both the network and/or the UE may trigger the RRC connection release. However, we also understand the view of companies that support Option 1 in order to ensure a better alignment between the network and the UE on the discontinuous coverage and the triggering of the RRC release.

Observation 4: both explicit RRC release with a new cause value and UE autonomous release may be considered for releasing the UE upon detection of discontinuous coverage.

Proposal 3: Explicit RRC Release using a new RRC Release cause for discontinuous coverage is introduced to ensure better alignment between the network and the UE on the UE release due to discontinuous coverage.

- As for how the UE uses the explicit RRC release message, one way is for certain idle mode procedures to be paused due to the discontinuous coverage.

Proposal 4: Certain idle mode procedures are paused if a UE receives an RRC release with the new RRC release cause.

- Another way that the new RRC Release cause can be used is when a UE is re-directed from a non-discontinuous coverage network towards a discontinuous coverage network.

Proposal 5: The new RRC release cause is used to re-direct a UE from a non-discontinuous coverage network to a discontinuous coverage network.

- When being re-directed towards a discontinuous coverage NTN, there is likely more information needed. We thus propose that information related to discontinuous coverage, such as satellite assistance information is provided in a re-direction message. In RAN2#121 [5], it was also agreed that RAN2 can continue to check whether dedicated RRC signaling may be used to provide satellite information corresponding to discontinuous coverage. We believe that one useful case for providing this information is in the use case of moving from a non-discontinuous coverage network to a discontinuous coverage network.

Proposal 6: Discontinuous coverage-related information is provided in a re-direct message when being re-directed to a discontinuous coverage network.

# Conclusion

In this contribution, we discussed some of the remaining issues related to enhancements of discontinuous coverage. The following are the observations and proposals in this document:

Observation 1: The notion of “idle mode tasks related to NTN” can be enhanced in Release 18.

Observation 2: The UE may avoid performing a given RRC procedure in the case of insufficient satellite coverage time to complete this procedure.

- Observation 3: RAN2 is considering two options for RRC connection enhancements; (1) explicit RRC Release using a new cause, and (2) UE Autonomous release upon the UE detection of a coverage gap.

Observation 4: both explicit RRC release with a new cause value and UE autonomous release may be considered for releasing the UE upon detection of discontinuous coverage.

Proposal 1: Measurement assistance information can be provided to control how the UE shall perform idle mode tasks (i.e. whether a UE can power down specific frequencies during discontinuous coverage) in a discontinuous coverage NTN.

Proposal 2(a): The UE verifies whether it has sufficient coverage time to complete a given RRC procedure (or a new connection establishment).

Proposal 2(b): The UE initiates a given RRC procedure if it has sufficient coverage time to complete this procedure. Otherwise, the UE will not initiate the procedure or will wait for the next available satellite coverage period.

Proposal 3: Explicit RRC Release using a new RRC Release cause for discontinuous coverage is introduced to ensure better alignment between the network and the UE on the UE release due to discontinuous coverage.

Proposal 4: Certain idle mode procedures are paused if a UE receives an RRC release with the new RRC release cause.

Proposal 5: The new RRC release cause is used to re-direct a UE from a non-discontinuous coverage network to a discontinuous coverage network.

Proposal 6: Discontinuous coverage-related information is provided in a re-direct message when being re-directed to a discontinuous coverage network.

# Reference

R2-2304258 [offline-115] Discontinuous coverage enhancements,	Interdigital,RAN2#121bis-e, April 2023

RAN2-121bis - NR-NTN-IoT-NTN (Sergio) 2023-04-26_EOM.docx

RAN2-122 - NR-NTN-IoT-NTN (Sergio) EOM.docx

R2-2307497 Report of [Post122][113][IoT NTN Enh] Discontinuous coverage (Huawei).

RAN2#121 Chair Notes EOM

R2-2301870 IoT NTN Enhancements to discontinuous coverage, Ericsson, RAN2#121, March 2023.

R2-2301393 Considerations on AS deactivation of discontinuous coverage, Deutsche Telekom et al, RAN2#121, March 2023.

R2-2301952 [offline 102] Enhancements to discontinuous coverage, Mediatek, RAN2#121, March 2023.