3GPP TSG-RAN WG2 Meeting #123    	                             R2-2308287

Toulouse, France, 21 - 25 August 2023

Agenda item:	7.4.3

Source:	CMCC

Title:	Discussion on NR-DC with selective activation of cell groups

WID/SID:	NR_Mob_enh2-Core

Document for:	Discussion and Decision

# 1	Introduction

In previous meeting, RAN2 discussed the scenarios, baseline procedure, delta configuration and security issue for NR-DC with selective activation of the cell groups. The following agreements were achieved in last meeting[1] and some issues were still FFS:

RAN2#122 Agreements: For SN-initiated SCG selective activation, candidate SN generates execution conditions for subsequent CPC. FFS if it shall be possible to do something like MN-initiated CPA/CPC where Candidate SN generate execution conditions for subsequent CPC The UE shall skip the condition evaluation for a candidate which is a current PScell. The reference configuration is provided to all candidates involved in preparation, FFS which node initially generates it. Assume it can be provided in MN initiated and in SN initiated procedures. Will not spend specific efforts for supporting nested configurations for candidate cell configuration. Terminology is “Subsequent CPAC”

This contribution will further discuss the open issues and potential enhancements on NR-DC with selective activation of the cell groups.

- 2 Discussion

  - Condition to release subsequent CPAC configuration

In RAN2#121 meeting[2], RAN2 agreed that the CPC/CPA configurations should be released after PCell change, at least for inter MN (by explicit indication from network) and FFS for other cases.

In SCG selective activation, the CPC/CPA configurations of the UE should be released after Pcell change, at least for inter MN (by explicit indication from network, FFS other case).

As specified in TS 37.340 [3], in legacy MN/SN initiated conditional SN change procedure, MN sends to the UE a RRCReconfiguration message which possibly includes an MCG configuration. When PCell is changed, the corresponding MCG configuration will be changed. And the UE will stop evaluating the execution condition for legacy CPC once PCell change is triggered regardless of inter-MN or intra-MN PCell change.

10.5.2 MR-DC with 5GC <omitted> MN initiated conditional SN Change <omitted> - The MN sends to the UE an RRCReconfiguration message including the CPC configuration, i.e. a list of RRCReconfiguration* messages and associated execution conditions, in which each RRCReconfiguration* message contains the SCG configuration in the RRCReconfiguration** message received from the candidate SN in step 2 and possibly an MCG configuration. Besides, the RRCReconfiguration message can also include an updated MCG configuration, e.g., to configure the required conditional measurements. <omitted> 10.6 PSCell change <omitted> A Conditional PSCell Change (CPC) is defined as a PSCell change that is executed by the UE when execution condition(s) is met. The UE starts evaluating the execution condition(s) upon receiving the CPC configuration, and stops evaluating the execution condition(s) once PSCell change or PCell change is triggered. Intra-SN CPC without MN involvement, inter-SN CPC initiated either by MN or SN are supported. <omitted>

For R18 subsequent CPAC, we think the similar mechanism to R17 can be reused, i.e. the stored subsequent CPAC configuration will be invalid and should be released upon PCell change, regardless of inter-MN or intra-MN PCell change. Therefore, the explicit indication from network is also not needed.

Proposal 1: The stored subsequent CPAC configuration should be released upon PCell change, regardless of inter-MN or intra-MN PCell change. The explicit indication from network is not needed.

In addition to PCell change, other condition/indication for releasing the stored SACG configuration should be discussed. In RAN2#121bis-e meeting[4], RAN2 agreed to release CPC candidate explicitly by RRC reconfiguration procedure.

UE will keep R18 CPC configurations after CPC execution. It should be possible to release a CPC candidate explicitly by RRC reconfiguration procedure.

We understand that the legacy method can be reused to release CPC candidate, i.e. condReconfigToRemoveList IE in RRCReconfiguration as specified in TS 38.331 [5].

Proposal 2: Reuse the condReconfigToRemoveList IE in RRCReconfiguration to explicitly release the candidate subsequent CPAC configuration.

  - SCG release or CPAC failure handling

In RAN2#121bis-e meeting, RAN2 assumed that CPA configuration can be used for CPC but with different triggering conditions. In our understanding, the CPA configuration can be also used for subsequent CPA. For example, the SCG will be released if there is no or few data/signaling transmission between UE and SN, so the UE will enter to SA mode, and it is expected to perform CPA procedure to go back to DC mode when high data rate arrives. And another case is that when the CPAC failure happens, the CPC is not applicable since there is no serving PSCell and CPA procedure is required. Hence, if the CPA configuration is kept after the first execution of CPA, the UE can perform subsequent CPA when the condition of candidate PSCell addition is satisfied.

Observation 1: UE could return to SA due to SCG release or subsequent CPAC failure, and the CPA configuration could be used for the UE that returns to SA.

Proposal 3: The stored CPA configuration can be used for subsequent CPA when UE returns to SA mode.

Other issue is whether the stored CPC configuration can be used for subsequent CPA. Even if the delta configuration is applied, the UE can form the complete/full configuration by applying the delta configuration on top of the reference configuration. Thus, we think when the UE returns to SA mode, the UE can use the stored CPC configuration for subsequent CPA if the execution condition of CPA for the same candidate PSCell is configured.

Proposal 4: The stored CPC configuration can be used for subsequent CPA with different triggering conditions.

  - Execution condition

In last meeting, RAN2 agreed that candidate SN can generate execution conditions for subsequent CPC for SN-initiated SCG selective activation, but FFS for MN-initiated SCG selective activation.

For MN-initiated case, we prefer to follow the R17 principle that the execution condition is generated by MN. If the candidate SN generates the execution condition, the issue is that a target SN does not know the existence of candidate PSCells belonging to other candidate SNs. To support this feature, the MN should transmit all candidate PSCells of candidate SNs to each candidate SN, which introduce additional signaling and complexity. Moreover, since the execution conditions are configured based on measConfig for MN, it is more suitable for MN to generate all execution conditions.

Proposal 5: For MN-initiated inter-SN SCG selective activation, source MN generates the execution conditions for all subsequent CPC.

RAN2#121bis-e meeting agreed that for MN initiated procedure, execution conditions based on event A4 are supported. FFS whether A3/A5 are supported. In legacy CPA/CPC procedure, only A4 event is supported for MN initiated procedure, and A3/A5 is not supported. We think the similar principle can be reused for MN initiated inter-SN SCG selective activation.

Proposal 6: For MN initiated inter-SN SCG selective activation, A3/A5 event is not supported.

  - Reference configuration

In RAN2#121bis-e meeting, RAN2 agreed that the reference SCG configuration and target SCG configuration are included in the initial RRC reconfiguration, and there are two FFSs left for reference MCG configuration and MCG configuration associated with the target SCG configuration.

The following may be included in the initial RRC reconfiguration message containing the Rel-18 CPC configurations: - Reference SCG configuration (Optionality FFS). Assume as for LTM Reference configuration may be empty. - FFS whether MCG configuration is included. - FFS RRC model for the reference configuration. - Initial List of candidate target PSCells (this list can be updated by the network, e.g., cells may be added or removed) with associated target SCG configurations. FFS whether the MCG configurations associated with the target SCG configurations are included. - 3. The execution conditions associated with each candidate target PSCell. - a. For MN initiated procedure, execution conditions based on event A4 are supported. FFS whether A3/A5 are supported. For MN initiated procedure, execution conditions based on event A4 are supported. FFS whether A3/A5 are supported. - b. For SN initiated procedure, execution conditions based on events A3/A5 are supported.

In R17 inter-SN CPC procedure as quoted in section 2.1, the RRC reconfiguration message contains candidate SCG configurations and possibly MCG configurations because the MCG configuration may be impacted by the addition of candidate PSCell configuration (e.g. DRB level configuration). It seems the same as in R18 subsequent CPAC where the MCG configuration can be also impacted by different candidate SCGs. Thus, the MCG configurations can be associated with the target SCG configurations included in initial RRC reconfiguration message.

Proposal 7: The MCG configurations can be associated with the target SCG configurations in initial RRC reconfiguration message.

As analyzed above, the MCG configuration may be different for different target SCG, it could be beneficial to include a reference MCG configuration for delta configuration to reduce the signaling overhead.

Proposal 8: A reference MCG configuration can be included in reference configuration.

In last meeting, RAN2 agreed to provide the reference configuration to all candidates involved in preparation, and FFS which node initially generates it. RAN2 assumed it can be provided in MN initiated and in SN initiated procedures. In our understanding, the most straightforward way is that the node initiated the subsequent CPAC procedure generates the reference configuration, i.e. SN generates the reference configuration for SN initiated procedure, and MN generates the reference configuration for MN initiated procedure. Of course, MN can retrieve the current SCG configuration as reference configuration as legacy in R17. On the other hand, if RAN2 agrees to introduce reference MCG configuration, SN also can request MN to send the current MCG configuration to SN as reference MCG configuration.

Proposal 9: SN generates the reference configuration for SN initiated intra-SN/inter-SN subsequent CPAC and MN generates the reference configuration for MN initiated inter-SN subsequent CPAC.

  - Coexistence of supported scenarios

RAN2 agreed to support SN initiated intra-SN, MN initiated inter-SN and SN initiated inter-SN SCG selective activation scenarios in RAN2#121 meeting. For SN initiated intra-SN SCG selective activation, it may be without MN involvement. For SN initiated inter-SN scenario, MN must be aware of SN initiated procedure.

Whether the following supported scenarios can coexist needs to be clarified. We understand the benefit of MN and SN both configure inter-SN SCG selective activation is unclear, and the point is whether the intra-SN and inter-SN subsequent CPAC can be both configured to the UE. If we limit that intra-SN and inter-SN subsequent CPAC cannot be configured to the UE, the flexibility of network control is also restricted, and more interaction will be introduced between SN and MN. Thus, we prefer that intra-SN and inter-SN subsequent CPAC can be both configured to the UE.

Assume to support the following scenarios of SCG selective activation: - SN initiated intra-SN SCG selective activation - MN initiated inter-SN SCG selective activation - SN initiated inter-SN SCG selective activation

Proposal 10: The intra-SN and inter-SN subsequent CPAC can be both configured to the UE.

If the above proposal is approved, the behaviour of the UE should be further clarified, e.g. when the execution conditions of intra-SN candidate PSCell and inter-SN candidate PSCell are satisfied at the same time, or when the UE changes to an inter-SN candidate PSCell. For the former, we prefer that the UE selects an intra-SN candidate PSCell because it is simple within SN and can avoid data forwarding. For the latter, we think the stored intra-SN subsequent CPAC configuration should be released since it will be invalid after inter-SN PSCell change.

Proposal 11: If the execution conditions of intra-SN candidate PSCell and inter-SN candidate PSCell are satisfied at the same time, the UE selects an intra-SN candidate PSCell.

Proposal 12: The stored SN initiated intra-SN subsequent CPAC configuration should be released when the UE changes to an inter-SN candidate PSCell, if intra-SN and inter-SN subsequent CPAC procedure are both configured to the UE.

# 3	Conclusion

Here are the observations and proposals for NR-DC with selective activation cell of groups.

Condition to release subsequent CPAC configuration:

Proposal 1: The stored subsequent CPAC configuration should be released upon PCell change, regardless of inter-MN or intra-MN PCell change. The explicit indication from network is not needed.

Proposal 2: Reuse the condReconfigToRemoveList IE in RRCReconfiguration to explicitly release the candidate subsequent CPAC configuration.

SCG release or CPAC failure handling:

Observation 1: UE could return to SA due to the release of SCG or SCG failure, and the CPA configuration could be used for the UE that returns to SA.

Proposal 3: The stored CPA configuration can be used for subsequent CPA when UE returns to SA mode.

Proposal 4: The stored CPC configuration can be used for subsequent CPA with different triggering conditions.

Execution condition:

Proposal 5: For MN-initiated inter-SN SCG selective activation, source MN generates the execution conditions for all subsequent CPC.

Proposal 6: For MN initiated inter-SN SCG selective activation, A3/A5 event is not supported.

Reference configuration:

Proposal 7: The MCG configurations can be associated with the target SCG configurations in initial RRC reconfiguration message.

Proposal 8: A reference MCG configuration can be included in reference configuration.

Proposal 9: SN generates the reference configuration for SN initiated intra-SN/inter-SN subsequent CPAC and MN generates the reference configuration for MN initiated inter-SN subsequent CPAC.

Coexistence of supported scenarios:

Proposal 10: The intra-SN and inter-SN subsequent CPAC can be both configured to the UE.

Proposal 11: If the execution conditions of intra-SN candidate PSCell and inter-SN candidate PSCell are satisfied at the same time, the UE selects an intra-SN candidate PSCell.

Proposal 12: The stored SN initiated intra-SN subsequent CPAC configuration should be released when the UE changes to an inter-SN candidate PSCell, if intra-SN and inter-SN subsequent CPAC procedure are both configured to the UE.

# 4	Reference

- Draft_R2_122_meeting_report_v2

- R2-2302401 Report of 3GPP TSG RAN WG2 meeting #121, Athens, Greece

- TS 37.340 Evolved Universal Terrestrial Radio Access (E-UTRA) and NR; Multi-connectivity; Stage 2

- R2-2306553 Report of 3GPP TSG RAN WG2 meeting #121bis-e, Online

- TS 38.331 NR; Radio Resource Control(RRC)