3GPP TSG-RAN WG2 Meeting #123      	                       R2-2308290

Toulouse, France, 21 - 25 August, 2023

Agenda item:	7.6.3.2

Source:	CMCC

Title:	Discussion on CHO enhancements for eMTC NTN

WID/SID:	IoT_NTN_enh

Document for:	Discussion

# 1	Introduction

In last meeting [1], RAN2 discussed the CHO enhancements for eMTC NTN, e.g. whether the time/location trigger must be configured together with an RRM based trigger, but had no consensus. This contribution will discuss the remaining issues CHO enhancements for eMTC NTN.

- Discussion

## 2.1	CHO enhancements for eMTC NTN

In NR NTN, the time/location-based trigger condition is always configured together with one of the measurement-based trigger conditions, considering the UE should select a target cell in good radio condition to perform handover, and is specified in TS 38.300 [2]:

16.14.3.2.2 Conditional Handover The same principle as described in 9.2.3.4 applies to NTN unless hereunder specified. NTN supports the following additional trigger conditions upon which UE may execute CHO to a candidate cell, as defined in TS 38.331 [12]: - The RRM measurement-based event A4; - A time-based trigger condition; - A location-based trigger condition. A time-based or a location-based trigger condition is always configured together with one of the measurement-based trigger conditions (CHO events A3/A4/A5) as defined in TS 38.331 [12]. It is up to UE implementation how the UE evaluates the time- or location-based trigger condition together with the RRM measurement-based event.

According to the revised WID for Rel-18 IoT NTN [3], the objective of mobility enhancements is to reuse the solutions introduced in Rel-17 NR NTN with minimum necessary changes to adapt them to eMTC.

4.1.2 Mobility enhancements The following mobility enhancements objectives are listed. - Support of neighbour cell measurements and corresponding measurement triggering before RLF, using Rel-17 (TN) NB-IoT, eMTC as a baseline. [RAN2] - Support signalling in system information of neighbour cell ephemeris, for eMTC and NB-IoT [RAN2] - Re-use the solutions introduced in Rel-17 NR NTN for mobility enhancements for eMTC, with minimum necessary changes to adapt them to eMTC [RAN2] - Define UE RRM core requirements for the above mobility enhancement features [RAN4].

Therefore, we think the similar mechanism can be reused for eMTC NTN, and how the UE evaluates the time-based or location-based trigger condition together with the RRM measurement-based event is up to UE implementation.

Proposal 1: Same as NR NTN, time/location-based trigger condition is always configured together with one of the measurement-based trigger conditions (CHO events A3/A4/A5).

Another issue is whether the time-based and location-based trigger condition can be configured jointly. In R17 NR NTN, joint configuration is not supported. The same principle can also be reused in eMTC NTN.

Proposal 2: Same as NR NTN, joint time-based and location-based CHO execution triggering for the same candidate cell is not supported for eMTC NTN.

# 3	Conclusion

Here are the proposals for CHO enhancements for eMTC NTN.

Proposal 1: Same as NR NTN, time/location-based trigger condition is always configured together with one of the measurement-based trigger conditions (CHO events A3/A4/A5).

Proposal 2: Same as NR NTN, joint time-based and location-based CHO execution triggering for the same candidate cell is not supported for eMTC NTN.

# 4	Reference

- Draft_R2_122_meeting_report_v2

- TS 38.300, NR; NR and NG-RAN Overall Description; Stage 2

- RP-223519, Revised WID on IoT NTN enhancements, Mediatek Inc., RAN#98-e, December 2022