3GPP TSG-RAN WG2 Meeting #123	R2-2308305

Toulouse, France, August 21-25, 2023

Agenda Item:	7.11.2.2

Source:	CMCC

Title:	Discussion on multicast reception in RRC_INACTIVE UP issues

Document for:	Discussion

Introduction

At the RAN2#121bis meeting[1], the following agreements were agreed regarding the UP design for multicast reception in RRC_INACTIVE：

From the location&bandwidth and SCS configuration perspective, follow R17 MBS broadcast CFR principle (i.e. case A,C,E) to provide multicast CFR configuration in RRC_INACTIVE. Multicast CFR in RRC_INACTIVE and broadcast CFR can be configured differently. FFS whether we need to restrict that one CFR is completely contained within the other in this case (we should understand what the issue is otherwise). Case B and case D are not supported for multicast CFR in RRC_INACTIVE; Whether multicast CFR in RRC_CONNECTED and in RRC_INACTIVE are different is up to NW implementation. FFS whether this causes some issues which need to be addressed. HARQ feedback related information in the DCI is not needed or can be ignored for multicast transmission to RRC_INACTIVE UE. The HARQ operation for multicast reception in RRC_INACTIVE is same as the operation without HARQ feedback in RRC_CONNECTED state. The multicast transmission in RRC_INACTIVE is performed via beam sweeping based on SSB index like broadcast MBS (i.e. beam information is not needed in DCI). For MTCH, RAN2 assumes to reuse the same DCI format of R17 multicast (i.e. DCI format 4-1/4-2) for dynamic scheduling of multicast in RRC INACTIVE. RAN2 assumes for MCCH scheduling, DCI format 4-0 is used. We will ask RAN1 to confirm whether it is feasible and whether both 4-1 and 4-2 are needed. We will also indicate other relevant agreements to RAN1 (e.g. on beam sweeping etc.) On support of multicast SPS in RRC_INACTIVE, postpone RAN2 discussion to next meeting. On DRX operation for multicast in RRC_INACTIVE, take the multicast DRX as baseline. FFS handling on PTM related HARQ RTT Timer and DRX Retransmission Timer. The common LCID space is used for multicast MRB and unicast DRB regardless of UE RRC state (i.e. no change on the LCID table for MTCH). Postpone the UP discussion on L2 operation during RRC state transition until the signaling design of PTM configuration in RRCRelease message is concluded. Postpone the discussion on L2 operation during mobility to next RAN2 meeting. Including the following two issues in LS to RAN1: - Issue 1: RAN1 to confirm RAN2 understanding that PDSCH aggregation is supported for multicast MTCH in RRC_INACTIVE (as that is supported in Rel-17 multicast MTCH in RRC_CONNECTED as well as broadcast MTCH). - Issue 2: RAN1 to check the feasibility of following Rel-17 CSS design for multicast MTCH and MCCH: 1) reusing the same CSS for multicast MTCH in RRC_INACTIVE (same as multicast MTCH in RRC_CONNECTED); 2) separate CSS for MCCH and MTCH. Change the working agreement to the agreement below: - Agreement: The same CFR is used for multicast MCCH and MTCH. It can be revisited if there is any issue found, e.g. for RedCap UEs. UE in RRC CONNECTED state is not required to read multicast MCCH to be able to receive multicast MBS service i.e. the UE receives the PTM configuration via dedicated signalling. This can be revisited if issues with service continuity are identified.

In this contribution, we provide our further consideration on the related open issues.

Discussion

Multicast CFR in RRC_INACTIVE

In RAN2#121bis meeting, we agreed that “from the location&bandwidth and SCS configuration perspective, follow R17 MBS broadcast CFR principle (i.e. case A,C,E) to provide multicast CFR configuration in RRC_INACTIVE. ” and “Multicast CFR in RRC_INACTIVE and broadcast CFR can be configured differently.”

Since both multicast CFR in RRC_INACTIVE and broadcast CFR may have the bandwidth same with or larger than CORESET#0, in other words, multicast CFR in RRC_INACTIVE and broadcast CFR have some overlapping, at least overlapping with CORESET#0, therefore, some companies propose to restrict that one CFR is completely contained within the other. In the proponent point of view, it enables the UE receives both multicast service in RRC_INACTIVE state and broadcast service to monitor only a larger CFR, which may help to UE’s overhead reduction. But in Rel-17, the simultaneous reception of multicast and broadcast is also supported, and no there’s no such restriction to the CFR configuration, therefore, we think the same principle can be reused. Besides, even one CFR is completely contained the other one, the configuration of the two CFR may be different, we don’t see the clear gain for the UE. What’s more, the same broadcast service may be received by both Rel-17 MBS UEs and Rel-18 MBS UEs, the influence on Rel-17 MBS UEs with introducing such restriction is not clear.

Observation 1: There’s no CFR configuration limitation like one CFR is completely contained within the other for Rel-17 UEs receiving both multicast in RRC_CONNECTED and RRC_INACTIVE.

Observation 2: The configuration of the CFRs may be different, which may reduce the potential gains.

Observation 3: Restricting one CFR is completely contained within the other may have influence on Rel-17 broadcast UEs.

Based on the analysis above, we prefer not to introduce the restriction that for multicast CFR in RRC_INACTIVE and broadcast CFR, one CFR is completely contained within the other.

Proposal 1: For multicast CFR in RRC_INACTIVE and broadcast CFR, one CFR is not needed to be completely contained within the other.

As to the relationship of the multicast CFR in RRC_INACTIVE and multicast CFR in RRC_CONNECTED, the current agreement is “Whether multicast CFR in RRC_CONNECTED and in RRC_INACTIVE are different is up to NW implementation.” For multicast CFR in RRC_INACTIVE, it has overlapping with CORESET#0, while for multicast CFR in RRC_CONNECTED, it is within UE’s active BWP, which may have no overlapping with CORESET#0. And in RAN2#119 meeting, the following is taken as baseline: we assume the same PDCCH/PDSCH resources (e.g. resources used for MTCH) can be used for all UEs (including UEs in CONNECTED and/or INACTIVE states) for receiving the same multicast session. If the CFR for multicast in RRC_INACTIVE and RRC_CONNECTED is different, there’s possibility that the different PDSCH resources will be used based on the physical scheduling algorithm. Thus, RAN2 need to discuss if the multicast CFR in RRC_CONNECTED and in RRC_INACTIVE are different, how to insure the same PDSCH resources can be used.

Observation 4: If the multicast CFR in RRC_CONNECTED and in RRC_INACTIVE are different, there’s possibility that the different PDSCH resources will be used for the same data, which is not align with the current agreements.

Proposal 2: RAN2 need to discuss if the multicast CFR in RRC_CONNECTED and in RRC_INACTIVE are different, how to insure the same PDSCH resources can be used.

SPS for multicast in RRC_INACTIVE

In the Post121 email discussion [2], whether to support SPS for multicast in RRC_INACTIVE was discussed and there’s no consensus. With SPS, the PDCCH overhead can be reduced. But from our point of view, since in Rel-17 multicast, or in other features, it aims to RRC_CONNECTED UEs, and it is not supported by Rel-17 broadcast, and the capacity of PDCCH is not a big issue. Besides, since the multicast reception in RRC_INACTIVE does not support HARQ feedback, it is not feasible to support the SPS HARQ without HARQ feedback. Therefore, we think there’s no need to support SPS for multicast reception in RRC_INACTIVE.

Proposal 3: SPS is not supported for multicast reception in RRC_INACTIVE.

L2 operation during RRC state transition

At the beginning of the discussion of multicast reception for RRC_INACTIVE UEs, two scenarios are identified and agreed in RAN2 [3]:

- - Scenario 1: a UE has been receiving multicast in CONNECTED, and it enters INACTIVE and continues the multicast reception. - - Scenario 2: a UE has joined a multicast session and has been directed to INACTIVE, the UE starts to receive the multicast session

In scenario 1, the UE receives multicast service in RRC_CONNECTED state and RRC_INACTIVE state successively. When receiving multicast in RRC_CONNECTED, the UE establishes MRB as the configuration provided by the network, including MRB ID, and configurations for different protocol layers.

Based on TS 38.331, when the UE receives RRCRelease Message, the UE shall reset/suspend protocol layers, apply the suspendConfig and suspend all SRB(s) and DRB(s) and multicast MRB(s), except SRB0 and broadcast MRBs [4].

5.3.8.3 Reception of the RRCRelease by the UE The UE shall: …… 1> if the RRCRelease includes suspendConfig: 2> reset MAC and release the default MAC Cell Group configuration, if any; 2> apply the received suspendConfig except the received nextHopChainingCount; …… 2> suspend all SRB(s) and DRB(s) and multicast MRB(s), except SRB0 and broadcast MRBs; 2> indicate PDCP suspend to lower layers of all DRBs and multicast MRBs; ……

As agreed in last RAN2 meeting that suspendconfig is to provide the PTM configuration for the UE:

As a baseline, The PTM configuration in the RRCRelease message with suspendconfig has the same structure as the PTM configuration in multicast MCCH.

Therefore, for the UE receives multicast service in RRC_CONNECTED first and then switched to RRC_INACTIVE, the multicast MRB used in RRC_CONNECTED will be suspended and a new MRB for the same service will be established for RRC_INACTIVE reception, which may result in resource wasting in some sense.

Besides, due to the reset of MAC, the soft buffers for all DL HARQ processes will be flushed, except for the DL HARQ process being used for MBS broadcast, therefore, the service continuity will be affected.

Observation 5: When a multicast receiving UE is switched from RRC_CONNECTED to RRC_INACTIVE for continuous reception, the multicast MRB used in RRC_CONNECTED will be suspended and the related HARQ process will be flushed and a new multicast MRB used in RRC_INACTIVE may be established, which results in resource wasting and service interruption.

To avoid the resource wasting and service interruption mentioned above, it’s proposed to not suspend multicast MRB, and if the HARQ process used for multicast reception doesn’t support HARQ feedback, it is not flushed, if the UE is configured to continue receiving multicast in RRC_INACTIVE state, for example, the PTM configuration is included in suspendconfig.

Proposal 4: It’s proposed to not suspend multicast MRB and not flush the related HARQ process not supporting HARQ feedback used for multicast reception, if the UE is configured to continue receiving multicast in RRC_INACTIVE state, for example, the PTM configuration is included in suspendconfig.

Since the multicast MRB used in RRC_CONNECTED is not suspended, the multicast MRB in RRC_INACTIVE can inherit from the MRB in RRC_CONNECTED, for example PDCP configuration, RLC configuration, MRB ID, etc.

Proposal 5: The multicast MRB in RRC_INACTIVE can inherit from the MRB in RRC_CONNECTED, for example PDCP configuration, RLC configuration, MRB ID, etc.

Conclusions

In this contribution, we analyse the open issues of shared processing, following are our observations and proposals:

Observations:

Observation 1: There’s no CFR configuration limitation like one CFR is completely contained within the other for Rel-17 UEs receiving both multicast in RRC_CONNECTED and RRC_INACTIVE.

Observation 2: The configuration of the CFRs may be different, which may reduce the potential gains.

Observation 3: Restricting one CFR is completely contained within the other may have influence on Rel-17 broadcast UEs.

Observation 4: If the multicast CFR in RRC_CONNECTED and in RRC_INACTIVE are different, there’s possibility that the different PDSCH resources will be used for the same data, which is not align with the current agreements.

Observation 5: When a multicast receiving UE is switched from RRC_CONNECTED to RRC_INACTIVE for continuous reception, the multicast MRB used in RRC_CONNECTED will be suspended and the related HARQ process will be flushed and a new multicast MRB used in RRC_INACTIVE may be established, which results in resource wasting and service interruption.

Proposals:

Proposal 1: For multicast CFR in RRC_INACTIVE and broadcast CFR, one CFR is not needed to be completely contained within the other.

Proposal 2: RAN2 need to discuss if the multicast CFR in RRC_CONNECTED and in RRC_INACTIVE are different, how to insure the same PDSCH resources can be used.

Proposal 3: SPS is not supported for multicast reception in RRC_INACTIVE.

Proposal 4: It’s proposed to not suspend multicast MRB and not flush the related HARQ process used for multicast reception if the UE is configured to continue receiving multicast in RRC_INACTIVE state, for example, the PTM configuration is included in suspendconfig.

Proposal 5: Some of the PTM configuration for the multicast MRB in RRC_INACTIVE should be consistent with the previous one, such as the MRB ID, which can be treated as the reconfiguration of the multicast MRB used in RRC_CONNECTED.

References

- R2_121bis_meeting_report;

- R2-2303420	Summary of [Post121][607][eMBS] UP issues for Multicast in RRC Inactive (Apple)	Apple	discussion	Rel-18	NR_MBS_enh-Core

- R2_119_meeting_report;

- 3GPP TS 38.321 Medium Access Control (MAC) protocol specification.