3GPP TSG RAN WG2 #123 Toulouse, France	R2-2308311

Aug 21st-25th, 2023

Agenda item:	7.5.4.3

Source:	CMCC

Title:	Consideration on CG enhancement for XR

Document for:	Discussion

1	Introduction

In RAN2 122[1], consensus has been reached on CG enhancement for XR, which are shown in the following:

For retransmission less CG enhancement in XR, adapt the NTN solution by disabling the HARQ RTT timer per CG configuration. Specifically, the following modifications shall be introduced:

- •	A new RRC parameter for disabling drx-HARQ-RTT-TimerUL for a CG configuration;

- •	Changes in the procedural text of DRX operations for CG in the MAC specification;

- •	A new UE capability for supporting disabling drx-HARQ-RTT-TimerUL for a CG configuration.

In this contribution, a discussion on CG enhancement will be conducted for XR service.

2	Discussion

2.1 RAN1’s LS

RAN1 send an LS to RAN2[2] to inform RAN2 about the determination of HARQ process ID of CG PUSCHs of a multi-PUSCHs CG.

The specific determination formula is:

Agreement From RAN1 perspective, for determination of HARQ process IDs associated to PUSCHs in multi-PUSCHs CG assuming one TB per PUSCH: - The HARQ process ID for the first configured PUSCH in a period is determined based on the legacy CG procedure when cg-RetransmissionTimer is not configured, and applying the following formula, whichever is applicable - HARQ Process ID = [X*floor( (CURRENT_symbol ) / periodicity)] modulo nrofHARQ-Processes - HARQ Process ID = [X*floor((CURRENT_symbol ) / periodicity)] modulo nrofHARQ-Processes + harq-ProcID-Offset2 - X= the number of configured PUSCHs in the CG period - The HARQ process ID of the remaining configured and valid CG PUSCHs in the period is determined by incrementing the HARQ process ID of the preceding PUSCH in the period by one with module operation with nrofHARQ-Processes or module operation with (nrofHARQ-Processes + harq-ProcID-Offset2), whichever applicable. - Note: A configured CG PUSCH is invalid if the CG PUSCH is dropped due to collision with DL symbol(s) indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated or SSB.

It has impact on TS38.321, however, it can be handled later since other modifications (e.g., retransmission-less CG) also has impact on TS38.321.

2.2 Multi-PUSCH CG

Multiple-PUSCH CG still has some issue to discuss. The main issue is Multi-PUSCH CG configuration, and Multi-PUSCH CG configuration should be the responsibility of RRC messages (MAC entity is not preclude), and RAN1 also agrees with this point.

Agreement Indication of UTO-UCI by CG PUSCHs associated to a CG configuration, is enabled by configuration of an RRC parameter. FFS on whether/how to extend to multiple CG configurations

Considering the current progress of RAN1 and RAN2, For Multi-PUSCH configuration, compared to the legacy CG configuration, it may also be necessary to extend its content for transmission resource indication for UTO-UCI and Multi-PUSCH TO resourse, and also the retrransmission less CG related configruations.

For UTO-UCI, there currently exists a dispute in RAN1 whether UTO-UCI will be jointly encoded with ACK/NACK. If joint encoding is used, UTO-UCI will be sent together with ACK/NACK, which will have a smaller impact on the specification. If non-joint encoding is used, it may require some RRC configurations to semi-statically indicate UTO-UCI resources.

The existing CG-UCI encoding and multiplexing procedures are reused for encoding the “UTO-UCI” in a configured grant PUSCH in absence or presence of other UCIs being multiplexed in the PUSCH, by applying the following adjustments: The “UTO-UCI” is used instead of CG-UCI in the corresponding procedures for encoding of CG-UCI and/or HARQ-ACK and/or CSI, whichever is present. For determining the beta-offset, Beta offset is configured for the “UTO-UCI” and applied when applicable. If UTO-UCI and HARQ-ACK is not jointly encoded, the beta offset for the “UTO-UCI” is used in the procedures instead of CG-UCI beta offset, when applicable. If UTO-UCI and HARQ-ACK is jointly encoded, HARQ-ACK beta offset is used in the procedures instead of CG-UCI beta offset FFS on sequence generation order between UTO-UCI and HARQ-ACK FFS on dropping rule between UTO-UCI and HARQ-ACK when joint encoding is not configured Note: The term “UTO-UCI” refers to the “UCI that provides information about unused CG PUSCH transmission occasions” for convenience.

Hence, RAN2 can wait for RAN1’s decision.

Proposal 1: Wait for RAN1's conclusion on whether UTO-UCI and HARQ-ACK is jointly encoded or not then discuss PUCCH resource allocation for UTO-UCI.

For the configuration of Multi-PUSCH TO, the CG configuration requires the number of TOs and the interval between TOs to be configured. There are two options for the interval between TOs. The first option is that the time domain interval between all TOs is a single value, and the second option is that the time domain interval between TOs is configured by the RAN. We believe that both of these configurations should not be excluded at the current stage.

Proposal 2: RAN2 to agree that the number of TO and the time domain interval of TO should be provided in Multi-PUSCH CG configuration.

If interval of TOs is single valued, all TOs are uniformly distributed within CG period. If not, the interval between TOs is uneven. It is generally believed that jitter obeys the Poisson distribution in time domain, that is, the probability of small jitter is high, so it is not a wise choice to use a uniform interval between TOs.

We believe that the other reason for using uneven TOs is that it can cover a longer period with fewer TOs and makes it better for adopting jitter. And it can better adapt jitter and reduce signaling overhead.

This consideration comes with that too many TOs can affect the scheduler's resource allocation strategy, e.g., radio resources may not be used by other UEs due to potential TO transmissions, and too many TOs may also cause the signaling overhead of the indicated unused CG TOs to become significant.

Observation 1: Uneven interval between TOs is more appreciated since jitter obeys Poisson distribution other than Uniform distribution. Too many multi-PUSCHs CG TOs can cause overhead and difficulty in indicating unused CG PUSCH TOs.

We simulated different TO distributions using jitter within the range of [-4, 4]ms, where the jitter distribution is shown in the figure below:

The simulation results are shown in the figure, where the uneven interval and TO-number-fewer distributions achieved almost the same delay and jitter coverage as the even interval and TO-number-more distributions for the same TO range.

Hence, both even and uneven interval of TO should be discussed.

Proposal 3: The time domain interval of TO within CG period can be single or multiple value.

In addition, due to the non-integer period issue also existing in DRX, CG can refer to the solutions in DRX. According to the Chairs note in RAN2#122, the DRX cycle is configured based on rational numbers. Therefore, the CG period can also be a rational number.

Proposal 4: RAN2 to agree to introduce rational period for CG, as already agreed in DRX.

2.3 BSR for UTO indication

Another issue that affects RAN2 is whether to use TOs that have been indicated as unused. This issue has already been discussed in RAN1, where it was stated that unused TOs cannot be used by UEs for transmission, while TOs that have not been indicated as unused can be used or not used by UEs.

Agreement A CG PUSCH occasion indicated as “unused” earlier, is not allowed to be indicated as “NOT unused later”. A CG PUSCH occasion indicated as “NOT unused” earlier, can be indicated as “unused” later. FFS: Whether there is specification impact

However, when indicating an unused TO, besides UTO-UCI, padding BSR, especially in CG TO, can also be a similar indication. Any padding BSRs with zero buffer size can indicate that the UE has no data in uplink thus no more TO is needed.

Additionally, the time when the UE assemble TB and padding BSR is earlier than when it sends the corresponding UTO-UCI. Therefore, UE is aware whether TB contains padding BSR and decide whether to send UTO-UCI. If padding BSR exists, UE does not need to send the corresponding UTO-UCI, thus avoiding unnecessary signaling.

Proposal 5: RAN2 to agree that padding BSR with zero buffer can also indicate unused TO.

2.4 Retransmission-less CG

In the previous meeting, the main topic of discussion was about Retransmission-less CG, including specific configurations, UE capability-related issues, and specification impacts. The characteristics of HARQ Mode B and HARQ timers are largely derived from NTN, but the reliability requirements for traffic flow in NTN are different from those in XR. Data transmitted using Retransmission-less CG is generally considered to be pose and control data, and haptic data possibly being included in the future. This data is crucial for reducing user dizziness, and we therefore suggest that RAN2 discuss reliability enhancements for the data transmitted by Retransmission less CG, such as using PUSCH repetition and Low SE MCS table.

Proposal 6: RAN2 to introduce reliability assurance mechanism for retransmission-less CG, e.g., PUCCH repetition, using Low SE MCS table.

3	Summary

This contribution discusses potential capacity enhancement for XR, the following are the observations and proposals that may be taken into consideration.

Multiple-PUSCH CG configuration aspect:

Proposal 1: Wait for RAN1's conclusion on whether UTO-UCI and HARQ-ACK is jointly encoded or not then discuss PUCCH resource allocation for UTO-UCI.

Proposal 2: RAN2 to agree that the number of TO and the time domain interval of TO should be provided in Multi-PUSCH CG configuration.

Observation 1: Uneven interval between TOs is more appreciated since jitter obeys Poisson distribution other than Uniform distribution. Too many multi-PUSCHs CG TOs can cause overhead and difficulty in indicating unused CG PUSCH TOs.

Proposal 3: The time domain interval of TO within CG period can be single or multiple value.

Proposal 4: RAN2 to agree to introduce rational period for CG, as already agreed in DRX.

Indication of UTO aspect:

Proposal 5: RAN2 to agree that padding BSR with zero buffer can also indicate unused TO.

Reliability of Retx-less CG aspect:

Proposal 6: RAN2 to introduce reliability assurance mechanism for retransmission-less CG, e.g., PUCCH repetition, using Low SE MCS table.

References

- R2-2306542, Report on LTE legacy, XR, QoE and MUSIM, Vice Chairman (Nokia)