# 3GPP TSG-WG2 Meeting #123                                   R2-2308324

# Toulouse, France, Aug, 2023

Source: 			CMCC

Title:  			Considerations on multi-path scenario 2

Agenda item:	7.9.4

Document for: 	Discussion

Introduction

In last RAN2 meeting, we have the following conclusion for MP scenario 2. With the post email discussion#419, some open issues are left for further discussion. In this contribution, we will discuss those open issues of scenario 2.

- WA: For scenario 2, remote-UE reports the RRC_CONNECTED relay-UE C-RNTI and serving cell ID (e.g., NCGI) for indirect path addition.

Discussion

Terminology

During last meeting, RAN2 discussed how to capture the terminology of UE in scenario 2 to TS 38.300.

As we clarifies with the discussion on scenario 2, the typical scenario of UE aggregation is to address the restricted UL bitrates and fulfills the 5G UL wide bandwidth service requirement. The linking topology in scenario 2 we expect is the anchor UE connects to more than one associated UE. So, the terms “anchor UE” and “associated UE” are suggested to be used for scenario 2. In addition, the scenario 2 is suggested to definite as “UE aggregation” to distinguish from SL Relay scenario 1.

Proposal 1: Terms “UE aggregation” is suggested to definite scenario 2.

Proposal 2: Terms “anchor UE” and “associated UE” are suggested to be used for scenario 2.

UE-UE association

UE association between anchor UE and associated UE is assumed as pre-configured or static. gNB pre-configures the associated UE information to an authorized UE, before the indirect path is added to anchor UE. Considering the flexibility of associated UE selection in scenario 2, anchor UE can inform gNB with one or more candidate associated associated UE and gNB make decision on the selection of the associated UE.

Proposal 3: Anchor UE can inform gNB with one or more candidate associated UEs and gNB decides the associated UE.

How to identify the associated UEs in scenario 2 had been discussed, and RAN2 reached one WA that for associated UE in RRC_CONNECTED state, C-RNTI can be used as associated UE ID. However, for associated UEs in RRC_IDLE/RRC_INACTIVE, no conclusion was reached.

In our understanding, gNB can pre-configured local ID to associated UEs. When associated UEs in RRC_CONNECTED, the C-RNTI can be used; for associated UEs in RRC_IDLE/RRC_INACTIVE the pre-configured local ID can be use as UE identification.

Proposal 4 : For associated UEs in RRC_IDLE/RRC_INACTIVE, gNB pre-configured local ID can be used as UE identification.

For scenario 2, it was agreed that it is left to UE implementation to trigger the RRC_IDLE/RRC_INACTIVE target associated UE to initiate RRC connection establishment procedure. However, if the associated UE is in RRC_CONNECTED state before the gNB configure it, it will avoid latency caused by RRC establishment. The RRC state can be taken into account when anchor UE reports the associated UEs or when gNB select the associated UE. We have some concerns on the idea that anchor UE triggers the RRC_IDLE/RRC_INACTIVE associated UE into RRC_CONNECTED state while reporting the associated UE information. For the UE selected as associated UE, it may accelerate the multi-path establishment procedure. But, for UE who are not chosen, it brings extra expanses.

Proposal 5: The RRC state can be taken into account when anchor UE reports the candidate associated UE or when gNB decides the associated UE.

UE-UE link failure

RAN2 have discussed how to handle the failure over UE-UE link in scenario 2 and agreed that UE-UE link failure detection is supported. Anchor UE can report UE-UE link failure to network via direct path, once the link failure is detected. From network’s perspective, it can reconfigure another associated UE to anchor UE for indirect link recovery. The associated UE to be reconfigured as indirect path can be in the list of candidate associated UE as informed by anchor UE before. The anchor UE can also report one or more associated UEs to gNB along with the UE-UE link failure report.

Proposal 6: Anchor UE can report UE to UE link failure to network via direct path, once the UE-UE link failure is detected.

Proposal 7: Network can reconfigure another associated UE for indirect link recovery, where that associated UE can be the candidate associated UE as the informed by anchor UE.

Besides the scenario of UE-UE link failure and recovery, gNB can re-configure another associated UE to anchor UE to change the indirect path for better system performance with some trigger events. Comparing with the recovery for UE-UE link failure, indirect path change can lower the latency and avoid the possible data loss.

Proposal 8: Supporting indirect path change for scenario 2.

Indirect path adding

For scenario 2, the link between UE to UE are non-standard interface, the link establishment procedure is out of 3GPP scope. However, the trigger event for indirect path adding should be discussed in RAN2. As RAN2 concluded that anchor UE can report the C-RNTI to gNB for associated UE in RRC_CONNECTED, and we proposed in this contribution that anchor UE can report the pre-configured local ID to gNB for associated UE in in RRC_IDLE and in RRC_INACTIVE, those identifications of associated UE are used to inform gNB about the association between the anchor UE and its associated UE. Such reporting does not trigger the indirect path addition for scenario 2.

After receiving the associated UE identification from the anchor UE, gNB can decide not add the indirect path immediately unless it is necessary, to avoid wasting of transmission resource. The gNB can decide to add the indirect path, based on gNB implementation or UE report indirect path addition request. For gNB implementation, it can take into account some measurement result, traffic requirement, or UE assistant information, for example, the UE buffer size status report, PHR, high priority or special service type and some other conditions. Here, we suggest RAN2 to have further discussion on that.

Proposal 9: Ask RAN2 to discuss the trigger events for indirect path addition in scenario 2.

Protocol stack

With the progress on topology and the presence of adaption layer for scenario 2, it is very clear that the protocol stack for scenario 1 and 2 are different. The typical scenario 1 is multiple anchor UEs associated to one associated UE, the SRAP is necessary to distinguish the packets from different anchor UEs encapsulated in one logical channel. Conversely, as in scenario 2, one associated UE connects to more than one anchor UE, where SRAP is not needed and the data routing for split bearers can be taken over by PDCP, as illustrated in Figure 1-1 for UP and figure 1-2 for CP.

Figure.1-1 DC-like UP protocol stack for scenario 2

Figure.1-2 DC-like CP protocol stack for scenario 2

Alternatively, for UE aggregation, we propose another possible kinds of protocol stack, DAPS like, which is shown as Figure.1-3 and figure.1-4. For DAPS-like UE aggregation, the aggregated RB will be configured with two or multiple PDCP entities and RLC entities for different UEs respectively, which are involved in the delivery of the aggregated service. Besides, PDCP in anchor UE will response for the unified SN allocation across the anchor UE and associated UE(s). PDCP SDU with allocated SN will be further handled by PDCP in aggregated UE for security and ROHC.

Figure.1-3 UP protocol stack for scenario 2 (DAPS-like)

Figure.1-4 CP protocol stack for scenario 2 (DAPS-like)

Proposal 10: Agree TP to TR 38.836 as in Annex.

Conclusions

According the above discussion we have following proposals:

Proposal 1: Terms “UE aggregation” is suggested to definite scenario 2.

Proposal 2: Terms “anchor UE” and “associated UE” are suggested to be used for scenario 2.

Proposal 3: Anchor UE can inform gNB with one or more candidate associated UEs and gNB decides the associated UE.

Proposal 4 : For associated UEs in RRC_IDLE/RRC_INACTIVE, gNB pre-configured local ID can be used as UE identification.

Proposal 5: The RRC state can be taken into account when anchor UE reports the candidate associated UE or when gNB decides the associated UE.

Proposal 6: Anchor UE can report UE to UE link failure to network via direct path, once the UE-UE link failure is detected.

Proposal 7: Network can reconfigure another associated UE for indirect link recovery, where that associated UE can be the candidate associated UE as the informed by anchor UE.

Proposal 8: Supporting indirect path change for scenario 2.

Proposal 9: Ask RAN2 to discuss the trigger events for indirect path addition in scenario 2.

Proposal 10: Agree TP to TR 38.836 as in Annex.

Annex

- (TP to TR 38.836 for SL relay enhancement)

  - 4.x	Multi-path for L2 U2N relay

  - 4.x.1	Architecture and Protocol Stack

    - 4.x.1.1	Protocol Stack

The protocol stacks for user plane and control plane of multi-path scenario 2 are described in Figure 4.x.1.1-1 and Figure 4.x.1.1-2 for DC-like architecture, Figure 4.x.1.1-3 and Figure 4.x.1.1-4 for DAPS-like architecture.

Figure 4.x.1.1-1:User plane protocol stack for scenario 2 (DC-like)

Figure 4.x.1.1-2:Control plane protocol stack for scenario 2(DC-like)

Figure.4.x.1.1-3 User plane protocol stack for scenario 2 (DAPS-like)

Figure.4.x.1.1-4 Control plane protocol stack for scenario 2 (DAPS-like)