3GPP TSG-RAN WG2 Meeting #123	                                                         R2-2308332

Toulouse, France, August 21-25, 2023

Agenda Item:	7.24.2

Source:	CMCC

Title:	Discussion on the issue of unpredictable measurement sequence for inter-frequency measurement reporting and candidate solutions

Document for:	Discussion

Introduction

In RAN2#122 meeting, we shared an issue of unpredictable measurement sequence for inter-frequency measurement reporting in R2-2305774, which we observed in the real network. Here are the agreements achieved in last RAN2 meeting.

The issue is about latency from MO configuration to Measurement Report for the most interesting frequency(ies) e.g. for the service of the UE. More specifically at the time of measurement report, the network cannot know which frequencies the UEs has already measured, so there is no way for the network to decide to wait or not for another potential measurement report (expect the network to act immediately on the first measurement report).

There is support to attempt a solution for Rel-18. Solution discussion next meeting.

Background of unpredictable measurement sequence issue identified in network

In current specification, network can configure UE with MeasConfig IE, which contains a list of frequencies for the UE to measure as well as report criteria. When UE performs the measurement on the configured frequencies, the sequence of the measured frequencies is left to UE implementation.

However, such kind of UE implementation causes problems in our real network. We observed that different UEs have different implementation. For example, some UEs may follow the sequence of frequency in MeasConfig, while some UEs may prior to measure the frequency which UE latest comped on.

Observation 1: The sequence for UE to perform inter-frequency measurement is based on UE implementation. And the sequence varies for different UEs.

Figure 1. UE A performs measurement and reporting following the sequence of measID

Figure 2. UE B performs measurement and reporting NOT following the sequence of measID

In real network, UE is configured with multiple frequencies for measurement and reporting. As the log in figure 1 and figure 2 shows the procedure of inter-RAT inter-frequency measurement reporting and UE handover from 5G to 4G for VoLTE fallback. In the configuration procedure, both UE A and UE B are configured with measObjectEUTRA of EARFCN 38950 (Band 40), EARFCN 1300 (Band 3) and EARFCN 38400 (Band 39). The UE behaviours are quite different:

- The UE A in the first log perform measurement in the frequency sequence in measObjectEUTRA. UE A measured Band 40 firstly and report the measurement results of Band 40. The network then handover the UE A to Band 40.

- While the UE B in the second log didn’t follow the frequency sequence in measObjectEUTRA. UE B measured Band 3 firstly and report the measurement results of Band 3. The network then handover the UE to Band 3.

We observed that when the first measured frequency satisfies the trigger event for measurement reporting, UE will directly report the measurement of that frequency and not wait for the measurement of other frequencies. The reason for network side to send the frequency in the sequence of Band 40, Band 3, Band 39 is that, Band 40 has best coverage and dedicated optimized for voice call. So Band 40 is the best frequency for VoLTE fallback. Therefore, the network side send Band 40 in the first of the sequence of frequency expecting the UE to measure Band 40 firstly.

Some may argue why network wouldn’t wait until UE B report more measurement for other frequencies. From network point of view, network can neither foresee when UE B will report the measurement of Band 40, nor foresee if Band 40 failed to meet the measurement event. So these is no timer at the network side to wait for more measurements. In addition, the waiting time also causes voice fallback delay or handover delay. Network has to handover the UE to target cell whenever a measurement report is received.

Observation 2: During measurement reporting procedure, when the first measured frequency satisfies the trigger event for measurement reporting, UE will directly report the measurement of that frequency and not wait for the measurement of other frequencies.

Observation 3: During VoLTE Fallback procedure, whenever UE reports the measurement results, network will immediately HO or redirect the UE to target LTE cell. And it is impossible for network side to wait for more measurement reporting.

With the analysis above, we can observe that the unpredictable sequence of measurement and reporting results to the UE HO or redirect to an unexpected frequency.

We observe this problem during voice fallback from NR to LTE, as we have a specific frequency optimized for VoLTE. Due to UE implementation, the UE may measure the inter-frequency not in the sequence as network configured in the MeasConfig IE. And we can also foresee this problem may occur in some other scenarios. For instance, when more network slices are deployed on different NR frequencies, network may prefer to HO UE to a frequency which support the specific slices.

Observation 4: The unpredictable sequence of measurement and reporting results to the UE HO or redirect to an unexpected frequency.

Based on the analysis and observations above, we kindly ask RAN2 to identify and address the issue in TEI18 item. We propose to standardize the UE measurement sequence for inter-frequency for both intra-RAT and inter-RAT, which will make the UE measurement reporting predictable

In RAN2#122 meeting, RAN2 achieved agreement on the issue identification, i.e., the issue is about latency from MO configuration to Measurement Report for the most interesting frequency(ies) e.g. for the service of the UE. More specifically at the time of measurement report, the network cannot know which frequencies the UEs has already measured, so there is no way for the network to decide to wait or not for another potential measurement report (expect the network to act immediately on the first measurement report).

Candidate solutions

Candidate solution 1: UE indicates the frequencies which have already been measured but haven’t meet the reporting criteria in the measurement report

We aware there are candidate solutions, e.g., UE indicates the frequencies which have already been measured but haven’t meet the reporting criteria in the measurement report. But our concern is that such solution cannot address the issue completely. UE may still not perform the measurement for the most important frequency (voice frequency), during VoLTE fallback procedure.

Observation 5: The solution of indicating the frequencies which have already been measured but haven’t meet the reporting criteria may not address the issue completely.

From operator point of view, we would prefer a solution that can address the issue completely.

Candidate solution 2: Network indicates the recommended sequence for intra/inter-RAT inter-frequency measurements

We would like to share an example to show the intention. In current UE implementation, if network configure UE with frequency A/B/C. UE may perform the measurement in the gap as C B A C B A C B A, or A B C A B C A B C. The intention of recommended sequence is to require UE follow the sequence configured by the network, e.g., A A A B B B C C C. That means, in the first 3 measurement gaps, UE firstly measure the first frequency. However, in case there is no SMTC for the first frequency, UE is allowed to measure the second frequency. After UE performs the measurement for frequency A in 3 measurement gaps, UE continue measure next frequency in next gap. Such solution can guarantee the most important frequencies (e.g., voice frequencies) can be measured firstly, during VoLTE fallback procedure.

RAN2 impact:

In order to implement the solution in TEI18, we are looking for ways to minimize the impact on the specification.

The correction to TS 38.331 is to capture in the field description of measIdToAddModList that the UE should measure the frequencies in the sequence of measID. The frequency ranked first in measID should be measured firstly.

RAN4 impact:

To resolve the issue completed, there would be impact on RAN4 RRM test cases and measurement requirement. RAN4 need to add a RRM test case to cover the in-sequence inter-frequency measurement reporting. In current RRM test case in the Annex 6.6.2 and Annex 6.6.3.1.1 of TS 38.133, it captured the inter-frequency and inter-RAT measurement reporting test cases. In order to support the testable of in-sequence measurement reporting, the Annex 6 of TS 38.133 need to add new cases to test whether the UE can in-sequence report the measurement. And how the UE perform in-sequence measurement can either be left to UE implementation or add new procedural text in RAN4 core specification. That part can be left for RAN4 decision.

Considering there would not be enough time for RAN4 to complete the corresponding correction during TEI18, there may be the following two approaches:

Approach 1: RAN4 only capture a RRM test case for in-sequence inter-frequency and inter-RAT measurements. While the change to core specification is postponed to Release 19.

Approach 2: Postpone all the RAN4 work, including core specification and RRM test case, to Release 19.

For either approach 1 or 2, in Release 18, we can expect the UE which support the 38.331CR to perform in sequence measurement based on UE implementation. After RAN4 finish the corresponding work in release 19, we will have a complete solution.

Proposal 1: In release 18, RAN2 introduces recommended sequence for intra/inter-RAT inter-frequency measurement.

Proposal 2: Before RAN4 finishes the corresponding standardization work, it can depend on UE implementation to apply the recommended measure sequence or priority.

Conclusions

In this contribution, we discuss on an issue we encountered in our real network. The observations and proposals are as follows:

Observation 1: The sequence for UE to perform inter-frequency measurement is based on UE implementation. And the sequence varies for different UEs.

Observation 2: During measurement reporting procedure, when the first measured frequency satisfies the trigger event for measurement reporting, UE will directly report the measurement of that frequency and not wait for the measurement of other frequencies.

Observation 3: During VoLTE Fallback procedure, whenever UE reports the measurement results, network will immediately HO or redirect the UE to target LTE cell. And it is impossible for network side to wait for more measurement reporting.

Observation 4: The unpredictable sequence of measurement and reporting results to the UE HO or redirect to an unexpected frequency.

Observation 5: The solution of indicating the frequencies which have already been measured but haven’t meet the reporting criteria may not address the issue completely.

Proposal 1: In release 18, RAN2 introduces recommended sequence for intra/inter-RAT inter-frequency measurement.

Proposal 2: Before RAN4 finishes the corresponding standardization work, it can depend on UE implementation to apply the recommended measure sequence or priority.