3GPP TSG-RAN WG2 Meeting #123	R2-2308336

Toulouse, France, 21 – 25 August 2023

Agenda item:	7.5.1

Source:	Nokia, Qualcomm (Rapporteurs)

Title:	XR SA4 Status

WID/SID:	NR_XR_enh-Core - Release 18

Document for:	Discussion and Decision

# 1	Introduction

This contribution gives a short overview of the status of the XR work in SA4.

# 2	Status

Since the last RAN2 meeting, the following has happened on SA4 side for XR:

-	TR 29.926 on Traffic Models and Quality Evaluation Methods for Media and XR Services in 5G Systems was updated to version 1.5.0 to describe some details of the Quality Metrics and Computation, and to describe a pose uplink model:

Based on these considerations, the following is proposed for the uplink modelling: - XR Viewer Pose information is provided to the application in 1kHz sampling frequency - The size of the XR Viewer Pose packets are considered 100 byte (60 payload, 40 header) - The Pose is sent with a frequency of controlled by the application, but we assume alignment with the rendering frame rate, i.e. 10 ms. In order to minimize the rendering impact, the expected latency of the pose information is at most 10ms in the uplink. This means that the content is rendered with a pose of typically 10-15ms age.

-	A CR on XR Traffic Characteristics was endorsed in S4-230989:

A summary of expected bitrates and traffic characteristics are provided: - Viewport-independent 6DOF streaming - Downlink only - HTTP Streaming - Up to 100 Mbps to address high-quality 6DOF VR services to allow 2k per eye streaming at 90 fps (see TR 26.928, clause 4.2 and 6.2.2) - Viewport-dependent 6DOF streaming - Downlink only - HTTP streaming, HTTP/TCP level information and responses are exchanged every 100-200 ms in viewport-dependent streaming. - Up to 25 - 50 Mbps to address high-quality 6DOF VR services to allow 2k per eye streaming at 90 fps (see TR 26.928, clause 4.2 and 6.2.3) - Raster-based split rendering - Primarily downlink - for H.264/AVC the bitrates are in the order of up to 50 Mbps per eye buffer, i.e. up to 100 Mbit/s. - for H.265/HEVC the bitrates are in the order of 20 - 30 Mbps per eye buffer, i.e. 40 – 60 Mbit/s - latency requirements are in the range of 15ms - uplink pose information (see TR 26.926 [47], clause 5.8) - typically every 10 -15ms constant packet size of up to 100 bytes - latency requirements are 10 – 15 ms - Generalized split rendering - details are FFS - XR conversational - details are FFS

-	TS 26.522 on 5G Real-time Media Transport Protocol Configurations version 0.1.0 was agreed, and contains a description of the PDU set RTP header extensions:

The one-byte RTP Header Extension for the marking of PDU Sets and End of Bursts is defined as follows: 0 1 2 3 0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1 +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+ ¦ 0xBE ¦ 0xDE ¦ length ¦ +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+ ¦ ID ¦ L=5 ¦E¦ EDB ¦ PSI ¦ PSSN ¦ PSN ¦ +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+ ¦ PSSize ¦ +.+.+.+.+.+.+.+.+.+.+.+.+.+.+.+.+.+.+.+.+.+.+.+.+ The two-byte RTP Header Extension for the marking of PDU Sets and End of Bursts is defined as follows: 0 1 2 3 0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1 +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+ ¦ 0x100 ¦ appbits ¦ length ¦ +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+ ¦ ID ¦ L=6 ¦E¦ EDB ¦ PSI ¦ PSSN +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+ ¦ PSN ¦ PSSize ¦ +-+-+-+-+-+-+-+-+.+.+.+.+.+.+.+.+.+.+.+.+.+.+.+.+.+.+.+.+.+.+.+.+ The semantics of the fields of the RTP Header Extension for the marking of PDU Set and End of Bursts are defined as follows: - End PDU of the PDU Set [E] (1 bit): This field is a flag that shall be set to 1 for the last PDU of the PDU Set and set to 0 for all other PDUs of the PDU Set. - End of Data Burst [EDB] (3 bits): The EDB field is 3 bits in length and indicates the end of a Data Burst. The 3 bits encode the End of Data Burst indication as per the encoding and guidelines provided in Clause 4.4.2.6.1. - PDU Set Importance [PSI] (4 bits): The PDU Set Importance field indicates the importance of this PDU Set compared to other PDU Sets within the same RTP stream. Lower values shall indicate a higher importance PDU Set with the highest importance PDU Set indicated by 0 and the lowest importance PDU Set indicated by 15. - PDU Set Sequence Number [PSSN] (10 bits): The field encodes the sequence number of the PDU Set to which the current PDU belongs acting as a 10-bit numerical identifier for the PDU Set. NOTE 2: This value wraps around at 1023, however, using the RTP packet sequence number and PSSN pair a receiver may uniquely distinguish between any PDU Sets. - PDU Sequence Number within a PDU Set [PSN] (6 bits): The sequence number of the current PDU within the PDU Set. The PSN shall be set to 0 for the first PDU in the PDU Set and incremented monotonically for every PDU in the PDU set in order of transmission from the sender. NOTE 3: A receiver may use the RTP packet sequence number together with the PSN to distinguish between PDUs within a PDU Set that contains more than 64 PDUs. - PDU Set Size [PSSize] (24 bits): The PDU Set Size indicates the total size of all PDUs of the PDU Set to which this PDU belongs. This field is optional and subject to an SDP signaling offer/answer negotiation, where the Application Server may indicate whether it will be able to provide the size of the PDU Set for that RTP stream. If not enabled, the field should not be present. If enabled, but the Application Server is not able to determine the PDU Size for a particular PDU Set, it should set the value to 0 in all PDUs of that PDU Set. The PSSize shall indicate the size of [a PDU Set including RTP/UDP/IP header encapsulation overhead of its corresponding PDUs] / [sum of RTP payload sizes of all PDUs present in a PDU Set]. The PSSize is expressed in bytes. This field may be optionally present given the signaling of the “pdu-set-size” extension attribute in the SDP offer/answer negotiation.