3GPP TSG-RAN WG2 Meeting #123	R2-2308338

Toulouse, France, 21 – 25 August 2023

Agenda item:	7.5.2

Source:	Nokia, Nokia Shanghai Bell

Title:	Jitter and End of Data Burst Signalling

WID/SID:	NR_XR_enh-Core - Release 18

Document for:	Discussion and Decision

# 1	Introduction

As discussed in the last meeting, PDU Set information is considered useful for RRM operations in uplink and especially jitter, reference time, periodicity and End of Data Burst (EoDB) indicator. In particular, the following agreements were made

UL jitter information: - 1. UE reports to RAN the range of jitter in its UL traffic, defined in the similar way as the one for N6 jitter. - 2. Reference time is defined in similar way as BAT (Burst Arrival Time) at UE side. - 2. UL assistance information (burst arrival time, UL jitter, FFS on periodicity) is reported per QoS flow. Network can configure for which QoS flow UE should report assistance information. - 3 . RRC UAI framework is updated for Rel-18 to support signalling UL assistance information agreed so far for XR (Jitter, burst arrival time, FFS on periodicity). EoDB determination for UL: - 1. Do not use PIN delay budget request for jitter reporting for XR services. - 3. Reuse existing mechanisms (e.g. (Padding) BSR with BS value equal to zero) as implicit End of Data Burst (EoDB) indicator for the RAN. - 1. On the UL, the identification of PDU sets, data bursts and PSI is left to UE implementation. This doesn’t mean UE cannot use information provided by upper layers, but RAN2 does not intend to specify how.

# 2	Signalling of Uplink Jitter

## 2.1	Periodicity, reference time, jitter

The traffic model for XR applications agreed in RAN1 consists of a periodic arrival process on top of which jitter and offset are considered. Specifically, according to TR38.838 the periodic arrival with jitter gives the arrival time for packet with index k (k=1,2,3….) as offset + k/F*1000 + J [ms], where F is the given frame generation rates (per second) and J is a random variable capturing jitter. Note that actual traffic arrival timing of traffic for each UE could be shifted by the UE specific arbitrary offset.

While the jitter in the XR traffic model was designed mainly to capture deviation of traffic arrivals in DL direction, the traffic model is general enough to also capture UL jitter. As discussed in our last contribution [R2-2305190], both the media encoder and the network contribute to the jitter of XR services. Specifically, the media encoder may require different time for the generation of two frames/slices due to the type of encoding process (e.g. intra- or inter-frame), while the network latency can vary depending on the quality of radio links and traffic load (i.e., buffering) especially in threating scenarios. It is worth mentioning that RAN1 with the help of SA4 identified the three following main elements of the traffic model for XR video traffic:

-	F: the frame rate, with 1/F equal to the average periodicity of the frame generation process.

-	J: jitter, which capture the deviation from the expected arrival time introduced by the codec and/or network.

-	Offset: shift in time accounting for actual arrival time specific for each UE.

The offset parameter permits to define a reference time (also known as central point of expected arrival window) specific for each UE and it can be used to model additional delays before the first frame of the video stream is transmitted including the PDU session establishment and RRC procedures. Figure 1 illustrates two cases with offset equal to 0 and -12ms, respectively. For the sake of simplicity, we consider an interarrival time equal to 16ms in the figure. The timeline shows the subframe numbers.

Figure 1 – Example of XR video traffic with offset=0 (case A) and offset=-12 (case B). 
The offset permits to shift in time the arrival time of the first frame, 
thus effectively modelling the reference time of the arrival process.

In the last meeting, it was agreed to define reference time in a similar way as Burst Arrival Time (BAT), which has been defined in TS 23.501 Section 5.27.2 TSC Assistance Information (TSCAI) and TSC Assistance Container (TSCAC). According to TS 23.501, BAT is defined as the latest possible time when the first packet of the data burst arrives at either the ingress of the RAN (downlink flow direction) or the egress of the UE (uplink flow direction). We observe that the reference time is implicitly modelled in the XR traffic model using the offset parameter. Therefore, reference time is essential to fully define the pattern of XR video traffic.

| Assistance Information | Description |
| --- | --- |
| Burst Arrival Time (optional) | The latest possible time when the first packet of the data burst arrives at either the ingress of the RAN (downlink flow direction) or the egress of the UE (uplink flow direction). |
| Burst Arrival Time Window (BAT Window) (optional) (NOTE 1) (NOTE 2) | Indicates the acceptable earliest and latest arrival time of the first packet of the data burst at either the ingress of the RAN (downlink flow direction) or the egress of the UE (uplink flow direction). |
| N6 Jitter Information (optional) (NOTE 3) | Jitter information associated with the Periodicity in downlink (see clause 5.378.1). |
| Periodicity Range (optional) (NOTE 4) | It indicates that the AF will adjust the periodicity and provides the acceptable range (which is formulated as lower bound and upper bound of the Periodicity) or acceptable Periodicity value(s) (which is formulated as a list of values for the Periodicity). |
| NOTE 1: Only one of the parameters (BAT Window or Capability for BAT adaptation) can be provided. NOTE 2: The parameter can only be provided together with Burst Arrival Time. NOTE 3: Only one of the parameters Burst Arrival Time or N6 Jitter Information may be provided for a given Traffic Flow. NOTE 4: The Periodicity Range can only be provided together with Periodicity when Burst Arrival Time and Burst Arrival Time Window are present. |  |

Observation 1: The XR video traffic model implicitly includes the reference time.

While the network can roughly estimate periodicity, jitter and reference time by measuring statistical properties of XR video traffic, this process requires time and the accuracy is affected by multiple factors including multiplexing of XR traffic, additional sources of delay and jitter unknown to the network like in the case of tethering. Therefore, all three information elements shall be provided by the UE in order to support real-time optimization of scheduling and RRM functions. Specifically, the UE can report assistance information in the following form:

-	Reference time: the time instant used to generate the sequence of expected traffic arrival windows. This information can be provided using an explicit or implicit indication. An explicit indication consists in a value provided using some kind of signaling scheme like RRC UAI, while an implicit indication can be the evaluated based on the transmission/reception of a certain control or data packet.

-	Nominal periodicity: the average period between two consecutive traffic arrivals. This information can be provided as explicit indication using RRC UAI. Nominal periodicity can be provided in the form of a value representing a period or a frequency.

-	Jitter: the size of the deviation of the periodicity that defines the window around the expected arrival time. Jitter can be indicated using an explicit singling scheme like RRC UAI. Examples of the jitter information include a maximum deviation, a range, a distribution, a mean and variance for the distribution.

Observation 2: The three key elements to identify the expected time window of every traffic/burst arrival for XR video traffic are a reference time, the periodicity, and the jitter.

Referring again to the examples illustrated in Figure 1, the assistance information provided by the UE related to reference time, periodicity and jitter can be summarized as follows:

Table 1 – Examples of assistance information provided by the UE

|  | Reference time | Nominal periodicity | Jitter [min,max] |
| --- | --- | --- | --- |
| Case A | 12 | 16 | [0,8] |
| Case B | 0 | 16 | [0,8] |

We further observe that the reference time discussed in this paper differs from the agreement made in the past meeting on using the PDCP discard timer as reference time for delay reporting in BSR [R2-2306542]. In particular, the reference time discussed in this paper identifies an anchor point in time for drawing the traffic pattern. This information together with periodicity and jitter does not change frequently and semi-static reconfiguration using RRC signalling is enough to follow long-term variations. In contrast, the remaining delay budget information (or equivalently buffering time, and remaining time) reported using BSR (or new MAC CE) as discussed in [R2-2307789] varies more dynamically due to e.g. UE buffering, network load, and scheduling restrictions. Additionally, the remaining delay budget is used for instantaneous rather than semi-static radio resource management (RRM) actions.

Observation 3: The reference time for XR traffic pattern is different from the reference time agreed for delay reporting in BSR.

Proposal 1: Consider both explicit and implicit indication for reference time.

In the last meeting, it has been discussed the possibility of reporting of reference time, nominal periodicity, and jitter either per DRB or per QoS flows. We observe that when multiple QoS flows are multiplexed into a single DRB, the resulting traffic pattern of the DRB is the superposition of all traffic patterns of the multiplexed QoS flows. In this case, the reporting per DRB does not provide additional information to the network with respect to the information that can be measured by observing the traffic of the DRB. Additionally, when DRB carries traffic of multiple QoS flows with different QoS requirements such as delay-critical and non-delay-critical QoS flows, the assistance information cannot distinguish among the different type of QoS flows. In contrast, when assistance information is provided per-QoS flow, even if the QoS flow is multiplexed with other type of traffic, the reporting can be enabled only for delay-critical services.

Observation 4: When multiple QoS flows are multiplexed into a single DRB, the resulting traffic pattern of the DRB is the superposition of all traffic patterns of the multiplexed QoS flows. In this case, the reporting per DRB does not provide additional information to the network.

Proposal 2: Report assistance information per QoS flow.

## 2.2	RRC UAI

The UE assistance information procedure is for the UE to inform the network of [38.331]:

-	its delay budget report carrying desired increment/decrement in the connected mode DRX cycle length, or;

-	its overheating assistance information, or;

-	its IDC assistance information, or;

-	its preference on DRX parameters for power saving, or;

-	its preference on the maximum aggregated bandwidth for power saving, or;

-	its preference on the maximum number of secondary component carriers for power saving, or;

-	its preference on the maximum number of MIMO layers for power saving, or;

-	its preference on the minimum scheduling offset for cross-slot scheduling for power saving, or;

-	its preference on the RRC state, or...

As discussed earlier when used for XR, the UE assistance information procedure should also allow the UE to inform the network of uplink jitter and reference time. There is no agreement on the provision of periodicity yet but with the frame rate of the display being possibly adjusted dynamically to save power at the UE side, we believe periodicity should also be signalled.

Proposal 3: the UE assistance information procedure should also allow the UE to inform the network of uplink periodicity.

Because the UE may not be able to determine all pieces of information, and because only a subset of the pieces of information may change, uplink jitter, burst arrival time and periodicity should all be optional.

Proposal 4: uplink jitter, burst arrival time and periodicity are all optional in UEAssistanceInformation.

# 3	End of Data Burst (EoDB)

In the last meeting, it was agreed to adopt (Padding) BSR with BS value equal to zero as implicit End of Data Burst (EoDB) indicator for the RAN. However, using only (Padding) BSR as EoDB indicator comes with the following drawbacks:

-	Padding BSR is transmitted opportunistically only when the grant is larger than the data to be transmitted. In this case, the UE can add Padding BSR to get an additional grant or to indicate the EoDB.

Observation 5: Padding BSR is transmitted only when the grant is larger than the data to be transmitted.

To solve this issue new conditions can be considered to trigger the transmission of BSR as EoDB. For example, when the buffer status of a logical channel (LCH) or logical channel group (LCG), which carries the XR traffic, becomes empty a regular BSR with BS equal to zero is triggered. This additional trigger can be explicitly configured by the gNB via RRC signalling, for instance for logical channels carrying QoS flows enabled with PDU Set based QoS handling by 5GC only.

Proposal 5: Define a new trigger for BSR as EoDB indication based on the status of the LCH or LCG buffers.

In many cases just triggering regular BSR as EoDB indicator soon after the buffer becomes empty may not be enough and provide wrong indication to the network. For example, when a single Data Burst contains multiple PDU Sets and consecutive PDU Sets enter the UE buffer with a certain delay as illustrated in in Figure 2, this solution may not work properly as EoDB indication. Indeed, the accuracy of BSR as EoDB indication depends on several parameters, procedures, and triggers including the time when the last PDU of the Data Burst is detected by upper layers, the preparation time of the BSR report, the time when the BSR is set by the UE, and the time the PDU of each PDU Sets are pushed into the UE layer 2 buffer (i.e., either LCH or LCG buffer). For example, a delay between the first and second PDU Sets within Data Burst 1 may lead the UE to erroneously trigger a BSR for EoDB when the last PDU of the PDU Set 1 is pulled from the buffer.

Figure 2 – Multiple PDU Sets are included in one Data Burst.

Observation 6: Triggering regular BSR as EoDB indication as soon as the buffer becomes empty may result in wrong EoDB indication to the network.

Note that when the network receives the EoDB indication, it may indicate to the UE to enter in sleep mode using WUS or skip the monitoring of the PDCCH by switching SSSG. The only way for the UE to rectify this early EoDB indication is to send a Scheduling Request (SR) to indicate to the NW that additional data is ready to be transmitted. However, the data transmission may be delayed beyond the PDB depending on when this early EoDB indication occurs and system settings including the frame structure, and numerology. This problem can be solved by considering an additional EoDB timer, which is handled according to the following logic:

-	when the LCH/LCG buffer becomes empty, the EoDB timer is triggered;

-	any PDU entering the buffer causes the reset and stop of the EoDB timer.

When the EoDB timer expires, the UE triggers the transmission of a BSR as EoDB indication. This way the EoDB is indicated only after the EoDB timer expires and unpredictable yet small delays between PDUs within the same data burst will not trigger premature EoDB indication.

Observation 7: Delay between consecutive PDUs within the same Data Burst may cause the early triggering of BSR as EoDB indication when the buffer becomes empty. This eventually increases the scheduling delay of the remaining data of the burst possibly beyond the PDB or PSDB.

Proposal 6: Define a new timer to trigger a BSR as EoDB indication to avoid early triggering of EoDB indication.

While the EoDB Timer solves the issue of early EoDB indication, using BSR for EoDB indication can still exhibit other problems due to the rules and conditions indicated in the TS 38.321 for triggering the BSR report. For example, when the UE can fill the whole grant with the buffered data and MAC CEs with higher priority than BSR, the UE shall remove the transmission of the BSR since there is no additional space for the (Padding) BSR, because TS 38.321 states:

“All triggered BSRs may be cancelled when the UL grant(s) can accommodate all pending data available for transmission but is not sufficient to additionally accommodate the BSR MAC CE plus its subheader.”

When the BSR as EoDB indication is cancelled, the network is unaware that the UE has no more data to transmit until the next data burst. Therefore, the network can schedule another grant for the UE (for example based on the last BSR it has received). The grant will not allow the UE to enter sleep mode since a DCI command will be sent by the network and the DCI will trigger the DRX inactivity timer, which keeps the UE active even if the UE has no data in the buffer to transmit. While this problem does not cause the loss (or delay) of any data, the UE loses an opportunity to save some power by entering in sleep mode and the network wastes radio resources since the granted resources are used by the UE only to send a BSR for EoDB – something that was already available as information at the UE, but it could not be transmitted due to the cancelling of BSR.

Observation 8: The cancellation of BSR as EoDB indication when there is no space in the grant to accommodate BSR and its subheader may cause higher UE power consumption and waste of radio resources.

This problem can be solved if the EoDB indication is provided by higher layers (e.g., in PDCP). Specifically, the new design properties of the EoDB indication are:

-	new BSR trigger based on application EoDB indication in PDCP;

-	new dedicated LCID as indication of EoDB;

-	new priority arrangement for MAC CE construction.

The new dedicated LCID (point 1) together with a new BSR trigger based on application EoDB (point 2) eliminate the need of using BSR with BS=0 and enable the possibility of triggering the EoDB indication even when some buffer is not empty, respectively. For example, in case of eMBB and XR traffic mapped into two different LCGs, the EoDB of the XR traffic can be signaled to the network using a BSR with new LCID together with a BSR for the size of buffered eMBB data. This provides more flexibility to decide and control data transmission and UE sleep operations. Finally, the new priority arrangement for MAC CE construction (point 3) enables the transmission of BSR as EoDB indication when there is not enough space for all MAC CEs. Specifically, to make sure the new LCID is used as EoDB indication, the new trigger must be with higher priority than Regular or periodic BSR and by extension padding BSR.

Proposal 7: Define a new BSR trigger based on application EoDB indication in PDCP and a new dedicated LCID as indication of EoDB.

Proposal 8: Update the priority of logical channels indicated in section 5.4.3.1.3 of TS 38.321.

# 3	Conclusion

This document has made the following observations:

Observation 1: The XR video traffic model implicitly includes the reference time.

Observation 2: The three key elements to identify the expected time window of every traffic/burst arrival for XR video traffic are a reference time, the periodicity, and the jitter.

Observation 3: The reference time for XR traffic pattern is different from the reference time agreed for delay reporting in BSR.

Observation 4: When multiple QoS flows are multiplexed into a single DRB, the resulting traffic pattern of the DRB is the superposition of all traffic patterns of the multiplexed QoS flows. In this case, the reporting per DRB does not provide additional information to the network.

Observation 5: Padding BSR is transmitted only when the grant is larger than the data to be transmitted.

Observation 6: Triggering regular BSR as EoDB indication as soon as the buffer becomes empty may result in wrong EoDB indication to the network.

Observation 7: Delay between consecutive PDUs within the same Data Burst may cause the early triggering of BSR as EoDB indication when the buffer becomes empty. This eventually increases the scheduling delay of the remaining data of the burst possibly beyond the PDB or PSDB.

Observation 8: The cancellation of BSR as EoDB indication when there is no space in the grant to accommodate BSR and its subheader may cause higher UE power consumption and waste of radio resources.

And proposed the following:

Proposal 1: Consider both explicit and implicit indication for reference time.

Proposal 2: Report assistance information per QoS flow.

Proposal 3: the UE assistance information procedure should also allow the UE to inform the network of uplink periodicity.

Proposal 4: uplink jitter, burst arrival time and periodicity are all optional in UEAssistanceInformation.

Proposal 5: Define a new trigger for BSR as EoDB indication based on the status of the LCH or LCG buffers.

Proposal 6: Define a new timer to trigger a BSR as EoDB indication to avoid early triggering of EoDB indication.

Proposal 7: Define a new BSR trigger based on application EoDB indication in PDCP and a new dedicated LCID as indication of EoDB.

Proposal 8: Update the priority of logical channels indicated in section 5.4.3.1.3 of TS 38.321.