3GPP TSG RAN WG2#123	R2-2308341

Toulouse, France, 21st - 25th August, 2023

Agenda item:	7.19.3

Source:	Nokia, Nokia Shanghai Bell

Title:	On enhanced RedCap capabilities

WID/SID:	NR_redcap_enh-Core - Release 18

Document for:	Discussion and Decision

# 1	Introduction

The following is captured in TS 38.306 running CR:

### 4.2.x eRedCap Parameters #### 4.2.x.1 Definition of eRedCap UE eRedCap UE is the UE with reduced capability: - TBD further details, if any #### 4.2.x.2 General parameters Definitions for parameters; Per; M; FDD-TDD DIFF. supportOfEnhancedRedCap-r18 Indicates that the UE is an eRedCap UE.TBD further details, if any; TBD; TBD; TBD.

In this TDoc further details on enhanced RedCap capabilities are discussed .

# 2	Discussion

The following is captured in TS 38.306 for REL-17 RedCap:

#### 4.2.21.2 General parameters Definitions for parameters; Per; M; FDD-TDD DIFF. ncd-SSB-ForRedCapInitialBWP-SDT-r17 Indicates that the UE supports using RedCap-specific initial DL BWP associated with NCD-SSB for SDT. If absent, the UE only supports SDT in an initial DL BWP that includes the CD-SSB. UE supporting this feature shall indicate support of supportOfRedCap-r17 and ra-SDT-r17 and/or cg-SDT-r17.; UE; No; No. supportOf16DRB-RedCap-r17 Indicates whether the RedCap UE supports 16 DRBs. This capability is only applicable for RedCap UEs.; UE; No; No. supportOfRedCap-r17 Indicates that the UE is a RedCap UE with comprised of at least the following functional components: - Maximum FR1 RedCap UE bandwidth is 20 MHz; - Maximum FR2 RedCap UE bandwidth is 100 MHz; - Support of RedCap early indication based on Msg1, MsgA (if UE indicated support of twoStepRACH-r16) and Msg3 for random access; - Separate initial UL BWP for RedCap UEs; - It includes the configuration(s) needed for RedCap UE to perform random access - Enabling/disabling of frequency hopping for common PUCCH resources - Separate initial DL BWP for RedCap UEs; - It includes CSS/CORESET for random access - For separate initial DL BWP used for paging, CD-SSB is included - For separate initial DL BWP only used for RACH, SSB may or may not be included - For separate initial DL BWP used in connected mode as BWP#0 configuration option 1, CD-SSB is included - 1 UE-specific RRC configured DL BWP per carrier; - 1 UE-specific RRC configured UL BWP per carrier; - UE-specific RRC-configured DL BWP with CD-SSB or NCD-SSB; - NCD-SSB based measurements in RRC-configured DL BWP. A RedCap UE shall set the field to supported.; UE; CY; No.

Based the REL-17 specification the following is proposed for REL-18 enhanced RedCap:

Proposal 1: eRedCap UE shall support functionalities of supportOfRedCap-r17 except:

- Support of RedCap early indication based on Msg1, MsgA (if UE indicated support of twoStepRACH-r16) and Msg3 for random access;

- Maximum FR1 RedCap UE bandwidth is 20 MHz;

- Maximum FR2 RedCap UE bandwidth is 100 MHz;

Proposal 2:  eRedCap UE shall support eRedCap early indication based on Msg1, MsgA (if UE indicated support of twoStepRACH-r16) and Msg3 for random access

Proposal 3: eRedCap UE shall support “20 MHz + PR1” or “BW3/PR3 + PR1”

Proposal 4: A eRedCap UE shall set supportOfEnhancedRedCap-r18 field to supported.

The following is captured in TS 38.306 for REL-17 RedCap:

#### 4.2.21.1 Definition of RedCap UE RedCap UE is the UE with reduced capability: - The maximum bandwidth is 20 MHz for FR1, and is 100 MHz for FR2. UE features and corresponding capabilities related to UE bandwidths wider than 20 MHz in FR1 or wider than 100 MHz in FR2 are not supported by RedCap UEs; - The maximum mandatory supported DRB number is 8; - The mandatory supported PDCP SN length is 12 bits while 18 bits being optional; - The mandatory supported RLC AM SN length is 12 bits while 18 bits being optional; - For FR1, 1 DL MIMO layer if 1 Rx branch is supported, and 2 DL MIMO layers if 2 Rx branches are supported; for FR2, either 1 or 2 DL MIMO layers can be supported, while 2 Rx branches are always supported. For FR1 and FR2, UE features and corresponding capabilities related to more than 2 UE Rx branches or more than 2 DL MIMO layers, as well as UE features and capabilities related to more than 1 UE Tx branch or more than 1 UL MIMO layer are not supported by RedCap UEs; - CA, MR-DC, DAPS, CPAC and IAB (i.e., the RedCap UE is not expected to act as IAB node) related UE features and corresponding capabilities are not supported by RedCap UEs. All other feature groups or components of the feature groups as captured in TR 38.822 [24] as well as capabilities specified in this specification remain applicable for RedCap UEs same as non-RedCap UEs, unless indicated otherwise.

Proposal 5: The text in TS 38.306 clause 4.2.21.1 is applicable to eRedCap UE except “The maximum bandwidth is 20 MHz for FR1, and is 100 MHz for FR2. UE features and corresponding capabilities related to UE bandwidths wider than 20 MHz in FR1 or wider than 100 MHz in FR2 are not supported by RedCap UEs”

# 3	Conclusion

Based on the discussion following is proposed:

Proposal 1: eRedCap UE shall support functionalities of supportOfRedCap-r17 except:

- Support of RedCap early indication based on Msg1, MsgA (if UE indicated support of twoStepRACH-r16) and Msg3 for random access;

- Maximum FR1 RedCap UE bandwidth is 20 MHz;

- Maximum FR2 RedCap UE bandwidth is 100 MHz;

Proposal 2:  eRedCap UE shall support eRedCap early indication based on Msg1, MsgA (if UE indicated support of twoStepRACH-r16) and Msg3 for random access

Proposal 3: eRedCap UE shall support “20 MHz + PR1” or “BW3/PR3 + PR1”

Proposal 4: A eRedCap UE shall set supportOfEnhancedRedCap-r18 field to supported.

Proposal 5: The text in TS 38.306 clause 4.2.21.1 is applicable to eRedCap UE except “The maximum bandwidth is 20 MHz for FR1, and is 100 MHz for FR2. UE features and corresponding capabilities related to UE bandwidths wider than 20 MHz in FR1 or wider than 100 MHz in FR2 are not supported by RedCap UEs”