3GPP TSG-RAN WG2 #123 	R2-2308348

Toulouse, France, 21st – 25th August, 2023

Agenda item:	7.18.3

Source:	Huawei, HiSilicon

Title:	User plane aspects of MT-SDT

Document for:	Discussion

# Introduction

In RAN2#120 and RAN2#121 meeting, the following agreements were made.

RAN2#120 Agreements

- For RAN paging, MT-SDT indication (at least one bit) is explicitly included per UE via a paging message.  FFS if more information for MT-SDT are needed FFS what the indication will be called.  FFS signalling details

- Rel-18 MT-SDT after the MT-SDT paging trigger is detected, RA-SDT and CG SDT solutions/procedures specified in Rel-17 is re-used as a baseline.  The detailed triggers will be discussed on case by case.  FFS on resources used for access

- UE can use non-SDT random access resources for accessing the network for an MT-SDT transfer.  The UE can also use the configured grant resources and/or MO-RA resources.

- The network should be able to differentiate why the UL access was triggered, i.e. implicit or explicit indication by the UE.

- MT-SDT is data that belongs to bearers that are configured for SDT.    FFS whether the configuration is MO-SDT or MT-SDT specific.  The network can only trigger MT-SDT if the data belongs to those bearers.

- It is possible for the network to configure only MT-SDT without MO-SDT RA resources and/or CG-SDT.  Subsequent UL/DL data belonging to SDT bearers while in INACTIVE is allowed like MO-SDT procedure.  FFS stage 3 details

- New Resume cause in RRC resume will be introduced, one code point MT-SDT indication

RAN2#121 Agreements

1.	Include a one-bit indication in paging to trigger MT-SDT.   We will ensure that the CCCH message can be transmitted over CG.

2.	Indication is per UE.  FFS on signalling.

3.	In case condition for paging triggered MT-SDT is not fulfilled the UE initiates RRC Resume procedure. Resume cause FFS

4.	Upon receiving MT-SDT trigger, the UE shall initiate SDT procedure if the following checks are satisfied (all these same as Rel-17)

-     FFS 3a: Check for DVT (if UL data becomes available in UL)

-     3b: Check for SDT RSRP threshold

-     3c: Check for TA validation before selecting CG (if applicable)

-     3d: Check for SSB level RSRP threshold for CG resource (if applicable)

5.  When UE resumes for MT-SDT, UE resumes all RBs configured for SDT

6.	RBs configured for SDT are common for MO-SDT and MT-SDT

7.	If there is valid CG-SDT resources, the UE should use CG-SDT to transmit the response.   FFS on whether we need to optimize for case when CG periodicity is too long

8. To confirm that when SDT is initiated due to MT-SDT, UE can exchange subsequent DL/UL SDT data on the resumed RBs. This clarifies the RB behaviour of related RAN2#120 agreement.

1.	Specify an RRC procedure for RRCResume for MT-SDT initiation without checking for availability of UL data (i.e. if MT-SDT is initiated first the resume cause will be set to MT-SDT)

2.	UE is allowed to initiate either MO-SDT based resume or non-SDT based resume at any point (before initation RRCResumeRequest for MT-SDT) using separate procedures

3.	If MT-SDT procedure is initiated, for RACH during subsequent data transfer (i.e. RACH triggered due to SR), UE uses only the non-SDT RACH resources (i.e. like legacy)

In RAN2#122 meeting, the following agreements were made.

RAN2#122 Agreements

1. 	Allow support of only MT-SDT in a cell.  A separate SIB configuration will be introduced.  FFS what is put in there.

2. 	For paging indication signalling, a new list of paging records for MT-SDT indication is optionally included in paging message using non critical extension. Each record in this list optionally includes 1 bit MT-SDT indication. UE identity and access type are not included in paging record of this list.

3. 	gNB may include MT-SDT indication in paging message only if UE’s I-RNTI is included in the paging message (i.e. MT-SDT is only used by RAN initiated paging).

4. 	UE selects '0' as the Access Category when the resumption of the RRC connection is triggered by response to the MT-SDT triggering in a PAGING message

5. 	MT-SDT is only applicable to the legacy MT-Access use case (i.e. it is not applicable to access identities 1, 2 and 11-15).

6. 	SRB2 can be used for MT-SDT (i.e. similar to MO-SDT)

7. 	No additional enhancement is needed specifically for RedCap UE to monitor paging for MT-SDT

8. 	When RRC resume is triggered due to MT-SDT and in the case the condition for paging triggered MT-SDT is not fulfilled, the UE initiates RRC Resume procedure with Resume cause “mt-Access”.

9. 	A separate sdt-RSRP threshold for MT-SDT can be configured, at least in the case where MO-SDT is not configured in the cell.

1. 	RRC explicitly indicates to MAC whether resume is trigged due to MT-SDT

2. 	LCH restrictions are checked for DRBs as in MO-SDT (if UL data is available during SDT procedure).  Ensure CCCH can be transmitted in CG-SDT when MT-SDT is triggered in stage 3 discussion.

3. 	Assumption is that if the UE has UL data the UE can still check and trigger MO-SDT (it is up to UE implementation)

4. 	RA-SDT resources are not used for MT-SDT initiation RACH

5. 	In case CG-SDT resources cannot be used or are not available for MT-SDT, UE uses non-SDT RACH for RA-based MT-SDT.  FFS whether new triggers are defined

6. 	There is no need to define new Rel-18 CG configurations specific to MT-SDT.

7. 	When the UE is configured with both MO and MT SDT the radio bearer configuration is common for both.

8. 	For both MO and MT-SDT, if the next CG-SDT resource is too far, then RACH resource can be selected first.   This is checked at the point of initial resource selection (e.g. CG SDT selection).   FFS what is too far and how this is configured.   Assumption is that we will continue this discussion in SDT session.  CONFIRM with main session [CB]

9. 	SPS will not be supported for Rel-18 SDT

In this paper, we further discuss the issues with respect to user plane for MT-SDT.

# Discussion

## 2.1 UL data volume checking

For UL data volume checking, RAN2#121 agreed the following for RRC procedure of MT-SDT:

1. Specify an RRC procedure for RRCResume for MT-SDT initiation without checking for availability of UL data (i.e. if MT-SDT is initiated first the resume cause will be set to MT-SDT) 2. UE is allowed to initiate either MO-SDT based resume or non-SDT based resume at any point (before initation RRCResumeRequest for MT-SDT) using separate procedures

The above agreement means that UE does not need to check the UL data volume upon reception of paging for MT-SDT, and UE can initiate MO-SDT procedure as long as RRC resume procedure for MT-SDT is not performed. Furthermore, during RAN2#122 meeting, RAN2 made the agreement:

- RA-SDT resources are not used for MT-SDT initiation RACH

Combining all these agreements together, it becomes evident there is no reason to check data volume threshold or availability at the MAC layer as well.

Proposal 1: The condition for data volume threshold is not checked by MAC for MT-SDT.

## 2.2 CG-SDT Resources

RAN2#121 agreed that “If there is valid CG-SDT resources, the UE should use CG-SDT to transmit the response. FFS on whether we need to optimize for case when CG periodicity is too long”, which means that UE will consider CG-SDT resource in the first place when performing resource selection for MT-SDT. According to the FFS, it might happen that a delay between the MT-SDT data arrival and the available CG resource is not acceptable, e.g. the QoS requirements of the service may be violated or the network may assume the UE is not reachable and Paging failure may be assumed. To avoid such issues, RAN2#122 also agreed that “For both MO and MT-SDT, if the next CG-SDT resource is too far, then RACH resource can be selected first.   This is checked at the point of initial resource selection (e.g. CG SDT selection).   FFS what is too far and how this is configured.   Assumption is that we will continue this discussion in SDT session.  CONFIRM with main session [CB]”

To define what is too far, NW can configure a “maximum waiting time” for UE. If the next CG-SDT occasion occurrence is not within the “maximum waiting time”, then UE should use RA-based resource for the MO or MT-SDT procedure. This is how the mechanism is currently implemented in the running RRC and MAC CRs. However, one issue with the current implementation is that it assumes that maximum waiting time is configured per CG-SDT configuration. Considering different kinds of traffic may have different QoS requirements, the “maximum waiting time” should be configured per LCH.

Proposal 2: The maximum waiting time to next CG-SDT occasion should be configured per LCH.

## 2.3 RSRP threshold for MT-SDT

There was a question raised in [1] “whether it is possible to configure two different RSRP thresholds for MO-SDT and MT-SDT”. Since regardless of whether SDT procedure is triggered by the UE or by the network, the data is usually exchanged in both UL and DL directions, we see no reason to differentiate between the RSRP thresholds used for checking MO-SDT and MT-SDT triggering conditions.

Proposal 3: A separate RSRP threshold for MT-SDT is not needed.

# Conclusion

Based on the discussion in this paper, we recommend RAN2 to discuss and adopt the following proposals:

Conditions for initiating MT-SDT:

Proposal 1: The condition for data volume threshold is not checked by MAC for MT-SDT.

CG-SDT Resources:

Proposal 2: The maximum waiting time to next CG-SDT occasion should be configured per LCH.

RSRP threshold for MT-SDT:

Proposal 3: A separate RSRP threshold for MT-SDT is not needed.

# References

- R2-2306958 [Post122][309][MT-SDT] 38.321 Running CR (Huawei), Huawei, HiSilicon (Rapporteur)