3GPP TSG-RAN WG2 #123 	R2-2308349

Toulouse, France, 21st – 25th August, 2023

Agenda item:	7.18.2

Source:	Huawei, HiSilicon

Title:	Control plane aspects of MT-SDT

Document for:	Discussion

# Introduction

In RAN2#120 and RAN2#121 meeting, the following agreements were made.

RAN2#120 Agreements

- For RAN paging, MT-SDT indication (at least one bit) is explicitly included per UE via a paging message.  FFS if more information for MT-SDT are needed FFS what the indication will be called.  FFS signalling details

- Rel-18 MT-SDT after the MT-SDT paging trigger is detected, RA-SDT and CG SDT solutions/procedures specified in Rel-17 is re-used as a baseline.  The detailed triggers will be discussed on case by case.  FFS on resources used for access

- UE can use non-SDT random access resources for accessing the network for an MT-SDT transfer.  The UE can also use the configured grant resources and/or MO-RA resources.

- The network should be able to differentiate why the UL access was triggered, i.e. implicit or explicit indication by the UE.

- MT-SDT is data that belongs to bearers that are configured for SDT.    FFS whether the configuration is MO-SDT or MT-SDT specific.  The network can only trigger MT-SDT if the data belongs to those bearers.

- It is possible for the network to configure only MT-SDT without MO-SDT RA resources and/or CG-SDT.  Subsequent UL/DL data belonging to SDT bearers while in INACTIVE is allowed like MO-SDT procedure.  FFS stage 3 details

- New Resume cause in RRC resume will be introduced, one code point MT-SDT indication

RAN2#121 Agreements

1.	Include a one-bit indication in paging to trigger MT-SDT.   We will ensure that the CCCH message can be transmitted over CG.

2.	Indication is per UE.  FFS on signalling.

3.	In case condition for paging triggered MT-SDT is not fulfilled the UE initiates RRC Resume procedure. Resume cause FFS

4.	Upon receiving MT-SDT trigger, the UE shall initiate SDT procedure if the following checks are satisfied (all these same as Rel-17)

-     FFS 3a: Check for DVT (if UL data becomes available in UL)

-     3b: Check for SDT RSRP threshold

-     3c: Check for TA validation before selecting CG (if applicable)

-     3d: Check for SSB level RSRP threshold for CG resource (if applicable)

5.  When UE resumes for MT-SDT, UE resumes all RBs configured for SDT

6.	RBs configured for SDT are common for MO-SDT and MT-SDT

7.	If there is valid CG-SDT resources, the UE should use CG-SDT to transmit the response.   FFS on whether we need to optimize for case when CG periodicity is too long

8. To confirm that when SDT is initiated due to MT-SDT, UE can exchange subsequent DL/UL SDT data on the resumed RBs. This clarifies the RB behaviour of related RAN2#120 agreement.

1.	Specify an RRC procedure for RRCResume for MT-SDT initiation without checking for availability of UL data (i.e. if MT-SDT is initiated first the resume cause will be set to MT-SDT)

2.	UE is allowed to initiate either MO-SDT based resume or non-SDT based resume at any point (before initation RRCResumeRequest for MT-SDT) using separate procedures

3.	If MT-SDT procedure is initiated, for RACH during subsequent data transfer (i.e. RACH triggered due to SR), UE uses only the non-SDT RACH resources (i.e. like legacy)

In RAN2#122 meeting, the following agreements were made.

RAN2#122 Agreements

1. 	Allow support of only MT-SDT in a cell.  A separate SIB configuration will be introduced.  FFS what is put in there.

2. 	For paging indication signalling, a new list of paging records for MT-SDT indication is optionally included in paging message using non critical extension. Each record in this list optionally includes 1 bit MT-SDT indication. UE identity and access type are not included in paging record of this list.

3. 	gNB may include MT-SDT indication in paging message only if UE’s I-RNTI is included in the paging message (i.e. MT-SDT is only used by RAN initiated paging).

4. 	UE selects '0' as the Access Category when the resumption of the RRC connection is triggered by response to the MT-SDT triggering in a PAGING message

5. 	MT-SDT is only applicable to the legacy MT-Access use case (i.e. it is not applicable to access identities 1, 2 and 11-15).

6. 	SRB2 can be used for MT-SDT (i.e. similar to MO-SDT)

7. 	No additional enhancement is needed specifically for RedCap UE to monitor paging for MT-SDT

8. 	When RRC resume is triggered due to MT-SDT and in the case the condition for paging triggered MT-SDT is not fulfilled, the UE initiates RRC Resume procedure with Resume cause “mt-Access”.

9. 	A separate sdt-RSRP threshold for MT-SDT can be configured, at least in the case where MO-SDT is not configured in the cell.

1. 	RRC explicitly indicates to MAC whether resume is trigged due to MT-SDT

2. 	LCH restrictions are checked for DRBs as in MO-SDT (if UL data is available during SDT procedure).  Ensure CCCH can be transmitted in CG-SDT when MT-SDT is triggered in stage 3 discussion.

3. 	Assumption is that if the UE has UL data the UE can still check and trigger MO-SDT (it is up to UE implementation)

4. 	RA-SDT resources are not used for MT-SDT initiation RACH

5. 	In case CG-SDT resources cannot be used or are not available for MT-SDT, UE uses non-SDT RACH for RA-based MT-SDT.  FFS whether new triggers are defined

6. 	There is no need to define new Rel-18 CG configurations specific to MT-SDT.

7. 	When the UE is configured with both MO and MT SDT the radio bearer configuration is common for both.

8. 	For both MO and MT-SDT, if the next CG-SDT resource is too far, then RACH resource can be selected first.   This is checked at the point of initial resource selection (e.g. CG SDT selection).   FFS what is too far and how this is configured.   Assumption is that we will continue this discussion in SDT session.  CONFIRM with main session [CB]

9. 	SPS will not be supported for Rel-18 SDT

In this paper, we further discuss the issues with respect to control plane for MT-SDT.

# Discussion

## 2.1 RSRP threshold for MT-SDT

There was a question raised in [2] “whether it is possible to configure two different RSRP thresholds for MO-SDT and MT-SDT” and a similar issue has been raised during the discussion on the running RRC CR. Since regardless of whether SDT procedure is triggered by the UE or by the network, the data is usually exchanged in both UL and DL directions, we see no reason to differentiate between the RSRP thresholds used for checking MO-SDT and MT-SDT triggering conditions. Hence, mt-SDT-ConfigCommon will only be required in case the cell supports MT-SDT only, otherwise the existing sdt-ConfigCommon parameter can be reused.

Observation 1: MT-SDT specific common configuration is only required in a cell that supports MT-SDT only (i.e. does not support MO-SDT). Otherwise, the existing sdt-ConfigCommon parameter can be reused.

Based on the considerations above, we propose the following:

Proposal 1: MT-SDT specific common SDT configuration (comprising T319a and SDT RSRP threshold) is only configured in case legacy sdt-ConfigCommon is not present (i.e. only if the cell does not support MO-SDT).

## 2.2 Non-SDT data arrival during MT-SDT

While the MT-SDT procedure is ongoing, if DL non-SDT data/signalling arrives, the network can straightaway move the UE to RRC_CONNECTED state by sending the RRCResume message if the UE responds to paging and resumes in the last serving gNB.

However, both with and without UE context relocation procedures will be supported for MT-SDT, and if the DL non-SDT data arrives in the last serving gNB while it is anchoring the MT-SDT procedure, it will not be able to move the UE to RRC_CONNECTED state straight away by sending RRCResume message. Instead, the gNB will have to send RRCRelease message to the UE to terminate the ongoing MT-SDT procedure and then again perform paging in the RNA. When the UE responds to Paging, the UE’s context will be relocated and the UE will be moved to RRC_CONNECTED state and only then the gNB will be able to send the DL non-SDT data to the UE. Such procedure will clearly lead to unnecessary signaling overhead and latency in sending, potentially time critical, DL data.

To avoid this, it would be worth allowing the last serving gNB to include an indication in the RRCRelease message informing the UE it should initiate a new resume procedure right-away.

When the UE initiates a new legacy resume procedure, the UE context can be relocated from the last serving gNB to the receiving gNB and the UE can be transitioned to the RRC_CONNECTED state by reusing the legacy procedure.

In addition, RAN3 has also discussed this issue in the RAN3#119bis meeting and as captured in [1], a large number of companies think that such an indication will be helpful and RAN3 agreed to leave the decision to RAN2 as below.

RAN3#119bis agreement: For the issue on DL non-SDT data arrives during the ongoing MT-SDT procedure, RAN3 waits for RAN2 on whether any signaling enhancements are needed.

Hence, we propose the following.

Proposal 2: When DL non-SDT data arrives during the ongoing MT-SDT procedure, the last serving gNB shall end the ongoing SDT procedure by sending RRCRelease message to the UE with an indication to initiate a new Resume procedure, to avoid having to page the UE.

## 2.3 Capability for MT-SDT

MT-SDT will be an optional feature for the UE to support and the network should be aware of such capability to be able to configure the UE properly and know whether MT-SDT Paging can be initiated towards the UE.

In RAN2#122 meeting, RAN2 agreed the following for MT-SDT:

RAN2#122 Agreement: 4. RA-SDT resources are not used for MT-SDT initiation RACH

According to the above agreement, UE can initiate MT-SDT using legacy RACH resource with new resume cause “mt-SDT”, instead of using separated RA-SDT resource, which is only used for MO-SDT. Therefore, a new capability is needed to let the network know that the UE supports MT-SDT where legacy RACH resource is used as a reply.

Another possibility to reply to MT-SDT Paging is via CG-SDT resource. Considering CG-SDT resource configuration is in general the same as the one used in Rel-17 for MO-SDT, we believe cg-SDT-r17 capability can be reused. Similar approach can also be applied to indicate a support of SRB for MT-SDT, because even in Rel-17 the data can be exchanged in both UL and DL directions over SRB.

Therefore, a new capability “mt-SDT-r18” can be introduced for R18 MT-SDT, which is specified as follows::

- If UE supports “mt-SDT-r18” only, i.e. without any other SDT-related capabilities, then NW can page UE for SDT DRB data, and UE can use legacy RACH resource to initiate the MT-SDT related resume procedure;

- If UE supports “mt-SDT-r18” and “cg-SDT-r17”, then NW can page UE for SDT DRB data, UE can use R17 CG-SDT or legacy RACH resource to initiate the MT-SDT related resume procedure;

- If UE supports “mt-SDT-r18” and “srb-SDT-r17”, then NW can page UE for SDT DRB data or SRB signalling, UE can use legacy RACH resource to initiate the MT-SDT related resume procedure;

- If UE supports “mt-SDT-r18” and “cg-SDT-r17” and “srb-SDT-r17”, then NW can page UE for SDT DRB data or SRB signalling, UE can use R17 CG-SDT or legacy RACH resource to initiate the MT-SDT related resume procedure.

Proposal 3: An “mt-SDT-r18” capability is introduced for MT-SDT, specified as follows:

  - If UE supports “mt-SDT-r18” only, i.e. without any other SDT-related capabilities, then NW can page UE for SDT DRB data, and UE can use legacy RACH resource to initiate the MT-SDT related resume procedure;

  - If UE supports “mt-SDT-r18” and “cg-SDT-r17”, then NW can page UE for SDT DRB data, UE can use R17 CG-SDT or legacy RACH resource to initiate the MT-SDT related resume procedure;

  - If UE supports “mt-SDT-r18” and “srb-SDT-r17”, then NW can page UE for SDT DRB data or SRB signalling, UE can use legacy RACH resource to initiate the MT-SDT related resume procedure;

  - If UE supports “mt-SDT-r18” and “cg-SDT-r17” and “srb-SDT-r17”, then NW can page UE for SDT DRB data or SRB signalling, UE can use R17 CG-SDT or legacy RACH resource to initiate the MT-SDT related resume procedure.

# Conclusion

Based on the discussion in this paper, we recommend RAN2 to discuss and adopt the following proposals:

Conditions for initiating MT-SDT:

Observation 1: MT-SDT specific common configuration is only required in a cell that supports MT-SDT only (i.e. does not support MO-SDT). Otherwise, the existing sdt-ConfigCommon parameter can be reused.

Proposal 1: MT-SDT specific common SDT configuration (comprising T319a and SDT RSRP threshold) is only configured in case legacy sdt-ConfigCommon is not present (i.e. only if the cell does not support MO-SDT).

Non-SDT data arrival during MT-SDT:

Proposal 2: When DL non-SDT data arrives during the ongoing MT-SDT procedure, the last serving gNB shall end the ongoing SDT procedure by sending RRCRelease message to the UE with an indication to initiate a new Resume procedure, to avoid having to page the UE.

MT-SDT capability:

Proposal 3: An “mt-SDT-r18” capability is introduced for MT-SDT specified as follows:

  - If UE supports “mt-SDT-r18” only without any other SDT-related capabilities, then NW can page UE for SDT DRB data, and UE can use legacy RACH resource to initiate the MT-SDT related resume procedure;

  - If UE supports “mt-SDT-r18” and “cg-SDT-r17”, then NW can page UE for SDT DRB data, UE can use R17 CG-SDT or legacy RACH resource to initiate the MT-SDT related resume procedure;

  - If UE supports “mt-SDT-r18” and “srb-SDT-r17”, then NW can page UE for SDT DRB data or SRB signalling, UE can use legacy RACH resource to initiate the MT-SDT related resume procedure;

  - If UE supports “mt-SDT-r18” and “cg-SDT-r17” and “srb-SDT-r17”, then NW can page UE for SDT DRB data or SRB signalling, UE can use R17 CG-SDT or legacy RACH resource to initiate the MT-SDT related resume procedure.

# References

- R3-231895 Summary of Offline Discussion on CB: SDT_MTSDT, 17 – 26 April 2023

- R2-2306958 [Post122][309][MT-SDT] 38.321 Running CR (Huawei), Huawei, HiSilicon (Rapporteur)