3GPP TSG RAN WG2 Meeting #123	                                                    R2-2308355

# Toulouse, France, 21 – 25 August, 2023

Agenda item:		7.14.4

Source:			Huawei, HiSilicon

Title:		Discussion on QoE measurements in NR-DC

Document for:	 	Discussion and decision

# Introduction

For QoE measurements for NR-DC, RAN2#122 made some progress, but there are some leftover issues to resolve. In the RAN2#123 agenda, some guidance has been put for this topic:

7.14.4	Support of QoE measurements for NR-DC

Including discussion on how the QoE report transmission is handled (e.g. if the QoE report is not configured to use the currently configured SRB, whether this works if SCG is deactivated/released, what do the RAN3 agreements state on this, etc.)

Including discussion on how MN knows to corrrectly forward SN-associated QoE reports received via SRB4

Including discussion on RRC configuration of QoE reporting and measurements for NR-DC

In this paper, we will focus on the above guidance as well as leftover issues from the previous RAN2 meetings.

# Discussion

## QoE configuration and reporting

### Reporting SRB selection

RAN2 FFS is listed as below:

  - FFS how to handle the QoE report transmission if there is only one SRB and the QoE report is not (explicitly) configured for that SRB (to be checked if this is possible according to latest RAN3 agreements)

Let us assume UE has only SRB4 configured, and the UE has got the following configurations from NW:

- QoE config1 is configured to use SRB4 for reporting

- QoE config2 is configured to use SRB5 for reporting

Some companies think the above configuration is valid and should be allowed and the UE should be able to send QoE reports (associated with QoE config2) via SRB4 rather than SRB5 in this case. However, we see several issues with this approach. Firstly, we think the explicit reporting SRB configuration is sufficient and can be used in this case. There is no good reason to specify additional UE behavior which unnecessarily complicates both UE and network implementations.  Secondly, such approach would go against RAN3 agreements:

The SN can indicate to the MN that the reports are to be sent via the SRB5. The SN can request the use of the SRB4 for reporting, which the MN can confirm or reject. FFS whether the indication is explicit or implicit. The network can explicitly instruct a UE in NR-DC to switch the reporting leg. The leg switching command can be sent to the UE by the node that configured that specific QoE configuration. The node that currently receives the QoE reports via the Uu can send a request to the peer node, asking that the QoE reporting leg is switched to the peer node. The leg switch for QoE reporting needs to be approved by both nodes serving the UE.

Based on the agreements above, we see that RAN3 only agreed that the reporting leg can be switched via explicit command from the node which configured that specific QoE configuration. Even more importantly, RAN3 agreed that switching of the leg needs to be approved by both nodes. This means that the UE cannot implicitly switch the reporting leg as this could violate the network’s intention. Considering the above, we propose to agree the following:

Proposal 1: Only the reporting leg indicated via explicit configuration to the UE can be used for QoE reporting by the UE.

### SCG deactivation

We think the following use cases should be considered:

- UC1: Both SRB4 and SRB5 are configured for QoE reporting (for different QoE configurations)

- UC2: only SRB4 is configured

- UC3: only SRB5 is configured

When SCG is deactivated, the SCG configuration is maintained at the UE and hence also SN QoE configurations can be kept. However, some action is required for the QoE configurations (from both MN and SN) which are configured to utilize SRB5 as the reporting leg. Several options are possible:

- UE autonomously switches QoE reporting leg to SRB4.

- UE autonomously pauses these QoE configurations.

- We rely on network implementation to either switch the reporting leg or pause these QoE configurations.

Options 1 and 2 would require specifying new UE behaviour in RRC. Furthermore, it would lead to discrepancy of UE’s RRC configuration and UE’s actual behaviour, e.g. the UE would pause the QoE configuration even though its RRC configuration would indicate the QoE configuration as “not paused”. Similarly, the UE would report over SRB4 even though SRB5 would be configured as the reporting leg in the RRC configuration. Such mechanism would be overcomplicated in our opinion and would require many additional specifications changes. We believe it is much easier to rely on proper network reconfiguration before SCG deactivation occurs.

Proposal 2: When SCG is deactivated, the network should ensure that there are no QoE configurations which are configured with SRB5 as the reporting leg, e.g. by releasing or pausing such QoE configurations or by switching the reporting leg to SRB4.

### SCG release

SCG release can be initiated by the NW or by the UE, i.e.:

- Case (a): SCG release initiated by NW: In this case, the network sends an indication to the UE to explicitly release the SCG. It is noted that the UE can indicate a preference to NW for SCG release, and then it is up to NW whether/how to respond.

- Case (b): SCG release initiated by UE: As defined in TS 38.331, the UE shall release the SCG configuration upon initiation of the RRC connection re-establishment procedure.

As defined in TS 37.340: UE can implicitly indicate a preference for NR SCG release by indicating zero number of carriers and zero aggregated maximum bandwidth in both FR1 and FR2. As defined in TS 38.331: When the UE initiates RRC connection re-establishment procedure, it is specified: Upon initiation of the procedure, the UE shall: 1> if UE is not configured with attemptCondReconfig: <partially omitted> 2> if MR-DC is configured: 3> perform MR-DC release, as specified in clause 5.3.5.10; #### 5.3.5.10 MR-DC release The UE shall: 1> as a result of MR-DC release triggered by E-UTRA or NR: <partially omitted> 2> if the UE is configured with NR SCG: 3> release the SCG configuration as specified in clause 5.3.5.4;

In the last meeting, RAN3 made the following agreements:

If the SN configured a UE with QoE measurements, at SN release, the QoE/RVQoE configuration can be released. Whether the SN-configured QoE/RVQoE configuration information can be passed to the MN in case of SN release needs to be further discussed.

In R17 QoE, the UE releases the QoE measurement when the UE enters RRC_IDLE. In NR-DC, the UE will release the whole SCG configuration when the SCG is released. However, in order to keep the continuity of services, the MN can transmit the services which were served by the SN. For example, the MN can reconfigure or add DRBs for these services. For these cases, we need to keep the QoE measurement continuity because the services are still on-going, the UE can also report the QoE results to the MN so that MCE can still obtain the QoE results and MN can use the RAN visible QoE results to optimize the scheduling.

It may be argued that it is strange that UE sends the QoE results to MN for QoE configurations which were not configured by the MN. However, it has been already agreed that the network can switch the QoE reporting leg. It means that UE can send the QoE reporting to MN even if MN did not configure the QoE measurement for UE. Therefore, we think the network can use the same principle to switch the QoE reporting during SN release.

In order to forward the QoE results to MCE by MN after the SN release and optimize the scheduling based on the RAN visible QoE results, MN needs to know the MCE address and the RAN visible QoE configuration.

Proposal 3: For the QoE measurement configured by the SN, continue the QoE measurement if the services are still on-going in the SN release case. MN receives the QoE/RAN visible QoE configuration information from SN and informs UE whether to keep the QoE measurement.

## Forwarding of SN-associated QoE reports received via SRB4

The RAN2 made the following agreement:

9: For NR-DC, if SRB5 is not configured (FFS on the SCG deactivation case), UE can transmit the SN-associated QoE reports via SRB4. FFS whether there are some ambiguities how MN knows where to forward this.

At RAN3#119b-e, the following agreements were made:

- If the SN is asked by the MN to forward to the MCE the QoE reports pertaining to a measurement configured by the MN, the MN should indicate to the SN the QoE Reference, the MCE IP Address and the RRC ID.

- If the MN is asked by the SN to forward to the MCE the QoE reports pertaining to a measurement configured by the SN, the SN should indicate to the MN the QoE Reference and the MCE IP Address.

- If the SRB5 is not configured, the RVQoE reports can be sent on the SRB4 from the UE via the MN to the SN.

As can be seen from the agreements, MN forwarding SN-associated QoE reports received via SRB4 will be anyway discussed by RAN3 and specified in XnAP.

Observation 1: RAN3 has made some progress regarding how MN forwards SN-associated QoE reports received via SRB4, and RAN2 can rely on RAN3 on solving the issue.

RAN2 can then focus on deciding which message to utilize for QoE reporting in different scenarios without having to consider details of MN-SN coordination and QoE reports forwarding.

In MR-DC, SRB3 is optional. If SRB3 is not configured, UE transfers the SN MeasurementReport, SN RRCReconfigurationComplete, SN UEAssistanceInformation, SN FailureInformation, and SN IABOtherInformation messages in the MN ULInformationTransferMRDC message. In MR-DC, the MN ULInformationTransferMRDC is transmitted via the SRB1 and the scheduling priorities of this message and the SN information included in this message are the same.

In R17 QoE measurement, the QoE results are transmitted in the MeasurementReportAppLayer message via SRB4. In R18, the scheduling priorities of QoE results sent to MN and SN are the same. Therefore, in case SRB5 is not configured and SRB4 is configured as the reporting leg, the SN QoE results can be included in the MN MeasurementReportAppLayer message. UE can reuse the existing R17 IEs in MeasurementReportAppLayer message to transfer the SN QoE results. RAN3 and RAN2 have agreed that if both MN and SN send the QoE configurations to the UE, MN and SN should not use the same set of measConfigAppLayerId. Therefore the MN knows whether the QoE results received from UE is associated with the SN QoE measurement configuration and can forward the QoE results to SN based on this.

Also, according to the agreements of RAN3, both nodes may be interested in knowing the RAN visible QoE results. In such case, if the SN MeasurementReportAppLayer is included in the MN ULInformationTransferMRDC message, MN needs to send the SN MeasurementReportAppLayer to SN and only then SN sends the RAN visible QoE results back to MN. It will increase the delay of RAN visible QoE results and increase signaling over Xn interface.

Observation 2: MN knows whether the measurement report needs to be forwarded to SN based on MN-SN coordination and by knowing the measConfigAppLayerId of the report.

Observation 3: Using MN ULInformationTransferMRDC message for sending the SN MeasurementReportAppLayer may increase the delay and signaling overhead of RVQoE transfer.

Furthermore, the following agreements should be considered

RAN2#122 agreed that: RAN2 will follow RAN3 agreements (for NR-DC when SRB5 is not configured) on forwarding the received encapsulated QoE reports to the correct recipient (i.e. MCE or SN) RAN3#119b-e agreed the following: When SN indicates its interest in configuring m-based QoE a measurement to a UE: The SN can indicate to the MN that the reports are to be sent via the SRB5. The SN can request the use of the SRB4 for reporting, which the MN can confirm or reject. FFS whether the indication is explicit or implicit. RAN3#120 agreed the following: If the MN is asked by the SN to forward to the MCE the QoE reports pertaining to a measurement configured by the SN, the SN should indicate to the MN the QoE Reference and the MCE IP Address.

Based on the above agreements from RAN3, it can be concluded that at least reporting via MeasurementReportAppLayer is needed, so that the case agreed by RAN3 can be supported. Following this, if using ULInformationTransferMRDC would be allowed, it would require the UE to know whether reports are intended to be delivered to SN or whether the MN is supposed to forward them to MCE to decide which message to use. At the same time, we see no benefits of using ULInformationTransferMRDC as the reports received by MN can be simply forwarded to SN using XnAP protocol, if they are supposed to be received by SN.

Observation 4: Based on the current RAN3 agreements, RAN2 needs to support reporting of QoE reports associated with QoE configurations configured by the SN via at least MeasurementReportAppLayer message over SRB4 to support the case where these reports are forwarded to MCE directly by the MN.

Based on the observations above, we propose the following:

Proposal 4: When QoE reports associated with QoE configurations configured by the SN are sent over SRB4, they are included in the MeasurementReportAppLayer message.

## Pause of QoE measurement

The RAN2 FFS is listed as below:

14: As a baseline, Rel-17 pause/resume procedure is reused to pause/resume reporting of one or multiple QoE measurement configurations in a UE for NR-DC. Details are FFS, e.g. whether paused QoE reports can be reported to SN (if SN is not overload).

In TS 38.300, the pause/resume mechanism is defined (since Rel-17 NR QoE) as follows:

### 21.2.3 Handling of QMC during RAN Overload The QoE measurement collection pause/resume procedure is used to pause/resume reporting of one or multiple QoE measurement configurations in a UE in RAN overload situation. The gNB can use the RRCReconfiguration message to temporarily stop the UE from sending application layer measurement reports associated with one or multiple application layer measurement configurations. When the UE receives the QoE measurement collection pause indication, the UE temporarily stores application layer measurement reports in AS layer. When the UE receives the QoE measurement collection resume indication, the UE sends the stored application layer measurement reports to the gNB.

It was proposed by some companies that pause indication should be leg-specific, i.e. in case one node pauses the QoE configuration, then the reports can be sent over the other leg. However, it should be noted there is already a possibility for the RAN to change the reporting leg of the QoE configuration and it is unclear why another mechanism is needed. Hence, there is no reason to modify the pause indication mechanism and we think the Rel-17 principle should be followed also in R18.

Observation 4: There is already a possibility for RAN to change the reporting leg of the QoE configuration and it is unclear why pause indication should be used for that.

Proposal 5: Rel-17 defined pause/resume mechanism is kept for NR-DC case, i.e. when the UE receives pause indication, the UE suspends sending of the reports associated with the concerned QoE configuration.

# Conclusion

Based on the discussion in this paper, the following observations and proposals are made:

Reporting SRB selection

Proposal 1: Only the reporting leg indicated via explicit configuration to the UE can be used for QoE reporting by the UE.

SCG deactivation

Proposal 2: When SCG is deactivated, the network should ensure that there are no QoE configurations which are configured with SRB5 as the reporting leg, e.g. by releasing or pausing such QoE configurations or by switching the reporting leg to SRB4.

SCG release

Proposal 3: For the QoE measurement configured by the SN, continue the QoE measurement if the services are still on-going in the SN release case. MN receives the QoE/RAN visible QoE configuration information from SN and informs UE whether to keep the QoE measurement.

Forwarding of SN-associated QoE reports received via SRB4

Observation 1: RAN3 has made some progress regarding how MN forwards SN-associated QoE reports received via SRB4, and RAN2 can rely on RAN3 on solving the issue.

Observation 2: MN knows whether the measurement report needs to be forwarded to SN based on MN-SN coordination and by knowing the measConfigAppLayerId of the report.

Observation 3: Using MN ULInformationTransferMRDC message for sending the SN MeasurementReportAppLayer may increase the delay and signaling overhead of RVQoE transfer.

Observation 4: Based on the current RAN3 agreements, RAN2 needs to support reporting of QoE reports associated with QoE configurations configured by the SN via at least MeasurementReportAppLayer message over SRB4 to support the case where these reports are forwarded to MCE directly by the MN.

Proposal 4: When QoE reports associated with QoE configurations configured by the SN are sent over SRB4, they are included in the MeasurementReportAppLayer message.

Pause of QoE measurement

Observation 4: There is already a possibility for RAN to change the reporting leg of the QoE configuration and it is unclear why pause indication should be used for that.

Proposal 5: Rel-17 defined pause/resume mechanism is kept for NR-DC case, i.e. when the UE receives pause indication, the UE suspends sending of the reports associated with the concerned QoE configuration.