3GPP TSG-RAN WG2 Meeting #123	R2-2308362

Toulouse, France, August 21-25, 2023

Agenda Item:	7.14.3

Source:	CATT

Title:	Discussion on Rel-17 leftover issues for QoE

Document for:	Discussion and Decision

# 1	Introduction

In RAN2#122 meeting [1], the R17 leftover issues were discussed and the following agreements were achieved.

- 2: RAN2 confirms that buffer level threshold-based triggering of RVQoE reporting is triggered by application layer. - 3: RAN2 waits for RAN3’s conclusion on the buffer level threshold-based RVQoE reporting. - 3: RAN2 will align with RAN3’s agreement on support of radio-related event triggers for RVQoE reporting in Rel-18.

Based on the RAN2’s, there are still several FFS needed to be discussed further in RAN2. In this contribution, we provide some analysis on the remaining issues.

# 2 Discussions

## 2.1 RV QoE trigger event

For RV QoE trigger event, it was agreed to introduce the buffer level threshold-based trigger.

As RV QoE report is mainly used by gNB to optimize the network immediately, the buffer level threshold can be configured by gNB based on the network’s need. In current spec, the RV QoE configuration is send to UE AS layer, AS layer should forward the related RV QoE parameters including measConfigAppLayerId, ran-VisiblePeriodicity, numberOfBufferLevelEntries, reportPlayoutDelayForMediaStartup to APP layer. Similar as other RV QoE parameters, the buffer level threshold should be included in ran-visibleParameters IE sent to UE AS layer and then AS layer forward the buffer level threshold to APP layer. Then APP layer can trigger the RV QoE reporting based on the buffer level threshold. After UE AS layer receives the RV QoE report from APP layer, UE sends the RV QoE report to gNB.

Proposal 1: The buffer level threshold should be included in ran-visibleParameters IE to be sent to UE AS layer and then AS layer forwards it to APP layer.

## 2.2 The enhancement on RAN Overload scenario

When NG-RAN is overload, QoE measurement may be paused. On the contrary, if overload is relieved, QoE measurement can be resumed. But whether we will introduce the prioritization mechanism for RAN overload scenario is still discussed in both RAN2 and RAN3. Some agreements are achieved in RAN2 [2] as follow.

- 7: RAN2 to postpone the discussion of the QoE reporting enhancement for overload scenario to the next meeting (based on the progress of RAN3). - 8: FFS on whether to send the priority information 1) UE and gNB or 2) only to gNB - 9: To wait for RAN3 decision on granularity of priority.

According to RAN3’s LS [3], when assistance information for handling of QoE reporting upon overload is sent to the RAN, it is sent together with QoE measurement configuration. RAN3 need to further discuss what the assistance information is and an LS [4] is sent to SA5 to ask whether OAM can provide priority to gNB. But from RAN3’s perspective, there is no need to send assistance information to UE and RAN2 can provide feedback if necessary. So we can wait for SA5 and RAN3’s progress on the priority information.

Proposal 2: Whether the priority can be send to UE can wait for SA5 and RAN3’s progress.

If it is agreed that the assistance information can include priority, we think we can give feedback to RAN3 that the priority information is useful to UE. As in RAN2, there are still some companies that see the benefits if the priority information can be send to UE. So we think we can discuss the necessity of sending priority information to UE in RAN2. If we agree that UE need the priority information from gNB, we can send the LS to RAN3.

We also think it is beneficial for UE to get priority information from gNB. As in current specification, multiple QMC jobs may be inactivated for a UE when NG-RAN overload. If the NG-RAN wants to pause some QMC jobs, it can only randomly select some QMC jobs without any assistant information. It is not reasonable for NG-RAN to pause important QMC while keep less important QMC jobs. Besides, when the NG-RAN is overload, the UE AS layer is responsible to store the RV QoE report. It is possible that the RAN overload is not resolved until the memory size is full. In this case, the UE may discard the stored QoE report which is important for RAN. The priority information sent to UE can help UE discard the QoE report with less importance.

Besides, when overload is relieved, there also may be many UEs which store QoE measurement reports during the overload period. If many UEs send QoE measurement reports to network at the same time, it may lead to overload again. So it is beneficial to send the QoE report with higher priority first to avoid the overload occurs again.

Proposal 3: If the priority information can be provided by OAM, it is beneficial to send the priority information to UE.

# 3 Conclusions

According to the above discussion, the following proposals are made:

Proposal 1: The buffer level threshold should be included in ran-visibleParameters IE to be sent to UE AS layer and then AS layer forward it to APP layer.

Proposal 2: Whether the priority can be send to UE can wait for SA5 and RAN3’s progress.

Proposal 3: If the priority information can be provided by OAM, it is beneficial to send the priority information to UE.

# 4 References

- Report of 3GPP TSG RAN2 meeting #122

- R2-2211101  Report of 3GPP TSG RAN WG2 meeting #119bis-e, Online

- R2-2302425 LS on assistance information for handling of QoE reporting upon RAN overload

- R2-2304625 LS on the feasibility of introducing assistance information for handling of QoE reporting during RAN overload