3GPP TSG-RAN WG2 #123	R2-2308373

Toulouse, France, August 21- 25, 2023

Agenda Item:	7.7.4.2 Handover enhancements

Source:	NEC

Title:	Satellite Switch: PCI change without L3 handover

Document for:	Discussion, Decision

# Introduction

Regarding satellite switch scenario, without PCI changing solution has been agreed, and the solution is only applicable to hard satellite switch case:

In quasi-earth fixed cell case, for hard satellite switch in the same SSB frequency and same gNB (no key change), satellite switching without PCI changing (not requiring L3 mobility) is supported, unless major technical issues are identified by RAN1 (as usual RAN2 will aim at minimizing the specification impact so that it fits in Rel-18)

In this contribution, we propose to support changed PCI with no L3 mobility (in addition to unchanged PCI solution) for better support the satellite switch scenario including soft switch case.

# Discussion

## Issue and benefit of unchanged PCI solution

Satellite switch scenario is depicted in below. It was agreed in Rel17 that PCI will be changed after satellite switch. From UE point of view, UE would then see two different cells (one appear, and another one disappear), consequently UE will execute handover or RRC Re-establishment. This would involve L3 mobility signaling and service interruption due to the protocol reset/reestablishment.

It has been extensively discussed to not change the PCI and consequently not to trigger L3 mobility and security key update. technically speaking, since all UEs will reconnect to the same gNB and consume/share the same radio resource and hardware, It is not worth to trigger RRC reconfiguration, security key change and user plane protocol reestablishment for the change of relay satellite. There is great benefit and possibility to not trigger L3 mobility

Observation 1: the benefit of unchanged PCI is to save L3 mobility procedure.

At the same time, the main concern of unchanged PCI solution is not support soft satellite change case in which it will cause interference and confusion at UE side. Some other concerns include not exact matched coverage before and after satellite change, backward compatibility etc..

To solve the issue and at the same time keep the benefit of unchanged PCI solution, we propose to change PCI (to solve the issues) but without L3 mobility (to keep the benefits). In next section, we look into the details and feasibility

Observation 2: the main issue of unchanged PCI is the interference caused in soft satellite switch, this can be solved by changing the PCI

## PCI change without L3 mobility

In SIB19, T-service is corresponding to the satellite switch time, this could be reused to tell UE the timing of satellite switch and PCI change. Moreover, a replacement PCI can be added in SIB19 as follows. Since the satellite will be changed, at least the ntn-Config-r17 should be updated from the old satellite’s info to new satellite’s info upon satellite switch.

SIB19 information element -- ASN1START -- TAG-SIB19-START SIB19-r17 ::= SEQUENCE { ntn-Config-r17 NTN-Config-r17 OPTIONAL, -- Need R t-Service-r17 INTEGER (0..549755813887) OPTIONAL, -- Need R referenceLocation-r17 ReferenceLocation-r17 OPTIONAL, -- Need R distanceThresh-r17 INTEGER(0..65525) OPTIONAL, -- Need R ntn-NeighCellConfigList-r17 NTN-NeighCellConfigList-r17 OPTIONAL, -- Need R lateNonCriticalExtension OCTET STRING OPTIONAL, ..., [[ ntn-NeighCellConfigListExt-v1720 NTN-NeighCellConfigList-r17 OPTIONAL -- Need R ]] [[ReplacementCellConfig-r18 ::= SEQUENCE { physCellId-r18 PhysCellId } ]] OPTIONAL, -- Cond t-service } NTN-NeighCellConfigList-r17 ::= SEQUENCE (SIZE(1..maxCellNTN-r17)) OF NTN-NeighCellConfig-r17 NTN-NeighCellConfig-r17 ::= SEQUENCE { ntn-Config-r17 NTN-Config-r17 OPTIONAL, -- Need R carrierFreq-r17 ARFCN-ValueNR OPTIONAL, -- Need R physCellId-r17 PhysCellId OPTIONAL -- Need R } -- TAG-SIB19-STOP -- ASN1STOP

PCI change without L3 mobility (together with RACH-less concept ) then can be simply modelled as UL synchronization loss due to invalid. Small changes are enough:

- gNB informs the replacement PCI in system information (e.g., SIB19)

- at the time around T-service

  - gNB updates the NTN-Config corresponding to new satellite

  - UE expires T430 at the time of T-service, re-acquire SIB19 for synchronization to the replacement cell

The main specification change is shown in below:

RRC: #### 5.2.2.6 T430 expiry The UE shall: 1> if ReplacementCellConfig is included in SIB19, at the time t-Service: 2> expire T430 for the serving cell 2> inform lower layer the PCI of serving cell is changed into ReplacementphysCellId 1> if T430 for serving cell expires and if in RRC_CONNECTED: 2> inform lower layers that UL synchronisation is lost; 2> acquire SIB19 as defined in clause 5.2.2.3.2; 2> upon successful acquisition of SIB19: 3> inform lower layers when UL synchronisation is obtained; NOTE: The exact time when UL synchronisation is obtained (after SIB19 is acquired) is left to UE implementation, which can be from the subframe indicated by epochTime and optionally before the subframe indicated by epochTime. UNRELEVANT PART OMITTED ##### 5.2.2.4.21 Actions upon reception of SIB19 Upon receiving SIB19, the UE in RRC_CONNECTED shall: 1> start or restart T430 for serving cell with the timer value set to ntn-UlSyncValidityDuration for the serving cell from the subframe indicated by epochTime for the serving cell; NOTE: UE should attempt to re-acquire SIB19 before the end of the duration indicated by ntn-UlSyncValidityDuration and epochTime by UE implementation.

MAC spec (no change) : ## 5.2a Maintenance of UL Synchronization The MAC entity shall for each Serving Cell: 1> if an indication of uplink synchronization has been received from upper layers (see clause 5.2.2.6 of TS 38.331 [5]): 2> allow uplink transmission on the Serving Cell. 1> if an indication of uplink synchronization loss is received from upper layers: 2> flush all HARQ buffers; 2> not perform any uplink transmission on the Serving Cell.

From UE perspective, the detailed procedure of changed PCI without L3 mobility could be as follows:

- UE read SIB19, if it includes both T-service and also the PCI of the replacement cell, at T-service, UE consider it as uplink synchronization loss and hence suspend all uplink transmissions.

- UE detaches from the serving cell identified by PCI A and DL synchronize to the new cell identified by PCI B

- UE re-enquire SIB19 from cell identified by PCI B, within which the NTN-config IE should be corresponding to the replacement satellite and cell information now.

- UE recalculate/apply the TA based on re-enquired SIB19. Then UE consider as uplink synchronization is obtained.

- Continue with the RRC connection with the same configuration. Neither RACH nor L3 mobility (including key change) is needed

If the UE detects other cell than the replacement PCI, normal RRC re-establishment can be applied

Above procedure in our understanding is simple with limit specification changes as shown in the TP in the appendix section. At the same time, it keep the big benefit of unchanged PCI solution. even though PCI is changed, there is no need to change security key as same as for intra-gNB handover or LTM.

## Compatibility

Compatibility with unchanged PCI solution:

To support unchanged PCI solution, simply gNB can set the replacement PCI as same as current PCI of serving cell. UE still can following the same procedure to expire T430, trigger the DL synchronization to new satellite, calculate the UL timing/frequency compensation, and resume the connection.

Observation 3: same specification change and procedure can be shared with unchanged PCI and change PCI solution.

Back-ward compatibility

For UE does not support change/unchanged PCI solution, they still rely on handover procedure or RRC re-establishment procedure

# Summary

In this contribution we discuss the benefit and issue of unchanged PCI solution for satellite switch scenario, to keep the benefit and solve the issue, we propose to support changed PCI but without L3 mobility in addition to unchanged PCI solution. Details model and procedure are provided.

It would be up to network whether PCI will be changed or not during satellite switch scenario. Likely, unchanged PCI would be used for hard satellite switch and changed PCI would be used for soft satellite switch. Both will avoid to trigger L3 mobility procedure.

Observation 1: the benefit of unchanged PCI is to save L3 mobility procedure.

Observation 2: the main issue of unchanged PCI is the interference caused in soft satellite switch, this can be solved by changing the PCI

Observation 3: same specification changes and procedure can be shared by unchanged PCI and change PCI solutions (without L3 mobility) .

Following these observations, we have following proposal:

Proposal 1: Support PCI change without L3 mobility for satellite switch scenario (In addition to unchanged PCI solution).

Proposal 2: Agree the TP in appendix as baseline for satellite switch

# Appendix: TP to TS38.331

#### 5.2.2.6	T430 expiry

The UE shall:

1> if ReplacementCellConfig is included in SIB19, at the time t-Service:

2> expire T430 for the serving cell

2> inform lower layer the PCI of serving cell is changed into ReplacementphysCellId

1>	if T430 for serving cell expires and if in RRC_CONNECTED:

2>	inform lower layers that UL synchronisation is lost;

2>	acquire SIB19 as defined in clause 5.2.2.3.2;

2>	upon successful acquisition of SIB19:

3>	inform lower layers when UL synchronisation is obtained;

NOTE:	The exact time when UL synchronisation is obtained (after SIB19 is acquired) is left to UE implementation, which can be from the subframe indicated by epochTime and optionally before the subframe indicated by epochTime.

UNRELEVANT PART OMITTED

### 6.3.1	System information blocks

UNRELEVANT PART OMITTED

#### SIB19

SIB19 contains satellite assistance information for NTN access.

SIB19 information element

-- ASN1START

-- TAG-SIB19-START

SIB19-r17 ::= SEQUENCE {

ntn-Config-r17                           NTN-Config-r17                                  OPTIONAL,       -- Need R

t-Service-r17                            INTEGER (0..549755813887)                       OPTIONAL,       -- Need R

referenceLocation-r17                    ReferenceLocation-r17                           OPTIONAL,       -- Need R

distanceThresh-r17                       INTEGER(0..65525)                               OPTIONAL,       -- Need R

ntn-NeighCellConfigList-r17              NTN-NeighCellConfigList-r17                     OPTIONAL,       -- Need R

lateNonCriticalExtension                 OCTET STRING                                    OPTIONAL,

...,

[[

ntn-NeighCellConfigListExt-v1720         NTN-NeighCellConfigList-r17                     OPTIONAL        -- Need R

]]

[[ReplacementCellConfig-r18 ::=              SEQUENCE {

ReplacementphysCellId-r18                   PhysCellId

}

]] OPTIONAL, -- Cond t-service

}

NTN-NeighCellConfigList-r17 ::=          SEQUENCE (SIZE(1..maxCellNTN-r17))  OF NTN-NeighCellConfig-r17

NTN-NeighCellConfig-r17 ::=              SEQUENCE {

ntn-Config-r17                           NTN-Config-r17                                  OPTIONAL,       -- Need R

carrierFreq-r17                          ARFCN-ValueNR                                   OPTIONAL,       -- Need R

physCellId-r17                           PhysCellId                                      OPTIONAL        -- Need R

}

-- TAG-SIB19-STOP

-- ASN1STOP

SIB19 field descriptions

distanceThresh Distance from the serving cell reference location and is used in location-based measurement initiation in RRC_IDLE and RRC_INACTIVE, as defined in TS 38.304 [20]. Each step represents 50m.

ntn-Config Provides parameters needed for the UE to access NR via NTN access such as Ephemeris data, common TA parameters, k_offset, validity duration for UL sync information and epoch.

ntn-NeighCellConfigList, ntn-NeighCellConfigListExt Provides a list of NTN neighbour cells including their ntn-Config, carrier frequency and PhysCellId. This set includes all elements of ntn-NeighCellConfigList and all elements of ntn-NeighCellConfigListExt. If ntn-Config is absent for an entry in ntn-NeighCellConfigListExt, the ntn-Config provided in the entry at the same position in ntn-NeighCellConfigList applies.

referenceLocation Reference location of the serving cell provided via NTN quasi-Earth fixed system and is used in location-based measurement initiation in RRC_IDLE and RRC_INACTIVE, as defined in TS 38.304 [20].

t-Service Indicates the time information on when a cell provided via NTN quasi-Earth fixed system is going to stop serving the area it is currently covering. The field indicates a time in multiples of 10 ms after 00:00:00 on Gregorian calendar date 1 January, 1900 (midnight between Sunday, December 31, 1899 and Monday, January 1, 1900). The exact stop time is between the time indicated by the value of this field minus 1 and the time indicated by the value of this field.

ReplacementCellConfig Indicate the replacement cell information in case of satellite switch within the same frequency and same gNB

| Conditional Presence |  | Explanation |  |
| --- | --- | --- | --- |
| t-Service | The field is optionally present, Need R, if t-Service is present; otherwise the field is not present. |